This page covers notes for PrintGL  1.22  that are not included in the

v1.20 manual.

/FA supports the HP DeskJet 500C (and the Mannesmann Tally MT92C).

/FB drives the Canon BJC-800 in Epson LQ mode.

/FF drives Fujitsu 24 pin printers in Fujitsu mode.

/FW is an alternate Epson 9  pin driver that uses different indexing

commands than /FN.  This  provides compatibility  for some not quite

compatible printers.

The IBM 9 pin printer driver (/F1) has a  * resolution - 240x144 and

allows  color.   This  driver   will   not  work  correctly  on  all

Proprinters.

The  IBM  24  pin  alternate  drivers, /F5  and  /FX, now have  a  *

resolution that give 360x360 print  and they allow color.  This only

works on printers with 1/360 indexing such as the 2390, not the X24E

or 24P. /F6* will give 360x364 print with the X24E and 24P.

/FY appends a TIFF preview bit map to a PostScript file.  To use it,

create a PostScript file with PrintGL  and  then run  PrintGL again,

specifying  the  same  options  except  /FY  instead  of  /FS.   The

destination  file is appended  by default.  /FY assumes a PostScript

file, generated by PrintGL, is being appended.

/FV+ without a mode number sets up VESA 800x600 mode.

/Bi,j,x specifies tiling -  i,j are the number of tiles horizontally

and vertically. X is the overlap in inches.

Pen  overwrite mode can be set  for each  pen  with the  /C  option.

Insert  O for opaque or T for  translucent in front a pen  to switch

modes. The mode remains in effect until changed with another O or T.

Blank area compression  mode is the default on many printers, but  !

is still allowed.

/XH suppresses  the  help  screen.  /XP causes a pause  for  loading

paper. /XC causes a break on clipped plots.

0 width pens are allowed. This just turns that pen off.

The  PMI  options "Run  printgl",  "Alt settings", and "eXit" can be

invoked directly by pressing the R, A, or X keys.

In PMI "create bat" creates a BAT file that can be run with "batfile

hpglfile /opt1 /opt2 ..".  The BAT file contains the options defined

by the current PMI settings and accepts additional options. The name

of the BAT file is specified under "other"