Documentation for DarkSE

WARNING - WARNING - WARNING - WARNING - WARNING

Use this program ONLY a Toshiba T1000SE.

Although this program is copyrighted I give permission for

you to use it.

This program turns off the backlight screen of a Toshiba

T1000SE after 5 minutes of no keypresses.

Any keyboard input will immediately restore the screen after it

goes dark.  Your program will continue to run whether the screen

is dark or not.

Neither toshutil or other programs seem to work if ac power

is applied.  They seem to be only for battery saving.  However,

the screen life is limited.  This program does not care if you are

on ac or battery power - it will save both batteries and possibly

prolong your screen life.

This program is a TSR which takes up about 512 bytes.  Load

it in your autoexec.bat file with the command:

DarkSE

The screen blanking function can be disabled at any time with

the command:

DarkSE d

This program actually works telling the T000SE to use an

external monitor.  At the present time, there are no external

monitors because the docking adaptor is not yet on the market.

At some time in the future I hope to know the proper internal

command to stop the backlighting but retain the letters on the

screen at which time I will come out with a revision.

If you really insist on changing the timeout from 5 minutes

look for the eyecatcher TIME: near the front of the com file.  The

next two bytes will be 1554h (5460 decimal) which represents 5

minutes of internal timer ticks.  Change that.

The called routines for screen blanking and unblanking are

after the eyecatchers DARK: and LIGHT: near the front of the code.

Thanks go to David Markovitch who wrote and released to

public domain a program Dscrnsav in January 1990 which I used as

the basis of this one.

I also thank Don Taylor who found the port code in Toshutil

to enable and disable the external monitor.

I can be reached for questions or comments on Compuserve.

Enjoy,

Larry Alkoff

CIS 72117,1254