BIOSSIG is a BIOS ROM Signature Identification Database/Program.  BIOSSIG

is a "Work In Progress".  It is hoped that the program, written in Borland's

Turbo Prolog 2.0, can someday be made to appear "artifically intelligent" in

such a manner as to accurately determine the Signature of a BIOS ROM not in its

current database.  In its current implementation BIOSSIG simply matches the

contents of its database with the contents of your BIOS.  One option looks

for an exact match and reports its results very quickly.  Another option

only looks at BIOSs that have the same IBM Model Id Byte as your BIOS.  This

option takes longer, but allows you to then scan the signatures of BIOS's with

signatures similar to the one in your PC.  You may also print a formatted

report describing the signatures of all of the BIOSs known to us today.

BIOSSIG starts in Color, without suppressing CGA "Snow".  It's Color scheme may

be modified from the Optional Customization menu.  Some Monochrome adapters

do not display the default BIOSSIG color scheme (others do).  If your monochrome

screen blanks and all disk activity stops without any messages appearing, then

monochrome may be selected on start up by including "mono" on the command line.

Some Color Graphics Adapters display "snow" on your color screen if they

are accessed via direct writes to video memory.  If you do see "snow", try

setting Snow Suppression Required.  Using this setting slows down the display

slightly to prevent generating the snow.  No Snow Suppression will restore

BIOSSIG to its fastest display speed.  CGA Snow suppression may be selected on

start up by including "snow" on the command line or by exercizing the optional

customization menu.

BIOSSIG prints 132 Columns of information per page.  If you select 6 Lines/Inch,

BIOSSIG will print 5 Signatures/Page.  If you select 8 Lines/Inch, BIOSSIG

will print 7 Signatures/Page.

To give some indication that the program is working while it scans its database

(Now at 390+ entries and growing) BIOSSIG displays the file name of each BIOS

Signature and a percentage of match between your BIOS and each database entry.

Sample BIOS Signature Screen(s)

�������������������������������������������� BIOS Signature 3.2  �������������ͻ

� Model Id Byte at FFFFE matches 100% �������������������������������Ŀ�����Ŀ �

� FC                                  �      Overall Match: 100%      �� CRC � �

�                                     �Your BIOS Signature is in our  ��F0000� �

� Date          at FFFF5 matches 100% �Database as shown below, but   ��FFFFF� �

� 08/12/87                            �its CRC does not match.  Please�� C2F0� �

�                                     �submit this BIOS and its Info. �� C2F1� �

� Part Number   at FE000 matches 100% ���������������������������������������� �

� NCR'S VERSION IBM CORP. AT ROM                                               �

�                                                                              �

� Compatibility at FE00E matches 100%                                          �

� IBM                                                                          �

�                                                                              �

� Copyright     at FE01E matches 100%                                          �

� COPR. NCR CORP. 1985,1987COPR. COMPUTER DEVICES INC. 1982                    �

�                                                                              �

� OEM Revision  at FFFEA matches 100%                                          �

� NCR4.5                                                                       �

�                                                                              �

�                                                                              �

�     PC: Honeywell AP  | NCR P8 Test with Bad CR BIOS: NCR 4.5                �

� Source: Mike Focke                                                           �

�   Date: 5 July 1988                        BIOS Signature File: NCR4-5x      �

������������������������������������������������������������������������������ͼ

Esc to return to Main Menu, F10 to attempt another Exact match

�������������������������������������������� BIOS Signature 3.2  �������������ͻ

� Model Id Byte at FFFFE matches 100% �������������������������������Ŀ�����Ŀ �

� FC                                  �      Overall Match: 100%      �� CRC � �

�                                     �                               ��F0000� �

� Date          at FFFF5 matches 100% �Your BIOS Signature is in our  ��FFFFF� �

� 08/12/87                            �Database as shown below. Please�� C2F0� �

�                                     �inform us if you disagree.     ��     � �

� Part Number   at FE000 matches 100% ���������������������������������������� �

� NCR'S VERSION IBM CORP. AT ROM                                               �

�                                                                              �

� Compatibility at FE00E matches 100%                                          �

� IBM                                                                          �

�                                                                              �

� Copyright     at FE01E matches 100%                                          �

� COPR. NCR CORP. 1985,1987COPR. COMPUTER DEVICES INC. 1982                    �

�                                                                              �

� OEM Revision  at FFFEA matches 100%                                          �

� NCR4.5                                                                       �

�                                                                              �

�                                                                              �

�     PC: Honeywell AP  | NCR P8                  BIOS: NCR 4.5                �

� Source: Mike Focke                                                           �

�   Date: 5 July 1988                        BIOS Signature File: NCR4-5       �

������������������������������������������������������������������������������ͼ

Esc to return to Main Menu, F10 to attempt another Exact match

�������������������������������������������� BIOS Signature 3.2  �������������ͻ

� Model Id Byte at FFFFE matches 100% �������������������������������Ŀ�����Ŀ �

� FC                                  �      Overall Match:  98%      �� CRC � �

�                                     �Your BIOS does not match the   ��F0000� �

� Date          at FFFF5 matches  62% �currently displayed database   ��FFFFF� �

� 08/12/87                            �entry.  The second line of any �� C2F0� �

� 05/02/88                            �pair comes from the database.  �� A916� �

� Part Number   at FE000 matches 100% ���������������������������������������� �

� NCR'S VERSION IBM CORP. AT ROM                                               �

�                                                                              �

� Compatibility at FE00E matches 100%                                          �

� IBM                                                                          �

�                                                                              �

� Copyright     at FE01E matches 100%                                          �

� COPR. NCR CORP. 1985,1987COPR. COMPUTER DEVICES INC. 1982                    �

�                                                                              �

� OEM Revision  at FFFEA matches  83%                                          �

� NCR4.5                                                                       �

� NCR4.6                                                                       �

�                                                                              �

�     PC: Honeywell AP  | NCR P8                  BIOS: NCR 4.6                �

� Source: Cathy Cockrum Brower                                                 �

�   Date: 3 November 1988                    BIOS Signature File: NCR4-6       �

������������������������������������������������������������������������������ͼ

Press F10 for next worst match, Press Esc to Quit

For the purposes of the BIOS Signature Project a BIOS ROM Signature consists of

the following six pieces of information:

Model Id Byte

Date

Part Number

Compatibility

Copyright

OEM Revision

Model Id Byte:

IBM puts a coded byte in BIOS ROM memory location FFFFE.  PC Magazine

June 28, 1988 published a list of the IBM Model Id Bytes as follows:

FF - IBM PC

FE - IBM PC-XT

FD - IBM PCjr

FC - IBM PC AT

FB - IBM PC-XT(rev 1)

FA - IBM PS/2 Model 30

F9 - IBM PC Convertible

F8 - IBM PS/2 Model 80

Date:

IBM puts the Date the BIOS ROM was issued in BIOS ROM memory location FFFF5.

Part Number:

IBM put the Part Number of the BIOS ROM in the early BIOS ROM chips in BIOS

ROM memory location FE000.  The BIOS Signature Database uses the Part Number

if appropriate or other identifying text if not an IBM BIOS ROM.

Compatibility:

Nominally all of the BIOS ROMs of interest are IBM compatible.  The story is

that when the first IBM PC was introduced some programs looked for "IBM" in

the BIOS ROM.  If "IBM" was not there the programs (a) didn't run or

(b) trashed something.  This is the theory behind looking for "IBM" in BIOS

ROM memory.  Not all modern (nominally IBM compatible) ROMs contain the IBM

trademark.  Those BIOSs that do contain the "IBM" normally put it where IBM

puts its copyright notice.

Copyright:

IBM puts a Copyright notice for the BIOS ROM in BIOS ROM memory location FE008.

Most other manufacturers also put copyright notices in their own BIOS Roms.

There is some discussion as to what it takes to be a legal copyright notice.

Minimally there should be the Copyright symbol, a "C" within a circle.  Most

BIOS ROMs approximate this symbol with a (C) or (c) using matched parentheses

to enclose the "C". The word Copyright should be spelled out if the symbol is

not present.  The Year for which the copyright is claimed is required.  The

name of the entity claiming the copyright is the last required information.

OEM Revision:

Original Equipment Manufacturers (OEMs) have added information to uniquely

identify a BIOS ROM.  This is an OEM Revision (Number).  When an OEM BIOS ROM

initializes the PC it normally displays its revision on the Console.

Address:

This is the BIOS ROM memory address (shown in Hexadecimal Notation) where the

information may be found in matching BIOS ROMs.

Match:

This is the Percentage Match between Your BIOS ROM and the BIOS ROM from

the BIOS Signature Database.  It is computed by recursively matching the

longest common substrings between the contents of the database and the

corresponding areas of your BIOS ROM memory.  An Exact Match yields 100%.

If 10 characters out of 20 are the same, even if they are not in the exact

order of the database, then the match is 50%.

The Overall match is computed by arbitrarily assigning a length of one to the

Model Id Byte and the Date.  For each item in the signature the Match is

multiplied by the length in characters, the sum of these products is divided

by the total length giving the Overall Match as a percentage of the total

characters.

PC:

The Brand Name of the Manufacturer mentioning any identifying characteristics.

BIOS:

The Name of the BIOS Manufacturer (from the ASCII Copyright Statement if

available).

Source:

The Name of the Person who first supplied the BIOS Signature Project with the

required information.

Date:

The Date the information was acquired.

BIOS Signature File:

The File containing the contents of that BIOS ROM.  Also used as a unique

identifier.

CRC:

To differentiate between multiple BIOSs with the same ASCII BIOS Signature as

defined above, version 3.1c implemented a Cyclic Redundancy Check of your BIOS

ROM.  A CRC is a mathematical calculation which is used in communications to

detect errors in a block of data.  The operation is performed on a data block,

the block and CRC are transmitted, the receiver independently computes the

CRC of the received block and compares it to the CRC computed by the sender.

If the two CRCs are the same it is assumed that the block has been transmitted

and received correctly.  According to W.  David Schwaderer's CRC Calculation

article in the April 1985 issue of PC Tech Journal, "Using these polynomials

allows detection of all errors involving 16 bits or less and about 99.95

percent of errors involving more than 16 bits."

The two 5 digit hex addresses are the address of the first and last bytes of

the block of information upon which the CRC computation is calculated.  The

next 4 digits = the 16-bit CRC of your BIOS.  If the database contains a

different CRC it is displayed below yours.  If the ASCII BIOS Signature of

your BIOS ROM is an exact ( 100 percent) match, but the CRCs are different,

your BIOS Manufacturer has issued 2 ROMS with identical signatures but

different code.  There are, for example, 2 Zenith entries in the database

with the same signature but different CRCs.  There were only 5 bytes different

between the 2 BIOS.  One BIOS came from a Z-183 the other from a Z-184.

The Future:

Even after examining 390+ BIOSs, we are still learning how to recognize those

of a particular brand.  Once we feel we know enough about how Phoenix, Award,

AMI, Compaq and IBM typically store the signature information, then we hope

to be able to include logic to produce/recognize the signatures of BIOSs we

haven't seen.

BIOSTREE

BIOSTREE displays a family tree of BIOSs.

Legal Concerns:

Both Rick and I work for a computer manufacturer who will buy us any BIOS

we want. Our intent is to add to the litterature about BIOS Signatures and

the only way to do this is to examine the code to find the signatures.

We don't use the BIOS you send us for any other purpose. Please don't ask

us to send you the latest BIOS. We won't.

AMI BIOS can be bought from Herschell Fleming 1-800-828-9294

Phoenix BIOS can be bought from Komputerwerk 1-800-666-6715 or

Virginia 804-320-8835