///////////////////////////\

/////  Printer Test!  ////  /

//////////////////////////  /

\_________________________\/

v1.1

Author: Julio Monroy

September 18, 1991

���������������Ŀ

�  Description  �

�����������������

Printer Test is a simple program that displays the current

status of the parallel printer port.  It performs a direct port

read, then displays the status of each bit along with a diagnostic

message. Printer Test is a simple, yet powerful little program

that helps overcome printing problems.

���������������Ŀ

�    Purpose    �

�����������������

Print Test is designed primarily as a simple diagnostic tool

for those people who are experiencing printing problems.  It is

also intended for debugging use by software programmers who

are experiencing printing problems with their software.

���������������Ŀ

�    Usage      �

�����������������

Printer Test requires an IBM PC/XT/AT compatible with an 8088,

80286, 80386, or 80486 micro-processor with at least 1 printer port

attached.  Without a printer port attached, the program will

not recognize anything... rendering the program useless.

Usage is:

=========

PRNTST [/p] [/R]

[p]  =  Printer port to sample.  1 = LPT1, 2 = LPT2, 3 = LPT3.

[R]  =  Tells the program to run test repeatedly.  This parameter

makes the program run in a loop, constantly sampling the

status of the printer interrupt, until a key is pressed.

Examples of usage:

==================

PRNTST /R /2   =  Repeatedly tests LPT2 until a key is pressed.

PRNTST /3      =  Shows status of LPT3  (one sample).

PRNTST         =  Shows status of LPT1  (one sample).

PRNTST /?      =  Displays the internal help menu.

*    If no parameters are used, the default is one sample of LPT1.

*    The program accepts any delimeters, even no delimeters.

For example, all of these parameters are legal:

/R   or  -R  or  R

/2   or  -2  or  2

*    Command line parameters are non-case sensitive.

���������������Ŀ

� Public Domain �

�����������������

This program is hereby officially declared as Public Domain.

Anyone, anywhere, can take this source code and incorporate into their

program without any expressed written or verbal consent permission from

the author.  So go for it!  Tear up this code!  Although nothing new

is being covered, and this program could probably be easily put

together by reading any MS-DOS and IBM-PC compatible reference books.

���������������Ŀ

�     Notes     �

�����������������

*  Print Test was assembled using Microsoft Macro-Assembler

(MASM) version 5.10.

*  Directly reads the port adrress to monitor printer activity.

*  This program WILL work correctly with redirected

devices (i.e., mapped printers on a network), or with

print spooler software that directly mask INT 17h.

The reason for this is because PRNTST performs a direct

port interrogation.  It checks in the BIOS data area

for the port address, and performs a direct port read

to determine the printer status.  Contrary to the prior

version of PRNTST, this version will work regardless

of device redirection (i.e., mapped printers on a LAN).

*  Thanks for Vincent Wu for his advice and suggestions

on direct port interrogation for the PRNTST program.

���������������Ŀ

� ... Finally   �

�����������������

Comments can be forwarded to the following locations:

Turbo Sys:              415-365-7491

Incipits Remote:        415-324-2601

Invisible BBS:          415-345-5509

All are 2400,N,8,1

Both are PC Pursuitable via CAPAL or CASJO.

{end of document}