Quick Hardware Identification (QHI)

Version 1.1

September 29, 1991

Copyright 1991:   George Spafford

Purpose:

To identify the basic hardware on an IBM compatible computer.

Procedure:

From the DOS prompt, invoke the program by typing QHI and then

pressing enter.  It will then access the BIOS and DOS services

to identify the equipment on your machine.

Output Definitions:

Processor Class:        This is an attempt to identify the processor

chip that was used in the computer.  The results

fall into four catagories at this time:

1.      8088, 8086 or V20.

2.      80188 or 80186

3.      80286

4.      80386 or 80486

I hope to improve this routine to be more accurate

in regards of the V20 and 486 chip sets.

Approximate Speed:      This is the approximated speed rating of the

processor in terms of MegaHertz.  It may take

the program several seconds to determine this value.

Any TSR's or multitaskers could affect this value.

Floating Point Unit:    This identifies whether a FPU (i.e. math coprocessor)

is present in the system.

BIOS Date:              This is the date of the BIOS that is installed in

the system.  It is NOT the system date.

Parallel Port(s):       The number of parallel ports that the program

identifies on the system.

Serial Port(s):         The number of serial ports that the program

identifies on the system.

Game Port(s):           The number of game ports that the program

identifies on the system.

Base Memory:            This is the amount, in terms of Kilobytes, of

base memory installed on the system.  (The

first 640Kb)

Free Extended Memory:   On a 286, 386 or 486, this is the amount of free

extended memory on the system.  Note, that is FREE

memory - not installed memory.

Expanded Memory:        This is the amount of EMS installed on a computer.

This value assumes that 16Kb pages are used and

arrives at this value by multiplying the total

pages by 16.  This routine identifies LIM 4.0 EMS.

Total Pages:            If EMS exists, this identifies the total number of

16Kb pages that are available in total - not

necessarily free.

Free Pages:             If EMS exists, this identifies the number of FREE

16Kb pages that are available for use.

Free EMS:               This is the amount of free EMS in Kilobytes.

History:

v1.1   09/29/91    v1.0's access of extended memory caused some XMS routines

to crash.  I have changed to a routine that only accesses

the amount of XMS installed.  The older version caused

a misinitializattion of XMS handles in some instances.

v1.0   09/26/91    Initial release date.

I hope this program helps you evaluate some of your problems

or even makes some solutions/benefits clear.  This program is SHAREWARE.

IT IS NOT PUBLIC DOMAIN.  If you find that this program aids you, you are

required ethically and legally to register it after a 30 day evaluation period.

This software is priced at $5 for each concurrently used copy in an

organization.

Sincerely,

George Spafford

3001 LakeShore Drive, #329

St. Joseph, MI 49085

Data:    (616) 468-5026  Queued Access BBS 19200-8-N-1

And now for the unpleasantries:

QHI IS DISTRIBUTED AS IS.  THE AUTHOR (GEORGE SPAFFORD)

MAKES NO WARRANTY OF ANY KIND, EXPRESSED OR IMPLIED,

INCLUDING BUT NOT LIMITED TO TO WARRANTIES OF MERCHANTABILITY

OR FITNESS FOR A PARTICULAR PURPOSE, WITH RESPECT TO THIS

SOFTWARE AND DOCUMENTATION. IN NO EVENT SHALL THE AUTHOR BE

LIABLE FOR ANY DAMAGES, INCLUDING LOST PROFITS, LOST SAVINGS,

OR ANY OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES ARISING OUT

OF THE USE OF OR THE INABILITY TO USE THIS PROGRAM.

----------------------------------------------------------------

Other Products:

Cache Test	Find out just how much your cache really

helps!

CutIt		Parse streamed files by x number of bytes

and add a CR/LF at the end.

FLC            Compare two file lists and generate a list

of duplicates and/or unique files.

PadIt		Convert Variable length records to a Fixed

length format.

ZipCodeTool    Look up zip codes by state or city.  If you

know the zip code, you can look up the city

and state.

ZipWhen        Insure that the Zip file's date matches the

date of the newest file it contains.  This

reverses the annoying time stamping that some

BBS packages do.

-- EOF --