_______

____|__     |               (tm)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

There are 4 files here:

BIOSDUMP.EXE

BIOSCRC.EXE

MERGER.EXE

SPLITTER.EXE

All of the programs are written in Turbo Pascal 5.5 and make use of dynamic

objects.  Buffer size is 32KB and MERGER and SPLITTER use 5 buffers so you

need at least 160KB + about 16KB for object allocation.

BIOSDUMP will copy the memory addresses F0000 thru FFFFF to the filename

specified on the command line.

BIOSCRC  will tell you the calculated CRC (8 bit) , file's CRC and the new

CRC of the filename specified on the command line.  This program

returns 3 numbers:

1. the calculated CRC for the file excepting the last byte.

2. The last byte which is the current value use to make

the CRC equal 0.

3. The new value that needs to be used to make the modified

file work.  This value replaces the last byte in the file.

It then asks you if you want to change the value and if you answer

"Y" then it writes the new value to the file.

MERGER	 will take 2 or 4 files and merge them into 1 big file interleaving

them.	src1 = bytes 0,4,8,12,...  or 0,2,4,6,8,...

src2 =       1,5,9,13,...  or 1,3,5,7,9,...

src3 =       2,6,10,14,...

src4 =       3,7,11,15,...

All of the source files must be the same size or the program will

abort.

format is: MERGER src1 src2 <src3 src4> dst

SPLITTER will reverse MERGER's process.

format is: SPLITTER src dst1 dst2 <dst3 dst4>

The source file must be evenly divisible by the number of destination

files you give on the command line.  If you want to split into EVEN

ODD files then src must be divisible by 2.  If you want to split into

4-way interleave then the src must be evenly divisible by 4.  If this

condition is not met the program will abort.

Use of the programs is as follows:

You can use an EPROM programmer to read in the BIOS chips to

binary files and then use MERGER to make the final file or you

can use BIOSDUMP to get the final file straight from memory.

Then use a binary file editor such as Norton's NU program or CalTech

CCO File Modify program or you can use DEBUG to make any changes in

the BIOS (i.e. add a new disk type or change an old one).

Then use BIOSCRC to calculate the new CRC byte that goes into the

last position in the big file and then you can let BIOSCRC change the

value for you or you can insert that value using an editor.

Then use SPLITTER to split the big file into the EVEN / ODD files

and burn a new set of EPROMS.

** Caution **

The files this program uses are straight binary.  DON'T write files

from an EPROM programmer using any other format (i.e. Intel hex,

Jedec, ASCII HEX, ...).

In the IBM PC AT the disk type tables start at $FE401 and is 512 bytes

long.

Following is an example of the table:  I can't guarantee that it is the same

in all machines although types 1 thru 24 are pretty standard.

Type  Size     Cyls  Heads  Pre-Comp    Landing Zone     Sec/Trk

Control

1     10MB	306	4	128	0	305		17

2     20MB	615	4	300	0	615		17

3     32MB	615	6	300	0	615		17

4     64MB	940	8	512	0	940		17

5     48MB	940	6	512	0	940		17

6     20MB	615	4	FFFF	0	615		17

7     32MB	462	8	256	0	511		17

8     30MB	733	5	FFFF	0	733		17

9    115MB	900	15	FFFF	8	901		17

10    20MB	820	3	FFFF	0	820		17

11    36MB	855	5	FFFF	0	855		17

12    50MB	855	7	FFFF	0	855		17

13    20MB	306	8	128	0	319		17

14    44MB	733	7	FFFF	0	733		17

15    Rsvd	0000	0	0000	0	000		00

Types 16 thru 24 are found only in BIOS ROMs dated from 05NOV86

16    20MB	612	4	0000	0	663		17

17    42MB	977	5	300	0	977		17

18    58MB	977	7	FFFF	0	977		17

19    60MB	1024	7	512	0	1023		17

20    30MB	733	5	300	0	732		17

21    44MB	733	7	300	0	732		17

22    30MB	733	5	300	0	733		17

23    10MB	306	4	0000	0	336		17

24    80MB	1024	9	FFFF	8	1024		17

24    53MB	925	7	0000	0	336		17

25    70MB	925	9	0000	8	925		17

26    43MB	754	7	0000	0	925		17

27    68MB	754	11	0000	8	754		17

28    40MB	699	7	0000	0	699		17

29    68MB	823	10	0000	8	823		17

30    53MB	918	7	0000	0	918		17

31    93MB	1024	11	0000	8	1024		17

32    127MB	1024	15	0000	8	1024		17

33    42MB	1024	5	0000	0	1024		17

34    10MB	612	2	0000	0	611		17

35    76MB	1024	9	0000	8	1024		17

36    68MB	1024	8	0000	0	1024		17

37    40MB	615	8	0000	0	615		17

38    24MB	987	3	0000	0	987		17

39    57MB	987	7	0000	0	987		17

40    40MB	820	6	0000	0	820		17

41    32MB	977	4	0000	0	977		17

42    32MB	981	4	0000	0	981		17

43    48MB	830	7	0000	0	830		17

44    68MB	830	10	0000	8	830		17

45    20MB	612	4	305	0	663		17

46    10MB	306	4	none	0	340		17

47    20MB	612	4	none	0	670		17

48    40MB	698	7	300	0	732		17

49    40MB	976	5	488	0	977		17

50    10MB	306	4	0000	0	340		17

51    20MB	611	4	306	0	663		17

52    42MB	732	7	300	0	732		17

53    42MB	1023	5	none	0	1023		17

54     5MB	306	2	none	0	305		17

55    68MB	1024	8	512	0	1024		17

Format of the Disk Type entry.

Offset	Size	Label

0	2	Number of Cylinders

2	1	Number of Heads

3	2	Unused

5	2	Write Pre-Compensation Cylinder

7	1	Unused

8	1	Unused

9	3	Unused

12	2	Landing Zone

14	1	Number of Sectors / Track

15	1	Unused

These programs are ShareWare and are not free.  If you like it

and continue to use it the price is $10.00 US.

Also any bug comments or any feedback at all are welcome.

Please address payment to:

Donald Prescott

POB 165133

Miami, FL 33116-5133

Please address comments to:

CIS PPN : 74045, 462

GEnie   : DONP

or the above address