The mformat.exe was d/l'ed from the Maxtor BBS and is the Maxtor equivalent

of the WDFORMAT.ZIP that is circulating for WD IDE drives.

It is only for use on LXT-200 series IDE drives.

These instructions were then FAXed to me and typed by my secretary

to form this file.

I was extremely disappointed in MAXTOR (and told them so) that they

would miss the chance to provide a description of the low level process

for their drives, thesteps they take at the factory and how you the

end user should best maintain their drives.  I told them they were missing

marketing oppertunity as well as failing to do something that would reduce

the number of aclls to tech support (thus saving them movey).  This file is

all I could get out of them....sorry.

mike focke 6/17/91

INTRODUCTION

The ATA interface FORMAT TRACK COMMAND provides only a few limited

functions.  The host system sends the interleave table with sector

numbers and flag bytes, matching its BIOS drive type.  The drive

will then translate the interleave table information to the

physical location on the disk.  A seek and head select operation

will then take place as required.  Since the drive is zone bit

recorded, the number of sectors specified in the interleave table

is only a partial amount of the physical sectors contained on the

track.  During the translate routine the number of sectors and

interleave requested is reordered into a one to one interleave

sequence.  Also included in the interleave table is the flag byte

information.  The LXT product only accepts flag bytes of 00, 80,

and 40.  The next operation is to update the sector ID's and write

a 00 pattern on the data field of the sectors that are affected on

the track.  During this update the sectors headers must be read

first, which verifies their existence.  If all the sectors check

out to be alright, then the update operation begins.  During the

initial LOW LEVEL FORMAT operation, if a defective sector was

mapped out, the area of the disk has a high frequency pattern

written for a sector's length.  Therefore, during the FORMAT TRACK

operation no ID field exists where a mapped defect has been

identified and the defect cannot be mapped.

This format utility MFORMAT.EXE has an AT VENDOR UNIQUE command

which allows a user to perform a LOW LEVEL FORMAT restoring the

header and creating the data fields for all tracks on the drive.

PRECAUTIONARY STEPS TO FOLLOW BEFORE LOW LEVEL FORMATTING:

1.   POWERUP YOUR SYSTEM FIRST.

2.   BACKUP YOUR DATA.

3.   INSERT THE MFORMAT DISKETTE IN YOUR DRIVE A:

4.   TYPE : A> MFORMAT.EXE

5.   THE SCREEN WILL DISPLAY: C: FOR DRIVE O; D: FOR DRIVE 1

6.   FOR ONE DRIVE USE C: DRIVE O.

7.   THE SCREEN WILL DISPLAY :" FORMATTING IN PROGRESS ".

8.   AFTER IT COMPLETES THE FORMAT, "FORMAT SUCCESSFUL " WILL THEN

BE DISPLAYED.

9.   PROCEED TO PARTITION THE DRIVE WITH FDISK AFTER THE FORMAT HAS

SUCCESSFULLY BEEN COMPLETED.