FIXED DISK PARKING

------------------

1. I wrote this program (parkplus) to overcome some of the annoying

'features' (annoying to me anyway) of some of the other park programs

I have come across lately.

These 'features' being:---

:: Park only the fist drive of a 2 drive system.

:: Park your drive(s) heads and then start a LOUD audible

'core melt down' alarm noise, requiring the PC to be switched

off - (this is actually a good idea in some areas of the work-

place but not ALL).

:: Purposely lock up the system after parking, again, a good idea

for some users but you don't have the option to return to DOS

and save your last hour's vdisk / ramdisk work that you just

remembered about, 3 seconds after parking!

2. There are 2 park utilities supplied in this 'ZIP'.

The park2.exe (and source code park2.c) is a 'regular/vanilla

garden variety' C program which shows how the parking of the

heads was done. Most of the popular C compilers should produce

an executable file without complaining.

Compilers not supporting the 'PC' specific dos.h include and

int86 function will of course give forth errors.

The parkplus.exe file (source not included) contains the same

'core' C code as the plain version, however it has been jazzed up

a small amount for those of us who like flashy windows on a color

display etc :-).

3. There is no checking to see if a hard disk exists.

Note also that parking is only required on HDD's using 'stepper motor'

head actuation, modern voice coil HDD's self retract the heads

when powered down.

4. As this program (like most park programs) moves the heads to the last

cylinder (+1) as reported by the BIOS function only, in many cases

this is not the actual physical last cylinder on the drive, however

it is the end of the formatted data area.

Disclaimer

No extensive research on disk parking was carried out to write

these programs, they are therefore supplied 'as is' without warranty

of any kind, either expressed or implied. The entire risk as to the

quality and performance of this software lies with the user.

Ken Palliser,

Box 119 Nunawading Vic. 3131

Australia

PH (03) 872 4545