QUADRIVE

Copyright 1991 Ronald Q. Smith

QUADRIVE is a DOS diskette driver for use with 1.2MB 5 1/4" diskette drives.  It

works with many IBM and other fully compatible personal computers to add support

for a 720KB format using standard 360KB floppy disks.  QUADRIVE will allow you

to read, write, format, and use most DOS compatible utilities as if your drive

was a 3 1/2" 720KB drive.  This is the format used by the Tandy model 2000 and a

few other vendors, so you can interchange diskettes between those systems and

your new system.  It may also be useful for making exact copies of 720KB 3 1/2"

diskettes when you have one 3 1/2" drive and the 1.2MB drive.  If you have a

system with a true 720KB 5 1/4" drive, QUADRIVE will allow you to use that

drive with standard DOS functions.

DEFINITION OF SHAREWARE

Shareware distribution gives users a chance to try software before buying it. If

you try a Shareware program and continue using it, you are expected to register.

Individual programs differ on details -- some request registration while others

require it, some specify a maximum trial period. With registration, you get

anything from the simple right to continue using the software to an updated

program with printed manual.

Copyright laws apply to both Shareware and commercial software, and the copy-

right holder retains all rights, with a few specific exceptions as stated below.

Shareware authors are accomplished programmers, just like commercial authors,

and the programs are of comparable quality. (In both cases, there are good

programs and bad ones!) The main difference is in the method of distribution.

The author specifically grants the right to copy and distribute the software,

either to all and sundry or to a specific group. For example, some authors

require written permission before a commercial disk vendor may copy their

Shareware.

Shareware is a distribution method, not a type of software. You should find

software that suits your needs and pocketbook, whether it's commercial or

Shareware. The Shareware system makes fitting your needs easier, because you can

try before you buy.  And because the overhead is low, prices are low also.

Shareware has the ultimate money-back guarantee -- if you don't use the product,

you don't pay for it.

DISCLAIMER - AGREEMENT

Users of QUADRIVE must accept this disclaimer of warranty:

"QUADRIVE is supplied as is.  The author disclaims all warranties, expressed or

implied, including, without limitation, the warranties of merchantability and of

fitness for any purpose.  The author assumes no liability for damages, direct or

consequential, which may result from the use of QUADRIVE."

QUADRIVE is a "shareware program" and is provided at no charge to the user for

evaluation.  Feel free to share it with your friends, but please do not give it

away altered or as part of another system.  The essence of "user-supported"

software is to provide personal computer users with quality software without

high prices, and yet to provide incentive for programmers to continue to develop

new products.

If you find this program useful and find that you are using QUADRIVE and

continue to use QUADRIVE after a reasonable trial period, you must make a

registration payment of $20 to Ronald Q. Smith at the address below.  The

$20 registration fee will license one copy for use on any one computer at any

one time.  For one year after receipt of the this registration fee, you will

recieve notification of the availability of any updates.  If you report a bug

within 3 months of receipt of the registration fee, you will receive a

correction to the problem or the fee will be refunded to you.  For a $45

registration fee you will receive the latest version including a copy of the

source code and technical documentation on the source code.  You will also

automatically receive any updates for one year.  If you report a bug within 3

months of receipt of the registration fee, you will receive a correction to

the problem or the fee will be refunded to you.  If you license the source

code, you may use it to maintain your copy of QUADRIVE and as a source of

information.  You may not use any or all of it in your own products without

first obtaining authorization from Ronald Q. Smith.

You must treat this software just like a book.  An example is that this

software may be used by any number of people and may be freely moved from one

computer location to another, so long as there is no possibility of it being

used at one location while it's being used at another.  Just as a book cannot

be read by two different persons at the same time.

Commercial users of QUADRIVE must register and pay for their copies of QUADRIVE

within 30 days of first use or their license is withdrawn.  Site-License

arrangements and volume discounts may be made by contacting Ronald Q. Smith.

Anyone distributing QUADRIVE for any kind of remuneration must first contact

Ronald Q. Smith at the address below for authorization.

Send fees and any inquiries to:

Ronald Q. Smith

11 Black Oak Road

North Oaks, Mn. 55127-6204

You may also contact me via CompuServ mail at userid 71620,514.  I will be happy

to respond to any problems and suggestions for future capabilities.

You are encouraged to pass a copy of QUADRIVE along to your friends for evalua-

tion.  Please encourage them to register their copy if they find that they can

use it.

USING QUADRIVE

QUADRIVE allows you to use any mixture of 160KB, 180KB, 320KB, 360KB, 720KB, and

1.2MB formats on the drive.  The new 720KB format is identical in all respects

except pysical size to the 3 1/2" 720KB format.  In fact, DOS utilities such as

FORMAT and DISKCOPY will believe that it is a 3 1/2" 720KB media.  QUADRIVE will

also allow you to read and write your Tandy and DEC 720KB diskettes that your

new system doesn't support.  At least these vendors and probably several others

supported a 720KB capacity on 5 1/4" diskettes.  That is the capacity that

QUADRIVE supports.  The full capabilities of QUADRIVE are only available with

DOS data formats and structures.  It will not support CP/M or other non-standard

formats for file access.  You will be able to format, disk copy, and disk

compare any 8 or 9 sector-per-track and 40 or 80 cylinder format even if it is

not otherwise a DOS format.  There are many very good commercial software

packages that provide support for file access for CP/M and many other formats.

Note that a DOS device driver is only effective when all access to the device is

via DOS.  This includes both files and the drive oriented interfaces such as INT

25H and INT 26H.  Any application that uses the BIOS directly, such as many

backup utilities, will not be able to use QUADRIVE.  DOS BACKUP and RESTORE

will, however, work correctly.  Non-DOS formatters such as Central Point

Software's PCFORMAT will not correctly support this capacity.  They make direct

calls to the BIOS and so don't take advantage of QUADRIVE's BIOS helper that

tells the BIOS how to handle the 720KB format.

The 720KB capacity is inherent in the 1.2MB disk controller and in all 360KB

floppy media.  No drive or media specifications are being exceeded.  This is

just a format the Microsoft and IBM chose not to support.

The driver is configured in CONFIG.SYS with the statement:

DEVICE=QUADRIVE.SYS /D:d1 [/D:d2] [/D:d3] [/D:d4]

QUADRIVE will support up to four devices concurrently.  The parameters d1

through d4 are physical device numbers for which you want 720KB support.

They must represent 1.2MB drives and controllers.  These are the physical drive

numbers 0 and 1 and not DOS drive letters A:, B:, etc.  Thus if you have a

single floppy drive, you would specify DEVICE=QUADRIVE.SYS /D:0.

You may repeat the same physical device number two or more times.  This will

assign multiple drive letters to the same drive just as DOS assigns both A:

and B: to drive 0 if you don't have a drive 1.  You will be prompted to change

diskettes with the familiar DOS message when you change the drive letter.

Be sure that you have LASTDRIVE set large enough to accommodate the new drive

letters that will be created.  If you have one or two floppy disks, a hard disk,

and a RAM disk, the default value for LASTDRIVE of E will allow you to add

QUADRIVE for one of the floppy drives.

Once QUADRIVE is installed you may refer to the same drive by at least two

names.  In fact, you will often have three names by which to refer to that

drive.  If you have a single floppy system, DOS will assign the names A: and B:

to that drive and QUADRIVE will add D: or E:.  While DOS will carefully prompt

you when switching between drives A: and B:, it is unaware that D: is also the

same drive.  Thus you will not be able to successfully execute a COPY A:file

D:file command as you will not have time to change floppies.

If you have a single floppy system and used the following CONFIG.SYS statement,

you will have four drive letters by which you may refer to the drive.

DEVICE=QUADRIVE.SYS /D:0 /D:0

When toggling between A: and B: or D: and E: you will see the prompt message.

However, DOS thinks that A: and B: refer to a 1.2MB drive and that D: and E:

refer to a 3 1/2" 720KB drive and will not prompt you when changing between

A: and D: or E: (or between B: and D: or E:).

Since QUADRIVE will let you read and write all of the existing 5 1/4" formats

as well as the new 720KB format, it may occur to you to wonder why it doesn't

simply add that capability to drive letter A: (and B:).  There are actually

two reasons.  One is that DOS does not define anyway for a device driver to

redefine any of the standard drives (usually A: through C:).  It is possible

only by using undocumented interfaces and we want to avoid that in order to

retain the greatest level of compatibility.  The more important reason is that

FORMAT.COM is just too smart for its own good.  FORMAT "knows" that you can't

mix those capacities on the same drive.  If it thinks the drive is a 1.2MB

drive, it will not let you select the 720KB capacity and vice versa.  This is

built right into FORMAT and we really don't want to require a non-standard

FORMAT program when simply using an additional drive letter avoids it.

So, you can read, write, and perform any other function that doesn't require

formatting using the QUADRIVE drive letter, and you can do it using all of

supported formats.  If you use a function that requires formatting, you must

use the QUADRIVE letter for 720KB diskettes and the DOS A: or B: drive letter

for other formats.  Functions that may require formatting are FORMAT, DISKCOPY,

and BACKUP.  DISKCOPY and BACKUP only require formatting if the target disk is

not already formatted.

You can use DISKCOPY, DISKCOMP, and many other copying programs with your

QUADRIVE drive.  Just use DISKCOPY E: E: or DISKCOPY E: F: to copy from one

720KB floppy to another.  DISKCOPY and others include their own prompting

capability and will notify you when to change diskettes if you use the same

drive letter for both the source and the target.  Due to limitations in

DISKCOPY you may not be able to copy from a 720KB diskette to a previously

formatted 360KB diskette without reformatting the target first.  While

DISKCOPY will normally format the target as it writes if it discovers that the

target is not formatted, it will see the first track on the previously formatted

360KB diskette as correctly formatted.  When it tries to write the second track

and gets an error, DISKCOPY assumes that the diskette is actually damaged and

not just unformatted.  That is, DISCOPY will only format if it is unable to

write the first track, and a 360KB floppy has a correct appearing first track

in 720KB mode.  If this happens to you, simply format the target floppy with

FORMAT E: (or whatever the QUADRIVE drive letter is) and then repeat your

DISKCOPY.  The same rule applies to BACKUP.

TECHNICAL STUFF

The earliest floppy drives supported one or two sides, 40 tracks, and 8 or 9

sectors per track for capacities of 160KB, 180KB, 320KB, and 360KB.  The

read/write heads in the drives required a wide data area and a wide gap (by

today's standards) for each track.  As the technology for heads and media

improved, the width of the data area and gap were reduced to half the earlier

value.  When a modern 360KB drive reads or writes a floppy disk, it leaves a

very wide gap between the tracks.  It is basically using every other track on

the device even though it calls them track (cylinder) 0, 1, etc.

By the mid-1980s several PC vendors were using the 720KB format.  These included

some DEC and Tandy systems.  They simply made small changes to the drive

controller and the BIOS to use every track.  When they had to handle an "old"

360KB floppy, the BIOS simply doubled the track (cylinder) number requested.

This format never caught on generally, but QUADRIVE will successfully handle

most of those old diskettes.

When the 1.2MB drive was introduced, it supported the use of both all the tracks

on the drive and higher density media that permitted 15 sectors per track.  Like

the other early implementations, when presented with a 360KB diskette, the BIOS

simply doubles the track number for all operations.  This then provides the

opportunity for QUADRIVE to use those intervening tracks.

The IBM PC/AT BIOS defined a state machine for its floppy BIOS.  As part of the

implementation, the BIOS attempts to sense the format of the media and respond

properly without you having to tell it in advance what capacity you are using.

The BIOS keeps a value that tells it what kind of media it is currently

handling.  If you insert a new diskette, the BIOS sets the media type to

unknown.  Whenever the BIOS handles a request for an "unknown" media, it first

tries to use the 360KB format.  If that gets an I/O error, it tries the 1.2MB

format.  Whichever one succeeds causes the BIOS's state for the drive to be set

to "determined" and the media type.  The media type state is actually a bit

mask which tells the BIOS what transfer rate to use, whether to double track

numbers, and how many sectors per track to permit.

The BIOS only has two values that it places in the state.  These are a 1.2 MB

media type, 15 sectors, and no doubling or 360KB media type, 9 sectors, and

doubling.  QUADRIVE takes advantage of this design to create another state value

that looks identical to the BIOS 360KB value except that the doubling bit is not

set.  As long as QUADRIVE sets this value in the state before every I/O opera-

tion, it all works.

The reason that you need a full device driver rather than just a little TSR that

sets the state is that you want to be able to continue to use the 360KB and

1.2MB formats.  With a device driver, DOS tells QUADRIVE what format it is

expecting and QUADRIVE knows whether to set the state or not.  This is the

technique that allows you to use drive letter A: for 1.2MB and 360KB formats and

drive letter E: for 720KB formats.

Actually, QUADRIVE will automatically adjust for reading and writing all media

formats from 160KB, 180KB, 320KB, and 360KB to 720KB and 1.2MB.  However, you

cannot FORMAT anything other than the 720KB with the QUADRIVE drive letter.

Unfortunately, while DOS would happily allow QUADRIVE to handle all of these and

does, the FORMAT program itself has an error check which says that the 720KB

format cannot be used on the same drive as the other formats.  It is trying to

keep you from accidentally asking to format a true 3 1/2" diskette at 360KB or

1.2 MB or vice versa.  Thus it will display an error message and not even try.

Basically, a few programs like FORMAT ask the device driver what physical type

of drive they support and will only allow the capacity options that they know

about for those physical types.  In order for QUADRIVE to get FORMAT to work at

all, it must always respond that this is a 3 1/2" drive type.  FORMAT then

refuses to accept a request for any of the other capacities.

QUADRIVE will only work with those BIOS implementations (IBM, Mitsubishi,

Phoenix, etc.) that compatibly implement the multi-media state machine described

in the IBM PC/AT Technical Reference manual.

INSIDE QUADRIVE

If you choose to license the source code, you will see that it is fully

annotated with an explanation of what DOS expects from a device driver and

the mechanics of the BIOS state machine.  In writing QUADRIVE, I discovered

that the DOS documentation is often incomplete or misleading and is actually

wrong in a few places.  It takes a lot of trial and error and time spent with

a debugger to determine what DOS actually wants in all cases.

QUADRIVE is written in Microsoft MASM 6.00.  You would need that version of the

assembler to use the source code, but you can use it as a source of information

without anything more than an understanding of assembler code and a copy of a

DOS technical reference manual such as the "Microsoft MS-DOS Programmer's

Reference".  In spite of my comment about its completeness, it is an invaluable

source of information for anyone writing system-level programs.  I also

recommend "Undocumented DOS" by Andrew Schulman, Raymond J. Michels, Jim Kyle,

Tim Paterson, David Maxey, and Ralf Brown.  It does not have as much information

on device drivers but the insite into the inner workings of DOS were very

valuable in trying to figure out what DOS really wants.

IBM is a registered trademark of International Business Machines Corporation.

MS-DOS and Microsoft are registered trademarks of Microsoft Corporation.

REVISION HISTORY

911006  -       Version 2.10:  Added support for up to four drives.  Added

support for multiple drive letters for a drive including

generating diskette change messages.

910927  -       Version 2.00:  Updated for DOS 3.3 through 5.0.  Use 1.2MB

drive to support 720KB.

850612  -       Version 1.10:  Added support for 720KB drives on Sperry PC-1

and PC/HT.

840819  -       Version 1.00:  Original version.  Support 720KB drive on IBM

PC-1 under DOS 2.10.