USING GRAPHIC MACROS ON A HEWLETT PACKARD LASERJET PLUS, SERIES II, SERIES III

OR COMPATIBLE PRINTER.

Before you can use graphic macros, you must load the images to your HP LaserJet

Plus, Series II or compatible printer's memory.  This can be done by using the

DOS command:

A> COPY *.MAC PRN

A> COPY *.OVR PRN

These commands are included in the batch file, LOADPRTR.BAT.

This will copy the images to the printer.  These images have the following

macro ids:

LTR101.OVR     Letterhead form                 101

SIGNATUR.MAC   Sample Signature                  1

These numbers will be referenced later in this document.

To use the Graphic Macros within your word processor or other application

program, you must have the ability to insert a printer command (escape sequence)

at the point where you wish the image to print.

The escape sequence remains constant for a given image.  The method of entering

the escape sequence will vary depending upon what software you are using.

To print the signature, position your cursor where you want the image to appear.

The escape sequence is:

<ESC>&f1y3X          print the signature at the current position

To print a page form, position yourself anywhere on the current page and

enter the following escape sequence:

<ESC>&f101y3X        print the letterhead form on the current page

To print an page automatic overlay, position yourself anywhere on the current

page and enter the following escape sequence:

<ESC>&f101y4X        print the letterhead form on all pages

Brief instructions for various word processors follow.  Please check your word

processor documentation to clarify any questions you may have.  The following

represent only a single method of inserting an escape sequence in a document for

each application program.  There may be other methods available for

accomplishing the same thing.

DISPLAYWRITE 4

Displaywrite 4 will require the modification of your Printer Function Table.  If

you are using a PFT supplied by IBM, the PFT may be too large to edit.  If you

are using a PFT that you created yourself or received from Hewlett Packard, you

will probably have no problems.

You should create a User Defined Code for each graphic macro you wish to use.

Start at the Displaywrite 4 Main Menu:

9              profiles

6              revise table

ENTER          next screen

filename.PFT   name of the LaserJet printer function table

6              User-Defined Controls

#              enter a number from 1-999 for the control number/macro

number you will use. For this example, use 1 for the

signature and 101 for the letterhead.

Enter the escape sequence in hexadecimal representation:

1B 26 66 31 79 33 58                     print the signature or,

1B 26 66 31 30 31 79 33 58               print the letterhead

ENTER          to save the control code

ESC            exit from control code menu

ESC            exit from Revise Table

ENTER          end and save table

ESC            return to main menu

To use the graphic macro within your document, place the cursor where you would

want the image to appear.  Then:

F8             instructions

5              User-Defined Controls

#              the number of the control code you wish to use:

1 for the signature or 101 for the letterhead

MICROSOFT WORD

At the point where you would like the image to appear, enter the escape

character by holding down the ALT key and entering 027 from the numeric keypad.

Release the ALT key and enter the balance of the escape sequence.

ALT-027 &f1y3X (no space) to print the signature, or

ALT-027 &f101y3X (no space) to print the letterhead

MULTIMATE

At the point where you would like the image to appear, you must enter the escape

sequence by entering each character from the numeric keypad.  Each character

must be preceded by an ALT-A.

The decimal equivalent of the escape sequence is:

027 038 102 049 121 051 088         print the signature, or,

027 038 102 049 048 049 121 051 088 print the letterhead

To print the signature, you would enter the following:

ALT-A     release the ALT key

027       enter the first character

ALT-A     release the ALT key

038       enter the first character

ALT-A     release the ALT key

102       enter the first character

ALT-A     release the ALT key

049       enter the first character

etc.

NOTA BENE

Nota Bene allows custom print commands in a document with the use of the

Print Insert Command on the command line.

At the point where you would like the image to appear, simply insert the

following on the command line:  (<ESC> is Shift-Alt-PrtSc)

PI <ESC>&f101y3X              print the letterhead, or,

PI <ESC>&f1y3X                print the signature

OFFICE WRITER 5

OFFICE WRITER 6

OfficeWriter allows the definition of special characters or codes to be sent to

the printer which are then accessed similar to a macro.

To enter the printer commands, start from the main menu:

F5     Special Functions

F3     Special Symbols

Choose the CTRL + key combination you wish to assign the macro to. This example

will use CTRL-L for the letterhead and CTRL-S for the signature.  You must enter

the escape sequence using the decimal equivalents of the individual characters.

The first character is the decimal equivalent of the character that will display

on the screen.  Enter the control sequence as follows:

CTRL-L   242 27 38 102 49 48 49 121 51 88 print the logo, or

CTRL-S   243 27 38 102 49 121 51 88       print the signature

At the point where you would like the image to appear, simply insert a CTRL-L to

print the letterhead, or a CTRL-S to print the signature.

PFS:WRITE

Q & A WORD PROCESSOR

At the point where you would like the image to appear, you must enter the escape

sequence by entering each character as numeric digits in the following format:

The decimal equivalent of the escape sequence is:

*P 27,38,102,49,48,49,121,51,88*     print the letterhead or,

*P 27,38,102,49,121,51,88*           print the signature

SAMNA WORD IV

Samna Word IV allows the entering of printer control codes within a document by

the use of the MARK function.  You must enter the hexadecimal values of each of

the characters in the escape sequence.  The hexadecimal values of the above

escape sequences are:

1B 26 66 31 30 31 79 33 58          print the letterhead, or,

1B 26 66 31 79 33 58                print signature

Do NOT enter the spaces, they are for readability only.

Place the cursor where you would like the image to appear.

Press MARK

Press E

Press ENTER to enter the escape sequence.

Enter one of the above escape sequences.

SPRINT 1.0

Sprint allows a printer command to be imbedded in a document using the

style command.

ALT-S    to select the Style Menu

O        to choose other format

enter the following line to print the signature

ESCAPE S "@CHAR(27)&f1y3X"

or            enter the following line to print the letterhead

ESCAPE S "@CHAR(27)&f101y3X"

C        to identify this as a command

WORDPERFECT 4.2

The way that Word Perfect sends data to the printer, requires entering a single

space character immediately before the escape sequence.

To enter the escape sequence:

CTRL-F8           print format menu

A                 enter printer control codes

<27>&f101y3X      print the letterhead, or,

<27>&f1y3X        print the signature

F7                exit back to your document

WORDPERFECT 5.x

The way that Word Perfect sends data to the printer, requires entering a single

space character immediately before the escape sequence.

To enter the escape sequence:

SHIFT-F8          format menu

4                 other formats

6                 printer functions

2                 printer commands

1                 enter a command

<27>&f101y3X      print the logo, or,

<27>&f2y1X        print the signature

F7                exit back to your document

WORDSTAR 2000

WordStar 2000 allows the use of printer control codes within a document by using

the 'PRINT EXTRA' command.

To enter the printer commands, use the following procedure:

At the printer install menu, choose PRINT EXTRA

Enter the escape sequence: (^[ is Ctrl-[)

^[&f101y3X         print the letterhead, or,

^[&f1y3X           print the signature

Assign a name to the PRINT EXTRA command.

When you want to invoke the image in your document, enter CTRL-P X to invoke the

PRINT EXTRA feature.  Select the item you wish to include and press enter.

WORDSTAR 4

WORDSTAR 5

WordStar allows custom print commands with the CTRL-P Q, CTRL-P W, CTRL-P E, and

CTRL-P R commands.  To define these codes, enter the following in your document

prior to using one of the commands.

.XQ 1B 26 66 31 30 31 79 33 58     print the letterhead, or,

.XW 1B 26 66 31 79 33 58           print the signature

At the point where you would like the image to appear, simply insert a CTRL-P Q

to print the logo, or a CTRL-P W to print the signature.

XYWRITE 3

XyWrite allows custom print commands in a document with the use of the

Print Insert Command on the command line.

At the point where you would like the image to appear, simply insert the

following on the command line:  (<ESC> is the actual escape key)

PI <ESC>&f101y3X              print the letterhead, or,

PI <ESC>&f1y3X                print the signature

DBASE, FOXBASE, ETC.

Once a graphic macro is loaded in the printer's memory, it can be activated

from within dBASE programs by simply "printing" the escape sequence.

For example, if you are formatting a report with the "@ xx,yy SAY" you would

simply

@ xx,yy SAY CHR(27)+"&f101y3X"      print the letterhead, or,

@ xx,yy SAY CHR(27)+"&f1y3X"        print the signature

SPECIAL OFFER

Be the first to provide us with a description of how to use a graphic macro

within a word processor not described above, and we will give you a FREE

signature scan ($60.00 value).  To avoid disappointment, please call to check

if we have already received the description for the word processor you wish to

submit.  We will make a reservation for you that will ensure that yours will

be the first.

If you send in a description of how to use a graphic macro within a word

processor without making a reservation, you run the risk of duplicating

something that we already have a description of.  We will offer only one free

signature scan per word processor submitted.

Orbit Enterprises, Inc.

PO Box 2875

Glen Ellyn, IL  60138

Voice:          (708) 469-3405 or (800) 767-6724

LaserStuff BBS: (708) 469-4850

FAX:            (708) 469-4895