The original Panasonic KX-P4450 Laser Printer driver for Word 5.0,

PANA4450.PRD, erroneously assumed that the the unprintable area at the

top of the page would be skipped.  Unfortunately, this potentially

caused the first line of text to not be printed if the top margin is

set to zero.

The first (and easiest) way to correct the problem is to download and

use P4450.PRD.  [The original PANA4450.PRD was modified to set

AutoSkipTop = 0; and the Printer Control Sequence Description at

byte:14 was shortened to "p0e0L".]

The second (and more complicated) method allows the use of the

orginial PANA4450.PRD driver, but requires permanent modification of

printer paramaters via the front panel of the printer.  The "ORIGIN Y"

parameter must be set to "060 DOT ROWS", and the default "TOP MARGIN"

may optionally be set to "00.30 INCHES" to maintain consistency.  Both

parameters are accessed via the FORMAT key.  Be sure to set the

"PERMANENT SAVE" parameter (accessed via the OTHER key) to "YES" in

order to permanently save the changes.  Otherwise, they will be lost

when the power is shut off.  CONSULT THE PRINTER MANUAL (section 4.3)

for details on how to make these changes.