COMPLETE.SYS Documentation

COMPLETE.SYS is a DOS device driver that allows the Complete PC Scanner

to be used with PC Paintbrush. Dither and Brightness are set on the

scanner itself. Resolution is dependent on the scanner type:

Scanner                          Resolution(s) in DPI

Complete Hand Scanner/400        400,300,200

Complete Half Page Scanner       200

Complete Half Page Scanner/400   400,300,200

It is unnecessary to run the bundled software that comes with the

Complete PC Scanner. Insert COMPLETE.SYS into the CONFIG.SYS file using

the appropriate options. At boot the driver will respond with a message

based on the options. When Paintbrush is run, the scanner will light up

briefly before the tools are drawn on the screen. If this happens, you are

in business. If not, you must power down and re-address the dip switches

and try again. COMPLETE.SYS does not require the use of any interrupts,

so don't set them unless you wish to use the bundled software that comes

with the scanner.

Options are:

(Default)  Complete Hand Scanner/400

/1  Complete Hand Scanner/400

/2  Complete Half Page Scanner

/3  Complete Half Page Scanner/400

/C  Channel for DMA, 1 or 3 (default is channel 3)

/D  Double buffering switch (for some EMS boards)

/H  Hex port value, default is 3E0H

Examples:

device=complete.sys /H2E0

(port = 2E0H)

Available Port Settings:

220H 260H 2A0H 2E0H 320H 360H 3A0H 3E0H

Scanner interface card DIP Switches:

Port   #1    #2    #3    #4    #5    #6    #7    #8

N.A.  IRQ2  IRQ3  IRQ4  IRQ5

3E0H   off   off   off   off   off   off   off   off

3A0H   off   off   on    off   off   off   off   off

360H   off   on    off   off   off   off   off   off

320H   off   on    on    off   off   off   off   off

2E0H   on    off   off   off   off   off   off   off

2A0H   on    off   on    off   off   off   off   off

260H   on    on    off   off   off   off   off   off

220H   on    on    on    off   off   off   off   off

Note: DIP switch  UP = ON

If you are using the software that comes with the hand scanner, make sure

that only one interrupt is set on.

If you are using 386MAX Version 4.06 or less, it may be impossible to use

the scanner. Qualitas has fixed the problem in Version 4.07.

LIMITATIONS:

THIS DRIVER IS GAMMA RELEASE LEVEL.  THIS MEANS THAT IT WORKS WELL ENOUGH

TO PASS OUR TESTING DEPARTMENT; HOWEVER, WITH THE MYRIAD OF PC HARDWARE

AVAILABLE ON THE MARKET TODAY, THERE MAY BE SOME PROBLEMS REMAINING.

WE DO KNOW THAT THIS DRIVER (VERSION 4.01) WILL PRODUCE SEAMLESS IMAGES

WITH EMS (LIM 4.0 SPEC.) AND AN 'AT' COMPUTER.