NISCAN.SYS

The Niscan Handy Scanner is completely programmable using the PaintBrush

scanning menus.

This scanner supports:

Resolutions: 200 & 100 DPI

Brightness: 1-9

Dithers: 3

The Niscan may be addressed in four different locations. These port addresses

can be switched using a combination of command line switches and in setting

the jumpers, according to this chart:

258h/0  (default, no command line switch)

278h/1  (switch JB2)

358h/2  (switch JB3)

378h/3  (switch JB2 & JB3)

LIMITATIONS:

THIS DRIVER IS GAMMA RELEASE LEVEL. THIS MEANS THAT IT WORKS. THOUGH IT PASSED

OUR TESTING DEPARTMENT, IT WAS JUDGED MARGINALLY ACCEPTABLE. THE REASON IS

THAT THIS HAND SCANNER DOES NOT SUPPORT DMA TRANSFER OF DATA (AS DO ALL OUR

OTHER HAND SCANNER DRIVERS), AND IS EXCEPTIONALLY SLOW.

WITH THE MYRIAD OF PC HARDWARE AVAILABLE ON THE MARKET TODAY, YOU MAY FIND

OTHER PROBLEMS WITH THIS DRIVER.

THIS DRIVER IS CAPABLE OF PRODUCING ALMOST SEAMLESS IMAGES WITH EMS (LIM 4.0)

AND AN 'AT' CLASS COMPUTER.