LOGITECH.SYS Documentation

LOGITECH.SYS is a DOS device driver that allows the Logitech ScanMan to be

used with PC PaintBrush. The resolutions are 400, 300, 200 and 100 dpi.

Dither and Brightness are set on the scanner itself.

Options are:

/C  Channel for DMA, 1 or 3 (default is channel 1)

/D  Double buffering switch (for some EMS boards)

/H  Hex port value, default is 280H

Examples:

device=logitech.sys

(set up for DMA channel 1 & Port 280H)

device=logitech.sys /c1

(set up for DMA channel 1)

device=logitech.sys /c3 /h3e0

(set up for DMA channel 3 & Port 3E0H)

LIMITATIONS:

THIS DRIVER IS GAMMA RELEASE LEVEL.  THIS MEANS THAT IT WORKS WELL ENOUGH

TO PASS OUR TESTING DEPARTMENT; HOWEVER, WITH THE MYRIAD OF PC HARDWARE

AVAILABLE ON THE MARKET TODAY, THERE MAY BE SOME PROBLEMS REMAINING.

WE DO KNOW THAT THIS DRIVER (VERSION 4.01) WORKS PERFECT WITH EMS

(LIM 4.0 SPEC.).