MARS.SYS

ZSoft Corporation's MARS.SYS is designed to work with the MarsTek 105 and

128 hand scanners. This scanner device driver has only two options:

scanner type and DMA channel.

Scanner type:  /1 MarsTek 105

/2 MarsTek 128

DMA channel:   /C1 DMA Channel 1

/C3 DMA Channel 3

Example CONFIG.SYS entry:

device=mars.sys /2 /c3

(setup for the MarsTek 128 and DMA channel 3.)

These Marstek hand scanners do not support interrupts, therefore to obtain

the best performance, MARS.SYS requires EMS. Paint Brush may save buffers

to the hard disk during the scan.

When the disk is accessed, scan lines can be lost, resulting in gaps in the

image. The entire scan may be scanned to Expanded Memory and then read back

as required, greatly reducing the chance of losing scan lines.

LIMITATIONS:

THIS DRIVER IS GAMMA RELEASE LEVEL. THIS MEANS THAT IT WORKS WELL ENOUGH TO

PASS OUR TESTING DEPARTMENT; HOWEVER, WITH THE MYRIAD OF PC HARDWARE

AVAILABLE ON THE MARKET TODAY, THERE MAY BE SOME PROBLEMS REMAINING.

WE DO KNOW THAT THIS DRIVER (VERSION 4.01) WILL PRODUCE SEAMLESS IMAGES

WITH EMS (LIM 4.0 SPEC.) AND AN 'AT' COMPUTER.