MAKING A REFERENCE BLOCK FRAME FOR MERGEIT

MergeIt uses black reference blocks in the upper left and right corners of the

scanned image segments to calculate the best image cut and perform the merge.

For best and consistent results, these blocks should be very black so that the

boundary to the adjacent white regions are distinct. Write protect stickers

for 5 1/4" floppies are often excellent, as long as one of the long edges is

cut back to remove the rounded corners. When placing a document on the frame,

the thickness of these stickers makes it easy to align the document with the

bottom edges.

Frame layout for letter length images:

�<......3.5"......>�<......3.5".......>�<......3.5".......>�

Black=>        �����Ŀ            �����Ŀ             �����Ŀ            ���Ĵ

Reference      �������            �������             �������            �����

Blocks         | ~1" |            | ~1" |             | ~1" |            1/4"

(height                                                                 (width)

> 0.1")                                                               (or more)

Hand Scanner GuideBar Setup:

�������������������������������������������������������������������������Ŀ

���������������������������������������������������������������������������

optional top slide guide

............... ���Ŀ

.  optional    � L �Ŀ             �����������������������������Ŀ

.   back      � E � �             �  Hand Scanner               �

.   triangle � F � �i            �                             �

.   support � T � �m            �                             �

.          �   � �a            �������Ŀ             ���������

.         � G � �g                    �             �

.        � U � �e                    �             �

.       � I � �                     �             �

.      � D � �c                    �             �

.     � E � �u                    �             �

.    �   � �t             ����������������������Ŀ

.   �   � �              ������������������������

.  �   � �e              hand scanner bottom guide

. �   � �d

.�   � �g                          � �

�   ���e             scan       \ � � /

�����                  image     \� �/

down      .

using left guide against

scanner body and bottom

guide.

Attaching a bottom guide to the hand scanner is extremely helpful in getting

accurate image scanning. Any turning of the scanner will distort the image

in ways that are not correctable by any software. The hand scanners should be

made with an easy way to add an edge guide. They aren't..., I use an aluminum

rod, 1/4" square and 6" long, epoxied to the bottom edge of the handle.

The left guide may be a straight piece of trim wood, a piece of utility

aluminum angle, or ?. It must be straight and long enough to guide the

scanner to the bottom of its scanning sweep.

The image cut edge is a piece of stiff white paper secured to the bottom of

the left guide.

Position the image cut edge so that it just slightly intrudes into the scanned

image. This will help you to best align the left guide when scanning in an

image. It will also define the left image edge since it will cut off the top

left reference block. The image cut edge is not essential (in which case the

scanner will naturally cut off the reference block at the left edge of the

image it scans).

Orienting paper for landscape scanning and merging:

Start scan

D               C              B     here     A

black   ���Ŀ reference ���Ŀ  blocks  ���Ŀ downward ���Ŀ

����������������������������������������������������Ĵ

�       ...    LEFT MARGIN EDGE .       . . .        �

�       ...     ..             ..       . . .        �

�       ...     ..             ..       . . .        �

�       ...     ..             ..       . . .     T  �

�       ...     ..             ..       . . .     O  �

�       ...     ..             ..       . . .     P  �

�       ...     ..             ..       . . .        �

�       ...     ..             ..       . . .        �

�       ...     ..             ..       . . .        �

�               D O C U M E N T                      �

������������������������������������������������������

Where possible, adjust document to put natural breaks in the reference

block regions. Typically this will be between text lines. Start scanning down

the document on the right side (at the top of the document) with the left

edge of the scanned image covering part of reference block "B" at its top edge.

Shift to the left so that the next scanned image covers part of the reference

block "C" at its top edge.

Finally, shift to the left so that the next scanned image includes part of the

reference block "D".

The width of each of the black reference blocks is written in the initial

values file. See "MergeIt.Doc".

Hints:

1) It is NOT required by MergeIt that you lay the input image document on

its side. I chose this convention as an introduction since it is most

appropriate for scanning in typed documents. (When scanning longer

documents in the upright position, you may find your hand scanner

software will not allow a long enough sweep at the higher DPI's.)

2) You may find the image cut edge casts a shadow into the scanned image.

If the edge is made to lay flat on the document, the shadow will be

minimized. You may also slightly undercut the edge in the region where

the image is scanned. You may also choose to eliminate the paper edge

and use a plastic spacer against the guide edge as you move the guide

to determine the next scan section. Cut the plastic spacer so it

indicates where your hand scanner left image edge starts.

3) To assure that each scanned section has reference block heights>=0.1",

you can make use of the top slide guide edge if you have one. Position

the reference blocks on the frame so that if your scanner uses the top

guide as a stop, the resultant downward sweep includes enough of the

reference blocks.

If you don't use a top slide guide, then you may cut the reference blocks'

height to about 1/8" (or more). When scanning, make sure the image starts

above the reference blocks.

Using MergeIt Without a Mechanical Scanning Frame

If the document to be scanned is part of a book or magazine that does not

fit the above scanning setup, you may still use MergeIt by making use of

the fact that you can make a movable strip with the reference blocks

appropriately positioned. Back this strip with a removable tacky glue, which

makes the strip similar to the common "Post-It" memo squares. Place the

strip at the top of the desired image. Carefully position a left guide edge

vertically to prevent slanting your image edges. Scan downwards against

this guide edge.