EZPLACE HELP FILE

Version 3.10

SEPTEMBER 6, 1991

Leading Edge Industries

2120 Myrtle Avenue

St Paul, Minnesota 55114

(612) 645-5254

and

101 Eagle Drive

Balsam Lake, Wisconsin 54810

$TOF

THE PURPOSE OF EZPLACE

EZPLACE is a program designed to allow ANYONE to enter parts,

placements and geometries of existing or custom Wire Wrap boards

and Printed Circuit Boards (PCBs).  EZPLACE has several purposes:

o   Simplify entry and reduce errors inherent in placing

parts on pre-pinned wire wrap boards.

o   Allow users to place components where THEY want them.

The data created by this program allows Leading Edge to deliver

to the user a Wire Wrapped assembly or PCB.  The user should:

o   Specify a board geometry or template

o   Specify board outlines and keep-out areas

o   Create special parts

o   Place parts where desired

o   Send the data to Leading Edge

EZPLACE was written by Rod May in 1990.  Dan Stover and Greg

Volk modified it in 1991 to include Library Part Creation,

Excellon Board Template Input, Redac Output, Keep Out Areas,etc.

$TOF

TABLE OF CONTENTS

I.   INTRODUCTION

1.  License to Use

2.  About Leading Edge

3.  Free Updates

4.  Hardware Requirements

5.  Definitions

II.  USING EZPLACE

1.  Setup

2.  Files

3.  Operation

III. MENU REFERENCE

1.  [A]DVANCED MENU

2.  [F]ILE MENU

3.  [K]EEP OUT MENU

4.  [L]IBRARY MENU

5.  PART COMMANDS

6.  CURSOR AND MAGNIFICATION COMMANDS

IV.  BOARD TEMPLATES

1.  Sources

2.  Format

3.  Example

V.   LIBRARY

1.  Definition

2.  Supplied Library Parts

3.  Creating Library Parts

4.  Editing  Library Parts

5.  Duplicating Library Parts

6.  Removing Library Parts

7.  DON'T INITIALIZE LIBRARY

UNTIL YOU READ THIS!

$TOF

VI.  PLACING PARTS

1.  Locating

2.  Rotating

3.  Inserting

4.  Deleting

5.  Moving

VII. KEEP OUT AREAS

1.  Definition

2.  Placing 'Keep Out' Lines

3.  Uses for 'Keep Out' Lines

VIII.OUTPUT

1.  What is Redac?

2.  Redac Output

3.  Net Lists

IX.  FUTURE

1.  Board Templates

2.  Net List Entry

3.  Off-Grid Parts

$TOF

I.   INTRODUCTION

1.  License to Use

There are NO restrictions on using EZPLACE!  You may

use it for Personal, Commercial or Industrial purposes.

You may give it to ANYONE (including competitive

companies) as long as the credits are displayed when

it is executed.

You will note the output is in ASCII, Redac format,

possibly the most common interface specification known.

No attempt is made to make use of the output difficult,

please use it.

2.  About Leading Edge

Leading Edge Industries are Minnesota Corporations

with Corporate offices in St Paul, MN and manufacturing

plants in St Paul, MN and Balsam Lake, WI.

Leading Edge Design (LED) produces PCB designs,

layouts and custom software in response to an engineers

manual or CAE design.

Leading Edge Assembly (LEA) is a quality vendor of Quick

Turn and production volume Wire Wrap assemblies.  LEA

does board stuffing, mechanical and electronic assembly

and miltary subcontracting. LEA claims 7 Vertical, 3

Horizontal and 5 Semi-Automatic Wire Wrap Machines.

Leading Edge Circuits (LEC) produces Quick Turn and

small production quantities of Printed Circuit Boards

(PCB) for a wide range of customers and usages.  LEC

fabricates PCB boards, down to 5 mil lines and up to

10 layers.

If you need it done, Leading Edge can do it!

3.  Free Updates

Write or call Leading Edge with your Company, Name,

Address and Telephone Number and we'll mail you free

updates and bug corrections.

There are more board templates available, if interested

please call.  If you have a board template you wish to

make available to the world, send it in.

4.  Hardware Requirements

A.  IBM PC 512K minimum memory

B.  Floppy disk drive or method of transferring files

C.  Hard Disk drive

D.  EGA or VGA monitor

5.  Definitions

A.  CURSOR

A cursor indicates where attention is currently

being focused.  For the purposes of EZPLACE a

part acts as the cursor, the default being a one-

pin 'TPOINT', shown as a white box.  When you

[C]hange Part, say to a 14-pin DIP, the new part

becomes the cursor and is shown in white.  Power

(Light Red) and Ground (Light Cyan) pins are shown

to aid placement.  Position the cursor where you

want to [I]nsert parts onto the board.

In [L]ibrary Mode the cursor is the next pin to be

placed and in [K]eep Out Mode the cursor is the next

'Keep Out' Line to be placed.

B.  SCROLL

While positioning a part or drawing a line, Pin 1

of the part or cursor might go beyond the viewport

on screen.  In this case the board is redisplayed

with Pin 1 of the cursor centered on the screen.

C.  ZOOM

The viewport on screen shows up to 20 x 20 inches

of the board.  Since this looks small on a CRT, a

magnification factor ranging from 2 (whole board)

to 16 (chip level view) dots per 100 mils has been

included.  Change the magnification with [Shift]

PgUp/PgDn and [A]dvanced [M]agnification keys.

$TOF

II.  USING EZPLACE

1.  Setup

Create your directory by typing MKDIR EZPLACE

Move to that directory by typing CD EZPLACE

Prepare to use easy place by then typing

'pkunzip a:\ezplace'. This will load all necessary

files into the directory "EZPLACE" as well as a

"DEMO" design.

2.  Files

There are several file types generated at one point

or another in the EZPLACE process:

A.  *.BRD and *.GRD files are the Excellon (Text) and

binary formats of board templates.

B.  LIBRARY.BIN contains the library parts.  [F]ile

[R]estoring from the floppy archive overwrites

this file, therebye deleting any custom parts

which were not present when the archive was created.

C.  *.JOB files hold the actual EZPlace parts data.

At this time the user is allowed 1000 parts per

design.

D.  *.KOA files contain the 'Keep Out Area' lines and

'Board Outlines'.  At this time the user is allowed

1000 lines per design.

3.  Operation

A.  If you desire a specific board template, assure that

the *.BRD template file is in your directory.  If no

Board template is specified, EZPlace will assume all

grid locations are valid for placement.  See BOARD

TEMPLATES(IV) for how to create a *.BRD Excellon file.

B.  Type 'EZPLACE'.

C.  Place your board or print on a convenient surface,

COMPONENT side up.

D.  Establish a reference point (0,0) at the UPPER

LEFT corner of your board.  This point should be on

a 100 mil grid, but need not be an actual pin.  ALL

placed pins must be below and to the right of this

point as EZPLACE uses only positive coordinates.

E.  To begin a new JOB enter the desired Board Template,

if you want one (see Step A), selected from the

[F]iles [B]oard menu entry.  Please note that the

Board Template is optional and exists to only to aid

the user.

OR...

To load an existing JOB use the [F]iles [L]oad or

[F]iles [M]erge menu entries.  The Board Template

formerly assigned to the job will be loaded.

F.  If creating a new design use the [K]eep Out Menu to

draw a board outline.

G.  [C]hange cursor to the first part you wish to place

or do a [L]ibrary [C]reate Part if the specified part

does not yet exist.

H.  Move the cursor part using the arrow keys or shifted-

arrow keys and the [R]otate option.  Use [I]nsert Part

to place a part at the current cursor position and the

[D]elete and [M]ove Part options to alter previously

placed parts. Continue placing, using [C]hange Part to

get a new Library Part.

I.  After placing all parts on the board use the

[K]eep Out Menu to enter your 'Keep Out Lines'.

Note that these 'lines' need not describe a closed

figure.

J.  Save the JOB using the [F]iles [S]ave option.

Enter a new job name or confirm the old name.

K.  After completing the job, use the [F]iles [A]rchive

option to save all relevant files to a floppy.  Send

this floppy to Leading Edge for wrapping or layout.

Please note that only one job may be archived per

floppy disk.  Only the last job archived on a floppy

is available for use.

$TOF

III. MENU REFERENCE

1.  [A]DVANCED MENU

[I]nitilize LIB

Delete ALL existing parts from the library with

the exception of the 'TPOINT' part.  BEWARE!

[M]agnification

Set the magnification factor directly (from 2 to

16 dots per 100 mils).

[P]rint Menu

[B]oard Description

Print a graphical representation of the board

template onto a generic printer.

[M]anual Print

Prints this manual to LPT1.

[E]pson Printer

Print the board on an Epson printer.

[8]0 Column Printer

Specifies 80 column printer.

[1]32 Column Printer

Specifies 132 column printer, default value.

2.  [F]ILE MENU

[D]elete Job

The *.JOB and *.KOA files are deleted from Disk.

[L]oad Job

Input *.JOB and *.KOA files.

[M]erge Job

Input *.JOB and *.KOA files to current job.

[S]ave Job

Output *.JOB and *.KOA files.

[A]rchive

Copy all required files to floppy disk.  The

disk then may be sent to Leading Edge for Wire

Wrapping or PCB Layout.

[R]etrive Archive

Bring data in from an archive floppy and put it

into the current directory.  Display the job.

[B]oard Template

Input *.BRD Excellon Board Template file.

[W]rite Template

Output current template to *.BRD Excellon file.

[O]utput Redac

Output the *.CDI Redac file.

3.  [K]EEP OUT MENU

[I]nsert Line

Enter the cursor line as a 'Keep Out' and begin a new

line at the movable end.

[D]elete Line

Delete the closest 'Keep Out' line to the cursor.

[BACK]up Line

Delete the last 'Keep Out' line entered.

[CR] New Line

Move the fixed end of the cursor line to the movable

end, thus allowing a new line to be entered.

4.  [L]IBRARY MENU

[C]reate Part

Create a new library part from scratch.

[D]uplic Part

Duplicate an existing part and edit the copy.

[E]dit Part

Edit the selected library part.  Addition pins may

be added as in [C]reate or existing pins deleted

with the backspace key.

[R]emove Part

Remove a library part from the library.  BE CAREFUL!

5.  PART COMMANDS

[C]hange Part

Highlight cursor device type using the arrow keys

and Select it with [CR].  The new cursor device type

will be used each [I]nsert Part until changed again.

[D]elete Part (also [DEL])

Highlight device to delete using the arrow keys and

Select it with [CR].  Confirm the deletion.

[I]nsert Part (also [INS])

Place a new part at the current cursor location and

name the part.  The part will not be placed if any

portion of the part is outside the placement area

or if the name is already used on the board.

[M]ove Part

Highlight device to move using the arrow keys and

Select it with [CR].  Use the arrow keys and [R]otate

Part to reposition and [I]nsert Part as above.

[R]otate Part

Rotate the cursor part 90 degrees clockwise.

[BACK]up Part

Delete the last device entered.

6.  CURSOR AND MAGNIFICATION COMMANDS

[->] and all other arrows

Move the cursor 1 pin in the arrow direction.

[Shift] [->] and all other arrows

Move the cursor 10 pins in the arrow direction.

PgUp/PgDn

Increase/decrease magnification incrementally.

[Shift] PgUp/PgDn

Increase/decrease magnification by 2.

Home/End

Refresh the display centered on cursor/center.

[Shift] Home/End

Zoom all the way in/out.

$TOF

IV.  BOARD TEMPLATES

1.  Sources

a) If you desire a custom board template, then you must

first design a part or parts which results in the desired

hole pattern, duplicate that part where needed, adding

it and other parts until all the holes on the custom

board exist.

To save this template then you enter [F][W] and give the

new *.BRD file a name; for example "BILL".  EZPlace will

write a file "BILL.BRD" which may be used ever after as

your Wire-Wrap Board Template.

The board you described may be either one you purchase

or Leading Edge can fabricate and pin or socket that

board for you.  [F][A] produces all the files to send

Leading Edge for a quote to be generated.

b) If you desire a specific board, drilled, socketed or

pinned you place parts until all the holes on the custom

board exist and all parts are in place.

To save this template then you enter [F][W] and give the

new *.BRD file a name; for example "BILL".  EZPlace will

write a file "BILL.BRD" which may be used ever after as

your Wire-Wrap Board Template.

The board you described may be either one you purchase

or Leading Edge can fabricate and pin or socket that

board for you.  [F][A] produces all the files to send

Leading Edge for a quote to be generated and the board

built.

c) If you have a fully pinned board or complete confidence

(snicker,snicker) in your measurements, the default

grid is there waiting for you.

$TOF

2.  Format

You can also hand enter a board template using the

following format.  All normal Excellon punctuation is

acceptable, but not required.  The default format for

EZPLACE is Leading Zeros, two integer places:

o   X 1"       Y 2"       entered as  X010Y020

o   X 1.5"     Y 2.1"     entered as  X015Y021

o   X 0.1000"  Y 0.2000"  entered as  X001Y002

3.  Example

X001Y001

X002

X003

X004

X001Y002

X002

X003

X004

$TOF

V.   LIBRARY

1.  Definition

The library is a collection of Supplied and Custom part

types from which the user selects the cursor, which in

turn is used for part placement.  Magnification will be

initialized to 16 dots per 100 mils in library mode.

2.  Supplied Library Parts

EZPLACE is shipped with a supplied library composed of

Single Inline Packages (SIP) and Dual Inline Packages

(DIPS) of various sizes.  The normal makeup of the

default library will be SIPs from one(1) to forty(40)

pins, DIPS from six(6) to forty(40) pins. The user may

not edit or delete these parts.

From version to version this library may change,

therefore you should keep the old versions of EZPLACE

when a new one arrives.  The library structure may be

different and your old design could have problems.

Each new primary version (3.xx,4.xx,5.xx) will remain

compatable and should have its own directory.

3.  Creating Library Parts:  [L]ibrary [C]reate

Using the arrow keys, position the cursor pin and press

[INS], [I]nsert, [P]ower or [G]round. The cursor becomes

part of the device and an incremented cursor is positioned

in the same direction as the last arrow key pressed.

If you make an error, [D]elete/[DEL]ete the last pin

placed.  Press [ESC] to finish and confirm the new part.

When you add a Library Part, it will appear at the end of

the displayed list of available parts.

The part pins MUST be on a 100 mil grid.  If you need

to place an off-grid part, create a part which takes

up the same space as this and has the same number of

pins.  Add a note to the files sent regarding the exact

part description.  This will be added into your board.

4.  Editing Library Parts:  [L]ibrary [E]dit

Much the same as creating a new part, except you start

where you left off last time.

5.  Duplicating Library Parts:  [L]ibrary [D]uplic

A cross between creating and editing a part.  Select an

existing part, name the new part as in [C]reate and edit

as in [E]dit.

$TOF

6.  Removing Library Parts:  [L]ibrary [R]emove

Move the arrow to the part you want to delete, press

[CR] and presto, it's gone.  EZPlace will check that

you have no parts on the current board of this type.

You must use caution with this option since you may

have boards not currently loaded which use this part

type.  You cannot delete the supplied library parts

with this option (see Initialize Library).

7.  DON'T INITIALIZE LIBRARY UNTIL YOU READ THIS!

The entire Parts Library is stored in a file called

LIBRARY.BIN.  If for any reason this file disappears,

you will have only the TPOINT part to work with.

[L]ibrary [I]nitialization develops a completely new

Parts Library and will destroy ALL but the TPOINT part.

You should use this option ONLY for unusual situations

and place your new LIBRARY.BIN in a special directory.

$TOF

VI.  PLACING PARTS

1.  Locating

Once a part has been [C]hanged from the Parts Library

selection menu, it appears in the placement window as

a rectangular box with pin one indicated by a small

square.  This is your cursor.  Use the arrow keys and

[Shift] arrow keys to position the cursor and PgUp/PgDn

or [Shift] PgUp/PgDn to increase/decrease the

magnification.

2.  Rotating

Press [R]otate to rotate the cursor part 90 degrees

clockwise about pin one.

3.  Inserting

Once the part properly located, press [I]nsert to fix

it at that location.  Then respond to the part name

prompt (U01,P3,R16,etc).

4.  Deleting

Press [D]elete and select which part to delete from the

parts menu.  Confirm your deletion with [Y]es.

5.  Moving

To Re-place an existing part use [M]ove Part and select

which part to move from the parts menu.  Then move or

[R]otate the cursor as before, remembering that the next

time you [I]nsert a part, it will move the part you

just selected to the current cursor location.  You can

type [ESC] to leave the part where it was.

$TOF

VII. KEEP OUT AREAS

1.  Definition

Every board has some area in which wires are not desired,

even if merely the extents of the board itself.  The

EZPlace 'Keep Out' capability allows the user to exclude

wires from entering restricted areas, thereby controlling

wire placement.

2.  Placing 'Keep Out' lines

Using this feature requires that you:

o   Enter [K]eep Out mode from the Main Menu

o   Use the arrow keys for cursor movement

o   Use [Shift] PgUp/PgDn for magnification

o   [I]nsert/[INS]ert 'Keep Out' when positioned

o   [D]elete/[DEL]ete the last 'Keep Out' entered

o   [ESC] to return to the Main Menu

Please note that the lines are kept with the JOB, not

the board template.  This allows different 'Keep Out

Areas' on the same physical board.

Don't overdo the 'Keep Out Areas'.  You could make it

IMPOSSIBLE to build the board!  If this is done, you'll

be hearing from the programming group.

3.  Uses for 'Keep Out' lines

One obvious use is to keep wires from going over an area

where something exists that might hinder assembly or

cause reliability issures in the future.

Another usage is to use just a single line, not a 'Keep

Out', but rather a 'Don't Cross' line.  This is useful

to make the finished product look neater, control the

direction of wires leaving a connector, isolate a noisy

section of the board, etc.

Leading Edge's Wire Wrap program will not allow a wire

to touch any 'Keep Out' line.  Vertical 'Keep Out' lines

tend to make the wires follow vertical channels, but too

many lines tend to make wire lengths longer.

$TOF

VIII.OUTPUT

1.  What is Redac?

The Redac CAD system is one of the oldest CAD systems

available in the US.  As such, Leading Edge Assembly

selected it to be the output format for EZPLACE.

At LEA information from EZPLACE is read directly into

a Redac CAD system for plotting, error checking and

output to the Wire Wrap program.

2.  Redac Output

When this is selected, from the [F]iles option, a Redac

*.CDI file is generated which is an initial data file

for the Redac system.  A new job is created in the

Redac CAD system and the input is error checked and

brought up for display and plotting.

An incidental benefit to this is that since Leading

Edge Assembly has a sister company, Leading Edge

Design, which does PCB layout, the output from EZPLACE

is also direct input to the Layout group.  The users

board can be turned into a PCB at a 25% cost reduction

over other options.  This is because the user has

already specified the components and locations,

reducing the designer's work.

3.  Net Lists

The Redac Netlist format is:

.REM NET23               { Net Name           }

U01 1 U02 2 U03 3 U04 4  { Any Number of Pins }

U04 4 U05 5 U06 6        { Continuation line  }

U06 6 U07 7              { Duplication OK     }

.REM NET24               { Start of new net   }

U22 2 U23 3

.REM POWER

Alpha pin designators are allowed but discouraged.

Although Redac format is preferred, a netlist in any

format is acceptable, for example:

o OrCad         o ViewLogic     o Valid

o PCad          o Mentor        o Calay

o Case          o Daisy         o FutureNet

$TOF

IX.  FUTURE

1.  Board Templates

There are plans to indicate Power(Red) and Ground(Cyan)

pins on the board templates to aid proper placement.

2.  Net List entry

Facility will be added to enter, check and transmit

Power/Ground/Signal nets.  This will aid layout and

Wire Wrap of the boards and reduce production times,

errors and programming cost significantly!

3.  Off Grid Parts

Some faculty will be added to allow parts which either

are placed on a location which is not on a 100 mil

grid, or has pins which are not on a 100 mil grid.

For now, enter the correct number of pins, make a part

which takes up the correct amount of space, and send a

part description with the floppy for fabrication.