The Tubesheet Helper v1.0   Evaluation Copy             December 18, 1991

TEMA Tubesheet Builder

By: CADsultants

PO BOX 5364

Deptford, N.J. 08096

U.S.A

(609) 384-1378

The Tubesheet Helper v1.0  Demo Version                December 18, 1991

Table of Contents ------------------------------------------------(1)

Copyright --------------------------------------------------------(2)

Shareware Concepts -----------------------------------------------(2)

Limited License --------------------------------------------------(2)

Registration -----------------------------------------------------(2)

Disclaimer -------------------------------------------------------(3)

Trademarks -------------------------------------------------------(3)

Introduction -----------------------------------------------------(4)

System Requirements ----------------------------------------------(5)

Installation -----------------------------------------------------(5)

Recommended DOS Settings -----------------------------------------(6)

Configuring ------------------------------------------------------(6)

How to Run The Tubesheet Helper --------------------------------(7-9)

DXF Importing ----------------------------------------------------(9)

Registration Form -----------------------------------------------(10)

(c) 1991  CADsultants - CAD/Computer Consultant      Pg 1 of 10

The Tubesheet Helper v1.0  Demo Version                 December 18, 1991

Copyright

Copyright 1991-1992 by CADsultants, All rights reserved. No part of the

publication may be reproduced, transcribed, copied or reduced to any

electronic medium or machine readable form without the written

permission of CADsultants.

Shareware Concept

The Tubesheet Helper is not public domain or free software, but is being

distributed under the shareware concept.  This enables the user to

first use the software before deciding whether or not to purchase it.

Limited License

NON-REGISTERED users of this software are hereby granted a limited

licence for a maximum of 30 days to try this Demo Version on a trial

basis.  Because this is an evaluation, it does not give full support to

all available options. At the end of this trial period, The Tubesheet

Helper must be registered, or use discontinued.

Registration

The registration fee for The Tubesheet Helper is US $395.00 (*).  You

must send US $395.00 (*) to CADsultants for one license per CPU

concurrently using The Tubesheet Helper.  You may register The Tubesheet

Helper only with CADsultants .  No other registrations will be

recognized.  If you are updating from a previous version, registration

will only cost US $95.00 (*).

Please submit all registration and updates using either a printed

copy of the REGISTER.FRM text file included in The Tubesheet Helper

(shareware version) or a photocopy of the registration form at the

end of the document.

If and when you register you will be given the latest version of

The Tubesheet Helper including all the supported options. You also will

receive a printed manual and a serial-numbered copy.

(*) New Jersey State residents please add 7% sales tax.

Shipping and Handling charges are US $5 and US $10 if outside USA.

Package will be send UPS Ground, other shipping available at

additional charge.

(c) 1991  CADsultants - CAD/Computer Consultant      Pg 2 of 10

The Tubesheet Helper v1.0  Demo Version                December 18, 1991

Disclaimer

CADsultants makes no representations or warranties with respect to

this publication or its contents and specifically disclaims any

implied warranties of merchantability or fitness for any particular

purpose. In addition, CADsultants reserves the right to revise this

publication and to make changes from time to time in its contents

without obligations of CADsultants to notify any person of such

revisions or changes.

USE THIS SOFTWARE AT YOUR RISK!

CADsultants

PO BOX 5364

Deptford, NJ

08096 U.S.A.

Trademarks

The Tubesheet Helper is a trademark of CADsultants.

IBM is a trademark of International Business Machines, Inc.

AutoCAD is a trademark of AutoDESK Inc.

MicroStation is a trademark of Intergraph Corp.

TEMA is a trademark of Tubular Exchanger Manufacturer Association, Inc.

(c) 1991  CADsultants - CAD/Computer Consultant      Pg 3 of 10

The Tubesheet Helper v1.0  Demo Version                 December 18, 1991

Introduction

The Tubesheet Helper is a PC based program for generating full scale Heat

Exchanger Tubesheet layouts in accordance to TEMA. All data is entered

via full screen input sheets.  After data has been entered, the user can

preview the layout On-Screen, then generate a DXF (Data Exchange Format)

file.  The DXF output can be used to import the Tubesheet layout into

your CAD (Computer Aided Design) System for future modification or

plotting.

*) All Tubesheets can be created per TEMA B, C, or R

*) Tubesheet data entered via user-friendly input screens

*) Maximizes number of tubes in a given shell.

*) Creates Triangular, Rotated Triangular, Square or

Rotated Square.

*) Horizontal and Vertical pass partition lanes.

*) Prints layouts On-Screen.

*) Generates DXF output files.

(c) 1991  CADsultants - CAD/Computer Consultant      Pg 4 of 10

The Tubesheet Helper v1.0  Demo Version                December 18, 1991

System Requirements

IBM or 100% Compatible: 8088, 286, 386, or 486

System Memory: 640k or More

Disk Storage: 512k

Floppy Drive: 5-1/4" 1.2Mb, 360k

or: 3-1/2" 720k, 1.44Mb

Color Monitor: CGA, EGA, or VGA Monitor

MS-DOS: 3.0 or Higher

Installation

Insert The Tubesheet Helper Program Disk in Drive A and enter

the following:

A:

A:\> INSTALL

Installation program will prompt you for a directory to place

support files.

Also Print-out the READ.ME file supplied on your Program Disk for

last minute items not covered in this document.

(c) 1991  CADsultants - CAD/Computer Consultant      Pg 5 of 10

The Tubesheet Helper v1.0  Demo Version                 December 18, 1991

Recommended DOS Setting

In the Root Directory of your Hard Drive there should be two files

called CONFIG.SYS and AUTOEXEC.BAT. If these files do not exist, t hey

must be created. If they do exist,  compare them to the settings

listed below and make the appropriate changes.

<CONFIG.SYS>

FILES=20

BUFFERS=10

<AUTOEXEC.BAT>

PATH=[PATH];C:\TSHELPER

Where [PATH] is the existing path setting, and C:\TSHELPER is the

location you installed The Tubesheet Helper.

Configuring The Tubesheet Helper

There is a configuration file copied with other files in the install

director.  The file TSHELPER.CFG contains values used to set the program

to work with your type of monitor.  Use any ASCII type editor to make

your changes if needed.  Each field in the CFG file is explained on the

previous line in configuration file.

Example:

'The following settings are used to setup your systems monitor type.

'[0]= Text Only and MGA Video

'[1]= CGA and EGA 320x200  16 colors  2 Bits Per Pixel

'[2]= CGA and EGA 640x200  16 colors  1 Bit Per Pixel

'[7]= EGA 320x200  16 colors  4 Bits Per Pixel

'[8]= EGA 640x200  16 colors  4 Bits Per Pixel

'[9]= EGA 640x300  16 colors  4 Bits Per Pixel

'

MONITOR TYPE

9

Lines starting with a single quote (') represents comments.  The line

labeled MONITOR TYPE is the heading for the next line, which in this case

is the number 9.  This sets TSH to display in EGA 640x300 16 color etc...

(c) 1991  CADsultants - CAD/Computer Consultant      Pg 6 of 10

The Tubesheet Helper v1.0  Demo Version                December 18, 1991

How to Run TSH

TSH is easily learned after using it a couple of times.  The first

thing to do is to load TSH,  this is done by entering TSH from the

install directory (C:\TSHELPER).  TSH data input consists of two screens

which allow you to move about each input field sequentially.  Move from

field to field by pressing the arrow up or arrow down keys.  Data fields

can be changed by entering new value over top of old, or by pressing the

delete key. Each of the data fields are described below.

*TEMA Class [B] [C] or [R]:

Sets the Mechanical Standard Class per TEMA Section 1.12

*Tubesheet Outside Diameter:

Self-Explanatory

(VALUE LOCKED AT 25.75" IN DEMO)

*Diameter of Bolt Circle:

*Specifies Tubesheet Bolt Circle if extended for bolting.

*Number of Bolt Holes:

Specifies Number of Tubesheet Bolt Holes if extended for bolting.

*Diameter of Bolt Holes:

Specifies Tubesheet Bolt Hole Diameter if extended for bolting.

*Shell Inside Diameter:

Self-Explanatory

(VALUE LOCKED AT 20.0" IN DEMO)

*Baffle Clearance on Diameter:

Specifies the difference between the Shell I.D. and the Baffle O.D.

If a value of 0 is entered, TSH will default to TEMA Section 4.3

*Outer Tube Limits Diameter:

Specifies the farthest distance from center of Tubesheet to the outside

of an extreme tubehole.  If a value of 0 is entered, TSH will calculate

an O.T.L.  (VALUE LOCKED AT 19.75" IN DEMO)

*Outside Diameter of Tube:

Self-Explanatory

(VALUE LOCKED AT .5000" IN DEMO)

*Choose a Pattern:

Specifies how tube holes shall be placed using standard pattern types as

shown in TEMA Section 2.4.  Patterns can be Triangular, Triangular

Rotated, Square, or Square Rotated.

(VALUE LOCKED AT "Triangular" IN DEMO)

*Tube Pitch:

Specifies the distance between each tube hole on a pattern.

If a value of 0 is entered, TSH will default to TEMA Section 2.5

(c)  1991  CADsultants - CAD/Computer Consultant      Pg 7 of 10

The Tubesheet Helper v1.0  Demo Version                December 18, 1991

How to Run TSH - Continued

*Number of Tie-Rods:

Self-Explanatory

If a value of 0 is entered, TSH will default to TEMA Section 4.71

If a value of -1 is entered, TSH will not place any Tie-Rods.

(VALUE LOCKED AT 8 IN DEMO)

*Diameter of Tie-Rods:

Self-Explanatory

If a value of 0 is entered, TSH will default to TEMA Section 4.71

*Vertical Pass Partition Clearance:

Specifies the distance between the outside of tubes straddling a vertical

pass partition plate or lane.

*Horizontal Pass Partition Clearance:

Specifies the distance between the outside of tubes straddling a

horizontal pass partition plate of lane.

*1st Tube Hole on Tubesheet C/L:

Specifies if the pattern will start with a hole in the center of the

tubesheet or with the pattern straddling.  This value will only be read

if the Horizontal and/or Vertical Pass Partition Clearance=0

*Pattern Offset by Window or Coordinates:

Specifies if the pattern is eccentric to the tubesheet center-line.  The

window option allow you to select an area where not tubes are to be

places.  The pattern will start by placing tubes up to, but not crossing

over the window dimension, then filling the remainder of the tubesheet.

Thereby shifting the pattern. If WINDOW is selected, then the AREA [1]

[2] [3] or [4] must be specified.  Only one window may be specified at a

time.  The areas are as follows:

AREA 1= UPPER Portion of Tubesheet (0)

AREA 2= LEFT Side of TubeSheet (270)

AREA 3= LOWER Portion of Tubesheet (180)

AREA 4= Right Side of Tubesheet (90)

If COORDINATES is selected, then X-Axis and Y-Axis Offset must be

specified.  These values can be any value within the range of the O.T.L.

Direction of offset can be controlled by entering positive or negative

values based on the Cartesian Coordinate System.

*Plot Pattern On Screen:

Allows defined pattern to be previewed On-Screen before creating DXF

file.  If the pattern is not the way you wanted, then you may return to

the input screens to make additional changes.  This process can be

continued indefinitely.

(c)  1991  CADsultants - CAD/Computer Consultant      Pg 8 of 10

The Tubesheet Helper v1.0  Demo Version                December 18, 1991

How to Run TSH - Continued

*Path:

Allows the DXF file to be made in a directory other then the root of the

current drive.  If a path is given it MUST NOT end with a backslash and

specified path MUST EXIST.  (DXF OUTPUT IS NOT AVAILABLE IN DEMO)

*Filename:

Allows you to specify the name of the DXF output file.  Filename must be

8 Characters or less and no extensions are allowed.

(DXF OUTPUT IS NOT AVAILABLE IN DEMO)

DXF Output File

Although DXF output is not available, you will find an actual DXF file

made from the registered version of The Tubesheet Helper.  DXF files have

been tested on AutoCAD Rel 9/10/11, and MicroStation PC v 3.3/4.0.  Here

we will describe the process for loading DXF files in to your CAD System.

**AutoCAD**

Load AutoCAD in your usual way, and stop at the Main Menu.  Enter "1" for

Create new drawing, then the filename you want the pattern to be called,

followed by an equal sign (=).  The equal sign tells AutoCAD not to

reference any prototypes with defined layers, ltypes, fonts, etc.  After

you press enter AutoCAD will load, at the command prompt enter "DXFIN"

followed by the DXF filename.  If the DXF file is not in the current

directory you will need to specify the path as well as the DXF filename.

**MicroStation**

Change directory to USTATION\DXF or wherever your Microstation DXF

directory is located on your system. Enter "DXFIN", then the name you

specified in TSH, followed by the design name you want the pattern to be

called in MicroStation.  After the design file has been created, return

to MicroStation's Main Directory, usually \USTATION. Load MircoStation in

your usual way, and load the design name specified in the DXF process.

(c)  1991  CADsultants - CAD/Computer Consultant      Pg 9 of 10

The Tubesheet Helper v1.0  Demo Version                 December 18, 1991

The registration ownership of The Tubesheet Helper v1.0,  print out and

complete the following form.  Include a check or money order for US

$395.00 (*) to CADsultants for one license per CPU concurrently using The

Tubesheet Helper.  You may register The Tubesheet Helper only with

CADsultants. No other registrations will be recognized. If you are updat-

ing from a previous version, registration will only cost US $95.00 (*).

Send to:

CADsultants

PO BOX 5364

Deptford, NJ 08096

U.S.A.

Name: __________________________________________________________________

Title: _________________________________________________________________

Company: _______________________________________________________________

Address: _______________________________________________________________

City: _____________________ Providence: __________ Zip: ________________

Phone: (______) _________________________ Date: ________________________

CAD Software/Version/Machine type:_____________________________________

________________________________________________________________________

How did you receive The Tubesheet Helper v1.0?

__ New Order

__ Friend

__ Upgrade from Version __   -- Serial Number ____________

__ User Group - Group Name ________________________ Phone: ____________

__ Software BBS - BBS Name ________________________ Phone: ____________

__ CompuServe GO ___________ Forum

Comments: __________________________________________________________

____________________________________________________________________

____________________________________________________________________

____________________________________________________________________

(c) 1991  CADsultants - CAD/Computer Consultant      Pg 10 of 10