ACUTERM REMARKS

REV 0.432

(REVISED 25-NOV-90)

�����������������������������������������������������������������������������

ACUTERM 0.432 IS EXPERIMENTAL!  IF YOU FIND BUGS, LET ME KNOW ABOUT THEM!

IF THE BUG BOTHERS YOU, WRITE FOR AN EARLIER VERSION.

IF YOU ARE READING THIS AFTER AUTOBOOTING, YOU MAY ABORT THE DISPLAY OF THESE

NOTES BY HITTING CTRL-C.  IF YOU WISH TO RUN ACUTERM, AT THE PROMPT CHOOSE TO

CONTINUE THE BATCH FILE.  OTHERWISE, CHOOSE TO TERMINATE THE BATCH FILE.

�����������������������������������������������������������������������������

CONTENTS

INTRODUCTION:

HARDWARE REQUIREMENTS:

WHAT'S NEW?

INSTALLATION OR UPDATE--SYSTEMS WITH NO HARD DRIVE:

INSTALLATION OR UPDATE--SYSTEMS WITH A HARD DRIVE:

GETTING ACQUAINTED--THE TERMINAL SCREEN:

GETTING ACQUAINTED--THE HELP SCREEN AND MAIN MENU:

OPERATING ACUTERM--LOGGING:

OPERATING ACUTERM--ID'ING:

OPERATING ACUTERM--READING FILES AND TRANSCRIPTS:

OPERATING ACUTERM--USING THE MINI-EDITOR:

OPERATING ACUTERM--USING TRANSCRIPT CAPTURE:

OPERATING ACUTERM--USING BEACON FILES:

OPERATING ACUTERM--USING BEACON CALL:

OPERATING ACUTERM--USING MORSE QSK LEVELS:

OPERATING ACUTERM--SPECIAL CHARACTERS:

OPERATING ACUTERM--CONTROLLING THE TRANSCEIVER:

OPERATING ACUTERM--RESETTING THE PK-232 FROM ACUTERM:

OPERATING ACUTERM--DATABASE AND ACCESSORIES:

OPERATING ACUTERM--TRANSLATING LOGS FROM ARIES AND LAN-LINK:

OPERATING ACUTERM--LEAVING THE PROGRAM:

TECHNICAL TOPICS--SETTING UP A VIRTUAL DISK:

TECHNICAL TOPICS--MODIFYING THE BATCH FILE:

TECHNICAL TOPICS--USING PARAMETER FILES:

TECHNICAL TOPICS--PORTS AND INTERRUPTS:

TECHNICAL TOPICS--EDITING THE LOG:

TECHNICAL TOPICS--ACCESSORIES:

TECHNICAL TOPICS--WORDOUT AND AUTOMATIC CARRIAGE RETURNS:

TECHNICAL TOPICS--LENGTH OF FILE PACKETS:

TECHNICAL TOPICS--FILE ORDERING:

TECHNICAL TOPICS--THE FILES SUBDIRECTORY:

BULLETIN--THE MISSING EXCLAMATION POINT:

CLOSING REMARKS:

�����������������������������������������������������������������������������

HARDWARE REQUIREMENTS:

ACUTERM is a terminal program for the AEA PK-232, and is intended to run on

IBM compatible clones with DOS 3.0 or above.  It runs the PK-232 in the host

mode.

It is suggested that you operate the PK-232 from COMM 1, and a TS-440S or

TS-940S from COMM 2.  Your printer should be connected to LPT1.

RS-232 cables must have at least five wires: DATA IN, DATA OUT, RTS, CTS,

and GROUND.

The use of batteries in the PK-232 is highly recommended.  This will avoid

the necessity of having a parameter file (INITIAL.PAR) loaded each time you

boot the program.

�����������������������������������������������������������������������������

WHAT'S NEW, 0.432 VS. 0.415?

o  ACUTERM 0.432 no longer uses the CUSTOM.DTA file used by earlier versions.

o  ACUTERM 0.432 allows most configuration parameters to be changed "on the

fly", including TBAUD.  Only COMM port and Transceiver changes require

rebooting.  To reconfigure, use CTRL-F1.  (Note that CTRL-F1 no longer

is an alternative way to bring up the BATCH FILE menu, but rather F6.)

o  ACUTERM 0.432 creates its own batch file, AUTO.BAT, for you whenever you

change its configuration.

o  ACUTERM 0.432 allows contest logs to be kept on floppies, or in a different

hard drive from the permanent log.

o  ACUTERM 0.432 no longer uses external help files.  Help screen information

is contained internally.

o  ACUTERM 0.432 no longer requires extensive customization of CQ and QRZ

files.  You can use the generic files to get started.

o  ACUTERM 0.432 has 7 packet channels, Channels 0-6.  Channel 6, while full

featured, is intended especially for sending CQ and monitoring.  Any connect

request made on Channel 6 is redirected to the lowest numbered unused

channel.  The F4 key initiates a connect request to CQ on Channel 6,

regardless of the current channel.

To realize the full benefit of the dedicated monitor channel, you should set

MCON to "ON" in your INITIAL.PAR file.

o  ACUTERM 0.432 provides optional direct screen writing, which is faster

and may provide cursor flash elimination on machines where it has been

a problem.

�����������������������������������������������������������������������������

INSTALLATION OR UPDATE--SYSTEMS WITH NO HARD DRIVE:

While ACUTERM was written primarily for use with hard drive systems, it is

possible to operate it from floppy-only systems.  To make that easier, this

distribution disk is provided in autobooting form.

The disadvantages of operation from a floppy are that the log will be

limited in size, and that log searches will be very slow.  Non-volatile

storage of transcripts is not advised.

To use ACUTERM in the autobooting mode, use DISKCOPY to make a working copy

of this distribution disk.  Before operating with the copy, you must strip all

unnecessary files from it, including these notes, so as to make room for your

log.  Here's how:

o  Print or jot down this procedure.

o  Make sure that your PK-232 is connected to a COMM port of your computer,

and note the number of the COMM port.

o  If there are batteries in your PK-232, reset it using another terminal

program, and immediately turn it off.  The PK-232 may also be reset by

temporarily removing JUMPER J1 or the batteries.

(You may normally skip this step, unless you have difficulty establishing

communications between ACUTERM and the PK-232.)

o  If you are already using an older version of ACUTERM, put your old working

disk in DRIVE B:.  Your existing log and customized files will be copied

to the new working copy automatically.

If not, use any text editor to replace my call and selcal with yours in the

file \ACUTERM\FILES\INITIAL.PAR.

o  Put the working copy (not the original!) of the distribution disk in your

DRIVE A:.

o  Turn the PK-232 on.  (If you have reset the PK-232, the BAUDOT light should

be on, indicating that the PK-232 is beginning the autobaud sequence.)

o  Reset your computer.  Hit ENTER until the display of these notes is over.

(If you think it takes too long, think about writing them.)  ACUTERM will

then autoboot from DRIVE A:.

o  The first time that you use ACUTERM 0.432, it will ask you questions about

your setup.  It is best to avoid initial complications, so you should avoid

setting up a transceiver interface and trying direct screen writing until

after you have everything else working.  You should initially select:

--DRIVE A:\ACUTERM for LOG ACCESS.

--DRIVE C:\ACUTERM for TRANSCRIPT ACCESS.

--The correct COMM PORT for the PK-232.  PORT 1 is highly recommended.

--NONE for TRANSCEIVER INTERFACE PORT.

When in doubt, accept the defaults by just hitting ENTER.  If you make a

mistake, leave the configuration session by hitting ESC, and enter it

again by hitting CTRL-F1.

(In floppy-only systems, DRIVE C:\ACUTERM is the proper choice for transcript

access even though there is no physical drive named C:.  That is because the

ACUTERM autobooting disk creates a RAM DRIVE by that name.)

o  After you have answered all the questions, ACUTERM should coax the PK-232

into communication.  If not, you will get a "no communications" warning.

o  When the initialization is completed, hit F1 to bring up the main menu.

Select BATCH FILE using the up/down cursors, and hit ENTER.

o  A batch file menu will appear.  Select INSTAL_A.BAT to remove all

unnecessary files to make room for your log.

If asked whether you want to erase or delete files, respond "Y".

If you have an old ACUTERM disk in DRIVE B:, INSTAL_A.BAT will then copy

all necessary files from your old disk to your new one, including your log,

customized calls, brags, and reference files.

o  Now you are ready to use ACUTERM.  Most of the information you need is on

the on-screen prompts and the help screens (F1), with some supplementary

remarks in these notes under OPERATING ACUTERM.

o  To boot up ACUTERM again, just put your working copy of the disk into your

DRIVE A: and reset your computer.

�����������������������������������������������������������������������������

INSTALLATION OR UPDATE--SYSTEMS WITH A HARD DRIVE:

The following remarks assume installation in HARD DRIVE C:.  If ACUTERM is

installed in HARD DRIVE D:, run INST_D.BAT instead of INST_C.BAT when indicated

below.  If still another drive, X, create an INST_X.BAT file patterning it

after INST_C.BAT.

To install ACUTERM 0.432 onto your hard drive:

o  Print or jot down this procedure.

o  Make sure that your PK-232 is connected to a COMM port of your computer,

and note the number of the COMM port.

o  Boot up your computer on its own DOS.

o  If there are batteries in your PK-232, reset it using another terminal

program, and immediately turn it off.  The PK-232 may also be reset by

temporarily removing JUMPER J1 or the batteries.

(You may normally skip this step, unless you have difficulty establishing

communications between ACUTERM and the PK-232.)

o  Put this distribution disk in your DRIVE A:.

o  Turn the PK-232 on.  (If you have reset the PK-232, the BAUDOT light should

be on, indicating that the PK-232 is beginning the autobaud sequence.)

o  Type A:\INSTAL_C.BAT (or A:\INSTAL_D.BAT) and hit enter.  After considerable

copying of files, ACUTERM 0.432 will boot up and ask you for selections.

If asked whether you want to erase or delete files, respond "Y".

o  NEW USERS:

It is best to avoid initial complications, so you should avoid setting up a

transceiver interface and trying direct screen writing until after you have

everything else working.  You should initially select:

--The hard drive that ACUTERM is installed in for LOG ACCESS.

--The hard drive that ACUTERM is installed in for TRANSCRIPT ACCESS.

--The correct COMM PORT for the PK-232.  PORT 1 is highly recommended.

--NONE for TRANSCEIVER INTERFACE PORT.

When in doubt, accept the defaults by just hitting ENTER.  If you make a

mistake, leave the configuration session by hitting ESC, and enter it

again by hitting CTRL-F1.

In hard drive systems, DRIVE C:\ACUTERM is the recommended initial choice

for TRANSCRIPT ACCESS if you are a new user.  After you get ACUTERM running,

it is strongly recommended that you set up a virtual disk for transcript

handling.  See:

TECHNICAL TOPICS--SETTING UP A VIRTUAL DISK:

and

TECHNICAL TOPICS--MODIFYING THE BATCH FILE:

OLD USERS:

You have probably already created a VDISK or RAMDRIVE, so feel free to

indicate that drive for TRANSCRIPT ACCESS.

If your computer is not compatible with direct screen access, you can

recover by erasing the file \ACUTERM\CONFIG.STR and rebooting.

o  After you have made all the selections, ACUTERM should coax the PK-232

into talking, and initialize it.

o  Now you are ready to use ACUTERM.  Most of the information you need is on

the help screens (F1), with some supplementary remarks in these notes under

OPERATING ACUTERM.

o  To run ACUTERM the next time, call \ACUTERM\AT.

�����������������������������������������������������������������������������

GETTING ACQUAINTED--THE TERMINAL SCREEN:

In the TERMINAL or MONITOR modes, where you will spend most of your time,

the upper two thirds of the screen is the TRANSCRIPT of your QSO or reception.

The line at the bottom of the TRANSCRIPT screen is a PROMPT line that usually

tells you about your principal options at any particular time.

Below the TRANSCRIPT screen is a three line LOG.  Depending on your setup

and equipment, most or all of the information on the left will be entered

automatically.  The fields of the top line of the LOG are indicated by the

PROMPT, the second line is usually for station information, and the bottom line

is usually for personal information.  However, the only manditory protocol

is that a callsign appear where indicated.

Below the LOG is a STATUS line that tells you the date, time, and PK-232

parameters of interest.  Parameters are generally changed by hitting the cursor

keys indicated by arrows, or the ALT keys indicated by the first letter in

the parameter.

Below the STATUS line is a three line typing MONITOR.

GETTING ACQUAINTED--THE HELP SCREEN AND MAIN MENU:

Read the HELP screens with F1 or ESC, noting that they contain a slider

main MENU.  On monocolor systems, the NO CHANGE menu selection should be

brighter than the others.  If not, adjust the brightness and contrast of

your monitor.

Move from one menu selection to another with the UP/DOWN cursor controls.

Select BAUDOT, hit ENTER.  When you re-enter the terminal mode, the status

information will change to indicate BAUDOT.  Hit the up and down cursor

controls, and verify that the baud rate changes on the screen.

Now hit F1 or ESC again, and note that the HELP information now pertains

to the BAUDOT mode.

The information surrounding the MENU is different in each mode, and

together with that in prompts, is all you need to know to operate ACUTERM.

Note that you can enter command keys while still in the HELP mode.

�����������������������������������������������������������������������������

OPERATING ACUTERM--LOGGING:

The best way to learn the logging system is by using it, guided by the

help screens and prompts, until you have tried all the options.  The key

things to remember are:

o Sign-in with ALT-F8.

o Enter/Exit editing with F8 as information comes in.

o Search with F7.  Tap F7 to find all matches.  If you find an old

log entry with information that you would like to copy to the

new one, hit ALT-C.  Tap until all entries are found, or hit ESC,

to return to the TERMINAL mode.

o Refresh the sign-out time at the end of the QSO by tapping F8 twice.

o Store the log entry with ALT-F8.  Hit ALT-F8 again when you want to

start a new one.

As long as the call area of the log is blank, no entry will be stored in the

log.  This means that a log entry can be aborted by just rubbing out the call.

You can do this with the space, backspace, or the erase-edit-field key, F5.

You don't have to worry about losing a log entry if you forget to store it

prior to quiting the program.  Log entries are stored automatically on exit.

However, turning the computer off, or resetting it, is another matter!  ALWAYS

USE THE QUIT SELECTION ON THE HELP SCREEN TO END AN OPERATING SESSION!

Searching:

You can search for the currently active call by hitting F7.

You can look for any call by hitting CTRL-F7 and entering the call or prefix

to be searched for in the box.

Note that ACUTERM searches by looking for a partial match.  This is

advantageous in that you can search for all entries with a certain prefix by

just typing in the prefix.  For example, the search will turn up all YV

stations by typing in "YV".  However, a search for N8BA will also turn up

N8BAA, N8BAB, etc.

You can flip through the log manually by hitting ALT-F7 from the terminal

mode, and use the up/down cursors to read entries.

You can jump back and forth between flipping and searching.

If log searches are slow, and there is quite a bit of head activity, run one

of the commercial programs that reorganizes your hard drive, such as the

COMPRESS utility in PC-TOOLS.

Editing:

Editing an old log entry is accomplished by displaying the old entry, by

searching or flipping, hitting F8 to edit, and hitting ESC to store the edited

entry.

�����������������������������������������������������������������������������

OPERATING ACUTERM--ID'ING:

Once you have entered your QSO partner's call in the log, the F2 keys may be

used for ID macro's in the following way:

o  F2 =     ID     : Used principally at beginning of transmission,

but may be used anytime:

YV1AQE DE N8BA..

or

YV1AQE DE N8BA =

o  ALT-F2 = ID + AR: Used to terminate a transmission and includes

the +? or CTRL-D:

YV1AQE DE N8BA KN

(CTRL-D)

or

YV1AQE DE N8BA

+?

o  SHFT-F2= ID + SK: Used to terminate a QSO:

YV1AQE DE N8BA

CLOSING AT 23:00 UTC...SK

or

YV1AQE DE N8BA

BREAKING LINK 23:00 UTC...SK

or

YV1AQE DE N8BA SK

�����������������������������������������������������������������������������

OPERATING ACUTERM--READING FILES AND TRANSCRIPTS:

The same scheme is used to read the transcipts and to read files.  Only the

entry method is different.  To read what has gone off the top of the screen,

engage the transcript reader with the PAGE UP key, and do the obvious things

with the up/down cursors and PAGE keys.  Use ESC to exit.

To read files, select READ FILES from the MAIN MENU.  Choose your file from

the directory presented.

Once you are reading a file or transcript, a number of features are shown

on the prompt.  These include editing the screen and saving it as the same or

a different file, loading the transceiver and ARQ/CONNECT boxes (provided the

cursor line has the required format), printing, etc.

If you want to cut out something and make it a file, hit ALT-E to enter the

MINI-EDITOR.  See the next section of these notes.

�����������������������������������������������������������������������������

OPERATING ACUTERM--USING THE MINI-EDITOR:

The same editor is used for editing the log, the transcripts, files, and

QSL labels.  In the non-log roles, the TAB's have a normal function.  In the

log, the TAB and SHIFT-TAB move you from field to field.  In all applications,

F5 clears the screen.

ACUTERM's internal file editor is intended for writing brief messages,

making changes to short files, or creating excerpts from longer files and

saving them under a different name.  Only what you see on the screen is

saved.

DO NOT ATTEMPT TO USE THE EDITOR TO CHANGE A FILE THAT RUNS OFF THE

SCREEN--YOU WILL LOSE WHAT YOU DO NOT SEE.  WHEN EXCERPTING FROM A LONG

FILE, MAKE CERTAIN YOU CHANGE THE NAME WHEN SAVING!

To use the mini-editor to change a file, display the file by choosing READ

FILES on the help screen and selecting the file.

To edit the file, hit ALT-E.  If you want to remove everything that is

currently there, hit F5.  Use the cursors to move around and enter text,

then hit ESC to end the editing.

A box will then appear with the file name in it.  If you want to save

the edited file under that name, hit ENTER.  If not, type a new name over

the old one and hit ENTER, or--if you decide to abort--hit ESC.

These steps are suggested by the prompts on the screen.

�����������������������������������������������������������������������������

OPERATING ACUTERM--USING TRANSCRIPT CAPTURE:

While the transcript file system provides a way to cut out screens and

make them files, longer transmissions can be recorded using the TCAPTURE

function.  This is toggled with the ALT-T key.

When you exit TCAPTURE, the program asks you to enter the file name where

the captured transcript should be stored.  A pre-name is provided; use it

if you wish, or type over it.  If you rub out, or clear with F5, the pre-name,

or hit ESC, you can leave the capture function "open", and resume capturing

to the file later.

Once a capture file is "closed", you can no longer add to its contents with

the TCAPTURE function.

If you give TCAPTURE a file name that already exits, it is rejected.

Your captured material is not lost.  Simply enter and exit TCAPTURE again,

and give it another file name.

Un-named captured material is retained when you turn your computer off.

�����������������������������������������������������������������������������

OPERATING ACUTERM--USING BEACON FILES:

The HELP screens desribe a BEACON CQ and sending a file as a BEACON.  What

does BEACON mean?  It means that the CQ or other file will be sent repetitively

until you hit the DELETE key or the file has been sent 10 times.  In AMTOR, the

transmissions will be aborted automatically if your station receives an ARQ or

FEC transmission.  The files are sent with the listening time that you selected

during configuration.

This feature is intended for sending a string of CQ's, or for repetitively

calling another station.  If the time-out feature limits your capability

to hog the band while away from your station, buy a KAM and set up a BBS.

�����������������������������������������������������������������������������

OPERATING ACUTERM--USING BEACON CALL:

The BEACON CALL is engaged with the CTRL-F2 key, and causes a beacon to be

sent, calling the station currently logged.

�����������������������������������������������������������������������������

OPERATING ACUTERM--USING MORSE QSK LEVELS:

You can change the QSK level by tapping ALT-Q.  Level 0 is the conventional

one where you put the PK-232 into the transmit mode with ALT-X, and put it

into the receive mode at the end of your transmission by ending it with

CTRL-D or ALT-F2.

Level 1 is identical except that XMITOK is turned off, so that the PTT line

is not keyed.  This allows you to activate your transmitter with VOX or use

full or semi-break-in.	By doing this, your transceiver will receive during

pauses, allowing you to copy by ear, while your PK-232 stays in the transmit

mode.

Level 2 is automatic.  The PK-232 will go into the transmit mode whenever

you type something, and automatically go back into the receive mode when its

buffer is empty.  No ALT-X is necessary, nor are CTRL-D's.  XMITOK is off

during transmit, so your transmitter must be in the semi- or full-break-in

mode.  This mode is fine for casual conversations, but you probably would not

want to use it for contests.

�����������������������������������������������������������������������������

OPERATING ACUTERM--SPECIAL CHARACTERS:

Certain special characters will be translated during sending if imbedded

in files.  These characters have no special meaning if typed into the buffer

directly from the keyboard.  When imbedded in files, the special characters

can be used to create generic files.  The special characters and their meaning

are:

CTRL-C      = YOUR CALLSIGN

CTRL-L      = THE CURRENT LOG NUMBER

CTRL-O      = THE OTHER STATIONS CALL

CTRL-S      = YOUR SELCAL

Note that the XT-PRO editor has some rather strange reactions to these CTRL

characters.  ACUTERM's editor, as well as PC-TOOLS and NORTON editors, can be

used.

Of course, the special PK-232 character CTRL-T continues to be the TIME and

does not have to be imbedded in a file to have that meaning.

�����������������������������������������������������������������������������

OPERATING ACUTERM--CONTROLLING THE TRANSCEIVER:

The transceiver control functions are as follows:

o  F9 and F10 control the RIT.  Shift-F10 clears the RIT.

o  Alt-F9 and ALT-F10 control the main frequency.

o  SHIFT-F9 tunes the transceiver to the current log entry from the terminal

mode.

o  ALT-X tunes the transceiver from non-terminal modes.

Note that the READ FILES function provides a prompt for loading the

transceiver from text files, and that the log reading functions (ALT-F7,

CTRL-F7, etc.) allow you to load the transceiver from a log entry.

The format of a text file entry for loading the transceiver is illustrated

below.  THIS FORMAT WAS CHANGED SLIGHTLY IN REV 0.407 TO ALLOW THE PK-232

MODE TO BE PROGRAMMED.

14.079.00/ 14.079.00 LSB  AMT  KR4P       LEE        PALM BAY, FLORIDA

14.077.00/ 14.077.00 FSK  AMT  YV1AQE     WILLY      MARACAIBO, VENEZUELA

14.115.00/ 14.115.00 LSB  PAC  AA6OK

28.187.00/ 28.187.00 LSB  PAC  KR4P V YV1AQE

����Fa���������Fb�����MDtx�MDpk��CALL��<<TO ENTER FROM LOG: ALT-L����EXIT:ESC�

Note how it aligns with the template on the edit prompt of the ACUTERM

mini-editor.  When using the mini-editor, also note that you can load a

file from a currently displayed log entry by positioning the cursor to

the line where you want the entry, and hitting ALT-L.

A decimal point, separating kilohertz and hertz, must be present. (The

decimal point between megahertz and kiloherts is optional.)

When you load the transceiver from a file, or the log, the first (receive)

frequency will be loaded into VFO A, the second (transmit), following the

slash, will be loaded into VFO B, and the modes of both transceiver and

PK-232 will be loaded as well.

The /(second frequency) is optional--if only one frequency is given, it

will be loaded into both VFO A and VFO B.

If your entry is intended for packet, you may enter the call starting

at column 32 with the "via" path extending as far as necessary across the

screen.  (Packet entries should not have name, qth, or comments after the

call.)

After loading the transceiver, and leaving the READ FILES mode, you will

find this call or its selcal in the CONNECT and ARQ boxes.  (Any logging

activity will erase or write over the selcal.)

You don't need a transceiver interface to find the above useful!

�����������������������������������������������������������������������������

OPERATING ACUTERM--RESETTING THE PK-232 FROM ACUTERM:

If there is a possibility that another program has left some non-default

parameters in the PK-232, you may want to reset the PK-232.  Enter the verbose

command mode (SHIFT-F1), type "RESET, [ENTER]", and slowly tap the "*" key

until you get the introductory message.

It is recommended that you exit ACUTERM at this point, and reboot it to

properly initialize the PK-232 for host mode communications.

�����������������������������������������������������������������������������

OPERATING ACUTERM--DATABASE AND ACCESSORIES:

KF8CU has written a DATABASE program, ACULOG. It allows you to sort and

print your ACUTERM log, allows you to handle QSL's as a batch process, and many

more functions.  These excellent programs are on this disk in the form of a

self-extracting ZIP file.  Run ACC.EXE to expand these programs.

He has also written a PRIMER for ACUTERM, and a tsr MOUSE SUPPORT program.

Write John if you are interested in these.

�����������������������������������������������������������������������������

OPERATING ACUTERM--TRANSLATING LOGS FROM ARIES AND LAN-LINK:

KA1FXY has written a program to translate an ARIES or LANLINK log to

ACUTERM, and vice versa.  It is included on this disk!

�����������������������������������������������������������������������������

OPERATING ACUTERM--LEAVING THE PROGRAM:

ALWAYS LEAVE ACUTERM BY USING THE QUIT SELECTION ON THE MAIN MENU.

If u don't, any active log entries will not be stored, your permanent

transcript will not be updated with the session's material, and you

will leave your PK-232 in the host mode (NOT A GOOD THING TO DO!).

�����������������������������������������������������������������������������

TECHNICAL TOPICS--SETTING UP A VIRTUAL DISK:

Hard drive users will very likely want to set up a virtual disk for working

access to their transcripts.  (Floppy users do not have to worry about this as

it is all taken care of or them by the ACUTERM disk.)

What is a virtual disk?  A virtual disk is a simulated disk drive using RAM

memory.  A virtual disk is very very fast, and suffers no wear and tear when it

is accessed.

The suggested way of handling storage of ACUTERM's transcripts, which are

updated each carriage return, is to copy the current transcript from a hard

drive to the virtual disk just before an ACUTERM operating session, and to copy

it back to the hard drive immediately after the session.  (This is the function

of AT.BAT.)

The benefits are faster speed, less wear on the disk drive, and one added

bonus.  When ACUTERM tries to write a transcript into a full disk drive, it

throws away all but the last 16 k of the transcript.  Because the virtual disk

is small (your choice, but usually 64 k), the transcripts can never grow to the

point where they take up the whole hard drive.  So the use of a virtual disk

or ram drive makes your transcript files self limiting.

Follow the instructions in your DOS MANUAL to set up a virtual disk.  Look

under either RAMDRIVE or VDISK in the index--they are the same thing, but the

name of the .SYS file depends on your version of DOS.

EXAMPLE:

You make the virtual disk by inserting just one line in your computer's

CONFIG.SYS file (which is always found in the root directory).

My computer calls the virtual disk setup program VDISK.SYS and has all DOS

accessory files, including VDISK.SYS, in a \DOS directory.  My CONFIG.SYS

file is:

COUNTRY=001,437

SHELL  =\COMMAND.COM /E:1024 /P

DEVICE =\DOS\ANSI.SYS

DEVICE =\DOS\VDISK.SYS

FILES  =20

BUFFERS=40

What's to the right of the equal sign must be the COMPLETE and CORRECT PATH

to VDISK.SYS or RAMDRIVE.SYS, as the case may be.

After you have modified your CONFIG.SYS, reset your computer and make sure

that you have a virtual disk.  For hard drive systems, it will be the letter

following that of the last physically present hard drive.

After you have gotten your virtual disk established, make use of it by doing

the following:

o  See the section of these notes called AT.BAT, and modify this batch file

according to your personal preference.  If you have just one hard drive, C:,

and your virtual disk is D:, no modification is necessary.

o  While running ACUTERM, go to the configuration routine (CTRL-F1) and choose

the drive letter of your virtual disk (usually DRIVE E:\ACUTERM).

�����������������������������������������������������������������������������

TECHNICAL TOPICS--MODIFYING THE BATCH FILE:

The primary purpose of the acuterm batch file is to allow the user lots

of flexibility in handling transcript files.

ACUTERM itself is really a file called TERM.EXE, and the batch file

prepares things for it, runs it, then cleans things up afterwards.

Beginning with ACUTERM 0.432, there is a default way of handling this

preparation and cleanup activity.  The default batch file is named AUTO.BAT.

If you are not interested in any bells and whistles, simply call \ACUTERM\AUTO

to run ACUTERM.

The automatic features can be combined with customized functions by having

AUTO.BAT contained in another batch file.  For example, AT.BAT can look like

this:

prompt SHELL--$p$g

call auto.bat

prompt $p$g

\mm

Note that some DOS versions may not accept the "call" in the second line.

The "call" is required by DOS 3.3 (at least) in order to return to the

"prompt $p$g" after running auto.bat, and to have it restore the normal prompt.

If AT.BAT bombs in your DOS, remove the word "call".  If you leave ACUTERM,

and still have an "acuterm" prompt, you can:

o  Avoid the whole issue by running AUTO.BAT instead of AT.BAT to run ACUTERM.

o  Avoid the whole issue by removing the prompt commands from AT.BAT

o  Confront the issue by learning how to nest batch files with the COMMAND

instruction.

Of course, you can still do everything for yourself, and write an AT.BAT

that keeps a non-volatile record of only Channel 0 like this:

prompt SHELL--$p$g               (Option:Makes a special "shell" DOS prompt)

cd \acuterm                      (Changes the directory to \ACUTERM)

copy xscript0.dta d:\acuterm\*.* (Copies only Channel 0's transcript to vdisk)

term.exe                         (Runs ACUTERM)

copy d:\acuterm\xscript0.dta *.* (Copies Channel 0's transcript to hard drive)

prompt $p$g                      (Restores standard DOS prompt)

\mm                              (Calls MY main menu program)

You can rewrite this file to concatenate the transcript files into one file,

or not store transcripts permanently at all.  See your DOS Manual.

Note that you also can end this file with an instruction that returns you

to your menu.

�����������������������������������������������������������������������������

TECHNICAL TOPICS--USING PARAMETER FILES:

PARAMETER FILES contain PK-232 commands to be run on booting up, changing

from HF to VHF packet or vice versa, or quiting the program.  Don't insert

CTRL-C's in the commands.  Note that .PAR files with these special names are

run automatically:

o INITIAL.PAR   (Run automatically on booting if no call is installed)

o FINAL.PAR     (Run automatically on quiting)

o VHFPKT.PAR    (Run automatically on going to VHF PACKET with ALT-V)

o HFPKT.PAR     (Run automatically on going to HF PACKET with ALT-V)

FINAL.PAR may be used to put your PK-232 parameters in a state suitable for

another terminal program on quiting.  If you normally run ACUTERM, delete it.

VHFPKT.PAR and HFPKT.PAR are very useful, and allow you to set up any number

of parameters any time you switch from VHF to HF packet.

Don't forget that you can have any number of *.PAR files, for special

circumstances, and run them as desired from a menu (ALT-F1).

�����������������������������������������������������������������������������

TECHNICAL TOPICS--PORTS AND INTERRUPTS:

In looking at some local friends' installations, I have discovered that

there is NO STANDARD meaning for serial ports COMM 3 and COMM 4.  For example,

on my AT, COMM 3 is at $02E8.  On K8SCA's AT, it is $03E8.  On W8ERN's AT, COMM

4 is at $03E0--on K8SCA's it is at $02E8.  This is no doubt a plot by Ma Bell

to cause me to get angry telephone calls at 9:00 AM on Sunday morning!

If a transceiver interface is selected, both IRQ 4 and 3 are used.  The two

COMM ports chosen for PK-232 and transceiver MUST BE SET UP TO HAVE DIFFERENT

INTERRUPTS!  THESE INTERRUPTS USUALLY ARE CONTROLLED BY DIP SWITCHES ON THE

SERIAL BOARD.

If no interface is selected, ACUTERM assumes that the COMM port has its dip

switches set according to the following interrupt scheme:

PK-232 COMM PORT     IRQ INTERRUPT ACTIVATED

1 ($03F8)     4 (STANDARD)

2 ($02F8)     3 (STANDARD)

? ($03E8)     4 (AVOID: NO CONVENTION)

? ($02E8)     4 (AVOID: NO CONVENTION)

ESPECIALLY IF YOU USE COMM 3 OR COMM 4 FOR THE PK-232, WITH NO TRANSCEIVER

INTERFACE, MAKE SURE THAT THE SERIAL PORT IS SET UP ACCORDING TO THIS CHART.

IF IT ISN'T, YOU CAN SOMETIMES MAKE ACUTERM WORK BY CONFIGURING IT (ALT-F1)

FOR A TRANSCEIVER INTERFACE EVEN THOUGH YOU DON'T HAVE ONE!

�����������������������������������������������������������������������������

TECHNICAL TOPICS--EDITING THE LOG:

Editing the log as a whole is not normally necessary, but may be required if

the log must be reconstructed following a disk mishap.  If you suddenly find

your log entries misaligned, something bad has happened to your CHRONLOG.DTA

file, and it must be repaired before logging any more entries on it.  Back it

up immediately, and erase it in \ACUTERM before continuing to operate.

The best way to assure that you don't lose your log is to back it up on

a regular schedule.

The log is easily edited with several text editors.  One is Microsoft's text

editor, called M.EXE, and comes as part of the Microsoft Macro Assembler 5.1.

Another is the NORTON text editor, which contains better internal instructions,

but must handle a large log in chunks.

TURN ANY TAB PADDING FEATURE IN THE EDITOR OFF!

DON'T ATTEMPT TO EDIT WITHOUT MAKING A BACKUP!

For your information, each log entry is a character stream of 248

characters.  240 of the characters are the log text, with no line feed/carriage

returns, and the final eight are six characters of padding, for future

information, followed by one line feed/carriage return pair.

If your log gets misaligned and you can't fix it, write for help.

�����������������������������������������������������������������������������

TECHNICAL TOPICS--ACCESSORIES:

Use ACUTERM'S batch file system to create your own set of accessories.

Bring up your menu of batch files with CTRL-F1, or from the slider menu

on the HELP screen.  You can access all of the programs on your hard

drive by writing a batch file that calls your menu program.  Use the DOS

exit of the program to return to ACUTERM.

You can enter DOS from ACUTERM by writing a batch file:

CD \

COMMAND

When you want to return to ACUTERM, enter the command "EXIT".  Our thanks

to AA6OK for this one!

You may also want to write batch files to access gray-line programs,

satellite tracking programs, and other frequently used amateur radio software.

�����������������������������������������������������������������������������

TECHNICAL TOPICS--WORDOUT AND AUTOMATIC CARRIAGE RETURNS:

Note that ACUTERM handles WORDOUT internally in all modes except MORSE.

Automatic carriage returns are built into ACUTERM for typed input,

but not for file input.  Files are sent "as is".

�����������������������������������������������������������������������������

TECHNICAL TOPICS--LENGTH OF FILE PACKETS:

Note that ACUTERM sends files in packets that terminate automatically

to end at the ends of lines.  Whenever 80 characters or less are to be

sent to the PK-232, it starts watching for a line feed, and ends the packet

on the next one it finds.  If it doesn't find one when it has sent a total

of 240 characters, it starts a new packet.

This results in files being sent so that the packets tend to end on

the ends of lines.  If PACLEN is large, 200 or more, text will be sent

in packets averaging three or more lines, usually ending at the end of a

line.

�����������������������������������������������������������������������������

TECHNICAL TOPICS--FILE ORDERING:

The operating system of your computer places new files at the first vacancy

(where a file was erased) in the directory.  It is suggested that you use a

utility such as NORTON'S ADVANCED UTILITIES to organize your CUSTOM.DTA,

HELP.*, CQ.*, QRZ.*, BRAG.*, and other little accessed files so that they are

at the beginning of the directory, and perhaps add some padding files (no

contents) to completely fill the first page of directory displays.

That way, the files that you do access often will be on the second and

subsequent pages, and you will not have to look through HELP, CQ, QRZ, and BRAG

files to find the one you are looking for.

Note that the mini-editor in ACUTERM allows you to edit files without

changing their position in the directory, but other editors sometimes do not.

If you want CUSTOM.DTA or a BRAG.* to stay put after editing, use the built-in

mini-editor.

�����������������������������������������������������������������������������

TECHNICAL TOPICS--THE FILES SUBDIRECTORY:

Several people have inquired as to why there is a FILES subdirectory. The

contents of FILES are very accessable, and can be changed easily or "killed".

To protect TERM.EXE, and your very important log--CHRONLOG.DTA, they are in a

separate directory, and not directly vulnerable to a wrong keystroke or two.

This concludes feature discussion.

�����������������������������������������������������������������������������

BULLETIN--THE MISSING EXCLAMATION POINT:

I have been buying AEA's firmware updates for several years now, each time

hoping for only one thing, an exclamation point in AMTOR.  What I got was new

features that I didn't want, old features that didn't work, and still no

exclamation point!

After avoiding the issue for some time, AEA has finally taken a position.

It now says that the U.S. Government won't allow it to provide an exclamation

point!

It now appears that this is just one more AEA tactic to cause accelerated

wear out of our equipment and to stiffle competition.  It is obvious that

sending "./" or ".'" rather than just "!" doubles the wear and tear on our

PK-232's and AEA ISO-LOOPS.

However, the anti-competitive angle to AEA's strategy is so devious as to

be terrifying.  AEA has been involved in a complex conspiracy with Charles

Keating, as well as Senators DeConcini, Cranston, and McCain.

These three Senators used constituent service as a pretense to give Keating

a tour of FCC headquarters.  While FCC staffers were distracted by the tour

guide (Ms. Gwendolynn van Paaschon), Keating spilled coffee on the "!" in a ITU

draft alphabet, making this character illegal.

AEA is now cleverly withholding the "!" while KANTRONICS and others, guided

by only common sense and no lawyers, are foolishly offering it.  Next spring,

simultaneous to introduction of a $1000 DSP MODEM and an EPROM OF THE MONTH

CLUB, AEA plans on blowing the whistle on KANTRONICS in the hopes of sending

their management to LEAVENWORTH.

(The above information was originally provided by Senator Riegle's office,

but now Riegle's people are saying they do not remember anything about it!)

If you are a good AMERICAN, and are sick of being pawns in this monopolistic

manuevering, now is your time to act.  Your free speech is at stake--the right

to say "!" on AMTOR!

Your congressmen have their addresses, if not their pictures, posted in your

POST OFFICE.  Write to them today!

The ARRL's fax number is 203-665-7531 (24 hours).  Send them a print of this

article and tell them you want them to stop advertising AEA products.

Finally, and most important, call AEA at 206-775-7373 and tell them that

you want them to get to the POINT!

�����������������������������������������������������������������������������

CLOSING REMARKS:

While there is a Copyright notice on the introductory screen, it is only to

protect the program from unauthorized exploitation.  All licensed amateur radio

operators are free to copy and distribute ACUTERM, provided that it is not for

profit, and that these notes are contained in the copy.

ACUTERM is not a commercial product, shareware or otherwise.  I would rather

have it used by a few hundred "friends" than a few thousand "customers".  If

ACUTERM stops being fun "activity" and starts becoming a bothersome "product",

I will go back to drinking again.  PLEASE DO NOT ADVERTISE ACUTERM ON BBS's!

If you request copies of ACUTERM from me, please keep in mind that mailing

a disk costs time and money, and that you will want updates.  Donations have

typically been $10-$20, and more than that is excessive.  Checks are preferred.

Do not send disks, mailers, stamps, or SASE's.  I simply cannot handle all

the various disk formats--only 360K 5-1/4 inch disks can be provided.

If you got ACUTERM from someone else, donations would be appreciated, but

only from happy users.  Please be considerate of the fact that I have quit

smoking and sniffing glue.  Ticking packages will be returned COD.  No, we have

two cats already!

Please do not encourage people to write me asking for "information about

ACUTERM".  Please give it to them yourself, and--if they are interested--have

them write me for a copy.  ACUTERM WILL NOT WORK ON KANTRONICS KAMS.

Correspondence may be sent to:

W.R.KISSEL--N8BA @ WA8DWO.MI.USA.NA

1331 E. DAWSON

MILFORD, MICH. 48381

313-685-8671 (7 PM to 10 PM EST)

CORRESPONDENCE SHOULD PROVIDE COMPUTER INFORMATION (PROCESSOR, VIDEO ADAPTOR

TYPE, COMM PORTS EQUIPPED, SERIAL CONFIGURATION, DISK CONFIGURATION, ANY

UNUSUAL ACCESSORIES, ETC.) AND PK-232 FIRMWARE REVISION.

PLEASE INCLUDE YOUR PHONE NUMBER AND--IF YOU HAVE ONE--VHF PACKET HOME BBS.

Finally, I'd like to express my thanks to ACUTERM's beta evaluators,

especially Lee--KR4P, Willy--YV1AQE, Brian--KA1FXY, Jack--AA6OK, Gary--AH6KM,

John--KF8CU, Fred--N4FG, Adolf--K8SCA, and Angie, W8ERN.

With their encouragement, interest, insight, nagging and contributions,

ACUTERM has become a group project and so much fun that we might run out of

REV numbers before its done!

73, BILL--N8BA