The DXer

By WA6JOO

19279 Santa Ana Ave

Bloomington, CA   92316

(c)1990

i. Release History

Version 1.0         Initial Release

Version 1.1         Fixed a few small bugs.  Program now works

correctly with Monochrome monitors.

Version 1.3         Fixed minor bugs in menuing system and FOT

9/3/90              calculations.  MUF and grayline functions run

MUCH faster (approx twice as fast on MUF, ten

times as fast on Gray Line). You may now

choose either six digit (Maidenhead) or four

digit Grid Square designations.  This is done

in the initial config. setup.

Version 1.31        Fixed really dumb bug in grid locator function

11/3/90             when using six digit designations.

Version 1.32        Fixed even dumber bug in bearing-distance

11/16/90            function.  Where do these things come from?

Version 1.33        Another bug. This one properly belongs to

1/11/91             MicroSoft and QuickBasic 4.5. Prevented

correct printing of custom bearing-distance

Tables. Also minor optimizing speeds up MUF

and Sunrise/set calculations by 10 to 15 percent.

1. Introduction

The DXer concentrates several functions of interest to the

serious DXer into one (hopefully) easy-to-use program.

A:   Bearing and Distance from Transmitter to Receiver.

Path ends may be selected by Lat-Lon, Grid Square,

Prefix, or by browsing through the data base.

B:   Sunrise and Sunset times for any location

C:   Maximum Usable Frequency and Frequency of Optimum

Traffic between any two locations.

D:   A listing of all locations sharing a common terminator

line (Gray Line).

E:   Prints custom bearing/distance charts for any location.

F:   A grid locator function using either six digit or four

digit coordinate system.

E:   A complete data base of all ARRL countries as well as

other locations around the world.  The data base shows

latitude, longitude,  continent and CQ zone of each

location.  Entries may be easily added, deleted or

edited.

2.  SYSTEM REQUIREMENTS

IBM PC, XT, AT or close clone with 384k minimum memory.  A

math coprocessor is highly recommended although not

required.  If you have no coprocessor, be prepared to wait

several seconds for the Gray Line and MUF calculations.

Monochrome, CGA, EGA, VGA monitor.  You may have some

readability difficulties if you use a composite b/w monitor

with a color board, since there is no way to disable color

with this combination.

One 360K floppy drive.  All data is kept in memory so,

except when loading or modifying the data base, there is no

real advantage to a hard disk.  If you have one, by all

means use it.

This program was developed on an 8MHz IBM AT and a 10MHz

clone.  The program has been tested on an IBM XT with mono

and CGA monitor, 8MHz, 10MHz, 12Mhz and 16 MHz '286 machines

with EGA and VGA, and a 16 MHz '386SX with VGA with no

problems.  It obviously runs a lot faster on fast machines.

There is no machine specific code in the program so it

should run on any 100% BIOS compatible.

A Printer is not required but, if available, many of the

data tables may be printed.

3.  GENERAL

The program is written in MicroSoft QuickBasic 4.0/4.5 with

a few routines in assembler and contains approximately 3000

lines of code.  The algorithms used in some of the

calculations were extracted from many sources and I make no

claim of originality for them except for the implementation

in QuickBasic.

The program was written for my own use and to become

proficient in the QuickBasic language and represents

literally hundreds of hours of programming time.  I am

releasing the program into the public domain for individual

use and enjoyment only!  Permission for any commercial use

is expressly denied.

Please feel free to upload or otherwise distribute copies of

The DXer. If you find this program to be useful, a small

($10 or so) donation would be appreciated, however, if you

choose not to make a donation,  go ahead and use the program

with a clear conscience. I doubt if I'll lose any sleep over

it.

4.  PROGRAM DESCRIPTION

The program is completely menu driven and is generally self

explanatory.  A couple of items to remember:

a:   When entering latitudes and longitudes, use decimal degrees,

not dd,mm,ss format.  Precede latitudes south of the equator

with a minus(-) sign.  Precede longitudes east of Greenwich

with a minus(-) sign.

b:   When running the program for the first time, you will be

prompted to create a Configuration file.  You will need to

know the Latitude and Longitude of your station, the

formfeed code (decimal) for your printer, and your summer

time change system to create the config file.  Once the file

has been created, it may be changed at any time from the

FILE UTILITIES menu.

If no printer is available, the BEARING TABLE function will

not show on the main menu and the print function will not

show on the PROPAGATION FORECAST and SUNRISE-SUNSET

selections.

c:   Two copies of the database file and the configuration file

are maintained by the program.  If the main file is damaged,

simply go into DOS and rename the main file to some other

name then rename the backup file to the primary name.  The

program will create a new backup file.

d:   When running the program, the active keys are shown at the

bottom of the screen.  Pressing an inactive key will

generate a beep from the computer.

e:   A moderate amount of error checking is done on any user

input although some types of errors are impossible to trap.

If you make a mistake on entry, use the backspace key to

edit.

f:   From any menu, use the up and down cursor keys to move the

highlite bar.  Press the Enter key to select.

g:   The Escape key will always back up to the previous logical

page.

h.   The program reads the system clock for local time and

converts to UTC as required for calculations.  If you don't

have a real-time clock be sure to set your system clock

using the DOS Time and Date functions before running the

DXer.  The Program recognizes Daylight Savings Time and

European Summer Time.  DST begins on the first Sunday in

April and Ends on the last Sunday in October.  EST begins on

the last Sunday in March and ends on the last Sunday in

September.  If your location does not observe one of these

time changes, enter "N" at the appropriate prompt when

creating your Config. file.  If some other time schedule is

observed at your QTH, you will have to make your own

correction by resetting your system clock.

5.  MENU DESCRIPTIONS

A:  BEARING-DISTANCE

Shows both short and long path bearings and distances from a

transmitter location (most often your home QTH) and a

receiver location.  The receiver location may be specified

by prefix, latitude/longitude, grid square, or chosen from

the data base.

When selecting by prefix, all locations with a common prefix

will be shown.  When searching for a prefix common to many

countries (i.e. VK), use the (C)ontinue Search key until you

find what you are looking for or exhaust all entries in the

data base.

When using the Select option, you will see the whole

database in tabular form.  Use the cursor movement keys to

browse through until you find the desired entry.  Press the

Enter key to select.

B:  PROPAGATION FORECAST

Transmitter and receiver locations are selected as above.

MUF calculations are by the MiniMUF '85 algorithm with an

initial hop length of 4000km.  Actual hop length and number

of hops are changed by the program so that the path between

transmitter and receiver is an even number of hops.  Since a

4000km hop length is very difficult to achieve due to the

extremely low radiation angle required, the program will not

extend the hop length beyond 4000km for calculations.

The FOT calculations are based on the calculated MUF with

corrections based on absolute sunspot number, UTC time, and

season.  FOT will generally be somewhere between 80 and 90%

of MUF.

The calculated values from the program have been checked

against the output from several other programs available in

the public domain as well as the propagation curves

published monthly in QST.  The plotted curves are in general

agreement although I have not observed point by point

agreement between any two programs.  The output from The

DXer seems to agree reasonably well with observed

propagation conditions on the bands.

Since the ionosphere has a long time constant, a 5 to 10 day

smoothed (average) solar flux number will generally give

more believable numbers.  No allowance is made for disturbed

conditions since I have yet to see an algorithm to do this.

If I ever find one, I will certainly add it to the program.

The radial MUF calculations are from a transmitter location

to receiver locations at a given distance from the

transmitter on bearings from 0 to 350 degrees from the

transmitter.  Maximum number of hops is 5 (20000km).

C:  SUNRISE-SUNSET TABLE

Calculates sunrise and sunset times for the selected

location over any one month period.  The first calculation

is always for the current month.

D: GRAY LINE

Calculates sunrise and sunset for all locations in the data

base and displays those locations whose sunrise or sunset is

within plus or minus one-half hour of the transmitter

location sunrise or sunset.

E:  BEARING TABLE

If you have a printer this routine will print a custom

bearing-distance table from any location to all locations in

the data base.  The printout will be in Prefix order and

includes all countries on the ARRL DXCC countries list as

well as any other location that you include in the data

base.

F:  GRID LOCATOR

This function is mainly useful by you VHF types out there.

Grid coordinates use either the six digit Maidenhead system

(i.e. DM14AA) as described in the RSGB VHF handbook or the

four digit system (i.e. DM14) more commonly used in the

states.  You may identify a grid square by entering the

latitude and longitude or may enter the grid square and

calculate latitude and longitude in which case the returned

values will be the center of the specified grid square.  If

the entered coordinates should fall on the dividing line

between two or more squares, the identification will default

to the square to the north and/or west of the entered

location.

G:  FILE UTILITIES

Allows you to maintain the data base.  You may add new

records, delete records, or edit existing records.  If you

change the data base in any way, the unchanged version is

saved as the backup file on disk.  The data base is

maintained in order sorted by prefix.

This function also allows creation of the configuration

file. The config file contains your home latitude and

longitude, preferred grid square system,  summer time change

system, and your printer formfeed code.

Well, that's about it.  I've pretty well debugged the code but there

is no such thing as a bug free program if it contains more than a few

lines of code.  If you find any significant bugs, drop me a note on

the back of a QSL and I'll fix it.