OTHER FCC RULES REFERENCED IN PART 97

0.121  Locations of field installations.

(a) and (b) reserved.

(c) Monitoring stations are located at the following

geographical  coordinates:

Station                   Latitude       Longitude

---------------------    -----------    ------------

Allegan, Michigan        42-36-20 N.     85-57-20 W.

Anchorage, Alaska        61-09-43 N.    149-59-55 W.

Belfast, Maine           44-26-42 N.     69-04-58 W.

Canandaigua, New York    42-54-48 N.     77-15-59 W.

Douglas, Arizona         31-30-02 N.    109-39-12 W.

Ferndale, Washington     48-57-21 N.    122-33-13 W.

Fort Lauderdale, Florida 26-06-08 N.     80-16-42 W.

Grand Island, Nebraska   40-55-21 N.     98-25-42 W.

Kingsville, Texas        27-26-29 N.     97-53-00 W.

Laurel, Maryland         39-09-54 N.     76-49-17 W.

Livermore, California    37-43-30 N.    121-45-12 W.

Powder Springs, Georgia  33-51-44 N.     84-43-26 W.

Sabana Seca, Puerto Rico 18-27-23 N.     66-13-37 W.

Waipahu, Hawaii          21-22-45 N.    157-59-54 W.

(Coordinates are given as "Degrees-Minutes-Seconds")

0.314  Additional authority delegated.

The Engineer in Charge at each installation is delegated

authority to act upon applications, requests, or other matters,

which are not in hearing status, and direct the following

activities necessary to conduct investigations or inspections:

.

.

.    (x) When deemed necessary by the Engineer-in-Charge of a

Commission field facility to assure compliance with the Rules, a

station licensee shall maintain a record of such operating and

maintenance records as may be necessary to resolve conditions of

interference or deficient technical operation.

1.1305  Actions which normally will have a significant impact upon

the environment, for which Environmental Impact Statements must be

prepared.

Any Commission action deemed to have a significant effect upon

the quality of the human environment requires the preparation of

a Draft Environmental Impact Statement (DEIS) and Final

Environmental Impact Statement (FEIS) (collectively referred to as

EISs) (see 1.1314, 1.1315 and 1.1317).  The Commission has reviewed

representative actions and has found no common pattern which would

enable it to specify actions that will thus automatically require

EISs.

Note - Our current application forms refer applicants to

1.1305 to determine if their proposals are such that the submission

of environmental information is required (see 1.1311).  Until the

application forms are revised to reflect our new environmental

rules, applicants should refer to 1.1307.  Section 1.1307 now

delineates those actions for which applicants must submit

environmental information.

1.1306  Actions which are categorically excluded from environmental

processing.

(a) Except as provided in 1.1307(c) and (d), Commission

actions not covered by 1.1307(a) and (b) are deemed individually

and cumulatively to have no significant effect on the quality of

the human environment and are categorically excluded from

environmental processing.

(b) Specifically, any Commission action with respect to any

new application, or minor or major modifications of existing or

authorized facilities or equipment, will be categorically excluded,

provided such proposals do not:

(1) Involve a site location specified under

1.1307(a)(1)-(5), or

(2) Involve high intensity lighting under 1.1307(a)(6).

(3) Results in human exposure to radiofrequency radiation

in excess of the applicable safety standards specified in

1.1307(b).

Note 1 - The provisions of 1.1307(a) do not encompass the

mounting of antennas on an existing building or antenna tower,

unless the antenna(s) to be mounted is (are) subject to the

provisions of 1.1307(b) and would result in human exposure to

radiofrequency radiation in excess of the applicable health and

safety guidelines cited in 1.1307(b).  Otherwise, the use of

existing buildings and towers is an environmentally desirable

alternative to the construction of new towers and is encouraged.

Note 2 - The specific height of an antenna tower or supporting

structure, as well as the specific diameter of a satellite earth

station, in and of itself, will not be deemed sufficient to warrant

environmental processing, see 1.1307 and 1.1308.

Note 3 - The construction of an antenna tower or supporting

structure in an established "antenna farm" (i.e., an area in which

similar antenna towers are clustered, whether or not such area has

been officially designated as an antenna farm), will be

categorically excluded unless one or more of the antennas to be

mounted on the tower or structure are subject to the provisions of

1.1307(b) and the additional radiofrequency radiation from the

antenna(s) on the new tower or structure would cause human exposure

in excess of the applicable health and safety guidelines cited in

1.1307(b).

1.1307  Actions which may have a significant environmental effect,

for which environmental assessments (EAs) must be prepared.

(a) Commission actions with respect to the following types of

facilities may significantly affect the environment and thus

require the preparation of EAs by the applicant (see 1.1308 and

1.1311) and may require further Commission environmental processing

(see 1.1314, 1.1315 and 1.1317):

(1) Facilities that are to be located in an officially

designated wilderness area.

(2) Facilities that are to be located in an officially

designated wildlife preserve.

(3) Facilities that will affect districts, sites,

buildings, structures or objects, significant in American history,

architecture, archeology or culture, that are listed in the

National Register of Historic Places or are eligible for listing.

(See 36 CFR paragraphs 60, 63 and 800.)   Note - The National

Register is updated and re-published in the Federal Register each

year in February.

(4) Facilities to be located in a floodplain (see

Executive Order 11990).

(5) Facilities whose construction will involve

significant change in surface features (e.g., wetland fill,

deforestation or water diversion).  (In the case of wetlands on

Federal property, see Executive Order 11990.)

(6) Antenna towers and/or supporting structures that are

to be equipped with high intensity white lights which are located

in residential neighborhoods, as defined by the applicable zoning

law.

(b) In addition to the actions listed in paragraph (a),

Commission actions granting construction permits, licenses to

transmit or renewals thereof, or Commission actions authorizing

modifications in existing facilities, will require the preparation

of an EA if the particular facility or operation would cause

exposure of workers or the general public to levels of

radiofrequency radiation in excess of the "Radio Frequency

Protection Guides" recommended in "American National Standard

Safety Levels with Respect to Human Exposure to Radio Frequency

Electromagnetic Fields, 300 kHz to 100 GHz," (ANSI C95-1-1982),

issued by the American National Standards Institute (ANSI), 1430

Broadway, New York, NY 10018. Copyright 1982 by the Institute of

Electrical and Electronics Engineers, Inc., 345 East 47th St., New

York, NY 10017.  Note - The provisions of paragraph (b) shall only

apply to facilities and services licensed or authorized under parts

5, 25, 73 and 74 (Subparts A and G only) of the FCC Rules and

Regulations.

(c) If an interested person alleges that a particular action,

otherwise categorically excluded, will have a significant

environmental effect, the person shall submit to the Bureau

responsible for processing that action a written petition setting

forth in detail the reasons justifying or circumstances

necessitating environmental consideration in the decision-making

process (see 1.1313). The Bureau shall review the petition and

consider the environmental concerns that have been raised.  If the

Bureau determines that the action may have a significant

environmental impact, the Bureau will require the applicant to

prepare an EA (see 1.1308 and 1.1311), which will serve as the

basis for the determination to proceed with or terminate

environmental processing.

(d) If the Bureau responsible for processing a particular

action, otherwise categorically excluded, determines that the

proposal may have a significant environmental impact, the Bureau,

on its own motion, shall require the applicant to submit an EA.

The Bureau will review and consider the EA as in paragraph (c) of

this section.

1.1308  Consideration of environmental assessments (EAs); findings

on No significant impact.

(a) Applicants shall prepare EAs for action that may have a

significant environmental impact (see 1.1307).  An EA is described

in detail in 1.1311 of this part of the Commission rules.

(b) The EA is a document which shall explain the environmental

consequences of the proposal and set forth sufficient analysis for

the Bureau or the Commission to reach a determination that the

proposal will or will not have a significant environmental effect.

To assist in making that determination, the Bureau or the

Commission may request further information from the applicant,

interested persons, and agencies and authorities which have

jurisdiction by law or which have relevant expertise.

(c) If the Bureau or the Commission determines, based on an

independent review of the EA, that the proposal will have a

significant environmental impact upon the quality of the human

environment, it will so inform the applicant.  The applicant will

then have an opportunity to amend its application so as to reduce,

minimize or eliminate environmental problems (see 1.1309).  If the

environmental problem is not eliminated, the Bureau will publish

in the Federal Register a Notice of Intent (see 1.1314) that EISs

will be prepared (see 1.1315 and 1.1317), or

(d) If the Bureau or Commission determines, based on an

independent review of the EA that the proposal would not have a

significant impact, it will make a finding of no significant

impact.  Therefore, the application will be processed without

further consideration of environmental effect.  Pursuant to CEQ

regulations (see 40 CFR 1501.4 and 1506.6), the applicant must

provide the community notice of the Commission's finding of no

significant impact.

1.1309  Application amendments.

Applicants are permitted to amend their applications to

reduce, minimize or eliminate potential environmental problems.

As a routine matter, an applicant will be permitted to amend its

application within thirty (30) days after the Commission or the

Bureau informs the applicant that the proposal will have a

significant impact upon the quality of the human environment (see

1.1308(c)).  The period of thirty (30) days may be extended upon

a showing of good cause.

1.1311  Environmental information to be included in the

environmental assessment (EA).

(a) The applicant shall submit an EA with each application

that is subject to environmental processing (see 1.1307).  The EA

shall contain the following information:

(1) For antenna towers and satellite earth stations, a

description of the facilities as well as supporting structures and

appurtenances, and a description of the site as well as the

surrounding area and uses.  If high intensity white lighting is

proposed or utilized within a residential area, the EA must also

address the impact of this lighting upon the residents.

(2) A statement as to the zoning classification of the

site, and communications with, or proceedings before and

determinations (if any) made by zoning, planning, environmental or

other local, state or federal authorities on matters relating to

environmental effect.

(3) A statement as to whether construction of the

facilities has been a source of controversy on environmental

grounds in the local community.

(4) A discussion of environmental and other

considerations which led to the selection of the particular site

and, if relevant, the particular facility; the nature and extent

of any unavoidable adverse environmental effects, and any

alternative sites or facilities which have been or might reasonably

be considered.

(5) Any other information that may be requested by the

Bureau or Commission.

(b) The information submitted in the EA shall be factual (not

argumentative or conclusory) and concise with sufficient detail to

explain the environmental consequences and to enable the Commission

or Bureau, after an independent review of the EA, to reach a

determination concerning the proposal's environmental impact, if

any. The EA shall deal specifically with any feature of the site

which has special environmental significance (e.g., wilderness

areas, wildlife preserves, natural migration paths for birds and

other wildlife, and sites of historic, architectural, or

archaeological value). In the case of historically significant

sites, it shall specify the effect of the facilities on any

district, site, building, structure or object listed or eligible

for listing, in the National Register of Historic Places.  It shall

also detail any substantial change in the character of the land

utilized (e.g., deforestation, water diversion, wetland fill, or

other extensive change of surface features).  In the case of

wilderness areas, wildlife preserves, or other like areas, the

statement shall discuss the effect of any continuing pattern of

human intrusion into the area (e.g., necessitated by the operation

and maintenance of the facilities).

(c) The EA shall also be accompanied with evidence of site

approval which has been obtained from local or federal land use

authorities.

(d) To the extent that such information is submitted in

another part of the application, it need not be duplicated in the

EA, but adequate cross-reference to such information shall be

supplied.

(e) An EA need not be submitted to the Commission if another

agency of the Federal Government has assumed responsibility for

determining whether the facilities in question will have a

significant effect on the quality of the human environment and, if

it will, for invoking the environmental impact statement process.

1.1312  Facilities for which no construction permit is required.

In the case of facilities for which no construction permit is

required or for which a waiver request for a construction permit

has been granted under 47 U.S.C. 319(d), the information required

by 1.1311 shall be submitted and ruled on by the Commission, and

the environmental processing (if invoked) shall be completed before

authorization of the facilities is granted.

1.1313  Objections.

(a) In the case of an application to which section 309(b) of

the Communications Act applies, objections based on environmental

considerations shall be filed as petitions to deny.

(b) Informal objections which are based on environmental

considerations must be filed prior to grant of the construction

permit, or prior to authorization for facilities that do not

require construction permits, or pursuant to the applicable rules

governing services subject to lotteries.

1.1314  Environmental impact statements (EISs).

(a) Draft Environmental Impact Statements (DEISs) (1.1315) and

Final Environmental Impact Statements (FEISs) (referred to

collectively as EISs)  (1.1317) shall be prepared by the Bureau

responsible for processing the proposal when the Commission's or

the Bureau's analysis of the EA (1.1308) indicates that the

proposal will have a significant effect upon the environment and

the matter has not been resolved by an amendment.

(b) As soon as practically feasible, the Bureau will publish

in the Federal Register a Notice of Intent to prepare EISs.  The

Notice shall briefly identify the proposal, concisely describe the

environmental issues and concerns presented by the subject

application and generally invite participation from affected or

involved agencies, authorities and other interested persons.

(c) The EISs shall not address non-environmental

considerations.  To safeguard against repetitive and unnecessarily

lengthy documents, the Statements, where feasible, shall

incorporate by reference material set forth in previous documents,

with only a brief summary of its content.  In preparing the EISs,

the Bureau will identify and address the significant environmental

issues and eliminate the insignificant issues from analysis.

(d) To assist in the preparation of the EISs, the Bureau may

request further information from the applicant, interested persons

and agencies and authorities, which have jurisdiction by law or

which have relevant expertise.  The Bureau may direct that

technical studies be made by the applicant and that the applicant

obtain expert opinion concerning the potential environmental

problems and costs associated with the proposed action, as well as

comparative analyses of alternatives.  The Bureau may also consult

experts in an effort to identify measures that could be taken to

minimize the adverse effects and alternatives to the proposed

facilities that are not, or are less, objectionable.  The Bureau

may also direct that objections be raised with appropriate local,

state or federal land use agencies or authorities (if their views

have not been previously sought).  Note - The Advisory Council of

Historic Preservation has adopted formal procedures for such

consultation.  See 36 CFR Part 800.

(e) The Bureau responsible for processing the particular

application and, thus, preparing the EISs shall draft supplements

to Statements where significant new circumstances occur or

information arises relevant to environmental concerns and bearing

upon the application.

(f) The application, the EA, the DEIS, and the FEIS and all

related documents, including the comments filed by the public and

any agency, shall be part of the administrative record and will be

routinely available for public inspection.

(g) If EISs are to be prepared, the applicant must provide the

community with notice of the availability of environmental

documents and the scheduling of any Commission hearings in that

action.

(h) The timing of agency action with respect to applications

subject to EISs is set forth in 40 CFR 1506.10.  No decision shall

be made until ninety (90) days after the Notice of Availability of

the Draft Environmental Impact Statement is published in the

Federal Register, and thirty (30) days after the Notice of

Availability of the Final Environmental Impact Statement is

published in the Federal Register, which time period may run

concurrently.  See 40 CFR 1506.10(c); see also 1.1315(b) and

1.1317(b).

(i) Guidance concerning preparation of the Draft and Final

Environmental Impact Statements is set out in 40 CFR Part 1502.

1.1315 The Draft Environmental Impact Statement (DEIS); Comments.

(a) The DEIS shall include:

(1) A concise description of the proposal, the nature of

the area affected, its uses, and any specific feature of the area

that has special environmental significance;

(2) An analysis of the proposal, and reasonable

alternatives exploring the important consequent advantages and/or

disadvantages of the action and indicating the direct and indirect

effects and their significance in terms of the short and long-term

uses of the human environment.

(b) When a DEIS and supplements, if any, are prepared, the

Commission shall send five copies of the Statement, or a summary,

to the Office of Federal Activities, Environmental Protection

Agency. Additional copies, or summaries will be sent to the

appropriate regional office of the Environmental Protection Agency.

Public Notice of the availability of the DEIS will be published in

the Federal Register by the Environmental Protection Agency.

(c) When copies or summaries of the DEIS are sent to the

Environmental Protection Agency, the copies or summaries will be

mailed with a request for comment to federal agencies having

jurisdiction by law or special expertise, to the Council on

Environmental Quality, to the applicant, to individuals, groups

and state and local agencies known to have an interest in the

environmental consequences of a grant, and to any other person who

has requested a copy.

(d) Any person or agency may comment on the DEIS and the

environmental effect of the proposal described therein within 45

days after notice of the availability of the statement is published

in the Federal Register. A copy of those comments shall be mailed

to the applicant by the person who files them pursuant to 47 CFR

1.47.  An original and one copy shall be filed with the Commission.

If a person submitting comments is especially qualified in any way

to comment on the environmental impact of the facilities, a

statement of his or her qualifications shall be set out in the

comments.  In addition, comments submitted by an agency shall

identify the person(s) who prepared them.

(e) The applicant may file reply comments within 15 days after

the time for filing comments has expired.  Reply comments shall be

filed with the Commission in the same manner as comments, and shall

be served by the applicant on persons or agencies which filed

comments.

(f) The preparation of a DEIS and the request for comments

shall not open the application to attack on other grounds.

1.1317  The Final Environmental Impact Statement (FEIS).

(a) After receipt of comments and reply comments, the Bureau

will prepare a FEIS, which shall include a summary of the comments,

and a response to the comments, and an analysis of the proposal in

terms of its environmental consequences, and any reasonable

alternatives, and recommendations, if any, and shall cite the

Commission's internal appeal procedures (see 47 CFR 1.101-1.120).

(b) The FEIS and any supplements will be distributed and

published in the same manner as specified in 1.1315.  Copies of the

comments and reply comments, or summaries thereof where the record

is voluminous, shall be attached to the FEIS.

1.1319  Consideration of the environmental impact statements.

(a) If the action is subject to a hearing:

(1) In rendering his initial decision, the Administrative

Law Judge shall utilize the FEIS in considering the environmental

issues. In a comparative context, the respective parties shall be

afforded the opportunity to comment on the FEIS, and the

Administrative Law Judge's decision shall contain an evaluation of

the respective applications based on environmental and

non-environmental public interest factors.

(2) Upon review of an initial decision, the Review Board

and/or the Commission will consider and assess all aspects of the

FEIS and will render its decision, giving due consideration to the

environmental and non-environmental issues.

(b) In all non-hearing matters, the Commission, as part of its

decision-making process, will review the FEIS, along with other

relevant issues, to ensure that the environmental effects are

specifically assessed and given comprehensive consideration.

2.106  Footnote US7.

In the band 420-450 MHz and within the following areas, the

peak envelope power output of a transmitter employed in the amateur

service shall not exceed 50 watts, unless expressly authorized by

the Commission after mutual agreement, on a case-by-case basis,

between the Federal Communications Commission Engineer in Charge

at the applicable District office and the Military Area Frequency

Coordinator at the applicable military base:

(a) Those portions of Texas and New Mexico bounded on the

south by latitude 31 degrees 45 minutes North, on the east by 104

degrees 0 minutes West, on the north by latitude 34 degrees 30

minutes North, and on the west by longitude 107 degrees 30 minutes

West;

(b) The entire State of Florida including the Key West area

and the areas enclosed within a 200-mile radius of Patrick Air

Force Base, Florida (latitude 28 degrees 21 minutes North,

longitude 80 degrees 43 minutes West), and within a 200 mile radius

of Eglin Air Force Base, Florida (latitude 30 degrees 30 minutes

North, longitude 86 degrees 30 minutes West);

(c) The entire State of Arizona;

(d) Those portions of California and Nevada south of latitude

37 degrees 10 minutes North, and the areas enclosed within a 200

mile radius of the Pacific Missile Test Center, Point Mugu,

California (latitude 34 degrees 9 minutes North, longitude 119

degrees 11 minutes West);

(e) In the State of Massachusetts within a 160-kilometer (100

mile) radius around locations at Otis Air Force Base, Massachusetts

(latitude 41 degrees 45 minutes North, longitude 70 degrees 32

minutes West);

(f) In the State of California within a 240-kilometer (150

mile) radius around locations at Beale Air Force Base, California

(latitude 39 degrees 8 minutes North, longitude 121 degrees 26

minutes West);

(g) In the State of Alaska within a 160-kilometer (100 mile)

radius of Clear, Alaska (latitude 64 degrees 17 minutes North,

longitude 149 degrees 10 minutes West). (The Military Area

Frequency Coordinator for this area is located at Elmendorf Air

Force Base, Alaska.)

(h) In the State of North Dakota within a 160-kilometer (100

mile) radius of Concrete, North Dakota (latitude 48 degrees 43

minutes North, longitude 97 degrees 54 minutes West). (The Military

Area Frequency Coordinator for this area can be contacted at: HQ

SAC/SXOE, Offutt Air Force Base, Nebraska 68113.)

(i) In the States of Alabama, Florida, Georgia and South

Carolina within a 200-kilometer (124 mile) radius of Warner Robins

Air Force Base, Georgia (latitude 32 degrees 38 minutes North, 83

degrees 35 minutes West).

(j) In the State of Texas within a 200-kilometer (124 mile)

radius of Goodfellow Air Force Base, Texas (latitude 31 degrees 25

minutes North, longitude 100 degrees 24 minutes West).

2.201  Emission, modulation and transmission characteristics.

The following system of designating emission, modulation and

transmission characteristics shall be employed.

(a) Emissions are designated according to their classification

and their necessary bandwidth.

(b) A minimum of three symbols are used to describe the basic

characteristics of radio waves. Emissions are classified and

symbolized according to the following characteristics:

(1) First symbol - type of modulation of the main

carrier;

(2) Second symbol - nature of signal(s) modulating the

main carrier;

(3) Third symbol - type of information to be transmitted.

Note - A fourth and fifth symbol are provided for additional

information and are shown in Appendix 6, part A of ITU Radio

Regulations.  Use of the fourth and fifth symbol is optional.

Therefore, the symbols may be used as described in Appendix 6, but

are not required by the Commission.

(c) First symbol - types of modulation of the main carrier:

(1) Emission of an unmodulated carrier - - - - - - - N

(2) Emission in which the main carrier is

amplitude-modulated (including cases where subcarriers are

angle-modulated):

- Double sideband  - - - - - - - - - - - - - - - - - A

- Single sideband, full carrier  - - - - - - - - - - H

- Single sideband, reduced or variable level

carrier  - - - - - - - - - - - - - - - - - - - - - - - - - - - R

- Single sideband, suppressed carrier  - - - - - - - J

- Independent sidebands  - - - - - - - - - - - - - - B

- Vestigal sideband  - - - - - - - - - - - - - - - - C

(3) Emission in which the main carrier is

angle-modulated:

- Frequency modulation - - - - - - - - - - - - - - - F

- Phase modulation - - - - - - - - - - - - - - - - - G

Note - Whenever frequency modulation (F) is indicated,

phase modulation (G) is also acceptable.

(4) Emission in which the main carrier is amplitude- and

angle-modulated either simultaneously or in a pre-established

sequence - - - - - - - - - - - - - - - - - - - - - - - - - - - D

(5) Emission of pulses (emissions where the main carrier

is directly modulated by a signal which has been coded into

quantized form (e.g., pulse code modulation) should be designated

under (2) or (3)):

- Sequence of unmodulated pulses - - - - - - - - - - P

- A sequence of pulses modulated in amplitude  - - - K

- A sequence of pulses modulated in width/duration - L

- A sequence of pulses modulated in position/phase - M

- A sequence of pulses in which the carrier is

angle-modulated during the period of the pulse - - - - - - - - Q

- A sequence of pulses which is a combination of the

foregoing or is produced by other means  - - - - - - - - - - - V

(6) Cases not covered above, in which an emission

consists of the main carrier modulated, either simultaneously or

in a pre-established sequence in a combination of two or more of

the following modes: amplitude, angle, pulse - - - - - - - - - W

(7) Cases not otherwise covered  - - - - - - - - - - N

(d) Second symbol - nature of signal(s) modulating the main

carrier:

(1) No modulating signal - - - - - - - - - - - - - - 0

(2) A single channel containing quantized or digital

information without the use of a modulating subcarrier, excluding

time-division multiplex  - - - - - - - - - - - - - - - - - - - 1

(3) A single channel containing quantized or digital

information with the use of a modulating subcarrier, time-division

multiplex  - - - - - - - - - - - - - - - - - - - - - - - - - - 2

(4) A single channel containing analog information - 3

(5) Two or more channels containing quantized or digital

information  - - - - - - - - - - - - - - - - - - - - - - - - - 7

(6) Two or more channels containing analog

information  - - - - - - - - - - - - - - - - - - - - - - - - - 8

(7) Composite system with one or more channels containing

quantized or digital information, together with one or more

channels containing analog information - - - - - - - - - - - - 9

(8) Cases not otherwise covered  - - - - - - - - - - X

(e) Third symbol - type of information to be transmitted:

(1) No information transmitted - - - - - - - - - - - N

(2) Telegraphy, for aural reception  - - - - - - - - A

(3) Telegraphy, for automatic reception  - - - - - - B

(4) Facsimile  - - - - - - - - - - - - - - - - - - - C

(5) Data transmission, telemetry, telecommand  - - - D

(6) Telephony (including sound broadcasting) - - - - E

(7) Television (video)   - - - - - - - - - - - - - - F

(8) Combination of the above - - - - - - - - - - - - W

(9) Cases not covered  - - - - - - - - - - - - - - - X

(f) Type B emission: As an exception to the above principles,

damped waves are symbolized in the Commission's rules and

regulations as type B emission. The use of type B emissions is

forbidden.

(g) Whenever the full designation of an emission is necessary,

the symbol for that emission, as given above, shall be preceded by

the necessary bandwidth of the emission as indicated in

2.202(b)(1).

2.202  Bandwidths.

(a) Occupied bandwidth: The frequency bandwidth such that,

below its lower and above its upper frequency limits, the mean

powers radiated are each equal to 0.5 percent of the total mean

power radiated by a given emission.  In some cases, for example

multi-channel frequency-division systems, the percentage of 0.5

percent may lead to difficulties in the practical application of

the definitions of occupied and necessary bandwidth; in such cases

a different percentage may prove useful.

(b) Necessary bandwidth: For a given class of emission, the

minimum value of the occupied bandwidth sufficient to ensure the

transmission of information at the rate and with the quality

required for the system employed, under specified conditions.

Emissions useful for the good functioning of the receiving

equipment as, for example, the emission corresponding to the

carrier or reduced carrier systems, shall be included in the

necessary bandwidth.

(1) The necessary bandwidth shall be expressed by three

numerals and one letter.  The letter occupies the position of the

decimal point and represents the unit of bandwidth.  The first

character shall be neither zero nor K, M or G.

(2) Necessary bandwidths:

- between 0.001 and 999 Hz shall be expressed in Hz

(letter H);

- between 1.00 and 999 kHz shall be expressed in kHz

(letter K);

- between 1.00 and 999 MHz shall be expressed in MHz

(letter M);

- between 1.00 and 999 GHz shall be expressed in GHz

(letter G).

(3) Examples:

-  0.002 Hz  =  H002

-  0.1 Hz    =  H100

-  25.3 Hz   =  25H3

-  400 Hz    =  400H

-  2.4 kHz   =  2K40

-  6 kHz     =  6K00

-  12.5 kHz  =  12K5

-  180.4 kHz =  180K

-  180.5 kHz =  181K

-  180.7 kHz =  181K

-  1.25 MHz  =  1M25

-  2 MHz     =  2M00

-  10 MHz    =  10M0

-  202 MHz   =  202M

-  5.65 GHz  =  5G65

(c), etc...rules on determining necessary bandwidth...

2.815  External radio frequency power amplifiers.

(a) As used in this Part, an external radio frequency power

amplifier is any device which,

(1) when used in conjunction with a radio transmitter as

a signal source is capable of amplification of that signal, and

(2) is not an internal part of a radio transmitter as

manufactured.

(b) After April 27, 1978, no person shall manufacture, sell

or lease, offer for sale or lease (including advertising for sale

or lease), or import, ship, or distribute for the purpose of

selling or leasing or offering for sale or lease, any external

radio frequency power amplifier or amplifier kit capable of

operation on any frequency or frequencies between 24 and 35 MHz.

(c) No person shall manufacture, sell or lease, offer for sale

or lease (including advertising for sale or lease), or import,

ship, or distribute for the purpose of selling or leasing or

offering for sale or lease, any external radio frequency power

amplifier or amplifier kit capable of operation on any frequency

or frequencies below 144 MHz unless the amplifier has received a

grant of type acceptance in accordance with Subpart J of this part

and Subpart C of Part 97 or other relevant Parts of this Chapter.

No more than 10 external radio frequency power amplifiers or

amplifier kits may be constructed for evaluation purposes in

preparation for the submission of an application for a grant of

type acceptance.

(d) The proscription in Paragraph (b) of this section shall

not apply to the marketing, as defined in that paragraph, by a

licensed amateur radio operator to another licensed amateur radio

operator of an external radio frequency power amplifier fabricated

in not more than one unit of the same model in a calendar year by

that operator provided the amplifier is for the amateur operator's

personal use at his licensed amateur radio station and the

requirements of Sections 97.75 and 97.76 * (outdated rules) of this

Chapter are met.

(e) The proscription in Paragraph (c) of this section shall

not apply to the marketing, as defined in that paragraph, by a

licensed amateur radio operator to another licensed amateur radio

operator of an external radio frequency power amplifier if the

amplifier is for the amateur operator's personal use at his

licensed amateur radio station and the requirements of Sections

97.75 and 97.76 * (outdated rules) of this Chapter are met.

73.682  Transmission standards.

(a) Transmission standards.

(1) The width of the television broadcast channel shall

be 6 MHz.

(2) The visual carrier frequency shall be nominally 1.25

MHz above the lower boundary of the channel.

(3) The aural center frequency shall be 4.5 MHz higher

than the visual carrier frequency.

(4) The visual transmission amplitude characteristic

shall be in accordance with the chart designated as Figure 5 of

Section 73.699; provided, however, that for stations operating on

Channel 15-69 and employing a transmitter with maximum peak visual

power output of 1 kilowatt or less the visual transmission

amplitude characteristic may be in accordance with the chart

designated as Figure 5(a) of Section 73.699.

(5) The chrominance subcarrier frequency is 63/88 times

precisely  5 MHz (3.57954545...MHz). The tolerance is plus or minus

10 Hz and the rate of frequency drift not to exceed 0.1 Hz per

second (cycles per second squared).

(6) For monochrome and color transmissions, the number

of scanning lines per frame shall be 525, interlaced two to one in

successive fields.  The horizontal scanning frequency shall be

2/455 times the chrominance subcarrier frequency; this corresponds

nominally to 15,750 hertz (with an actual value of 15,734.264 plus

or minus 0.044 hertz).  The vertical scanning frequency is 2/525

times the horizontal scanning frequency; this corresponds nominally

to 60 hertz (the actual value is 59.94 hertz).  For monochrome

transmissions only, the nominal values of line and field

frequencies may be used.

(7) The aspect ratio of the transmitted television

picture shall be 4 units horizontally to 3 units vertically.

(8) During active scanning intervals, the scene shall be

scanned from left to right horizontally and from top to bottom

vertically, at uniform velocities.

(9) A carrier shall be modulated within a single

television channel for both picture and synchronizing signals. The

two signals comprise different modulation ranges in amplitude in

accordance with the following:

(i) Monochrome transmissions shall comply with

synchronizing waveform specifications in Figure 7 of 73.699.

(ii) Color transmissions shall comply with the

synchronizing waveform specifications in Figure 6 of 73.699.

(iii) All stations operating on Channels 2 through

14 and those stations operating on Channels 15 through 69 licensed

for a peak visual transmitter output power greater than one

kilowatt shall comply with the picture transmission amplitude

characteristics shown in Figure 5 of 73.699.

(iv) Stations operating on Channels 15 through 69

licensed for a peak visual transmitter output power of one kilowatt

or less shall comply with the picture transmission amplitude

characteristics shown in Figure 5 or 5a of 73.699.

(10) A decrease in initial light intensity shall cause

an increase in radiated power (negative transmission).

(11) The reference black level shall be represented by

a definite carrier level, independent of light and shade in the

picture.

(12) The blanking level shall be transmitted at 75 plus

or minus 2.5 percent of the peak carrier level.

(13) The reference white level of the luminance signal

shall be 12.5 plus or minus 2.5 percent of the peak carrier level.

(14) (antenna polarization)

(15) The effective radiated power of the aural

transmitter must not exceed 22 percent of the peak radiated power

of the visual transmitter.

(16) The peak-to-peak radiation of transmitter output

within one frame of video signal due to all causes, including hum,

noise, and low-frequency response, measured at both scanning

synchronizing peak and blanking level, shall not exceed 5 percent

of the average scanning synchronizing peak signal amplitude.  This

provision is subject to change but is considered the best practice

under the present state of the art (1977).  It will not be enforced

pending a further determination thereof.

(17) The reference black level shall be separated from

the blanking level by the setup interval, which shall be 7.5 plus

or minus 2.5 percent of the video range from blanking level to the

reference white level.

(18) (chrominance equations, etc.)