SUBPART A - GENERAL PROVISIONS

97.1  Basis and purpose.

The rules and regulations in this Part are designed to provide

an amateur radio service having a fundamental purpose as expressed

in the following principles:

(a) Recognition and enhancement of the value of the amateur

service to the public as a voluntary noncommercial communication

service, particularly with respect to providing emergency

communications.

(b) Continuation and extension of the amateur's proven ability

to contribute to the advancement of the radio art.

(c) Encouragement and improvement of the amateur service

through rules which provide for advancing skills in both the

communications and technical phases of the art.

(d) Expansion of the existing reservoir within the amateur

radio service of trained operators, technicians, and electronics

experts.

(e) Continuation and extension of the amateur's unique ability

to enhance international goodwill.

97.3  Definitions.

(a) The definitions of terms used in Part 97 are:

(1) AMATEUR OPERATOR.  A person holding a written

authorization to be the control operator of an amateur station.

(2) AMATEUR RADIO SERVICES.  The amateur service, the

amateur-satellite service and the radio amateur civil emergency

service.

(3) AMATEUR-SATELLITE SERVICE.  A radiocommunication

service using stations on Earth satellites for the same purpose as

those of the amateur service.

(4) AMATEUR SERVICE.  A radiocommunication service for

the purpose of self-training, intercommunication and technical

investigations carried out by amateurs, that is, duly authorized

persons interested in radio technique solely with a personal aim

and without pecuniary interest.

(5) AMATEUR STATION.  A station in an amateur radio

service consisting of the apparatus necessary for carrying on

radiocommunications.

(6) AUTOMATIC CONTROL.  The use of devices and procedures

for control of a station when it is transmitting so that compliance

with the FCC Rules is achieved without the control operator being

present at a control point.

(7) AUXILIARY STATION.  An amateur station transmitting

communications point-to-point within a system of cooperating

amateur stations.

(8) BANDWIDTH.  The width of a frequency band outside of

which the mean power of the total (*) emission is attenuated at

least 26 dB below the mean power of the total emission, including

allowances for transmitter drift or Doppler shift.  (* This may be

a typographical error in the Federal Register; rather than "the

total," it probably should read "any" or "an.")

(9) BEACON.  An amateur station transmitting

communications for the purposes of observation of propagation and

reception or other related experimental activities.

(10) BROADCASTING.  Transmissions intended for reception

by the general public, either direct or relayed.

(11) CONTROL OPERATOR.  An amateur operator designated

by the licensee of a station to be responsible for the

transmissions from that station to assure compliance with the FCC

Rules.

(12) CONTROL POINT.  The location at which the control

operator function is performed.

(13) CSCE.  Certificate of successful completion of an

examination.

(14) EARTH STATION.  An amateur station located on, or

within 50 km of the Earth's surface intended for communications

with space stations or with other Earth stations by means of one

or more other objects in space.

(15) EIC.  Engineer in Charge of an FCC Field Facility.

(16) EXTERNAL RF POWER AMPLIFIER.  A device capable of

increasing power output when used in conjunction with, but not an

integral part of, a transmitter.

(17) EXTERNAL RF POWER AMPLIFIER KIT.  A number of

electronic parts, which, when assembled, is an external RF power

amplifier, even if additional parts are required to complete

assembly.

(18) FAA.  Federal Aviation Administration.

(19) FCC.  Federal Communications Commission.

(20) FREQUENCY COORDINATOR.  An entity, recognized in a

local or regional area by amateur operators whose stations are

eligible to be auxiliary or repeater stations, that recommends

transmit/receive channels and associated operating and technical

parameters for such stations in order to avoid or minimize

potential interference.

(21) HARMFUL INTERFERENCE.  Interference which endangers

the functioning of a radionavigation service or of other safety

services or seriously degrades, obstructs or repeatedly interrupts

a radiocommunication service operating in accordance with the Radio

Regulations.

(22) INDICATOR.  Words, letters or numerals appended to

and separated from the call sign during the station identification.

(23) INFORMATION BULLETIN.  A message directed only to

amateur operators consisting solely of subject matter of direct

interest to the amateur service.

(24) INTERNATIONAL MORSE CODE.  A dot-dash code as

defined in International Telegraph and Telephone Consultative

Committee (CCITT) Recommendation F.1 (1984), Division B, I. Morse

Code.

(25) ITU.  International Telecommunication Union.

(26) LINE A.  Begins at Aberdeen, WA, running by great

circle arc to the intersection of 48 degrees N, 120 degrees W,

thence along parallel 48 degrees N, to the intersection of 95

degrees W, thence by great circle arc through the southernmost

point of Duluth, MN, thence by great circle arc to 45 degrees N,

85 degrees W, thence southward along meridian 85 degrees W, to its

intersection with parallel 41 degrees N, thence along parallel 41

degrees N, to its intersection with meridian 82 degrees W, thence

by great circle arc through the southernmost point of Bangor, ME,

thence by great circle arc through the southernmost point of

Searsport, ME, at which point it terminates.

(27) LOCAL CONTROL.  The use of a control operator who

directly manipulates the operating adjustments in the station to

achieve compliance with the FCC Rules.

(28) NATIONAL RADIO QUIET ZONE.  The area in Maryland,

Virginia and West Virginia bounded by 39 degrees 15 minutes N on

the north, 78 degrees 30 minutes W on the east, 37 degrees 30

minutes N on the south and 80 degrees 30 minutes W on the west.

(29) QUESTION POOL.  All current examination questions

for a designated written examination element.

(30) QUESTION SET.  A series of examination (*) on a

given examination selected from the question pool.  (* Spelling is

correct.)

(31) RADIO REGULATIONS.  The latest ITU "Radio

Regulations" to which the United States is a party.

(32) RACES (radio amateur civil emergency service).  A

radio service using amateur stations for civil defense

communications during periods of local, regional or national civil

emergencies.

(33) REMOTE CONTROL.  The use of a control operator who

indirectly manipulates the operating adjustments in the station

through a control link to achieve compliance with the FCC Rules.

(34) REPEATER.  An amateur station that automatically

retransmits the signals of other stations.

(35) SPACE STATION.  An amateur station located more then

50 km above the Earth's surface.

(36) SPURIOUS EMISSION.  An emission, or frequencies

outside the necessary bandwidth of a transmission, the level of

which may be reduced without affecting the information being

transmitted.

(37) TELECOMMAND STATION.  An amateur station that

transmits communications to initiate, modify or terminate functions

of a space station.

(38) THIRD PARTY COMMUNICATIONS.  A message from the

control operator (first party) of an amateur station to another

amateur station control operator (second party) on behalf of

another person (third party).

(39) VE.  Volunteer examiner.

(40) VEC.  Volunteer-examiner coordinator.

(b) The definitions of technical symbols used in this Part

are:

(1) EHF (extremely high frequency).  The frequency range

30-300 GHz.

(2) HF (high frequency).  The frequency range 3-30 MHz.

(3) Hz.  Hertz.

(4) m.  Meters.

(5) MF (medium frequency).  The frequency range 300-3000

kHz.

(6) PEP (peak envelope power).  The average power

supplied to the antenna transmission line by a transmitter during

one RF cycle at the crest of the modulation envelope taken under

normal operating conditions.

(7) RF.  Radio frequency.

(8) SHF (super-high frequency).  The frequency range 3-30

GHz.

(9) UHF (ultra-high frequency).  The frequency range

300-3000  MHz.

(10) VHF (very-high frequency).  The frequency range

30-300 MHz.

(11) W.  Watts.

(c) The following terms are used in this Part to indicate

emission types.  Refer to Section 2.201 of the FCC Rules,

"Emission, modulation and transmission characteristics," for

information on emission type designators.

(1) CW.  International Morse code telegraphy emissions

having designators with A, C, H, J or R as the first symbol; 1 as

the second symbol; A or B as the third symbol; and emissions J2A

and J2B.

(2) DATA.  Telemetry, telecommand and computer

communications emissions having designators with A, C, D, F, G, H,

J or R as the first symbol; 1 as the second symbol; D as the third

symbol; and emission J2D.  Only a digital code of a type

specifically authorized in this Part may be transmitted.

(3) IMAGE.  Facsimile and television emissions having

designators with A, C, D, F, G, H, J or R as the first symbol; 1,

2 or 3 as the second symbol; C or F as the third symbol; and

emissions having B as the first symbol; 7, 8 or 9 as the second

symbol; W as the third symbol.

(4) MCW.  Tone-modulated international Morse code

telegraphy emissions having designators with A, C, D, F, G, H, J

or R as the first symbol; 2 as the second symbol; A or B as the

third symbol.

(5) PHONE.  Speech and other sound emissions having

designators with A, C, D, F, G, H, J or R as the first symbol; 1,

2 or 3 as the second symbol; E as the third symbol.  Also speech

emissions having B as the first symbol; 7, 8 or 9 as the second

symbol; E as the third symbol.  MCW for the purpose of performing

the station identification procedure, or for providing telegraphy

practice interspersed with speech.  Incidental tones for the

purpose of selective calling or alerting or to control the level

of a demodulated signal may also be considered phone.

(6) PULSE.  Emissions having designators with K, L, M,

P, Q, V or W as the first symbol; 0, 1, 2, 3, 7, 8, 9 or X as the

second symbol; A, B, C, D, E, F, N, W or X as the third symbol.

(7) RTTY.  Narrow-band direct-printing telegraphy

emissions having designators with A, C, D, F, G, H, J or R as the

first symbol; 1 as the second symbol; B as the third symbol; and

emission J2B.  Only a digital code of a type specifically

authorized in this part may be transmitted.

(8) SS.  Spread-spectrum emissions using

bandwidth-expansion modulation emissions having designators with

A, C, D, F, G, H, J or R as the first symbol; X as the second

symbol; X as the third symbol.  Only a SS emission of a type

specifically authorized in this Part may be transmitted.

(9) TEST.  Emissions containing no information having the

designators with N as the third symbol.  Test does not include

pulse emissions with no information or modulation unless pulse

emissions are also authorized in the frequency band.

97.5  Station license required.

(a) When a station is transmitting on any amateur service

frequency from a geographic location within 50 km of the Earth's

surface where the amateur service is regulated by the FCC, the

person having physical control of the apparatus must hold an

FCC-issued written authorization for an amateur station.

(b) When a station is transmitting on any amateur service

frequency from a location within 50 km of the Earth's surface and

aboard any vessel or craft that is documented or registered in the

United States, the person having physical control of the apparatus

must hold an FCC-issued written authorization for an amateur

station.

(c) When a station is transmitting on any amateur service

frequency from a location more than 50 km above the Earth's surface

aboard any craft that is documented or registered in the United

States, the person having physical control of the apparatus must

hold an FCC-issued written authorization for an amateur station.

(d) The types of written authorizations that permit amateur

station operation where the amateur service is regulated by the

FCC are:

(1) An operator/primary station license (FCC Form 660)

issued to the person by the FCC.  A primary amateur station license

is issued only to a person, together with an operator license on

the same document.  Every amateur operator licensed by the FCC must

have one, but only one, primary station license.  Except for a

representative of a foreign government, any person who qualifies

by examination is eligible to apply for an operator/primary station

license.

(2) A club station license (FCC form 660) issued to the

person by the FCC.  A club station license is issued only to the

person who is the license trustee designated by an officer of the

club.  The trustee must hold an FCC-issued Amateur Extra, Advanced,

General, or Technician operator license.  The club must be composed

of at least two persons and must have a name, a document of

organization, management and a primary purpose devoted to amateur

service activities consistent with this Part.

(3) A military recreation station license (FCC Form 660)

issued to the person by the FCC.  A military recreation station

license is issued only to the person who is the license custodian

designated by the official in charge of the United States military

recreational premises where the station is situated.  The custodian

must not be a representative of a foreign government.  The

custodian need not hold an amateur operator license.

(4) A RACES station license (FCC Form 660) issued to the

person by the FCC.  A RACES station license is issued only to the

person who is the license custodian designated by the official

responsible for the governmental agency served by that civil

defense organization.  The custodian must not be a representative

of a foreign government.  The custodian must be the civil defense

official responsible for coordination of all civil defense

activities in the area concerned.  The custodian need not hold an

amateur operator license.

(5) A reciprocal permit for alien amateur licensee (FCC

Form 610-AL) issued to the person by the FCC.  A reciprocal permit

for alien amateur licensee is issued only to a person who is a

citizen of a country with which the United States has arrangements

to grant reciprocal operating permits to visiting alien amateur

operators.  The person must be a citizen of the same country that

issued the amateur service license.  No person who is a citizen of

the United States, regardless of any other citizenship also held,

is eligible for a reciprocal permit for alien amateur licensee.

No person holding an FCC-issued amateur service license is eligible

for a reciprocal permit for alien amateur license.*

(* "license" should likely read "licensee," as elsewhere in the

paragraph.)

(6) An amateur service license issued to the person by

the Government of Canada.  The person must be a Canadian citizen.

(e) The written authorization for an amateur station

authorizes the use in accordance with FCC Rules of all transmitting

apparatus under the physical control of the station licensee at

points where the amateur radio service is regulated by the FCC.

The original written authorization document or a photocopy thereof

must be retained at the station.

97.7  Control operator required.

When transmitting, each amateur station must have a control

operator.  Only a person holding one of the following documents may

be the control operator of a station:

(a) An amateur operator/primary station license (FCC Form 660)

issued to the person by the FCC.

(b) A reciprocal permit for alien amateur licensee (FCC Form

610-AL) issued to the person by the FCC.

(c) An amateur service license issued to a Canadian citizen

by the Government of Canada.

97.9  Operator license.

(a) There are 5 classes of operator licenses: Novice,

Technician, General, Advanced and Amateur Extra.  An operator

license authorizes the holder to be the control operator of a

station with the privileges of the operator class specified on the

license.  The license document or a photocopy thereof must be in

the personal possession of the licensee at all times when the

person is the control operator of a station.

(b) A person holding a Novice, Technician, General, or

Advanced operator license who has properly filed with the FCC an

application for a higher operator class which has not yet been

acted upon, and who holds a CSCE indicating that the person

completed the necessary examinations within the previous 365 days

is authorized to exercise the rights and privileges of the higher

operator class.

97.11  Stations aboard ships or aircraft.

(a) The installation and operation of an amateur station on

a ship or aircraft must be approved by the master of the ship or

pilot in command of the aircraft.

(b) The station must be separate from and independent of all

other radio apparatus installed on the ship or aircraft, except a

common antenna may be shared with a voluntary ship radio

installation.  The station's transmissions must not cause

interference to any other apparatus installed on the ship or

aircraft.

(c) The station must not constitute a hazard to the safety of

life or property.  For a station aboard an aircraft, the apparatus

shall not be operated while the aircraft is operating under

Instrument Flight Rules, as defined by the FAA, unless the station

has been found to comply with all applicable FAA rules.

97.13  Restrictions on station location.

(a) Before placing an amateur station on land of environmental

importance or that is significant in American history, architecture

or culture, the licensee may be required to take certain actions

prescribed by Sections 1.1305 - 1.1319 of the FCC Rules.

(b) An amateur station within 1600 m (1 mile) of an FCC

monitoring facility must protect that facility from harmful

interference.  Failure to do so could result in imposition of

operating restrictions upon the amateur station by the EIC pursuant

to Section 97.221 of this Part.  Geographical coordinates of the

facilities that require protection are listed in Section 0.121(c)

of the FCC Rules.

97.15  Station antenna structures.

(a) Unless the amateur station licensee has received prior

approval from the FCC, no antenna structure, including and (*)

radiating elements, tower, supports and all appurtenances, may be

higher than 61 m (200 feet) above ground level at its site.

(b) Unless the amateur station licensee has received prior

approval from the FCC, no antenna structure, at an airport or

heliport that is available for public use and is listed in the

"Airport Directory" of the current "Airman's Information Manual"

or in either the "Alaska" or "Pacific Airman's Guide and Chart

Supplement;" or at an airport or heliport under construction that

is the subject of a notice or proposal on file with the FAA, and

except for military airports, it is clearly indicated that the

airport will be available for public use; or at an airport or

heliport that is operated by the armed forces of the United States;

or at a place near any of these airports or heliports, may be

higher than:

(1) 1 m above the airport elevation for each 100 m from

the nearest runway longer than 1 km within 6.1 km of the antenna

structure.

(2) 2 m above the airport elevation for each 100 m from

the nearest runway longer than 1 km within 3.1 km of the antenna

structure.

(3) 4 m above the airport elevation for each 100 m from

the nearest landing pad within 1.5 km of the antenna structure.

(c) An amateur station antenna structure no higher than 6.1

m (20 feet) above ground level at its site or no higher than 6.1

m above any natural object or existing manmade structure, other

than an antenna structure, is exempt from the requirements of

paragraphs (a) and (b) of this section.

(d) Further details as to whether an aeronautical study and/or

obstruction marking and lighting may be required, and

specifications for obstruction marking and lighting, are contained

in Part 17 of the FCC Rules, "Construction, Marking, and Lighting

of Antenna Structures." To request approval to place an antenna

structure higher than the limits specified in paragraphs (a), (b),

and (c) of this section, the licensee must notify the FAA on FAA

Form 7460-1 and the FCC on FCC Form 854.

(e) Except as otherwise provided herein, a station antenna

structure may be erected at heights and dimensions sufficient to

accommodate amateur service communications.  State and local

regulation of a station antenna structure must not preclude amateur

service communications.  Rather, it must reasonably accommodate

such communications and must constitute the minimum practicable

regulation to accomplish the state or local authority's legitimate

purpose.  [See PRB-1, 101 FCC 2d 952 (1985) for details.]

97.17  Application for new license.

(a) Any qualified person is eligible to apply for an amateur

service license.

(b) Each application for a new operator/primary station

license must be made on FCC Form 610.  Each application for a

reciprocal permit for alien amateur licensee must be made on FCC

Form 610-A.  No new license for a club, military recreation, or

RACES station will be issued.

(c) Each application for a new operator/primary station

license and each application involving a change in operator class

must be submitted to the VEs administering the qualifying

examination.

(d) Any qualified person is eligible to apply for a reciprocal

permit for alien amateur licensee.  The application must be

submitted to the FCC, P.O. Box 1020, Gettysburg, PA 17326.

(e) No person shall obtain or attempt to obtain, or assist

another person to obtain or attempt to obtain, an operator license

or reciprocal permit for alien amateur licensee by fraudulent

means.

(f) A call sign will be assigned systematically to each

station.  The FCC will issue public announcements detailing the

policies and procedures of the call sign assignment system.  The

FCC will not grant any request for a specific call sign.

97.19  Application for a renewed or modified license.

(a) Each application for a renewed or modified

operator/primary station license must be made on FCC Form 610.

Each application for a renewed or modified club, military

recreation or RACES station license must be made on FCC Form 610-B.

A reciprocal permit for alien amateur licensee is not renewable.

A new reciprocal permit may be issued upon proper application.

(b) Each application for a renewed or modified amateur service

license must be accompanied by a photocopy of the license document

or the original document.  Each application for a modified operator

license involving a change in operator class must be submitted to

the VEs administering the qualifying examination.  All other

applications must be submitted to: FCC, P.O. Box 1020, Gettysburg,

PA 17326.

(c) When the licensee has submitted a timely application for

renewal of an unexpired license (between 60 and 90 days prior to

the end of the license term is recommended), the licensee may

continue to operate until the disposition of the application has

been determined.  If a license expires, application for renewal

may be made during a grace period of 2 years after the expiration

date.  During this grace period, the expired license is not valid.

A license renewed during the grace period must be dated as of the

date of the renewal.

97.21  Mailing address and station location.

Each application for an amateur service license and each

application for a reciprocal permit for alien amateur licensee must

show a mailing address and a station location (the addresses may

be the same) in an area where the amateur service is regulated by

the FCC.  The mailing address must be one where the licensee can

receive mail delivery by the United States Postal Service.  The

station location must be a place where a station can be physically

located.  (A Postal Service box, RFD number, or general delivery

is unsuitable as a station location.)

97.23  License term.

(a) An amateur service license is normally issued for a

10-year term.

(b) A reciprocal permit for alien amateur license is normally

issued for a 1-year term.

97.25  FCC modification of station license.

(a) The FCC may modify an amateur station license, either for

a limited time or for the duration of the term thereof, if it

determines:

(1) That such action will promote the public interest,

convenience and necessity; or

(2) That such action will promote fuller compliance with

the provisions of the Communications Act of 1934, as amended, or

of any treaty ratified by the United States.

(b) When the FCC makes such a determination, it will issue an

order of modification.  The order will not become final until the

licensee is notified in writing of the proposed action and the

grounds and reasons therefor.  The licensee will be given

reasonable opportunity of no less than thirty days to protest the

modification; except that, where safety of life or property is

involved, a shorter period of notice may be provided.  Any protest

by a licensee of an FCC order of modification will be handled in

accordance with the provisions of 47 U.S.C. 316.

97.27  Replacement license.

Each licensee or permittee whose original document is lost,

mutilated or destroyed must request a replacement.  The request

must be made to: FCC, P.O. Box 1020, Gettysburg, PA 17236.  A

statement of how the document was lost, mutilated or destroyed must

be attached to the request.  A replacement license must bear the

same expiration date as the license that it replaces.

END OF A - !!!