SUBPART C - SPECIAL OPERATIONS

97.201  Auxiliary operation.

(a) Any amateur station licensed to a holder of a Technician,

General, Advanced or Amateur Extra Class operator license may be

an auxiliary station.  A holder of a Technician, General, Advanced

or Amateur Extra Class operator license may be the control operator

of an auxiliary station, subject to the privileges of the class of

operator license held.

(b) An auxiliary station may transmit only on the 1.25 m and

shorter wavelength bands, except the 220.0-220.5 MHz segment and

the UHF 431-433 MHz and 435-438 MHz segments.

(c) Where an auxiliary station causes harmful interference to

another auxiliary station, the two stations are equally and fully

responsible for resolving the interference unless one station's

operation is recommended by a frequency coordinator and the other

station's is not.  In that case, the licensee of the

non-coordinated auxiliary station has primary responsibility to

resolve the interference.

(d) An auxiliary station operation may be automatically

controlled only when it is part of a system that includes a

repeater station also being automatically controlled.

(f) An auxiliary station may transmit one-way communications.

97.203  Beacon station.

(a) Any amateur station licensed to a holder of a Technician,

General, Advanced or Amateur Extra Class operator license may be

a beacon.  A holder of a Technician, General, Advanced or Amateur

Extra Class operator license may be the control operator of a

beacon, subject to the privileges of the class of operator license

held.

(b) A beacon station must not concurrently transmit on more

than 1 channel in the same amateur service frequency band, from

the same station location.

(c) The transmitter power of a beacon must not exceed 100 W.

(d) A beacon may be automatically controlled when it is

transmitting on the 28.20-28.30 MHz, 50.06-50.08 MHz, 144.05-144.06

MHz, 220.05-220.06 MHz, 222.05-222.06 or 432.07-432.08 MHz

segments, or on the 33 cm and shorter wavelength bands.

(e) Before establishing an automatically controlled beacon in

the National Radio Quiet Zone or before changing the transmitting

frequency, transmitter power, antenna height or directivity, the

station licensee must give written notification thereof to the

Interference Office, National Radio Astronomy Observatory, P.O.

Box 2, Green Bank, WV 24994.

(1) The notification must include the geographical

coordinates of the antenna, antenna ground elevation above mean sea

level (AMSL), antenna center of radiation above ground level (AGL),

antenna directivity, proposed frequency, type of emission, and

transmitter power.

(2) If an objection to the proposed operation is received

by the FCC from the National Radio Astronomy Observatory at Green

Bank, Pocahontas County, WV, for itself or on behalf of the Naval

Research Laboratory at Sugar Grove, Pendleton County, WV, within

20 days from the date of notification, the FCC will consider all

aspects of the problem and take whatever action is deemed

appropriate.

(f) A beacon must cease transmissions upon notification by an

EIC that the station is operating improperly or causing undue

interference to other operations.  The beacon may not resume

transmitting without prior approval of the EIC.

(g) A beacon may transmit one-way communications.

97.205  Repeater station.

(a) Any amateur station licensed to a holder of a Technician,

General, Advanced or Amateur Extra Class operator license may be

a repeater.  A holder of a Technician, General, Advanced or Amateur

Extra Class operator license may be the control operator of a

repeater, subject to the privileges of the class of operator

license held.

(b) A repeater may receive and retransmit only on the 10 m and

shorter wavelength frequency bands except the 28.0-29.5 MHz,

50.0-52.0 MHz, 144.0-144.5 MHz, 145.5-146.0 MHz, 220.0-220.5 MHz,

431.0-433.0 MHz and 435.0-438.0 MHz segments.

(c) Where the transmissions of a repeater cause harmful

interference to another repeater, the two station licensees are

equally and fully responsible for resolving the interference unless

the operation of one station is recommended by a frequency

coordinator and the operation of the other station is not.  In that

case, the licensee of the non-coordinated repeater has primary

responsibility to resolve the interference.

(d) A repeater may be automatically controlled.

(e) Ancillary functions of a repeater that are available to

users on the input channel are not considered remotely controlled

functions of the station.  Limiting the use of a repeater to only

certain user stations is permissible.

(f) Before establishing a repeater in the National Radio Quiet

Zone or before changing the transmitting frequency, transmitter

power, antenna height or directivity, or the location of an

existing repeater, the station licensee must give written

notification thereof to the Interference Office, National Radio

Astronomy Observatory, P.O. Box 2, Green Bank, WV 24994.

(1) The notification must include the geographical

coordinates of the antenna, antenna ground elevation above mean sea

level (AMSL), antenna center of radiation above ground level (AGL),

antenna directivity, proposed frequency, type of emission, and

transmitter power.

(2) If an objection to the proposed operation is received

by the FCC from the National Radio Astronomy Observatory at Green

Bank, Pocahontas County, WV, for itself or on behalf of the Naval

Research Laboratory at Sugar Grove, Pendleton County, WV, within

20 days from the date of notification, the FCC will consider all

aspects of the problem and take whatever action is deemed

appropriate.

97.207  Space station.

(a) Any amateur station licensed to a holder of an Amateur

Extra Class operator license may be a space station.  A holder of

any class operator license may be the control operator of a space

station, subject to the privileges of the class of operator license

held by the control operator.

(b) A space station must be capable of effecting a cessation

of transmissions by telecommand whenever such cessation is ordered

by the FCC.

(c) The following frequency bands and segments are authorized

to space stations:

(1) The 15 m, 12 m, 10 m, 6 mm, 4 mm, 2 mm, and 1 mm

bands; and

(2) The 7.0-7.1 MHz, 14.00-14.25 MHz, 144-146 MHz,

2400-2450 MHz, 3.40-3.41 GHz, 5.83-5.85 GHz, 10.45-10.50 GHz and

24.00-24.05 GHz segments.

(d) A station in space operation may automatically retransmit

the radio signals of Earth stations and other space stations.

(e) A space station may transmit one-way communications.

(f) Results of measurements made in the space station,

including those related to the function of the station, transmitted

by a space station may consist of specially coded messages intended

to facilitate communications.

(g) The licensee of each space station must give two written,

pre-space station notifications to the Private Radio Bureau, FCC,

Washington, DC  20554.  Each notification must be in accord with

the provisions of Articles 11 and 13 of the Radio Regulations.

(1) The first notification is required no less than 27

months prior to initiating space station transmissions and must

specify the information required by Appendix 4, and Resolution No.

642 of the Radio Regulations.

(2) The second notification is required no less than 5

months prior to initiating space station transmissions and must

specify the information required by Appendix 3 and Resolution No.

642 of the Radio Regulations.

(h) The licensee of each space station must give a written,

in-space station notification to the Private Radio Bureau, FCC,

Washington, DC  20554, no later than 7 days following initiation

of space station transmissions.  The notification must update the

information contained in the pre-space notification.

(i) The licensee of each space station must give a written,

post-space station notification to the Private Radio Bureau, FCC,

Washington, DC  20554, no later than 3 months after termination of

the space station transmissions.  When the termination is ordered

by the FCC, notification is required no later than 24 hours after

termination.

97.209  Earth station.

(a) Any amateur station may be an Earth station.  A holder of

any class operator license may be the control operator of an Earth

station, subject to the privileges of the class of operator license

held by the control operator.

(b) The following frequency bands and segments are authorized

to Earth stations:

(1) The 15 m, 12 m, 10 m, 6 mm, 4 mm, 2 mm and 1 mm bands; and

(2) The 7.0-7.1 MHz, 14.00-14.25 MHz, 144-146 MHz, 435-438

MHz, 1260-1270 MHz, 2400-2450 MHz, 3.40-3.41 GHz, 5.65-5.67 GHz,

10.45-10.50 GHz and 24.00-24.05 GHz segments.

97.211  Telecommand station.

(a) Any amateur station designated by the licensee of a space

station is eligible to transmit as a telecommand station, subject

to the privileges of the class of operator license held by the

control operator.

(b) A telecommand station may transmit special codes intended

to obscure the meaning of telecommand messages to the station in

space operation.

(c) The following frequency bands and segments are authorized

to telecommand stations:

(1) The 15 m, 12 m and 10 m bands; 6 mm, 4 mm, 2 mm and

1 mm bands; and

(2) The 7.0-7.1 MHz, 14.00-14.25 MHz, 144-146 MHz,

435-438 MHz, 1260-1270 MHz and 2400-2450 MHz, 3.40-3.41 GHz,

5.65-5.67 GHz, 10.45-10.50 GHz and 24.00-24.05 GHz segments.

(d) A telecommand station may transmit one-way communications.

97.213  Remote control of a station.

An amateur station may be remotely controlled where:

(a) There is a radio or wireline control link between the

control point and the station sufficient for the control operator

to perform his/her duties.  If radio, the control link must use an

auxiliary station.  A control link using a fiber optic cable or

another telecommunication service is considered wireline.

(b) Provisions are incorporated to limit transmission by the

station to a period of no more than 3 minutes in the event of

malfunction in the control link.

(c) The station is protected against making, willfully or

negligently, unauthorized transmissions.

(d) A photocopy of the station license and the name, address,

and telephone number of the station licensee and at least one

designated control operator is posted in a conspicuous place at

the station location.

97.215  Remote control of model craft.

An amateur station transmitting signals to control a model

craft may be operated as follows:

(a) The station identification procedure is not required for

transmissions directed only to the model craft, provided that a

label indicating the station call sign and the station licensee's

name and address is affixed to the station transmitter.

(b) The control signals are not considered codes or ciphers

intended to obscure the meaning of the communication.

(c) The transmitter power must not exceed 1 W.