900227.23:07 ral hfmagic1.doc

HF MAGIC VERSION 1.0 DOCUMENTATION

1.  INTRODUCTION

HF MAGIC 1.0 is an HF propagation prediction program which is

different from those available from other sources today.  This program

is designed for use by the amateur or shortwave listener who wants to

maximize communications over "practical" HF distances, generally up to

2500 miles in length.  It not only presents the maximum usable

frequency (MUF), which is only part of the story, but computes the

received signal strength in "S" units including the very important

contribution of "D region absorption".  This factor is often missing

from propagation models, but is very important in determining total

signal strength during the daytime hours.  The program is based on

MINIMUF and NBS absorption expressions.

The outstanding features of this program include very compact

size and rapid execution.

2.  PROGRAMS

The program package consists of 4 files:

HFMAGIC1.FNL    main BASIC program

MAGICSRC.BAT    batch for screen output

MAGICFIL.BAT    batch file to send output to a file

MAGICPRT.BAT    batch file to send output to line printer

The version which provides only screen output can be directly

executed by entering the command:  BASICA HFMAGIC.FNL .  The file

version will load the output into a file A:MAGICOUT.FIL; you can edit

the batch file MAGICFIL.BAT to create the output file on any desired

drive or directory.

3.  INPUT DATA REQUIRED

The program will issue prompts for all required data.

The first prompt will be for which version of the program is

desired; enter 1 if you desire S-meter predictions for the 5

traditional ham bands (80,40,20,15, and 10 meters); enter 2 if you

desire 9 bands (5 traditional plus 160, 30, 18, and 12 meters); enter 3

if you desire the computation for arbitrary frequencies which you can

input; and enter 4 if you are only interested in MUF predictions.

The next prompt will ask for the number of solar flux points

and increment.  This permits you to have multiple runs which can be

referred to in order to evaluate the best band to use for a particular

set of solar flux conditions.  If only a single run is desired, enter

1,1 at this prompt.

The next prompt will ask for the number of runs to be

executed.  If you desire computations for more than 1 set of

conditions, enter the appropriate number here else 1 for only a single

set of conditions.

The fourth prompt will ask for the details for the path being

computed.  This data should be entered with a comma between each number

as a delineator.  The data consists of the latitude and longitudes of

the two endpoints of the path (in degrees and decimal degrees, north

and west positive), day of the month as a number, month as a number,

the starting value of solar flux (if multiple flux values are being

computed, else the particular flux value desired for the computation),

and a field of alphanumeric characters to document the run (these will

be printed out on the output). Current values of solar flux can be

determined by listening to station WWV or WWVH (2.5,5,10,15,20 Mhz) at

18 minutes past the hour.

After completion of the data entry, the entered parameters

will be echoed back for confirmation.  Hit enter if the string is

correct.

4.  OUTPUT

The output will consist of a matrix of time, frequency, and

signal strengths (in dB over/under S-9) based on the assumption of 100

watts transmitted and the use of non-directive antennas such as

vertical and dipole antennas.  A value of -98 in the output indicates

that the band will not be open, and a value of -99 indicates very high

D-region absorption.  The normal range of useful communications will be

down to an amplitude of -54, corresponding to 54 dB below S-9, or S-0

(an S unit is 6dB by convention).  An output of 20 indicates a signal

level of 20dB over S-9.

The time presented is Universal Time (UT), based on the local

time at the prime meridian at Greenwich, England.  Remember to convert

this time to your local time.

5.  ACCURACY

Most people who have used this program have reported that the

predictions match the actual conditions with a reasonable degree of

accuracy in predicting the best band to use and the signal strengths

which will result.	The accuracy is quite good for distances up to

about 2500 miles.

6.  COMPENSATION

Disemmination of this program via BBS systems is encouraged

so that many people can use it.  If you feel that this program is

useful for you, please send a token fee ($10 suggested) to:

Dick Lodwig W2KK	2201 Duvall Rd.  Woodbine, Md.	21797

7.  EXAMPLE OUTPUT

An example output for the case of a Maryland to Florida path

on the 27th of February with a solar flux of 200 is provided below.

Note that for communicating at midnight Eastern time (5 UT), the 160

thru 30 meter bands will give a path, but since the atmospheric noise

is much larger for the lower bands, the 40 or 30 meter bands will give

the best results.

For communicating at noon Eastern time (17 hrs UT), the 20

thru 12 meter bands will all give equivalent results and the choice of

band can be made based on equipment capabilities and QRM observed on

the bands.

------------------------------------------------------------------------

ENTER:

1-5 VAL SM

2-9 VAL SM

3-ARBITRARY SM

4-3 VAL MUF

? 2

ENTER NUM OF SF PTS,INCREMENT

? 1,1

NUMBER OF RUNS

? 1

STR FORMAT LAT,LON,LAT,LON,DAY,MO,SF START,ALPHA TITLE

ENTER STRING NO. 1

? 39,77,23,80,27,2,200,Md-Fla

FLUX PTS,INCR 1  1

INPUT DATA STRINGS

39  77  23  80  27  2	200 Md-Fla

PRESS ENTER TO CONTINUE?

#HFMAGIC 1.02 FOR Md-Fla BY W2KK.#

DATE: 27 FEB

TRANSMITTER LOCATION

LATITUDE 39  LONGITUDE 77

RECEIVER LOCATION

LATITUDE 23  LONGITUDE 80

SUNSPOT NUMBER=  155	 SOLAR FLUX=  200  (WWV AT HR+18 MIN)

RECEIVED SIGNAL FOR 100W TRANSMITTED. PERFECT

NON DIRECTIVE ANTENNAS. DB OVER/UNDER S9

UT	       FREQUENCY

HR  MUF    1.9	3.8  7.2 10.1 14.2 18.1 21.3 24.9 28.5

--  ---   ---- ---- ---- ---- ---- ---- ---- ---- ----

0 20.4     19	 13    7    5	 2   -1  -98  -98  -98

1 17.8     19	 13    7    5	 2  -98  -98  -98  -98

2 16.3     19	 13    7    5	 2  -98  -98  -98  -98

3 15.0     19	 13    7    5	 2  -98  -98  -98  -98

4 13.9     19	 13    7    5  -98  -98  -98  -98  -98

5 13.0     19	 13    7    5  -98  -98  -98  -98  -98

6 12.3     19	 13    7    5  -98  -98  -98  -98  -98

7 11.7     19	 13    7    5  -98  -98  -98  -98  -98

8 11.1     19	 13    7    5  -98  -98  -98  -98  -98

9 10.7     19	 13    7    5  -98  -98  -98  -98  -98

10 10.4     19	 13    7    5  -98  -98  -98  -98  -98

11 13.0     -4	  7    6    4  -98  -98  -98  -98  -98

12 19.2    -99	-21   -2   -0	-1   -2  -98  -98  -98

13 22.6    -99	-54  -11   -5	-3   -3   -4  -98  -98

14 24.7    -99	-85  -20   -9	-5   -5   -5  -98  -98

15 26.2    -99	-99  -27  -13	-7   -6   -6   -6  -98

16 27.2    -99	-99  -32  -16	-9   -7   -6   -7  -98

17 27.7    -99	-99  -35  -17	-9   -7   -7   -7  -98

18 28.0    -99	-99  -34  -16	-9   -7   -7   -7  -98

19 27.9    -99	-99  -30  -15	-8   -6   -6   -6  -98

20 27.4    -99	-99  -24  -11	-6   -5   -5   -6  -98

21 26.6    -99	-69  -15   -7	-4   -4   -5   -5  -98

22 25.3    -99	-37   -6   -2	-2   -3   -4   -4  -98

23 23.4    -54	 -5    2    2	 0   -1   -3  -98  -98

GRT CIR DIST=  1800 KM	 1125 STAT MILES   978 NAUT MILES.

PATH MIDPOINT=	78.6  LON    31  LAT

HFMAGIC 1.0 COPYRIGHT W2KK 2201 DUVALL RD. WOODBINE, MD 21797

---------------------------------------------------------------------