Automatic forwarding of messages to other MailBox systems.

The file FWD.MB contains information that drives the automatic forwarding

of messages. If the file does not exist, no forwarding is done. Forwarding

is attempted each hour at the minute specified for the port in PORTS.MB.

Forwarding will occur on those hours given in FWD.MB.

FWD.MB consist of a number of lists, and a "forwarding script" associated

with each list.  Each list has the information for one MailBox to which you

will forward.  The script associated with the list tells your MailBox how

to connect to the MailBox you will forward to.	The list contains information

that describes which messages will forward to that MailBox. Each list is

terminated by "*** EOF".

I have included an example FWD.MB file.

The overall structure of FWD.MB is thus:

Script1

List1

*** EOF

Script2

List2

*** EOF

... etc.

Routing lists.

Each list has a header line, and any number of callsigns, designators, or

sublists. The header line tells the MailBox when to do the forwarding to

that station, what MailBox port to use for the connection, and how to

identify the list when you use the X commands. Thus, a list will look like:

fa0023w0qrm

ntsmn

ntssd

w0qrm

k0cj

fh0205w7xyz

ntswa

98*

w7xyz

aa7abc

Format of the list header.

Columns     Data

1	    List type.

2	    Port identifier or filler. "A" = COM1, "B" = COM2, etc.

3-4	    Hour to activate this function.

5-6	    Hour to de-activate this function.

7-12	    Key used to select list with X or XI command.

For forwarding lists, the callsign of the MailBox to

forward to, WITHOUT SSID.

For Export/Import lists, the file name.

List types:

Type		Function

E      "Answer Reverse Forward Requests"

F      "Forward and Reverse Forward"

H      "Forward, Reverse Forward, and Poll"

E, F, and H lists are lists of stations for whom you should forward messages.

They are grouped in the list by the call of the MailBox to which the messages

An "F" list is used when you wish to initiate forwarding.

An "H" list acts the same as an "F" list, except that the

connect and probe for reverse forwaring will occur even

if you do not have any messages to forward.

An "E" list acts like an "F" list, except the forwarding is not done,

the list is used only when someone requests reverse forwarding from you.

Connect Scripts.

Connect scripts are supported through C, L, N, P, Q, R, and S items.

The Connect Script precedes the list that uses it.  The following are

some examples of common Connect Scripts:

For a direct connect to the MailBox you wish to forward to:

cc n7hae   <- Means "connect to n7hae from the tnc"

For a simple NET/ROM connect to the MailBox you wish to forward to:

cc k7zvv-7 <- Means "connect from the tnc (uplink) to your local node"

nc w7fbm-8 <- Means "connect from this node to to the distant node"

nc n7hae   <- Means "downlink from this node, to the MailBox"

To connect to a distant MailBox running the g8bpq switch, when you are

using the g8bpq switch also just connect to that MailBox call:

nc n7kmj

Summary of script items in FWD.MB

C     Connect TO this call FROM a normal tnc.

N     Connect TO this call FROM a node.

J     Connect TO this call, it is a KA-node.

K     Connect TO this call, FROM this KA-node.

P     Do this tnc command before connecting.

Q     Do this tnc command after disconnecting.

P!    Do this DOS command before connecting.

Q!    Do this DOS command after connecting.

L     Limit forwarding when you use this list.

S     Send this text.

R     Receive this text.

R!    Receive any text.

T     Set port timeout (in seconds) for this forward.

Some further examples:

CC W6NR-11

CC N7EQN-10 via A6DIG

NC W6NR

NC W6QRM v N6DIG

P items give TNC commands to be executed BEFORE the connection:

Pretry 10

Pmaxframe 3

Pfrack 8

Q items give TNC commands to be executed AFTER the disconnect:

Qretry 3

Qmaxframe 7

Qfrack 3

Be very careful using P and Q items. The MailBox assumes that the

TNC is setup in a "standard" manner. If you change CR, CP, or SE

in a script, it could cause problems.

An L item is used to place limits on what will forward when the list is used.

L nnnn	-  Will limit the size of messages forwarded.

l B	-  Will NOT forward bulletins.

Examples:

L 5000

Would cause any message larger than 5000 bytes to NOT forward.

S and R items are used to forward in unusual situations, so that

you can handle networks which do not use NET/ROM or KA-NODES.

S and R items come in pairs:

An S item is a line to send:

SBBS

An R item is the expected response:

R#SBAY1:N7EQN-10} Connected to #WWORM:WB6FFC-1

In the case that ANY response is valid use:

R!

There can only be one C item in a script, but may be as many N, P, Q, R

and S items as required. As an example, the script for W0RLI

in Santa Cruz using NET/ROM to connect with KA6IQA in San Diego might

be (using all the possible script features).

l 2000			  <-  Don't forward messages larger than 2K

t 240			  <-  Use 4 minute timeout for this forward

pretry 10		  <-  Tough path, retry a lot.

pmaxframe 3		  <-  And don't send very many packets.

pfrack 8		  <-  And wait a long time for ACK.

qretry 3		  <-  Put things back to normal after this forward.

qmaxframe 7		  <-  Ditto

qfrack 3		  <-  Ditto

cc w6amt		  <-  Connect to the local NET/ROM

nc w6amt-3		  <-  Connect to the NET/ROM closest to KA6IQA

nc ka6iqa v w6amt-4	  <-  Connect to KA6IQA

fb0023ka6iqa		  <-  Use port B. All hours. List name KA6IQA.

ka6iqa			  <-  Forward messages TO or AT KA6IQA

91*			  <-  Forward zip codes starting with 91

*** EOF 		  <-  End of list marker. Must be upper case.

Wildcards and special "callsigns" in lists.

When the designator in FWD.MB is compared to the TO or @ BBS call,

the characters '?', '*', '@', '!', '+', '"' in the designator act as

wildcards. See WILD.DOC for an explanation of how to use them.

Sublists.

At any place in the FWD.MB file you can refer to another file. The contents

of the file is treated exactly as if it was in the FWD.MB file. This feature

is very useful when you have several alternate paths to a given location.

FWD.MB need only contain the onnect Script for each different path, and a

list containing a reference to the file which contains the list contents.

A sublist file is given by a line starting with "@". The rest of the line is

the device, path, and file name of the sublist.

Example:

NC N4CHV

FC0023N4CHV

N4CHV

@C:\BBS\HF111.FWD

@C:\BBS\SILICON.FWD

*** EOF

NC W6CUS-1

FD0023W6AMT

NI6A

@C:\BBS\SILICON.FWD

*** EOF

Entry qualifiers.

A qualifier is appended to the line it affects. There is one qualifier at

this time: " /TSSEE" where SS is the start hour and EE is the end hour.

This qualifier causes the line to be ignored if the current time is not

within the time window.