PACK-PRO

Multimode Communications Program

for

PC Compatibles and the Kantronics KAM

Version 1.1

(c)Copyright 1991, Andrew C. Manti Jr.

This program and documentation are protected under United

States Copyright Laws and as such cannot be sold or in any

way distributed for profit without the prior written consent

of the author.  While you may distribute this program through

bulletin board systems, it must be distributed unmodified and

without charge of any kind.  As every possible use of this

program cannot be forseen, PACK-PRO is provided as-is, and

without warrantee of any kind.

SHAREWARE REGISTRATION

PACK-PRO is distributed as a Shareware product, meaning that

you have the opportunity to try the program before buying it.

You are free to copy and redistribute the entire file to

friends or other bulletin board systems as long as the files

are not modified in any way and no fee is charged. If, after

trying this program you feel it is of value to you, then you

are asked to pay the registration fee of $20 (US) plus

applicable sales tax if you are a Massachusetts resident.

Your registration fee will guarantee you telephone support as

well as a copy of the next release of PACK-PRO.  Releases

beyond the next will be made available to you at special

upgrade rates.

As you can imagine, your registration fee does NOT make me a

rich man, but rather helps defray the costs of further

program development and distribution. (Disks, Paper, Postage,

etc).  If for any reason you can not afford the $20, send

what you can afford.  You will still be registered, receive

phone support, and be able to obtain updates to the program

at much reduced rates. If nothing else, send me your comments

so that I can use them to make PACK-PRO an even better

program.

Please use the registration form at the end of this document

or in the file REGFORM.DOC to register you copy.

Thank you for your support!

Andrew C. Manti Jr.   W1VL

2 Deana Road

Methuen MA 01844

PACK-PRO         Pg 1.

INTRODUCTION

PACK-PRO is a multimode communications program designed for

use with IBM PCs and compatibles and the Kantronics KAM, all

mode TNC.  The program is designed to take the work out of

using the KAM by providing a user friendly interface for all

modes except WEFAX.  The program will run on floppy or a hard

disk and requires a minimum of 384K of memory.  An Epson or

IBM compatible printer is required if you intend to use the copy

to print option.  Both color and monochrome monitors are

supported.  The main features of this program are as

follows:

- Operates on COM1 or COM2 of your PC

- Modes: VHF/HF Packet, RTTY, AMTOR, CW, NAVTEX, ASCII

- Split screen display

- Brag file for RTTY/AMTOR

- Unlimited nine (80 characters each) message files

- Text Editor for files up to 300 lines in length

- Screen review (last 6 screens)

- ASCII Text file transmission

- Automatic CQ generator with auto answer detection

- Automatic SELCAL generation from inputed call

- Capture to Disk capability

- Automatic unattended mail capture for packet

- Routing file capability

- Multiple Monitor modes for packet

- Hot key selectable baud rates and shifts

- ASCII file upload/download capability for packet

- Script capability for packet multiple node connects

- Contest capability with serial number generator

- Toggles for :

- RTTY/ASCII/AMTOR tone selection

- DX CQ's

- Time Stamp for Packet

- Packet Beacons

- Mail Capture

- Auto capture to disk of all packet connects

- Local Echo

- Capture to printer

PACK-PRO         Pg 2.

SET-UP

Initial set up of PACK-PRO is very straightforward. The

program can be run from either floppy or a hard disk. Just

copy the files on the distribution disk to your working

floppy or a hard disk directory. The only other action

required is the set up of the PPACK.SYS file.  This file is

used to initialize both the KAM and the program to your

requirements.  A sample PPACK.SYS file is included to aid in

setting up your own.  You may generate this file using any

word processor which is capable of generating true ASCII

files or by using the built in text editor feature of PACK-

PRO. You invoke PACK-PRO with the dos command:

PACK_PRO <ENTER>

NOTE: THIS WILL BRING THE PROGRAM UP IN COLOR, IF YOU ARE

USING A MONOCHROME OR B/W MONITOR, TYPE:

PACK_PRO M <ENTER>.

The inital shareware screen will appear. Entering the letter

<E> at this point will take you immediately to the PACK-PRO

editor.  Any other key will load and execute PACK_PRO.  To

edit or create the PPACK.SYS file enter 'E' and then enter

the filename PPACK.SYS at the prompt.  You will then be in

editor mode.  Pressing <F1> while in the editor will give you

a help file showing the various editing keys you may use.

PACK-PRO         Pg 3.

The PPACK.SYS file MUST be set up in a specific format for

the program to function properly. This format is as follows:

Line #1      Your Call

Line #2      Your AMTOR SELCAL

Line #3      Drive Identifier for all Msg files (Drive Letter

Only  i.e  A B C D etc)

Line #4      CQ_Delay in Seconds (Time to wait before

repeating auto CQ's).

Line #5      Your QTH

Line #6      Text for Beacons (up to 80 characters)

Line #7      Communications port number (1,2,etc)

Line #8      Terminal to TNC baud rate code as follows:

0  -  110 baud

1  -  150 baud

2  -  300 baud

3  -  600 baud

4  -  1200 baud

5  -  2400 baud

6  -  4800 baud

7  -  9600 baud

(Note that most TNC's may not run reliably

at over 1200 baud)

Line #9      Number of stop bits (1,2)

Line #10     Number of Data Bits (7,8)

Line #11     Parity code as follows:

0  -  None

1  -  Odd

2  -  None

3  -  Even

Line #12     Number of times to repeat CQ text in auto mode

Line #13     BBS Read Message Command (usually RM)

Line #14     Default start up CW speed (default is 13 WPM)

Line #15     Next Sequential Serial Number (Prog will update

this number automatically each time you exit.

Line #16    Additional TNC Start up commands go on this line

and any following. (Line #16 must be present)

NOTE: All lines must be included and must end with a carriage

return (<ENTER>).  If you do not wish to use a specific

line just leave a blank line in its place.

It is assumed that your TNC has already been set up for

the proper baud rate etc and that these values have

been PERMed.  (See your TNC manual for more

information).

Review the included PPACK.SYS file as an example. Be sure to

change the calls and QTH information before running the

program.

PACK-PRO         Pg 4.

OTHER PROGRAM RELATED FILES

The PPACK.SYS file as mentioned above is the only file

required to run PACK-PRO.  However, additional files will be

needed if you wish to fully utilize all the features of the

program.  These files can be created with an ASCII editor

before you begin running PACK-PRO or by using the built in

editor.  These additional files are:

BRAG.MSG  - Brag file (usually contains information on

your station and equipment. Used

extensively on RTTY and AMTOR.)

*.MSG     - Message files used with the auto message send

option.  These files can each contain up to

nine (9) message lines of up to 78 characters

each.  Each line must end with a carriage

return.  These are very easily constructed

using the built in Create Message Option.

These messages are great for CW or for

contesting.  They may contain special

characters as outlined in the MAKE MESSAGE

option description later in this document.

CALL_LST.SYS - This is a PATH file for packet use. This

file, again generated by an ASCII editor or

the built in editor, includes preset paths to

specific stations.  The format of this file is

as follows:

Station Path <CR>

Be sure to leave ONE (1) space between the

station name and the path.  DO NOT put any

spaces in the station name. For example, if

the path to W1AW is W1AW V EMAS1,WRI,CCONN

then the associated line in the CALL_LST.SYS

file would be:

W1AW W1AW V EMAS1,WRI,CCONN <CR>

With this file you only need enter the station

name and the program will automatically look

up and connect via the path you have

established.  Note the use of commas to

separate the digipeaters in the path. Keep in

mind that the station name need not be call

letters. You may use any name you feel like.

If it is easier to remember JOHN rather than

John's call, use JOHN for the station name.

Then every time you wish to connect to John,

just enter his name as the call.

PACK-PRO         Pg 5.

This file may be as long as desired, but each

line must be less than 80 characters in

length.  The call of the station you wish to

work must be entered using the <F5> Enter Call

option and called using the <Alt><F4> Make

First Call option, both of which are described

later in this document.

CAPTURE.TXT - This is the capture to disk file. If

Capture to Disk is enabled, it will contain a

copy of all information which is displayed on

the screen.  The program will create this file

automatically.  Keep in mind that you may want

to edit or delete this file on occasion as it

can become very big, very fast.

MESSAGE.00x - These files, also created by the program

as needed, is where all <Ctrl><F4> option,

automatically captured packet mail will be

put.  All messages obtained with the Auto Mail

Capture option will be copied to this file,

one after the other.  Again you may want to

review these files on occasion and delete or

edit them.  Each time new mail is received the

.00x is incremented.

*.SCR  -  These are script files for use in packet

modes. These script files are for use when

connecting to another station through a series

of nodes.  This differs from digipeaters and

the CALL_LST.SYS file in that nodes commonly

require you to connect to each individually.

The script file which can have any name

(extension must be .SCR) is merely an ASCII

file of exactly what you would type to perform

the connect.  For example, to connect to W1VL

through the nodes SNH, EMASS, and port 4 of

METK the script file would look like this:

C SNH

C EMASS

C 4 W1VL V METK

Note that each line ends with a carriage

return and only one connect command per line.

These files also can be generated with any

ASCII text editor or the PACK-PRO editor.

PACK-PRO         Pg 6.

SPLIT SCREEN DISPLAY

PACK-PRO operates with a "split-screen" display.  This means

that information coming from the TNC to the computer is

displayed in the top or receive section of the screen, while

data which you type or is sent from the computer to the TNC,

will appear in the lower or transmit portion of the screen.

In the case of non-packet modes of operation, everything you

type in the transmit area, will also appear in the receive

area as the information is actually transmitted.  In TNC

lingo, this is the XMITECHO ON.  In packet modes, this will

not occur  so that incoming and outgoing data are not

intermixed on the screen.

At the bottom of the screen is a summary list of the most

common hot keys.  Pressing <F1> will give you a complete

listing of these keys.

The top line of the display will display the <F1>Help prompt,

followed by the time in 24 hour format. Note that this is the

time in your computer clock!  When in packet modes, the

existing Receive Mode will be displayed next.  In other modes this

will be replaced by the next serial number to be sent in Message

mode, if the serial number is greater than 1.  The end of

this line will show the name of the message file presently

loaded into memory, or if packet mail "auto capture" is on,

MAIL CAPTURE ON will appear.  If mail has been received, the

word MAIL will be flashing.

The box at the bottom of the screen is the operating status

line.  This contains the present mode of operation, baud rate

(or CW speed), shift and transmit/receive status.  The call

of the station you are working will also appear on this line

between the shift and transmit/receive areas.

This line also shows the status of various toggles just after

the mode of operation.  These indicators are as follows:

Toggle                On        Off

------                ---       ---

Capture to Disk            [CTD]      -

Invert (RTTY,AMTOR)        [INV]     [NOR]

DX Flag                    [DX]       -

Copy to Printer            [PRT]      -

LO Tones Toggle            [L]        -

PACK-PRO         Pg 7.

COMMAND KEYS AND PROGRAM OPTIONS

COMMAND KEY (Hot Key) Summary:

Function Keys:

<F1> - Help                  <F2> - Select Mode

<F3> - Send Brag Msg         <F4> - Send Calls

<F5> - Enter Call            <F6> - Call CQ

<F7> - Invert (Mark/Space)   <F8> - Receive Mode

<F9> - Delay Receive         <F10> - Transmit Mode

Control Keys:

<Ctl><F1> Make Txt File       <Ctl><F2> Edit Txt File

<Ctl><F3> Send Text File      <Ctl><F4> Mail Capture toggle

<Ctl><F5> D/L ASCII File      <Ctl><F6> U/L ASCII File

<Ctl><F7> Send Script         <Ctl><F8> Clr Rcv Screen

<Ctl><F10> Drop to DOS

Alt Keys:

<Alt><A> Auto Capture          <Alt><B> Beacon toggle

<Alt><C> COM Port Select       <Alt><D> DX Flag toggle

<Alt><E> Local Echo toggle     <Alt><F> Force Command

<Alt><G> Load Msg File         <Alt><H> Heard List

<Alt><I> Initialize KAM        <Alt><J> Low Tones

<Alt><K> Edit Msg File         <Alt><L> Lock on CW

<Alt><M> Make Msg File         <Alt><N> Reset S/N

<Alt><O> RTTY USOS toggle      <Alt><P> Printer toggle

<Alt><R> Packet Receive Mode   <Alt><S> Port Parameters

<Alt><T> Time Stamp toggle     <Alt><U> Unlock CW Speed

<Alt><F1> Disk Capture toggle  <Alt><F4> Make First Call

<Alt><n> Send Message n        <Alt><X> Exit Program

Other keys:

<PgUp> Screen Review          <->> Increment Baud/Speed

<<-> Decrement Baud/Speed     <Up Arrow> Change Shift

<END> End of transmission

NOTE: TNC commands may be issued manually at any time. If in

a Packet mode, simply type the command and press <ENTER>.

If in any mode other than Packet, press the <Alt><F>

Force Command hot key and then enter the command and

<ENTER>.

If you use the <Alt><F> combination, you must then press

<F2> and reselect your previous operating mode.

PACK-PRO        Pg 8.

The following details the options and funtions within the

program.  The functions are listed based on the operating mode

in use.

*****  ALL MODES (COMMON KEYS & FUNCTIONS) *****

END TRANSMISSION  <END>  - All Modes

The <END> key can be used to end transmissions in all

modes.  In the Packet modes, this key will send ' DE

your call >>' to signify to the other station that you

are done transmitting.

In AMTOR mode this key will send the '+?' code to

indicate you are waiting for the other station to now

transmit.

In these two modes, a switch to receive after pressing

<END> is automatic.

In all other modes, this key will send the call of the

station you are working (if entered with the <F5> option

described later) followed by 'DE', your call and  'K'.

In these modes, the program will return to receive mode

as soon as all information in the transmit buffer has

been sent.

SCREEN REVIEW  <PGUP> - All Modes

This option allows you to review the six previous

screens of information which have already scrolled by in

the receive text area.  Once this option is selected,

all further monitoring of the data from the TNC is

halted.

The use of the following keys allows you to scroll

through the previous screens:

B  - Back one page

F  - Forward one page

Q  - Quit and return to previous mode

CHANGE SHIFT  <UP ARROW> - All Modes

This key will cycle through the shifts available for

each respective mode.  Generally, you will shift through

the 170, 425, 850, and Modem shifts.  The shift in use

will appear in the appropriate place on the status line.

PACK-PRO         Pg 9.

INCREASE/DECREASE SPEED & BAUD - <LEFT & RIGHT ARROWS> - All

The left and right arrow keys are used to increase and

decrease baud rate or cw send speed.  The speed or baud

being used will appear in the status line.  The baud

rates will increase/decrease based on the rates

available for the particular mode you are in.  In CW,

the speed will increment or decrement in one (1) WPM

steps.  The default at startup is 13 WPM.

Should you change modes and later return, the program

will reenter the previous mode with the baud and shift

parameters set as when you left.  Once you exit the

program, the initial values will be the program

defaults.

HELP SCREEN  <F1> - All Modes

The Help Screen gives a listing of all hot keys used by

the program and a brief description of their function.

SELECT MODES  <F2> - All Modes

This option is used to select the operating mode you

wish to use.  Selecting by number will place the program

and TNC in the proper condition for the selected mode.

If you are returning to a previously used (during the

same session) mode, the program will return you with the

parameters (shift, baud,etc) that you were in when you

left it.

SEND CALLS <F4> - All Modes

This key will send the call of the station your are in

QSO with (if entered with <F5>) followed by ' DE ' and

your call. This will only be transmitted if in the

TRANSMIT mode or after you enter TRANSMIT mode. once

sent you will remain in this mode and may carry on your

contact.

ENTER CALL <F5> - All Modes

The enter call option is used to enter the call of the

station you are about to call or that has called you.

In packet mode enter the call of the station you wish to

connect to and use the FIRST CALL key <Alt><F4> to

perform the connect.  In this mode, the program will

check for any CALL_LST.SYS file and any paths that may

exist before accepting the call as entered.

PACK-PRO         Pg 10.

In CW, RTTY, and ASCII modes, enter the call of the

station are calling or is calling you.  This entry of

the call will insure that other options such as SEND

CALLS will have the correct call to use.  You should

enter this call before making contact.

In the AMTOR mode, after you have entered the call of

the station, the program will automatically determine

the stations SELCAL using accepted techniques. You will

see the SELCAL and be asked if it is correct (in case

the other station is not complying with accepted

practices). If it is simply enter 'Y'. If not enter 'N'

and then you will be able to enter the SELCAL manually.

Once entered you will be able to use the SEND FIRST CALL

<Alt><F4> option to actually make the link.

RECEIVE MODE  <F8> - All Modes

The Receive mode option allows you to return IMMEDIATELY

to the receive mode.  Any information in the type ahead

buffer or the TNC which has not yet been sent will be

cleared.  This allows you to clear the buffer, or to

stop an unintentional transmission.

In the packet modes, this option will issue a DISCONNECT

command to the TNC, disconnecting you from the station

you are talking to.

TRANSMIT  <F10> - All Modes

The F10 key will place you in transmit mode.  When in

one of the packet modes, you will be placed in CONVERSE

mode. Thus everything you type will be sent to the TNC

for transmission.  If operating Packet, the FIRST

Call <Alt><F4> option should be used when making a

connect.  This is really the Transmit switch when

operating in CW or RTTY modes. In the AMTOR mode this

key can be used to seize the link.

In CW, RTTY, and ASCII modes, this key will activate the

transmitter and all text already in the type ahead

buffer or typed real time at the keyboard will be

transmitted.  This allows you to prepare a transmission

while the other station is transmitting.  When ready to

send your text, just press <F10>. After the text already

in the buffer is sent, you will remain in a transmit

mode.

PACK-PRO         Pg 11.

MAKE TXT FILE  <Ctrl><F1> - All Modes

The Make Text File option allows you to create a pure

ASCII file of up to 300 lines in length.  These files

may be message files you wish to upload to a Packet BBS

or files you want to use to send information during a

QSO (Station equipment, etc).  The BRAG.MSG file

mentioned earlier is a good example of this type of

file.  When this option is selected, you will be

prompted for the filename to create. If a file by that

name already exists, you will not be able to create a

new one.  You may however use the EDIT TXT FILE

<Ctrl><F2> option to be described next to edit the

existing version.

This is a very simplistic text editor and does not have

a great deal of features. It is intended for creation

and editing of small files for use by the program.

Larger files should be created and edited using your

standard ASCII word processor or editor.  The following

editing keys can be used with this editor:

< -> >     - Move cursor Right

< <- >     - Move cursor Left

<Up Ar>    - Move up one line

<Dn Ar>    - Move down one line

<PG DN>    - Move down one page (15 lines)

<PG UP>    - Move up one page (15 lines)

<DEL>      - Delete character under the cursor

<INS>      - Toggle from Insert to Overwrite Mode

<HOME>     - Move to Start of line

<END>      - Move to End of line

<Ctl><HOME>- Move to Top of File

<Ctl><END> - Move to End of FIle

<Alt><I>   - Insert Line Above Cursor

<Alt><D>   - Erase entire line

<ESC>      - Exit Editor WITHOUT Saving file

<Ctl><S>   - Save File and Exit Editor

At this time (Version 1.1) word wrap does not exist so

files must be created in "typewriter mode", or in other

words, press <ENTER> at the end of each line as if you

were using a standard typewriter.

EDIT TXT FILE   <Ctrl><F2> - All Modes

This option allows you to edit previously created text

files in the same manner as above. When you have entered

the filename at the prompt, the program will check to be

sure the file exists. If no such file exists you must

use the MAKE TXT FILE <Ctrl><F1> option to create it.

The same editing keys mentioned above apply when

editing.

PACK-PRO         Pg 12.

SEND TEXT FILE  <Ctrl><F3>  - All modes

This option allows you to send any existing ASCII text

file.  When selected, you will be asked for the

filename. If the file exists it will be sent

immediately.  If the file does not exist, the request

will be ignored and you will be returned to the program.

CLEAR RECEIVE SCREEN   <Ctrl><F8> - All Modes

This option clears the receive portion of the screen

display.

DROP TO DOS   <Ctrl><F10>  - All Modes

This option allows you to drop to dos and execute other

dos commands without leaving the program.  This command

may not operate if you have less than 512K of RAM in

your system, since the program and COMMAND.COM must be

able to coexist.  If the program clears the screen and

then immediately returns to the program, you have

insufficient memory for this funtion to work.

Once you have dropped to DOS, you may execute other DOS

commands, (DIR, FORMAT, ERASE, etc).  Note that

COMMAND.COM must be in your root directory in order for

this option to work.

TOGGLE COM PORT - <Alt><C>  - All Modes

This option will toggle between COM1 and COM2 ports.

The program defaults to COM1.

LOCAL ECHO TOGGLE   <Alt><E>  - All Modes

This will toggle the program local echo on and off.  If

ON then everything you type in the Transmit area of the

screen will be displayed.  If OFF, nothing will be

displayed.  This defaults to ON.  Note that this is NOT

the local echo (or ECHO) command for your TNC.

FORCE COMMAND   <Alt><F>  - All Modes

This combination will force the TNC back into Command

(cmd:) mode.  Keep in mind that if you are in a mode

other than packet, you must reselect the <F2> select

mode command to return to the previous mode.

PACK-PRO         Pg 13.

LOAD MESSAGE FILE   <Alt><G>  - All Modes

This will load a message file prepared previously with

the <ALT><M> Make Message File Command.  Once loaded,

this group of up to nine message lines (see MAKE MESSAGE

FILE) will remain in memory until a new message file is

selected, or you exit the program.

HEARD LIST   <Alt><H>  - All Modes

This option will display a listing of the last 18

packet stations heard.  This is essentially the MH

command for the TNC.  Keep in mind that while this list

is displayed, the TNC will not be in Monitor mode, so no

additional information will be received until you return

to the program.

INITIALIZE  KAM  <Alt><I> - All Modes

This command will reinitialize the KAM with the same

parameters as when the program was started.  This is

handy if you have issued additional manual commands from

the keyboard and now wish to return to the original

setup.

EDIT MESSAGE FILE  <Alt><K> - All Modes

This is used to edit any of the Message files created

with the MAKE MESSAGE FILE  <Alt><M> command.

MAKE MESSAGE FILE  <Alt><M>  - All Modes

This option is used to generate the nine (9) part

message files for use by the program.  When selected, a

screen will appear where you can enter each of the nine

80 character messages.  As you enter each message, you

will be prompted for the next in that file.  Entering

just a carraige return <ENTER> instead of a message will

save the file.  The following special characters made be

used in these message files:

&&  - insert call of station in QSO with

#   - insert a sequential serial number

!   - repeat last serial number

For example a Sweepstake QSO file for W1VL in

his 235th QSO with W9ZZZ might take the form:

&& DE W1VL  NR # A W1VL 60 EMA =

This would transmit as: W9ZZZ DE W1VL NR 235 A W1VL 60 EMA (BK)

The next serial number to be sent can be

changed with the RESET SERIAL NUMBER option.

PACK-PRO         Pg 14.

RESET SERIAL NUMBER COUNTER  <Alt><N> - All Modes

This option will prompt you to change the next

sequential serial number to be sent with message files.

NOTE that each time you exit PACK-PRO, the Serial Number

counter will be saved in PPACK.SYS. Be sure not to remove

your program disk before this is saved.

PRINTER TOGGLE  <Alt><P> - All Modes

This option will toggle capture to printer on and off.

When on, all text displayed on the receive screen will

be copied to the printer.  If you are using an extremely

slow printer, the use of a print spooler is recommended

so as not to cause a slow down in program operation.

PORT PARAMETERS  <Alt><S>  - All Modes

This option allows you to change the computer to TNC

port parameters.  Note that this only changes the

settings in the computer.  If you wish to change TNC

parameters, you must issue the appropriate TNC commands.

DISK CAPTURE TOGGLE  <Alt><F1> - All Modes

This option allows you to capture all of the received

text to a disk file named CAPTURE.TXT.  This file is

created by the program if it does not exist.  When

toggled on, all received text will be copied to this

file.  When toggled again (OFF), no further text is

added to the file.  Be cautious since this file can

become very large very quickly.

MAKE FIRST CALL  <Alt><F4> - All Modes

In the Packet communications modes, this command will

use the call entered with the <F5> Enter Call option and

will issue the 'Connect' command to the TNC.  If Auto

Capture <Alt><A> is enabled, all incoming text will be

captured to the Capture.TXT file once the connect has

been made.

In all other modes, this option, when used after the

<F5>, Enter Call, routine will make the first call to a

station.  The difference between this and the <F4> Send

Calls option is that this option will send the calls

more than once.  Rather than W1ZZZ DE W1VL, this option

will send W1ZZZ W1ZZZ W1ZZZ DE W1VL W1VL W1VL K K.   In

the AMTOR mode, this option will issue a call to the

other stations SELCAL.

PACK-PRO         Pg 15.

SEND MESSAGE n  <Alt><n> (n = 1 to 9)  - All Modes

This option will send the message number 1 to 9 from the

message file loaded using the <Alt><G> Load Message File

option.  The message will be sent and you will remain in

the transmit mode.

EXIT PROGRAM  <Alt><X> - All Modes

This will load a shut down sequence into the KAM and

then will exit the PACK-PRO program.  All open files

will be closed as you exit the program. NOTE that all open

files will be closed and the contents of the program's

serial number counter for contesting etc. will be saved

in the PPACK.SYS file.  Be sure to allow time for this to

occur before removing your program disk.  When the program

returns you to the DOS prompt after exiting, all of these

clean up chores have been completed.

PACK-PRO         Pg 16.

*****   ALL MODES   E X C E P T   PACKET  *****

SEND BRAG <F3> - All Modes  EXCEPT Packet -

This key will automatically send the contents of the

BRAG.MSG file if it exists.  If you should press <F3>

while in the receive mode, the file will be opened but

will not be sent until you go to the TRANSMIT <F10>

mode.  As soon as you go into transmit, the file will be

sent.  If you are in transmit mode and press <F3> the

file will not begin sending until all text in the type

ahead buffer has been transmitted.

CALL CQ  <F6> - All Modes EXCEPT Packet -

The <F6> key invokes the Auto CQ function.  In this mode

a CQ will be generated with the number of lines entered

as the CQ number in your PPACK.SYS file.  The format of

the CQ will be different but appropriate for each mode.

If the DX toggle has been selected (as shown by [DX] on

the status line), a CQ DX will be called.

Once the CQ has been transmitted, the program will go

into LISTENING mode.  In this mode it will listen for a

preset time (set in PPACK.SYS) for a response to your

call.  If no response is detected after the preset time,

the CQ will be repeated.  This will continue until a

response is heard or you press the <F8> RECEIVE mode

key.

When listening for a response, the program looks for

your call followed by 'DE ' in an incoming text. It then

takes the incoming information after this text and

assumes it is the call of the station calling you.  If

this call is interpreted wrong or is missed, and you are

receiving a call, press <F8>, then enter the call using

the <F5> Enter Call functions.

DELAY RECEIVE  <F9> - All Modes except PACKET

The delay receive option is the normal method of

returning to the receive or listening mode.  When this

key is pressed, the TNC will return to Receive mode

AFTER all text in the type ahead buffer or messages in

process are sent.

DX FLAG TOGGLE - <Alt><D>  - All EXCEPT Packet

This key combination will toggle the DX flag on and off.

This flag when on will add 'DX' to all your CQ calls.

If off, a normal CQ will be sent.  The state of this

flag is displayed on the left of the status line as [DX]

if ON and nothing '   ' if OFF.

PACK-PRO          Pg 17.

*****   PACKET  ONLY  *****

MAIL CAPTURE TOGGLE <Ctrl><F4> - Packet ONLY -

MAIL CAPTURE allows you to receive mail from your local

BBS while the rig is unattended.  The <Ctrl><F4> key

combination will toggle mail capture on and off. This

function will only work in the 'CQ/BEACON' receive mode

for packet.  Once toggled on, the program waits for

beacons from the local BBS's.  As each beacon is

received, the program will first note the call of the

station, displaying it on the right side of the status

line.  It then watches for the word 'Mail'. Once this

has been found, the program will wait to see if your

call appears in the mail list.  If it does, the program

will automatically issue a connect request to the BBS,

and then issue a Read Mail (Mine) request.  The message

that it receives will be put in a file called

MESSAGE.00x on your default drive.  The file will be

created if it does not exist.  As additional messages

are retrieved, they will be added with extensions .002,

.003 etc.   These files can be erased at any time as

they will be recreated as needed.  Since this option

does not at present erase or kill the message after it

has been downloaded, please be sure to log back into the

BBS and kill all messages which have already been read.

DOWNLOAD ASCII FILE   <Ctrl><F5> -  Packet ONLY

This option allows you to download an ASCII text file

from your local BBS or another packet station.  When

selected, you will be prompted for a download command.

At this prompt, enter the entire download command as you

would normally do for the BBS involved. For example, to

download the file 'TEST.TXT' from your local BBS

you would enter  'D Files Test.Txt' then that is what

you would enter at the prompt.  The program will

transmit the command to the BBS and at the same time

will break the command down, based on the location of

the '.' and open a file of the same name.  Be sure to

include the '.' and extension when using this command.

Pressing any key while the file is being transferred

will abort the save to disk on your end and return you

to command mode.

UPLOAD ASCII FILE   <Ctrl><F6> - Packet ONLY

This option allows you to upload files from you to your

local BBS or another packet station.  When prompted,

enter the normal upload command as required by the

local BBS.  The program will then issue that command and

begin sending the file. Naturally, the file must exist

in order to be sent.  The file will be transmitted, one

line at a time with a 3-4 second delay between lines to

allow for acknowledgements to occcur.

PACK-PRO         Pg 18.

SEND SCRIPT FILE  <Ctrl><F7> - Packet ONLY

The script file option allows you to use a pre-defined

file when connecting to other stations which may require

the use of network nodes.  Many times you cannot simply

connect using vias in the path.  In these cases, the

script file can be used.  This is a standard ASCII file

with each line being one step in the connect process and

ending with a carriage return.  An example of this is:

C W1VL

C 3 EMASS

C W1XYZ V NETH

A file such as this would issue each of the individual

commands to the TNC, wait for a connect response and

then issue the next command.  If at any time a 'FAILURE

with' or 'BUSY from' response is received, the program

will abort the rest of the script and return you to

normal operating mode.

AUTO CAPTURE TO DISK   <Alt><A>  - Packet ONLY

When toggled on (see Help window), all connects made

using the ENTER CALL <F5> and SEND FIRST CALL <Alt><F4>

modes will automatically be captured to disk.  All text

being received and displayed on the screen will also

appear in the file CAPTURE.TXT.  This file will be

created if it does not exist.   Pressing the <Alt><A>

combination again will disable this funtion.  The

program defaults to Auto Capture ON.

BEACON TOGGLE   <Alt><B>  - Packet ONLY

This key combination will alternately enable and disable

the transmission of beacon signals.  Be sure your BTEXT

has the information you wish to include with your beacon

if you use this option.

RECEIVE MODE SELECT <Alt><R>  - Packet ONLY

This option will give you a choice of receive modes for

packet operation.  These are:

- Everything - all packet information heard

- CQ/Beacon  - CQ's and Beacons Only

- Traffic    - all connected packet information

- Myself     - only packets addressed to you.

TIME STAMP TOGGLE  <Alt><T>  - Packet ONLY

This option will toggle the time stamping of incoming

packets on and off.  When on, all received packet

headers will be followed by the date and time as

recorded by the TNC clock.

PACK-PRO          Pg 19.

*****   CW  ONLY  *****

LOCK ON CW  <Alt><L>   - CW Mode ONLY

This key combination will lock the KAM at the speed of

the CW signal presently being received.  Also see UNLOCK

CW SPEED <Alt><U>

UNLOCK CW SPEED  <Alt><U> - CW ONLY

This option will unlock the TNC CW tracking of speed and

allow you to start tracking another signal.

*****   RTTY  ONLY  *****

RTTY USOS TOGGLE  <Alt><O> - RTTY ONLY

This option will toggle RTTY unshift on space on and

off.

*****   RTTY/ASCII/AMTOR  ONLY  *****

INVERT <F7> - RTTY/ASCII/AMTOR Modes -

The <F7> key will toggle the mark and space tone

polarities for these modes.  The status of the tones

will be indicated on the left portion of the status line

as either [NOR]mal or [INV]verted.

LO TONES Toggle - <Alt><J> - RTTY/ASCII/AMTOR Modes -

The lo tone toggle will toggle between US and European

tone sets for these modes.

PACK-PRO Registration Form

Version 1.1

YES, please register me as a user of PACK-PRO.

I understand that payment of the requested $20

registration fee will entitle me to a copy of the next

release of PACK-PRO as well as unlimited telephone

support and reduced registration fees on future products

as they become available.

If the $20 registration fee is a problem, as it may be

in this day and age, a voluntary registration fee which

you feel is appropriate will still entitle you to full

registration and telephone support.  The next and later

PACK-PRO releases will be available at reduced registration

fees.

NAME: _________________________  Call:____________

STREET: __________________________________________

CITY: ____________________________________________

STATE: ____________  ZIP: __________________

PHONE: (optional) ________________________________

TNC: __________  COMPUTER: _________________ DISK SIZE: _____

REGISTRATION FEE ENCLOSED: $__________

(PLEASE - MA residents add 5% sales tax)

Please forward this application with the appropriate

registration fee to:

Andrew C. Manti Jr.

2 Deana Road

Methuen MA 01844

I thank you for supporting the shareware system!

NOTE: PLEASE USE THE BACK OF THIS FORM TO REPORT PROGRAM

BUGS OR SUGGESTIONS.

PLEASE NOTE MODE OF OPERATION ALONG WITH AS MUCH

DETAIL AS POSSIBLE ON WHAT WAS BEING DONE WHEN THE

PROBLEM OCCURRED.