DEFINITIONS OF TERMS USED IN THE PV COMPARE PROGRAM

Useable Watt: a watt which charges the battery or is used by a load

connected to the battery or Maximum Power Tracker.

Useable watts must be calculated based on the parameters

of each particular system, and cannot be determined

from the module type alone.

Maximum Power Tracker:

Aka Maximum Power Point Tracker, MPPT, MPT.  Forces the PV module

to operate at the knee of the I-V curve, thereby producing maximum

wattage.

I-V Curve:    Current (I) vs. Voltage (V) curve for a particular PV module,

recorded at a specific temperature and irradiance.  When a

PV module is short circuited, it produces maximum current at

almost zero voltage.  When a PV module is in an open circuit,

it produces maximum voltage and zero current.  The I-V curve

describes the voltage and current outputs that occur when

loading conditions are between open circuit and short circuit.

Note that output current decreases as output voltage rises -

thus increased voltage drop between the PV module and battery

decreases the PV module output current.

Array:  a group of PV modules in series and/or parallel combination.

Module: a group of PV cells in series and/or parallel combination, usually

in a sealed, pre-wired unit.

Cell:   an individual photovoltaic cell

Irradiance:   Solar radiation (sunlight) received.  Usually specified in

watts per meter squared.  1000 watts per meter squared is

essentially equivalent to unobscured sunlight at noon.

Voltage drop:

Voltage that can be measured across the component in question,

such as the wire connecting the PV array and battery.  Wiring

voltage drop multiplied by the current flowing = power

dissipated ("wasted") as heat.  However, in a battery

charging application, these watts would not have been produced

if the wiring resistance did not exist.  Therefore, voltage drop

is not a direct measure of watts gained or lost by using a particular

wire gauge.  Note that the PV array voltage operating point is

forced higher by any voltage drops between the array and battery.

I-V curve data must be used to determine which wire gauge will

be most cost effective for a particular system.

Cmf:    Circular mil foot: in this program, a conductor one thousandth

of an inch in diameter, one foot long.

AWG:    American Wire Gauge