INFO for the TECHNICALLY ADVANCED USER of PV COMPARE

Author: April Lorenzen

(401) 423-3939 voice

or  (717) 853-3599 BBS

or  Further Research

POB 293

Jamestown RI 02835

U.S.A.

NOTE:  This program is not intended to tell you how to design a PV system or

to definitely specify which components to use in it.  It is a

tool for answering what-if PV system design questions and making comparisons

between various system design options.  It is as accurate as the data fed

into it, but the value of the results is largely dependent on the skills of

the person analyzing those results.

TOPIC 1: I-V CURVE DATA FILES ( *.MOD )

PV COMPARE includes a few module I-V curve data files.  If you have I-V curve

data, you can create *.MOD files for other modules.  Use any "DOS text",

ASCII text, etc. editor or word processor.  Here's the format:

Manufacturer's model name/number          (any characters)

Price                                     (numeric only  )

Array temperature in Celsius              (numeric only  )

Irradiance in watts per meter squared     (numeric only  )

Short circuit current in amperes          (numeric only  )

This is followed by a list of module voltages and corresponding current at

each voltage.  Must start with the lowest voltage and progress to the highest.

Voltage and current are side by side on each line with at least one space

separating them, like this:

12.1      3.2267                          (numeric only, of course)

12.2      3.221

The list must include an I-V pair at the end where current is zero.  You can

use any voltage or current increment between pairs, or the increment can vary

randomly.  The list can be as long as 200 voltage and current pairs.  If you

need more than this, call me and I will increase the size for you in a

special version.

Before the release of PV COMPARE, I did not get much cooperation from PV

module manufacturers on I-V curve data.  The *.MOD files included with PV

COMPARE were derived from the ARCO PV module owners manual printed curves.

I enlarged the printed curves and painstakingly measured to find the

current levels at each whole volt, and/or between two points connected

by a straight line.  I then used the straight line data regression

feature in a spreadsheet to fill in the values to every tenth of a volt.

The accuracy of I-V curves found on manufacturer's spec sheets is question-

able.  I have heard everything from "we played with the data until the

curve looked good" to "we measured thousands of modules and averaged the

results".  Different manufacturers claim +-3% to +-10 for the expected

variance from the spec sheet power.

I hope that all PV module manufacturers will offer accurate I-V

curve data for use with this program in the near future.  Alternatively,

curve tests can be done independently by universities, laboratories, or

other interested parties.  I'm sure the curve data could be derived from

knowing short circuit current, open circuit voltage, and the maximum

power point current and voltage.  I'll add this option to the program

when I have the formula to do it.

To simulate a module at a particular irradiance or temperature, you must create

a *.MOD file containing the I-V curve data taken at that specific irradiance

and temperature.  The relationship of irradiance-to-current and

temperature-to-voltage is said by many to be essentially linear.  I have

elected not to include a program feature to compensate I-V curve data for

temperature and irradiance because I believe that the above mentioned

relationships are not "linear enough" (definitely not in amorphous modules)

and I believe that accuracy of the simulation would be significantly affected.

Name your I-V curve data files with a MOD extension.

TOPIC 2: WIRE DATA FILES ( *.WIR )

The DEFAULT.WIR file included with PV COMPARE contains per foot prices for

marine grade stranded wire from a discount mail order distributor: Jamestown

Distributors 1-800-423-0030.  This is not necessarily the most appropriate

wire for your particular application.  You may want to edit the prices shown

to match those from your supplier or you can create your own *.WIR data

files in the following format:

Conductor Material                     (such as copper, aluminum, etc)

Construction of cable                  (insulation type or designator)

Weight of conductor per circular mil foot in pounds

Ohms per circular mil foot at 0 degrees Celsius

Temperature co-efficient of conductor material

Temperature of conductor in degrees Celsius

Gauge Name, Circular Mils, Ampacity, Cost per foot

(Repeat until all gauges are listed)

You can list up to 30 wire gauges - you must begin with the largest physical

wire gauge and proceed to the smallest.  Use the DEFAULT.WIR file as an

example or template.

Name your wire data files with a WIR extension.

TOPIC 3: IMPORTING DATA TO A SPREADSHEET

The F7 key allows you to dump the system description and results for all wire

gauges to a *.TXT disk file.  You can import this into a spreadsheet because

it contains commas as delimiters.  Use the IMPORT feature of your spreadsheet,

do not try to read the *.TXT file in as a worksheet file.  This is handy for

graphing and data manipulation.  You can elect to print to paper instead of

disk file, but I have set up the printer output not to exceed 80 columns -

thus some decimal places are truncated.

TOPIC 4: CHANGING THE DEFAULT FILES

You can set any files you want as the DEFAULT.USR (system description),

DEFAULT.MOD  (I-V curve data), and DEFAULT.WIR (wire data).  Just rename

your preferred files with the default file names, or better yet, COPY your

files to the default file names.  However, please only distribute PV COMPARE

to others with the original DEFAULT.* files.

TOPIC 5: USING SINGLE CELL I-V CURVES TO SIMULATE MODULE PERFORMANCE

Cell interconnect resistance and other important factors are not taken into

consideration by PV COMPARE.

END