Network Working Group                                     V. Jacobson/1/

Request for Comments: 1144                                           LBL

February 1990

Compressing TCP/IP Headers

for Low-Speed Serial Links

Status of this Memo

This RFC is a proposed elective protocol for the Internet community and

requests discussion and suggestions for improvement.  It describes a

method for compressing the headers of TCP/IP datagrams to improve

performance over low speed serial links.  The motivation, implementation

and performance of the method are described.  C code for a sample

implementation is given for reference.  Distribution of this memo is

unlimited.

NOTE: Both ASCII and Postscript versions of this document are available.

The ASCII version, obviously, lacks all the figures and all the

information encoded in typographic variation (italics, boldface,

etc.).  Since this information was, in the author's opinion, an

essential part of the document, the ASCII version is at best

incomplete and at worst misleading.  Anyone who plans to work

with this protocol is strongly encouraged obtain the Postscript

version of this RFC.

----------------------------

1. This work was supported in part by the U.S. Department of Energy

under Contract Number DE-AC03-76SF00098.

Contents

1  Introduction                                                        1

2  The problem                                                         1

3  The compression algorithm                                           4

3.1 The basic idea . . . . . . . . . . . . . . . . . . . . . . . .  4

3.2 The ugly details . . . . . . . . . . . . . . . . . . . . . . .  5

3.2.1 Overview. . . . . . . . . . . . . . . . . . . . . . . . .  5

3.2.2 Compressed packet format. . . . . . . . . . . . . . . . .  7

3.2.3 Compressor processing . . . . . . . . . . . . . . . . . .  8

3.2.4 Decompressor processing . . . . . . . . . . . . . . . . . 12

4  Error handling                                                     14

4.1 Error detection  . . . . . . . . . . . . . . . . . . . . . . . 14

4.2 Error recovery . . . . . . . . . . . . . . . . . . . . . . . . 17

5  Configurable parameters and tuning                                 18

5.1 Compression configuration  . . . . . . . . . . . . . . . . . . 18

5.2 Choosing a maximum transmission unit . . . . . . . . . . . . . 20

5.3 Interaction with data compression  . . . . . . . . . . . . . . 21

6  Performance measurements                                           23

7  Acknowlegements                                                    25

A  Sample Implementation                                              27

A.1 Definitions and State Data . . . . . . . . . . . . . . . . . . 28

A.2 Compression  . . . . . . . . . . . . . . . . . . . . . . . . . 31

i

A.3 Decompression  . . . . . . . . . . . . . . . . . . . . . . . . 37

A.4 Initialization . . . . . . . . . . . . . . . . . . . . . . . . 41

A.5 Berkeley Unix dependencies . . . . . . . . . . . . . . . . . . 41

B  Compatibility with past mistakes                                   43

B.1 Living without a framing `type' byte . . . . . . . . . . . . . 43

B.2 Backwards compatible SLIP servers  . . . . . . . . . . . . . . 43

C  More aggressive compression                                        45

D  Security Considerations                                            46

E  Author's address                                                   46

ii

RFC 1144               Compressing TCP/IP Headers          February 1990

1  Introduction

As increasingly powerful computers find their way into people's homes,

there is growing interest in extending Internet connectivity to those

computers.  Unfortunately, this extension exposes some complex problems

in link-level framing, address assignment, routing, authentication and

performance.  As of this writing there is active work in all these

areas.  This memo describes a method that has been used to improve

TCP/IP performance over low speed (300 to 19,200 bps) serial links.

The compression proposed here is similar in spirit to the Thinwire-II

protocol described in [5].  However, this protocol compresses more

effectively (the average compressed header is 3 bytes compared to 13 in

Thinwire-II) and is both efficient and simple to implement (the Unix

implementation is 250 lines of C and requires, on the average, 90us (170

instructions) for a 20MHz MC68020 to compress or decompress a packet).

This compression is specific to TCP/IP datagrams./2/  The author

investigated compressing UDP/IP datagrams but found that they were too

infrequent to be worth the bother and either there was insufficient

datagram-to-datagram coherence for good compression (e.g., name server

queries) or the higher level protocol headers overwhelmed the cost of

the UDP/IP header (e.g., Sun's RPC/NFS). Separately compressing the IP

and the TCP portions of the datagram was also investigated but rejected

since it increased the average compressed header size by 50% and doubled

the compression and decompression code size.

2  The problem

Internet services one might wish to access over a serial IP link from

home range from interactive `terminal' type connections (e.g., telnet,

rlogin, xterm) to bulk data transfer (e.g., ftp, smtp, nntp).  Header

compression is motivated by the need for good interactive response.

I.e., the line efficiency of a protocol is the ratio of the data to

header+data in a datagram.  If efficient bulk data transfer is the only

objective, it is always possible to make the datagram large enough to

approach an efficiency of 100%.

Human-factors studies[15] have found that interactive response is

perceived as `bad' when low-level feedback (character echo) takes longer

----------------------------

2. The tie to TCP is deeper than might be obvious.  In addition to the

compression `knowing' the format of TCP and IP headers, certain features

of TCP have been used to simplify the compression protocol.  In

particular, TCP's reliable delivery and the byte-stream conversation

model have been used to eliminate the need for any kind of error

correction dialog in the protocol (see sec. 4).

Jacobson                                                        [Page 1]

RFC 1144               Compressing TCP/IP Headers          February 1990

than 100 to 200 ms.  Protocol headers interact with this threshold three

ways:

(1) If the line is too slow, it may be impossible to fit both the

headers and data into a 200 ms window:  One typed character results

in a 41 byte TCP/IP packet being sent and a 41 byte echo being

received.  The line speed must be at least 4000 bps to handle these

82 bytes in 200 ms.

(2) Even with a line fast enough to handle packetized typing echo (4800

bps or above), there may be an undesirable interaction between bulk

data and interactive traffic:  For reasonable line efficiency the

bulk data packet size needs to be 10 to 20 times the header size.

I.e., the line maximum transmission unit or MTU should be 500 to

1000 bytes for 40 byte TCP/IP headers.  Even with type-of-service

queuing to give priority to interactive traffic, a telnet packet has

to wait for any in-progress bulk data packet to finish.  Assuming

data transfer in only one direction, that wait averages half the MTU

or 500 ms for a 1024 byte MTU at 9600 bps.

(3) Any communication medium has a maximum signalling rate, the Shannon

limit.  Based on an AT&T study[2], the Shannon limit for a typical

dialup phone line is around 22,000 bps.  Since a full duplex, 9600

bps modem already runs at 80% of the limit, modem manufacturers are

starting to offer asymmetric allocation schemes to increase

effective bandwidth:  Since a line rarely has equivalent amounts of

data flowing both directions simultaneously, it is possible to give

one end of the line more than 11,000 bps by either time-division

multiplexing a half-duplex line (e.g., the Telebit Trailblazer) or

offering a low-speed `reverse channel' (e.g., the USR Courier

HST)./3/ In either case, the modem dynamically tries to guess which

end of the conversation needs high bandwidth by assuming one end of

the conversation is a human (i.e., demand is limited to <300 bps by

typing speed).  The factor-of-forty bandwidth multiplication due to

protocol headers will fool this allocation heuristic and cause these

modems to `thrash'.

From the above, it's clear that one design goal of the compression

should be to limit the bandwidth demand of typing and ack traffic to at

most 300 bps.  A typical maximum typing speed is around five characters

----------------------------

3. See the excellent discussion of two-wire dialup line capacity in

[1], chap. 11.  In particular, there is widespread misunderstanding of

the capabilities of `echo-cancelling' modems (such as those conforming

to CCITT V.32):  Echo-cancellation can offer each side of a two-wire

line the full line bandwidth but, since the far talker's signal adds to

the local `noise', not the full line capacity.  The 22Kbps Shannon limit

is a hard-limit on data rate through a two-wire telephone connection.

Jacobson                                                        [Page 2]

RFC 1144               Compressing TCP/IP Headers          February 1990

per second/4/ which leaves a budget 30 - 5 = 25 characters for headers

or five bytes of header per character typed./5/  Five byte headers solve

problems (1) and (3) directly and, indirectly, problem (2):  A packet

size of 100--200 bytes will easily amortize the cost of a five byte

header and offer a user 95--98% of the line bandwidth for data.  These

short packets mean little interference between interactive and bulk data

traffic (see sec. 5.2).

Another design goal is that the compression protocol be based solely on

information guaranteed to be known to both ends of a single serial link.

Consider the topology shown in fig. 1 where communicating hosts A and B

are on separate local area nets (the heavy black lines) and the nets are

connected by two serial links (the open lines between gateways C--D and

E--F)./6/ One compression possibility would be to convert each TCP/IP

conversation into a semantically equivalent conversation in a protocol

with smaller headers, e.g., to an X.25 call.  But, because of routing

transients or multipathing, it's entirely possible that some of the A--B

traffic will follow the A-C-D-B path and some will follow the A-E-F-B

path.  Similarly, it's possible that A->B traffic will flow A-C-D-B and

B->A traffic will flow B-F-E-A. None of the gateways can count on seeing

all the packets in a particular TCP conversation and a compression

algorithm that works for such a topology cannot be tied to the TCP

connection syntax.

A physical link treated as two, independent, simplex links (one each

direction) imposes the minimum requirements on topology, routing and

pipelining.  The ends of each simplex link only have to agree on the

most recent packet(s) sent on that link.  Thus, although any compression

scheme involves shared state, this state is spatially and temporally

----------------------------

4. See [13].  Typing bursts or multiple character keystrokes such as

cursor keys can exceed this average rate by factors of two to four.

However the bandwidth demand stays approximately constant since the TCP

Nagle algorithm[8] aggregates traffic with a <200ms interarrival time

and the improved header-to-data ratio compensates for the increased

data.

5. A similar analysis leads to essentially the same header size limit

for bulk data transfer ack packets.  Assuming that the MTU has been

selected for `unobtrusive' background file transfers (i.e., chosen so

the packet time is 200--400 ms --- see sec. 5), there can be at most 5

data packets per second in the `high bandwidth' direction.  A reasonable

TCP implementation will ack at most every other data packet so at 5

bytes per ack the reverse channel bandwidth is 2.5 * 5 = 12.5 bytes/sec.

6. Note that although the TCP endpoints are A and B, in this example

compression/decompression must be done at the gateway serial links,

i.e., between C and D and between E and F. Since A and B are using IP,

they cannot know that their communication path includes a low speed

serial link.  It is clearly a requirement that compression not break the

IP model, i.e., that compression function between intermediate systems

and not just between end systems.

Jacobson                                                        [Page 3]

RFC 1144               Compressing TCP/IP Headers          February 1990

local and adheres to Dave Clark's principle of fate sharing[4]:  The two

ends can only disagree on the state if the link connecting them is

inoperable, in which case the disagreement doesn't matter.

3  The compression algorithm

3.1  The basic idea

Figure 2 shows a typical (and minimum length) TCP/IP datagram header./7/

The header size is 40 bytes:  20 bytes of IP and 20 of TCP.

Unfortunately, since the TCP and IP protocols were not designed by a

committee, all these header fields serve some useful purpose and it's

not possible to simply omit some in the name of efficiency.

However, TCP establishes connections and, typically, tens or hundreds of

packets are exchanged on each connection.  How much of the per-packet

information is likely to stay constant over the life of a connection?

Half---the shaded fields in fig. 3.  So, if the sender and receiver keep

track of active connections/8/ and the receiver keeps a copy of the

header from the last packet it saw from each connection, the sender gets

a factor-of-two compression by sending only a small (<= 8 bit)

connection identifier together with the 20 bytes that change and letting

the receiver fill in the 20 fixed bytes from the saved header.

One can scavenge a few more bytes by noting that any reasonable

link-level framing protocol will tell the receiver the length of a

received message so total length (bytes 2 and 3) is redundant.  But then

the header checksum (bytes 10 and 11), which protects individual hops

from processing a corrupted IP header, is essentially the only part of

the IP header being sent.  It seems rather silly to protect the

transmission of information that isn't being transmitted.  So, the

receiver can check the header checksum when the header is actually sent

(i.e., in an uncompressed datagram) but, for compressed datagrams,

regenerate it locally at the same time the rest of the IP header is

being regenerated./9/

----------------------------

7. The TCP and IP protocols and protocol headers are described in [10]

and [11].

8. The 96-bit tuple <src address, dst address, src port, dst port>

uniquely identifies a TCP connection.

9. The IP header checksum is not an end-to-end checksum in the sense

of [14]:  The time-to-live update forces the IP checksum to be

recomputed at each hop.  The author has had unpleasant personal

experience with the consequences of violating the end-to-end argument in

[14] and this protocol is careful to pass the end-to-end TCP checksum

through unmodified.  See sec. 4.

Jacobson                                                        [Page 4]

RFC 1144               Compressing TCP/IP Headers          February 1990

This leaves 16 bytes of header information to send.  All of these bytes

are likely to change over the life of the conversation but they do not

all change at the same time.  For example, during an FTP data transfer

only the packet ID, sequence number and checksum change in the

sender->receiver direction and only the packet ID, ack, checksum and,

possibly, window, change in the receiver->sender direction.  With a copy

of the last packet sent for each connection, the sender can figure out

what fields change in the current packet then send a bitmask indicating

what changed followed by the changing fields./10/

If the sender only sends fields that differ, the above scheme gets the

average header size down to around ten bytes.  However, it's worthwhile

looking at how the fields change:  The packet ID typically comes from a

counter that is incremented by one for each packet sent.  I.e., the

difference between the current and previous packet IDs should be a

small, positive integer, usually <256 (one byte) and frequently = 1.

For packets from the sender side of a data transfer, the sequence number

in the current packet will be the sequence number in the previous packet

plus the amount of data in the previous packet (assuming the packets are

arriving in order).  Since IP packets can be at most 64K, the sequence

number change must be < 2^16 (two bytes).  So, if the differences in the

changing fields are sent rather than the fields themselves, another

three or four bytes per packet can be saved.

That gets us to the five-byte header target.  Recognizing a couple of

special cases will get us three byte headers for the two most common

cases---interactive typing traffic and bulk data transfer---but the

basic compression scheme is the differential coding developed above.

Given that this intellectual exercise suggests it is possible to get

five byte headers, it seems reasonable to flesh out the missing details

and actually implement something.

3.2  The ugly details

3.2.1  Overview

Figure 4 shows a block diagram of the compression software.  The

networking system calls a SLIP output driver with an IP packet to be

----------------------------

10. This is approximately Thinwire-I from [5].  A slight modification

is to do a `delta encoding' where the sender subtracts the previous

packet from the current packet (treating each packet as an array of 16

bit integers), then sends a 20-bit mask indicating the non-zero

differences followed by those differences.  If distinct conversations

are separated, this is a fairly effective compression scheme (e.g.,

typically 12-16 byte headers) that doesn't involve the compressor

knowing any details of the packet structure.  Variations on this theme

have been used, successfully, for a number of years (e.g., the Proteon

router's serial link protocol[3]).

Jacobson                                                        [Page 5]

RFC 1144               Compressing TCP/IP Headers          February 1990

sent over the serial line.  The packet goes through a compressor which

checks if the protocol is TCP. Non-TCP packets and `uncompressible' TCP

packets (described below) are just marked as TYPE_IP and passed to a

framer.  Compressible TCP packets are looked up in an array of packet

headers.  If a matching connection is found, the incoming packet is

compressed, the (uncompressed) packet header is copied into the array,

and a packet of type COMPRESSED_TCP is sent to the framer.  If no match

is found, the oldest entry in the array is discarded, the packet header

is copied into that slot, and a packet of type UNCOMPRESSED_TCP is sent

to the framer.  (An UNCOMPRESSED_TCP packet is identical to the original

IP packet except the IP protocol field is replaced with a connection

number---an index into the array of saved, per-connection packet

headers.  This is how the sender (re-)synchronizes the receiver and

`seeds' it with the first, uncompressed packet of a compressed packet

sequence.)

The framer is responsible for communicating the packet data, type and

boundary (so the decompressor can learn how many bytes came out of the

compressor).  Since the compression is a differential coding, the framer

must not re-order packets (this is rarely a concern over a single serial

link).  It must also provide good error detection and, if connection

numbers are compressed, must provide an error indication to the

decompressor (see sec. 4)./11/

The decompressor does a `switch' on the type of incoming packets:  For

TYPE_IP, the packet is simply passed through.  For UNCOMPRESSED_TCP, the

connection number is extracted from the IP protocol field and

IPPROTO_TCP is restored, then the connection number is used as an index

into the receiver's array of saved TCP/IP headers and the header of the

incoming packet is copied into the indexed slot.  For COMPRESSED_TCP,

the connection number is used as an array index to get the TCP/IP header

of the last packet from that connection, the info in the compressed

packet is used to update that header, then a new packet is constructed

containing the now-current header from the array concatenated with the

data from the compressed packet.

Note that the communication is simplex---no information flows in the

decompressor-to-compressor direction.  In particular, this implies that

the decompressor is relying on TCP retransmissions to correct the saved

state in the event of line errors (see sec. 4).

----------------------------

11. Link level framing is outside the scope of this document.  Any

framing that provides the facilities listed in this paragraph should be

adequate for the compression protocol.  However, the author encourages

potential implementors to see [9] for a proposed, standard, SLIP

framing.

Jacobson                                                        [Page 6]

RFC 1144               Compressing TCP/IP Headers          February 1990

3.2.2  Compressed packet format

Figure 5 shows the format of a compressed TCP/IP packet.  There is a

change mask that identifies which of the fields expected to change

per-packet actually changed, a connection number so the receiver can

locate the saved copy of the last packet for this TCP connection, the

unmodified TCP checksum so the end-to-end data integrity check will

still be valid, then for each bit set in the change mask, the amount the

associated field changed.  (Optional fields, controlled by the mask, are

enclosed in dashed lines in the figure.)  In all cases, the bit is set

if the associated field is present and clear if the field is absent./12/

Since the delta's in the sequence number, etc., are usually small,

particularly if the tuning guidelines in section 5 are followed, all the

numbers are encoded in a variable length scheme that, in practice,

handles most traffic with eight bits:  A change of one through 255 is

represented in one byte.  Zero is improbable (a change of zero is never

sent) so a byte of zero signals an extension:  The next two bytes are

the MSB and LSB, respectively, of a 16 bit value.  Numbers larger than

16 bits force an uncompressed packet to be sent.  For example, decimal

15 is encoded as hex 0f, 255 as ff, 65534 as 00 ff fe, and zero as 00 00

00.  This scheme packs and decodes fairly efficiently:  The usual case

for both encode and decode executes three instructions on a MC680x0.

The numbers sent for TCP sequence number and ack are the difference/13/

between the current value and the value in the previous packet (an

uncompressed packet is sent if the difference is negative or more than

64K). The number sent for the window is also the difference between the

current and previous values.  However, either positive or negative

changes are allowed since the window is a 16 bit field.  The packet's

urgent pointer is sent if URG is set (an uncompressed packet is sent if

the urgent pointer changes but URG is not set).  For packet ID, the

number sent is the difference between the current and previous values.

However, unlike the rest of the compressed fields, the assumed change

when I is clear is one, not zero.

There are two important special cases:

(1) The sequence number and ack both change by the amount of data in the

last packet; no window change or URG.

(2) The sequence number changes by the amount of data in the last

packet, no ack or window change or URG.

----------------------------

12. The bit `P' in the figure is different from the others:  It is a

copy of the `PUSH' bit from the TCP header.  `PUSH' is a curious

anachronism considered indispensable by certain members of the Internet

community.  Since PUSH can (and does) change in any datagram, an

information preserving compression scheme must pass it explicitly.

13. All differences are computed using two's complement arithmetic.

Jacobson                                                        [Page 7]

RFC 1144               Compressing TCP/IP Headers          February 1990

(1) is the case for echoed terminal traffic.  (2) is the sender side of

non-echoed terminal traffic or a unidirectional data transfer.  Certain

combinations of the S, A, W and U bits of the change mask are used to

signal these special cases.  `U' (urgent data) is rare so two unlikely

combinations are S W U (used for case 1) and S A W U (used for case 2).

To avoid ambiguity, an uncompressed packet is sent if the actual changes

in a packet are S * W U.

Since the `active' connection changes rarely (e.g., a user will type for

several minutes in a telnet window before changing to a different

window), the C bit allows the connection number to be elided.  If C is

clear, the connection is assumed to be the same as for the last

compressed or uncompressed packet.  If C is set, the connection number

is in the byte immediately following the change mask./14/

From the above, it's probably obvious that compressed terminal traffic

usually looks like (in hex):  0B c c d, where the 0B indicates case (1),

c c is the two byte TCP checksum and d is the character typed.  Commands

to vi or emacs, or packets in the data transfer direction of an FTP

`put' or `get' look like 0F c c d ... , and acks for that FTP look like

04 c c a where a is the amount of data being acked./15/

3.2.3  Compressor processing

The compressor is called with the IP packet to be processed and the

compression state structure for the outgoing serial line.  It returns a

packet ready for final framing and the link level `type' of that packet.

As the last section noted, the compressor converts every input packet

into either a TYPE_IP, UNCOMPRESSED_TCP or COMPRESSED_TCP packet.  A

----------------------------

14. The connection number is limited to one byte, i.e., 256

simultaneously active TCP connections.  In almost two years of

operation, the author has never seen a case where more than sixteen

connection states would be useful (even in one case where the SLIP link

was used as a gateway behind a very busy, 64-port terminal multiplexor).

Thus this does not seem to be a significant restriction and allows the

protocol field in UNCOMPRESSED_TCP packets to be used for the connection

number, simplifying the processing of those packets.

15. It's also obvious that the change mask changes infrequently and

could often be elided.  In fact, one can do slightly better by saving

the last compressed packet (it can be at most 16 bytes so this isn't

much additional state) and checking to see if any of it (except the TCP

checksum) has changed.  If not, send a packet type that means

`compressed TCP, same as last time' and a packet containing only the

checksum and data.  But, since the improvement is at most 25%, the added

complexity and state doesn't seem justified.  See appendix C.

Jacobson                                                        [Page 8]

RFC 1144               Compressing TCP/IP Headers          February 1990

TYPE_IP packet is an unmodified copy/16/ of the input packet and

processing it doesn't change the compressor's state in any way.

An UNCOMPRESSED_TCP packet is identical to the input packet except the

IP protocol field (byte 9) is changed from `6' (protocol TCP) to a

connection number.  In addition, the state slot associated with the

connection number is updated with a copy of the input packet's IP and

TCP headers and the connection number is recorded as the last connection

sent on this serial line (for the C compression described below).

A COMPRESSED_TCP packet contains the data, if any, from the original

packet but the IP and TCP headers are completely replaced with a new,

compressed header.  The connection state slot and last connection sent

are updated by the input packet exactly as for an UNCOMPRESSED_TCP

packet.

The compressor's decision procedure is:

- If the packet is not protocol TCP, send it as TYPE_IP.

- If the packet is an IP fragment (i.e., either the fragment offset

field is non-zero or the more fragments bit is set), send it as

TYPE_IP./17/

- If any of the TCP control bits SYN, FIN or RST are set or if the ACK

bit is clear, consider the packet uncompressible and send it as

TYPE_IP./18/

----------------------------

16. It is not necessary (or desirable) to actually duplicate the input

packet for any of the three output types.  Note that the compressor

cannot increase the size of a datagram.  As the code in appendix A

shows, the protocol can be implemented so all header modifications are

made `in place'.

17. Only the first fragment contains the TCP header so the fragment

offset check is necessary.  The first fragment might contain a complete

TCP header and, thus, could be compressed.  However the check for a

complete TCP header adds quite a lot of code and, given the arguments in

[6], it seems reasonable to send all IP fragments uncompressed.

18. The ACK test is redundant since a standard conforming

implementation must set ACK in all packets except for the initial SYN

packet.  However, the test costs nothing and avoids turning a bogus

packet into a valid one.

SYN packets are not compressed because only half of them contain a valid

ACK field and they usually contain a TCP option (the max. segment size)

which the following packets don't.  Thus the next packet would be sent

uncompressed because the TCP header length changed and sending the SYN

as UNCOMPRESSED_TCP instead of TYPE_IP would buy nothing.

The decision to not compress FIN packets is questionable.  Discounting

the trick in appendix B.1, there is a free bit in the header that could

be used to communicate the FIN flag.  However, since connections tend to

Jacobson                                                        [Page 9]

RFC 1144               Compressing TCP/IP Headers          February 1990

If a packet makes it through the above checks, it will be sent as either

UNCOMPRESSED_TCP or COMPRESSED_TCP:

- If no connection state can be found that matches the packet's source

and destination IP addresses and TCP ports, some state is reclaimed

(which should probably be the least recently used) and an

UNCOMPRESSED_TCP packet is sent.

- If a connection state is found, the packet header it contains is

checked against the current packet to make sure there were no

unexpected changes.  (E.g., that all the shaded fields in fig. 3 are

the same).  The IP protocol, fragment offset, more fragments, SYN,

FIN and RST fields were checked above and the source and destination

address and ports were checked as part of locating the state.  So

the remaining fields to check are protocol version, header length,

type of service, don't fragment, time-to-live, data offset, IP

options (if any) and TCP options (if any).  If any of these fields

differ between the two headers, an UNCOMPRESSED_TCP packet is sent.

If all the `unchanging' fields match, an attempt is made to compress the

current packet:

- If the URG flag is set, the urgent data field is encoded (note that

it may be zero) and the U bit is set in the change mask.

Unfortunately, if URG is clear, the urgent data field must be

checked against the previous packet and, if it changes, an

UNCOMPRESSED_TCP packet is sent.  (`Urgent data' shouldn't change

when URG is clear but [11] doesn't require this.)

- The difference between the current and previous packet's window

field is computed and, if non-zero, is encoded and the W bit is set

in the change mask.

- The difference between ack fields is computed.  If the result is

less than zero or greater than 2^16 - 1, an UNCOMPRESSED_TCP packet

is sent./19/  Otherwise, if the result is non-zero, it is encoded

and the A bit is set in the change mask.

- The difference between sequence number fields is computed.  If the

result is less than zero or greater than 2^16 - 1, an

----------------------------

last for many packets, it seemed unreasonable to dedicate an entire bit

to a flag that would only appear once in the lifetime of the connection.

19. The two tests can be combined into a single test of the most

significant 16 bits of the difference being non-zero.

Jacobson                                                       [Page 10]

RFC 1144               Compressing TCP/IP Headers          February 1990

UNCOMPRESSED_TCP packet is sent./20/  Otherwise, if the result is

non-zero, it is encoded and the S bit is set in the change mask.

Once the U, W, A and S changes have been determined, the special-case

encodings can be checked:

- If U, S and W are set, the changes match one of the special-case

encodings.  Send an UNCOMPRESSED_TCP packet.

- If only S is set, check if the change equals the amount of user data

in the last packet.  I.e., subtract the TCP and IP header lengths

from the last packet's total length field and compare the result to

the S change.  If they're the same, set the change mask to SAWU (the

special case for `unidirectional data transfer') and discard the

encoded sequence number change (the decompressor can reconstruct it

since it knows the last packet's total length and header length).

- If only S and A are set, check if they both changed by the same

amount and that amount is the amount of user data in the last

packet.  If so, set the change mask to SWU (the special case for

`echoed interactive' traffic) and discard the encoded changes.

- If nothing changed, check if this packet has no user data (in which

case it is probably a duplicate ack or window probe) or if the

previous packet contained user data (which means this packet is a

retransmission on a connection with no pipelining).  In either of

these cases, send an UNCOMPRESSED_TCP packet.

Finally, the TCP/IP header on the outgoing packet is replaced with a

compressed header:

- The change in the packet ID is computed and, if not one,/21/ the

difference is encoded (note that it may be zero or negative) and the

I bit is set in the change mask.

- If the PUSH bit is set in the original datagram, the P bit is set in

the change mask.

- The TCP and IP headers of the packet are copied to the connection

state slot.

----------------------------

20. A negative sequence number change probably indicates a

retransmission.  Since this may be due to the decompressor having

dropped a packet, an uncompressed packet is sent to re-sync the

decompressor (see sec. 4).

21. Note that the test here is against one, not zero.  The packet ID is

typically incremented by one for each packet sent so a change of zero is

very unlikely.  A change of one is likely:  It occurs during any period

when the originating system has activity on only one connection.

Jacobson                                                       [Page 11]

RFC 1144               Compressing TCP/IP Headers          February 1990

- The TCP and IP headers of the packet are discarded and a new header

is prepended consisting of (in reverse order):

- the accumulated, encoded changes.

- the TCP checksum (if the new header is being constructed `in

place', the checksum may have been overwritten and will have to

be taken from the header copy in the connection state or saved

in a temporary before the original header is discarded).

- the connection number (if different than the last one sent on

this serial line).  This also means that the the line's last

connection sent must be set to the connection number and the C

bit set in the change mask.

- the change mask.

At this point, the compressed TCP packet is passed to the framer for

transmission.

3.2.4  Decompressor processing

Because of the simplex communication model, processing at the

decompressor is much simpler than at the compressor --- all the

decisions have been made and the decompressor simply does what the

compressor has told it to do.

The decompressor is called with the incoming packet,/22/ the length and

type of the packet and the compression state structure for the incoming

serial line.  A (possibly re-constructed) IP packet will be returned.

The decompressor can receive four types of packet:  the three generated

by the compressor and a TYPE_ERROR pseudo-packet generated when the

receive framer detects an error./23/  The first step is a `switch' on

the packet type:

- If the packet is TYPE_ERROR or an unrecognized type, a `toss' flag

is set in the state to force COMPRESSED_TCP packets to be discarded

until one with the C bit set or an UNCOMPRESSED_TCP packet arrives.

Nothing (a null packet) is returned.

----------------------------

22. It's assumed that link-level framing has been removed by this point

and the packet and length do not include type or framing bytes.

23. No data need be associated with a TYPE_ERROR packet.  It exists so

the receive framer can tell the decompressor that there may be a gap in

the data stream.  The decompressor uses this as a signal that packets

should be tossed until one arrives with an explicit connection number (C

bit set).  See the last part of sec. 4.1 for a discussion of why this is

necessary.

Jacobson                                                       [Page 12]

RFC 1144               Compressing TCP/IP Headers          February 1990

- If the packet is TYPE_IP, an unmodified copy of it is returned and

the state is not modified.

- If the packet is UNCOMPRESSED_TCP, the state index from the IP

protocol field is checked./24/  If it's illegal, the toss flag is

set and nothing is returned.  Otherwise, the toss flag is cleared,

the index is copied to the state's last connection received field, a

copy of the input packet is made,/25/ the TCP protocol number is

restored to the IP protocol field, the packet header is copied to

the indicated state slot, then the packet copy is returned.

If the packet was not handled above, it is COMPRESSED_TCP and a new

TCP/IP header has to be synthesized from information in the packet plus

the last packet's header in the state slot.  First, the explicit or

implicit connection number is used to locate the state slot:

- If the C bit is set in the change mask, the state index is checked.

If it's illegal, the toss flag is set and nothing is returned.

Otherwise, last connection received is set to the packet's state

index and the toss flag is cleared.

- If the C bit is clear and the toss flag is set, the packet is

ignored and nothing is returned.

At this point, last connection received is the index of the appropriate

state slot and the first byte(s) of the compressed packet (the change

mask and, possibly, connection index) have been consumed.  Since the

TCP/IP header in the state slot must end up reflecting the newly arrived

packet, it's simplest to apply the changes from the packet to that

header then construct the output packet from that header concatenated

with the data from the input packet.  (In the following description,

`saved header' is used as an abbreviation for `the TCP/IP header saved

in the state slot'.)

- The next two bytes in the incoming packet are the TCP checksum.

They are copied to the saved header.

- If the P bit is set in the change mask, the TCP PUSH bit is set in

the saved header.  Otherwise the PUSH bit is cleared.

----------------------------

24. State indices follow the C language convention and run from 0 to N

- 1, where 0 < N <= 256 is the number of available state slots.

25. As with the compressor, the code can be structured so no copies are

done and all modifications are done in-place.  However, since the output

packet can be larger than the input packet, 128 bytes of free space must

be left at the front of the input packet buffer to allow room to prepend

the TCP/IP header.

Jacobson                                                       [Page 13]

RFC 1144               Compressing TCP/IP Headers          February 1990

- If the low order four bits (S, A, W and U) of the change mask are

all set (the `unidirectional data' special case), the amount of user

data in the last packet is calculated by subtracting the TCP and IP

header lengths from the IP total length in the saved header.  That

amount is then added to the TCP sequence number in the saved header.

- If S, W and U are set and A is clear (the `terminal traffic' special

case), the amount of user data in the last packet is calculated and

added to both the TCP sequence number and ack fields in the saved

header.

- Otherwise, the change mask bits are interpreted individually in the

order that the compressor set them:

- If the U bit is set, the TCP URG bit is set in the saved header

and the next byte(s) of the incoming packet are decoded and

stuffed into the TCP Urgent Pointer.  If the U bit is clear, the

TCP URG bit is cleared.

- If the W bit is set, the next byte(s) of the incoming packet are

decoded and added to the TCP window field of the saved header.

- If the A bit is set, the next byte(s) of the incoming packet are

decoded and added to the TCP ack field of the saved header.

- If the S bit is set, the next byte(s) of the incoming packet are

decoded and added to the TCP sequence number field of the saved

header.

- If the I bit is set in the change mask, the next byte(s) of the

incoming packet are decoded and added to the IP ID field of the

saved packet.  Otherwise, one is added to the IP ID.

At this point, all the header information from the incoming packet has

been consumed and only data remains.  The length of the remaining data

is added to the length of the saved IP and TCP headers and the result is

put into the saved IP total length field.  The saved IP header is now up

to date so its checksum is recalculated and stored in the IP checksum

field.  Finally, an output datagram consisting of the saved header

concatenated with the remaining incoming data is constructed and

returned.

4  Error handling

4.1  Error detection

In the author's experience, dialup connections are particularly prone to

data errors.  These errors interact with compression in two different

ways:

Jacobson                                                       [Page 14]

RFC 1144               Compressing TCP/IP Headers          February 1990

First is the local effect of an error in a compressed packet.  All error

detection is based on redundancy yet compression has squeezed out almost

all the redundancy in the TCP and IP headers.  In other words, the

decompressor will happily turn random line noise into a perfectly valid

TCP/IP packet./26/  One could rely on the TCP checksum to detect

corrupted compressed packets but, unfortunately, some rather likely

errors will not be detected.  For example, the TCP checksum will often

not detect two single bit errors separated by 16 bits.  For a V.32 modem

signalling at 2400 baud with 4 bits/baud, any line hit lasting longer

than 400us. would corrupt 16 bits.  According to [2], residential phone

line hits of up to 2ms. are likely.

The correct way to deal with this problem is to provide for error

detection at the framing level.  Since the framing (at least in theory)

can be tailored to the characteristics of a particular link, the

detection can be as light or heavy-weight as appropriate for that

link./27/  Since packet error detection is done at the framing level,

the decompressor simply assumes that it will get an indication that the

current packet was received with errors.  (The decompressor always

ignores (discards) a packet with errors.  However, the indication is

needed to prevent the error being propagated --- see below.)

The `discard erroneous packets' policy gives rise to the second

interaction of errors and compression.  Consider the following

conversation:

+-------------------------------------------+

|original | sent   |received |reconstructed |

+---------+--------+---------+--------------+

| 1:  A   | 1:  A  | 1:  A   | 1:  A        |

| 2:  BC  | 1,  BC | 1,  BC  | 2:  BC       |

| 4:  DE  | 2,  DE |  ---    |  ---         |

| 6:  F   | 2,  F  | 2,  F   | 4:  F        |

| 7:  GH  | 1,  GH | 1,  GH  | 5:  GH       |

+-------------------------------------------+

(Each entry above has the form `starting sequence number:data sent' or

`?sequence number change,data sent'.)  The first thing sent is an

uncompressed packet, followed by four compressed packets.  The third

packet picks up an error and is discarded.  To reconstruct the fourth

packet, the receiver applies the sequence number change from incoming

compressed packet to the sequence number of the last correctly received

----------------------------

26. modulo the TCP checksum.

27. While appropriate error detection is link dependent, the CCITT CRC

used in [9] strikes an excellent balance between ease of computation and

robust error detection for a large variety of links, particularly at the

relatively small packet sizes needed for good interactive response.

Thus, for the sake of interoperability, the framing in [9] should be

used unless there is a truly compelling reason to do otherwise.

Jacobson                                                       [Page 15]

RFC 1144               Compressing TCP/IP Headers          February 1990

packet, packet two, and generates an incorrect sequence number for

packet four.  After the error, all reconstructed packets' sequence

numbers will be in error, shifted down by the amount of data in the

missing packet./28/

Without some sort of check, the preceding error would result in the

receiver invisibly losing two bytes from the middle of the transfer

(since the decompressor regenerates sequence numbers, the packets

containing F and GH arrive at the receiver's TCP with exactly the

sequence numbers they would have had if the DE packet had never

existed).  Although some TCP conversations can survive missing data/29/

it is not a practice to be encouraged.  Fortunately the TCP checksum,

since it is a simple sum of the packet contents including the sequence

numbers, detects 100% of these errors.  E.g., the receiver's computed

checksum for the last two packets above always differs from the packet

checksum by two.

Unfortunately, there is a way for the TCP checksum protection described

above to fail if the changes in an incoming compressed packet are

applied to the wrong conversation:  Consider two active conversations C1

and C2 and a packet from C1 followed by two packets from C2.  Since the

connection number doesn't change, it's omitted from the second C2

packet.  But, if the first C2 packet is received with a CRC error, the

second C2 packet will mistakenly be considered the next packet in C1.

Since the C2 checksum is a random number with respect to the C1 sequence

numbers, there is at least a 2^-16 probability that this packet will be

accepted by the C1 TCP receiver./30/  To prevent this, after a CRC error

indication from the framer the receiver discards packets until it

receives either a COMPRESSED_TCP packet with the C bit set or an

UNCOMPRESSED_TCP packet.  I.e., packets are discarded until the receiver

gets an explicit connection number.

To summarize this section, there are two different types of errors:

per-packet corruption and per-conversation loss-of-sync.  The first type

is detected at the decompressor from a link-level CRC error, the second

at the TCP receiver from a (guaranteed) invalid TCP checksum.  The

combination of these two independent mechanisms ensures that erroneous

packets are discarded.

----------------------------

28. This is an example of a generic problem with differential or delta

encodings known as `losing DC'.

29. Many system managers claim that holes in an NNTP stream are more

valuable than the data.

30. With worst-case traffic, this probability translates to one

undetected error every three hours over a 9600 baud line with a 30%

error rate).

Jacobson                                                       [Page 16]

RFC 1144               Compressing TCP/IP Headers          February 1990

4.2  Error recovery

The previous section noted that after a CRC error the decompressor will

introduce TCP checksum errors in every uncompressed packet.  Although

the checksum errors prevent data stream corruption, the TCP conversation

won't be terribly useful until the decompressor again generates valid

packets.  How can this be forced to happen?

The decompressor generates invalid packets because its state (the saved

`last packet header') disagrees with the compressor's state.  An

UNCOMPRESSED_TCP packet will correct the decompressor's state.  Thus

error recovery amounts to forcing an uncompressed packet out of the

compressor whenever the decompressor is (or might be) confused.

The first thought is to take advantage of the full duplex communication

link and have the decompressor send something to the compressor

requesting an uncompressed packet.  This is clearly undesirable since it

constrains the topology more than the minimum suggested in sec. 2 and

requires that a great deal of protocol be added to both the decompressor

and compressor.  A little thought convinces one that this alternative is

not only undesirable, it simply won't work:  Compressed packets are

small and it's likely that a line hit will so completely obliterate one

that the decompressor will get nothing at all.  Thus packets are

reconstructed incorrectly (because of the missing compressed packet) but

only the TCP end points, not the decompressor, know that the packets are

incorrect.

But the TCP end points know about the error and TCP is a reliable

protocol designed to run over unreliable media.  This means the end

points must eventually take some sort of error recovery action and

there's an obvious trigger for the compressor to resync the

decompressor:  send uncompressed packets whenever TCP is doing error

recovery.

But how does the compressor recognize TCP error recovery?  Consider the

schematic TCP data transfer of fig. 6.    The confused decompressor is

in the forward (data transfer) half of the TCP conversation.  The

receiving TCP discards packets rather than acking them (because of the

checksum errors), the sending TCP eventually times out and retransmits a

packet, and the forward path compressor finds that the difference

between the sequence number in the retransmitted packet and the sequence

number in the last packet seen is either negative (if there were

multiple packets in transit) or zero (one packet in transit).  The first

case is detected in the compression step that computes sequence number

differences.  The second case is detected in the step that checks the

`special case' encodings but needs an additional test:  It's fairly

common for an interactive conversation to send a dataless ack packet

followed by a data packet.  The ack and data packet will have the same

sequence numbers yet the data packet is not a retransmission.  To

prevent sending an unnecessary uncompressed packet, the length of the

previous packet should be checked and, if it contained data, a zero

Jacobson                                                       [Page 17]

RFC 1144               Compressing TCP/IP Headers          February 1990

sequence number change must indicate a retransmission.

A confused decompressor in the reverse (ack) half of the conversation is

as easy to detect (fig. 7):    The sending TCP discards acks (because

they contain checksum errors), eventually times out, then retransmits

some packet.  The receiving TCP thus gets a duplicate packet and must

generate an ack for the next expected sequence number[11, p. 69].  This

ack will be a duplicate of the last ack the receiver generated so the

reverse-path compressor will find no ack, seq number, window or urg

change.  If this happens for a packet that contains no data, the

compressor assumes it is a duplicate ack sent in response to a

retransmit and sends an UNCOMPRESSED_TCP packet./31/

5  Configurable parameters and tuning

5.1  Compression configuration

There are two configuration parameters associated with header

compression:  Whether or not compressed packets should be sent on a

particular line and, if so, how many state slots (saved packet headers)

to reserve.  There is also one link-level configuration parameter, the

maximum packet size or MTU, and one front-end configuration parameter,

data compression, that interact with header compression.  Compression

configuration is discussed in this section.  MTU and data compression

are discussed in the next two sections.

There are some hosts (e.g., low end PCs) which may not have enough

processor or memory resources to implement this compression.  There are

also rare link or application characteristics that make header

compression unnecessary or undesirable.  And there are many existing

SLIP links that do not currently use this style of header compression.

For the sake of interoperability, serial line IP drivers that allow

header compression should include some sort of user configurable flag to

disable compression (see appendix B.2)./32/

If compression is enabled, the compressor must be sure to never send a

connection id (state index) that will be dropped by the decompressor.

E.g., a black hole is created if the decompressor has sixteen slots and

----------------------------

31. The packet could be a zero-window probe rather than a retransmitted

ack but window probes should be infrequent and it does no harm to send

them uncompressed.

32. The PPP protocol in [9] allows the end points to negotiate

compression so there is no interoperability problem.  However, there

should still be a provision for the system manager at each end to

control whether compression is negotiated on or off.  And, obviously,

compression should default to `off' until it has been negotiated `on'.

Jacobson                                                       [Page 18]

RFC 1144               Compressing TCP/IP Headers          February 1990

the compressor uses twenty./33/  Also, if the compressor is allowed too

few slots, the LRU allocator will thrash and most packets will be sent

as UNCOMPRESSED_TCP. Too many slots and memory is wasted.

Experimenting with different sizes over the past year, the author has

found that eight slots will thrash (i.e., the performance degradation is

noticeable) when many windows on a multi-window workstation are

simultaneously in use or the workstation is being used as a gateway for

three or more other machines.  Sixteen slots were never observed to

thrash.  (This may simply be because a 9600 bps line split more than 16

ways is already so overloaded that the additional degradation from

round-robbining slots is negligible.)

Each slot must be large enough to hold a maximum length TCP/IP header of

128 bytes/34/ so 16 slots occupy 2KB of memory.  In these days of 4 Mbit

RAM chips, 2KB seems so little memory that the author recommends the

following configuration rules:

(1) If the framing protocol does not allow negotiation, the compressor

and decompressor should provide sixteen slots, zero through fifteen.

(2) If the framing protocol allows negotiation, any mutually agreeable

number of slots from 1 to 256 should be negotiable./35/  If number

of slots is not negotiated, or until it is negotiated, both sides

should assume sixteen.

(3) If you have complete control of all the machines at both ends of

every link and none of them will ever be used to talk to machines

outside of your control, you are free to configure them however you

please, ignoring the above.  However, when your little eastern-block

dictatorship collapses (as they all eventually seem to), be aware

that a large, vocal, and not particularly forgiving Internet

community will take great delight in pointing out to anyone willing

----------------------------

33. Strictly speaking, there's no reason why the connection id should

be treated as an array index.  If the decompressor's states were kept in

a hash table or other associative structure, the connection id would be

a key, not an index, and performance with too few decompressor slots

would only degrade enormously rather than failing altogether.  However,

an associative structure is substantially more costly in code and cpu

time and, given the small per-slot cost (128 bytes of memory), it seems

reasonable to design for slot arrays at the decompressor and some

(possibly implicit) communication of the array size.

34. The maximum header length, fixed by the protocol design, is 64

bytes of IP and 64 bytes of TCP.

35. Allowing only one slot may make the compressor code more complex.

Implementations should avoid offering one slot if possible and

compressor implementations may disable compression if only one slot is

negotiated.

Jacobson                                                       [Page 19]

RFC 1144               Compressing TCP/IP Headers          February 1990

to listen that you have misconfigured your systems and are not

interoperable.

5.2  Choosing a maximum transmission unit

From the discussion in sec. 2, it seems desirable to limit the maximum

packet size (MTU) on any line where there might be interactive traffic

and multiple active connections (to maintain good interactive response

between the different connections competing for the line).  The obvious

question is `how much does this hurt throughput?'  It doesn't.

Figure 8 shows how user data throughput/36/ scales with MTU with (solid

line) and without (dashed line) header compression.  The dotted lines

show what MTU corresponds to a 200 ms packet time at 2400, 9600 and

19,200 bps.  Note that with header compression even a 2400 bps line can

be responsive yet have reasonable throughput (83%)./37/

Figure 9 shows how line efficiency scales with increasing line speed,

assuming that a 200ms. MTU is always chosen./38/  The knee in the

performance curve is around 2400 bps.  Below this, efficiency is

sensitive to small changes in speed (or MTU since the two are linearly

related) and good efficiency comes at the expense of good response.

Above 2400bps the curve is flat and efficiency is relatively independent

of speed or MTU. In other words, it is possible to have both good

response and high line efficiency.

To illustrate, note that for a 9600 bps line with header compression

there is essentially no benefit in increasing the MTU beyond 200 bytes:

If the MTU is increased to 576, the average delay increases by 188%

while throughput only improves by 3% (from 96 to 99%).

----------------------------

36. The vertical axis is in percent of line speed.  E.g., `95' means

that 95% of the line bandwidth is going to user data or, in other words,

the user would see a data transfer rate of 9120 bps on a 9600 bps line.

Four bytes of link-level (framer) encapsulation in addition to the

TCP/IP or compressed header were included when calculating the relative

throughput.  The 200 ms packet times were computed assuming an

asynchronous line using 10 bits per character (8 data bits, 1 start, 1

stop, no parity).

37. However, the 40 byte TCP MSS required for a 2400 bps line might

stress-test your TCP implementation.

38. For a typical async line, a 200ms. MTU is simply .02 times the line

speed in bits per second.

Jacobson                                                       [Page 20]

RFC 1144               Compressing TCP/IP Headers          February 1990

5.3  Interaction with data compression

Since the early 1980's, fast, effective, data compression algorithms

such as Lempel-Ziv[7] and programs that embody them, such as the

compress program shipped with Berkeley Unix, have become widely

available.  When using low speed or long haul lines, it has become

common practice to compress data before sending it.  For dialup

connections, this compression is often done in the modems, independent

of the communicating hosts.  Some interesting issues would seem to be:

(1) Given a good data compressor, is there any need for header

compression?  (2) Does header compression interact with data

compression?  (3) Should data be compressed before or after header

compression?/39/

To investigate (1), Lempel-Ziv compression was done on a trace of 446

TCP/IP packets taken from the user's side of a typical telnet

conversation.  Since the packets resulted from typing, almost all

contained only one data byte plus 40 bytes of header.  I.e., the test

essentially measured L-Z compression of TCP/IP headers.  The compression

ratio (the ratio of uncompressed to compressed data) was 2.6.  In other

words, the average header was reduced from 40 to 16 bytes.  While this

is good compression, it is far from the 5 bytes of header needed for

good interactive response and far from the 3 bytes of header (a

compression ratio of 13.3) that header compression yielded on the same

packet trace.

The second and third questions are more complex.  To investigate them,

several packet traces from FTP file transfers were analyzed/40/ with and

without header compression and with and without L-Z compression.  The

L-Z compression was tried at two places in the outgoing data stream

(fig. 10):    (1) just before the data was handed to TCP for

encapsulation (simulating compression done at the `application' level)

and (2) after the data was encapsulated (simulating compression done in

the modem).  Table 1 summarizes the results for a 78,776 byte ASCII text

file (the Unix csh.1 manual entry)/41/ transferred using the guidelines

of the previous section (256 byte MTU or 216 byte MSS; 368 packets

total).  Compression ratios for the following ten tests are shown

(reading left to right and top to bottom):

----------------------------

39. The answers, for those who wish to skip the remainder of this

section, are `yes', `no' and `either', respectively.

40. The data volume from user side of a telnet is too small to benefit

from data compression and can be adversely affected by the delay most

compression algorithms (necessarily) add.  The statistics and volume of

the computer side of a telnet are similar to an (ASCII) FTP so these

results should apply to either.

41. The ten experiments described were each done on ten ASCII files

(four long e-mail messages, three Unix C source files and three Unix

manual entries).  The results were remarkably similar for different

files and the general conclusions reached below apply to all ten files.

Jacobson                                                       [Page 21]

RFC 1144               Compressing TCP/IP Headers          February 1990

- data file (no compression or encapsulation)

- data -> L--Z compressor

- data -> TCP/IP encapsulation

- data -> L--Z -> TCP/IP

- data -> TCP/IP -> L--Z

- data -> L--Z -> TCP/IP -> L--Z

- data -> TCP/IP -> Hdr. Compress.

- data -> L--Z -> TCP/IP -> Hdr. Compress.

- data -> TCP/IP -> Hdr. Compress. -> L--Z

- data -> L--Z -> TCP/IP -> Hdr. Compress. -> L--Z

+-----------------------------------------------------+

|              | No data  | L--Z   |  L--Z  |  L--Z   |

|              |compress. |on data |on wire | on both |

+--------------+----------+--------+--------+---------+

| Raw Data     |     1.00 |   2.44 |   ---- |    ---- |

| + TCP Encap. |     0.83 |   2.03 |   1.97 |    1.58 |

| w/Hdr Comp.  |     0.98 |   2.39 |   2.26 |    1.66 |

+-----------------------------------------------------+

Table 1:  ASCII Text File Compression Ratios

The first column of table 1 says the data expands by 19% (`compresses'

by .83) when encapsulated in TCP/IP and by 2% when encapsulated in

header compressed TCP/IP./42/ The first row says L--Z compression is

quite effective on this data, shrinking it to less than half its

original size.  Column four illustrates the well-known fact that it is a

mistake to L--Z compress already compressed data.  The interesting

information is in rows two and three of columns two and three.  These

columns say that the benefit of data compression overwhelms the cost of

encapsulation, even for straight TCP/IP. They also say that it is

slightly better to compress the data before encapsulating it rather than

compressing at the framing/modem level.  The differences however are

----------------------------

42. This is what would be expected from the relative header sizes:

256/216 for TCP/IP and 219/216 for header compression.

Jacobson                                                       [Page 22]

RFC 1144               Compressing TCP/IP Headers          February 1990

small --- 3% and 6%, respectively, for the TCP/IP and header compressed

encapsulations./43/

Table 2 shows the same experiment for a 122,880 byte binary file (the

Sun-3 ps executable).  Although the raw data doesn't compress nearly as

well, the results are qualitatively the same as for the ASCII data.  The

one significant change is in row two:  It is about 3% better to compress

the data in the modem rather than at the source if doing TCP/IP

encapsulation (apparently, Sun binaries and TCP/IP headers have similar

statistics).  However, with header compression (row three) the results

were similar to the ASCII data --- it's about 3% worse to compress at

the modem rather than the source./44/

+-----------------------------------------------------+

|              | No data  | L--Z   |  L--Z  |  L--Z   |

|              |compress. |on data |on wire | on both |

+--------------+----------+--------+--------+---------+

| Raw Data     |     1.00 |   1.72 |   ---- |    ---- |

| + TCP Encap. |     0.83 |   1.43 |   1.48 |    1.21 |

| w/Hdr Comp.  |     0.98 |   1.69 |   1.64 |    1.28 |

+-----------------------------------------------------+

Table 2:  Binary File Compression Ratios

6  Performance measurements

An implementation goal of compression code was to arrive at something

simple enough to run at ISDN speeds (64Kbps) on a typical 1989

----------------------------

43. The differences are due to the wildly different byte patterns of

TCP/IP datagrams and ASCII text.  Any compression scheme with an

underlying, Markov source model, such as Lempel-Ziv, will do worse when

radically different sources are interleaved.  If the relative

proportions of the two sources are changed, i.e., the MTU is increased,

the performance difference between the two compressor locations

decreases.  However, the rate of decrease is very slow --- increasing

the MTU by 400% (256 to 1024) only changed the difference between the

data and modem L--Z choices from 2.5% to 1.3%.

44. There are other good reasons to compress at the source:  Far fewer

packets have to be encapsulated and far fewer characters have to be sent

to the modem.  The author suspects that the `compress data in the modem'

alternative should be avoided except when faced with an intractable,

vendor proprietary operating system.

Jacobson                                                       [Page 23]

RFC 1144               Compressing TCP/IP Headers          February 1990

+---------------------------------------+

|               |  Average per-packet   |

|    Machine    | processing time (us.) |

|               |                       |

|               | Compress | Decompress |

+---------------+----------+------------+

|Sparcstation-1 |       24 |         18 |

|   Sun 4/260   |       46 |         20 |

|   Sun 3/60    |       90 |         90 |

|   Sun 3/50    |      130 |        150 |

|  HP9000/370   |       42 |         33 |

|  HP9000/360   |       68 |         70 |

|   DEC 3100    |       27 |         25 |

|    Vax 780    |      430 |        300 |

|    Vax 750    |      800 |        500 |

|   CCI Tahoe   |      110 |        140 |

+---------------------------------------+

Table 3:  Compression code timings

workstation.  64Kbps is a byte every 122us so 120us was (arbitrarily)

picked as the target compression/decompression time./45/

As part of the compression code development, a trace-driven exerciser

was developed.  This was initially used to compare different compression

protocol choices then later to test the code on different computer

architectures and do regression tests after performance `improvements'.

A small modification of this test program resulted in a useful

measurement tool./46/  Table 3 shows the result of timing the

compression code on all the machines available to the author (times were

measured using a mixed telnet/ftp traffic trace).  With the exception of

the Vax architectures, which suffer from (a) having bytes in the wrong

order and (b) a lousy compiler (Unix pcc), all machines essentially met

the 120us goal.

----------------------------

45. The time choice wasn't completely arbitrary:  Decompression is

often done during the inter-frame `flag' character time so, on systems

where the decompression is done at the same priority level as the serial

line input interrupt, times much longer than a character time would

result in receiver overruns.  And, with the current average of five byte

frames (on the wire, including both compressed header and framing), a

compression/decompression that takes one byte time can use at most 20%

of the available time.  This seems like a comfortable budget.

46. Both the test program and timer program are included in the

ftp-able package described in appendix A as files tester.c and timer.c.

Jacobson                                                       [Page 24]

RFC 1144               Compressing TCP/IP Headers          February 1990

7  Acknowlegements

The author is grateful to the members of the Internet Engineering Task

Force, chaired by Phill Gross, who provided encouragement and thoughtful

review of this work.  Several patient beta-testers, particularly Sam

Leffler and Craig Leres, tracked down and fixed problems in the initial

implementation.  Cynthia Livingston and Craig Partridge carefully read

and greatly improved an unending sequence of partial drafts of this

document.  And last but not least, Telebit modem corporation,

particularly Mike Ballard, encouraged this work from its inception and

has been an ongoing champion of serial line and dial-up IP.

References

[1] Bingham, J. A. C. Theory and Practice of Modem Design. John Wiley

& Sons, 1988.

[2] Carey, M. B., Chan, H.-T., Descloux, A., Ingle, J. F., and Park,

K. I. 1982/83 end office connection study:  Analog voice and

voiceband data transmission performance characterization of the

public switched network. Bell System Technical Journal 63, 9 (Nov.

1984).

[3] Chiappa, N., 1988. Private communication.

[4] Clark, D. D. The design philosophy of the DARPA Internet

protocols. In Proceedings of SIGCOMM '88 (Stanford, CA, Aug.

1988), ACM.

[5] Farber, D. J., Delp, G. S., and Conte, T. M. A Thinwire Protocol

for connecting personal computers to the Internet. Arpanet Working

Group Requests for Comment, DDN Network Information Center, SRI

International, Menlo Park, CA, Sept. 1984. RFC-914.

[6] Kent, C. A., and Mogul, J. Fragmentation considered harmful. In

Proceedings of SIGCOMM '87 (Aug. 1987), ACM.

[7] Lempel, A., and Ziv, J. Compression of individual sequences via

variable-rate encoding. IEEE Transactions on Information Theory

IT-24, 5 (June 1978).

[8] Nagle, J. Congestion Control in IP/TCP Internetworks. Arpanet

Working Group Requests for Comment, DDN Network Information Center,

SRI International, Menlo Park, CA, Jan. 1984. RFC-896.

[9] Perkins, D. Point-to-Point Protocol:  A proposal for

multi-protocol transmission of datagrams over point-to-point links.

Arpanet Working Group Requests for Comment, DDN Network Information

Center, SRI International, Menlo Park, CA, Nov. 1989. RFC-1134.

Jacobson                                                       [Page 25]

RFC 1144               Compressing TCP/IP Headers          February 1990

[10] Postel, J., Ed. Internet Protocol Specification. SRI

International, Menlo Park, CA, Sept. 1981. RFC-791.

[11] Postel, J., Ed. Transmission Control Protocol Specification. SRI

International, Menlo Park, CA, Sept. 1981. RFC-793.

[12] Romkey, J. A Nonstandard for Transmission of IP Datagrams Over

Serial Lines:  Slip. Arpanet Working Group Requests for Comment,

DDN Network Information Center, SRI International, Menlo Park, CA,

June 1988. RFC-1055.

[13] Salthouse, T. A. The skill of typing. Scientific American 250, 2

(Feb. 1984), 128--135.

[14] Saltzer, J. H., Reed, D. P., and Clark, D. D. End-to-end arguments

in system design. ACM Transactions on Computer Systems 2, 4 (Nov.

1984).

[15] Shneiderman, B. Designing the User Interface. Addison-Wesley,

1987.

Jacobson                                                       [Page 26]

RFC 1144               Compressing TCP/IP Headers          February 1990

A  Sample Implementation

The following is a sample implementation of the protocol described in

this document.

Since many people who might have the deal with this code are familiar

with the Berkeley Unix kernel and its coding style (affectionately known

as kernel normal form), this code was done in that style.  It uses the

Berkeley `subroutines' (actually, macros and/or inline assembler

expansions) for converting to/from network byte order and

copying/comparing strings of bytes.  These routines are briefly

described in sec. A.5 for anyone not familiar with them.

This code has been run on all the machines listed in the table on page

24.  Thus, the author hopes there are no byte order or alignment

problems (although there are embedded assumptions about alignment that

are valid for Berkeley Unix but may not be true for other IP

implementations --- see the comments mentioning alignment in

sl_compress_tcp and sl_decompress_tcp).

There was some attempt to make this code efficient.  Unfortunately, that

may have made portions of it incomprehensible.  The author apologizes

for any frustration this engenders.  (In honesty, my C style is known to

be obscure and claims of `efficiency' are simply a convenient excuse.)

This sample code and a complete Berkeley Unix implementation is

available in machine readable form via anonymous ftp from Internet host

ftp.ee.lbl.gov (128.3.254.68), file cslip.tar.Z. This is a compressed

Unix tar file.  It must be ftped in binary mode.

All of the code in this appendix is covered by the following copyright:

/*

* Copyright (c) 1989 Regents of the University of California.

* All rights reserved.

*

* Redistribution and use in source and binary forms are

* permitted provided that the above copyright notice and this

* paragraph are duplicated in all such forms and that any

* documentation, advertising materials, and other materials

* related to such distribution and use acknowledge that the

* software was developed by the University of California,

* Berkeley.  The name of the University may not be used to

* endorse or promote products derived from this software

* without specific prior written permission.

* THIS SOFTWARE IS PROVIDED ``AS IS'' AND WITHOUT ANY EXPRESS

* OR IMPLIED WARRANTIES, INCLUDING, WITHOUT LIMITATION, THE

* IMPLIED WARRANTIES OF MERCHANTIBILITY AND FITNESS FOR A

* PARTICULAR PURPOSE.

*/

Jacobson                                                       [Page 27]

RFC 1144               Compressing TCP/IP Headers          February 1990

A.1  Definitions and State Data

#define MAX_STATES 16   /* must be >2 and <255 */

#define MAX_HDR 128     /* max TCP+IP hdr length (by protocol def) */

/* packet types */

#define TYPE_IP 0x40

#define TYPE_UNCOMPRESSED_TCP 0x70

#define TYPE_COMPRESSED_TCP 0x80

#define TYPE_ERROR 0x00 /* this is not a type that ever appears on

* the wire.  The receive framer uses it to

* tell the decompressor there was a packet

* transmission error. */

/*

* Bits in first octet of compressed packet

*/

/* flag bits for what changed in a packet */

#define NEW_C  0x40

#define NEW_I  0x20

#define TCP_PUSH_BIT 0x10

#define NEW_S  0x08

#define NEW_A  0x04

#define NEW_W  0x02

#define NEW_U  0x01

/* reserved, special-case values of above */

#define SPECIAL_I (NEW_S|NEW_W|NEW_U)        /* echoed interactive traffic */

#define SPECIAL_D (NEW_S|NEW_A|NEW_W|NEW_U)  /* unidirectional data */

#define SPECIALS_MASK (NEW_S|NEW_A|NEW_W|NEW_U)

/*

* "state" data for each active tcp conversation on the wire.  This is

* basically a copy of the entire IP/TCP header from the last packet together

* with a small identifier the transmit & receive ends of the line use to

* locate saved header.

*/

struct cstate {

struct cstate *cs_next;  /* next most recently used cstate (xmit only) */

u_short cs_hlen;         /* size of hdr (receive only) */

u_char cs_id;            /* connection # associated with this state */

u_char cs_filler;

union {

char hdr[MAX_HDR];

struct ip csu_ip;   /* ip/tcp hdr from most recent packet */

} slcs_u;

};

#define cs_ip slcs_u.csu_ip

Jacobson                                                       [Page 28]

RFC 1144               Compressing TCP/IP Headers          February 1990

#define cs_hdr slcs_u.csu_hdr

/*

* all the state data for one serial line (we need one of these per line).

*/

struct slcompress {

struct cstate *last_cs;            /* most recently used tstate */

u_char last_recv;                  /* last rcvd conn. id */

u_char last_xmit;                  /* last sent conn. id */

u_short flags;

struct cstate tstate[MAX_STATES];  /* xmit connection states */

struct cstate rstate[MAX_STATES];  /* receive connection states */

};

/* flag values */

#define SLF_TOSS 1       /* tossing rcvd frames because of input err */

/*

* The following macros are used to encode and decode numbers.  They all

* assume that `cp' points to a buffer where the next byte encoded (decoded)

* is to be stored (retrieved).  Since the decode routines do arithmetic,

* they have to convert from and to network byte order.

*/

/*

* ENCODE encodes a number that is known to be non-zero.  ENCODEZ checks for

* zero (zero has to be encoded in the long, 3 byte form).

*/

#define ENCODE(n) { \

if ((u_short)(n) >= 256) { \

*cp++ = 0; \

cp[1] = (n); \

cp[0] = (n) >> 8; \

cp += 2; \

} else { \

*cp++ = (n); \

} \

}

#define ENCODEZ(n) { \

if ((u_short)(n) >= 256 || (u_short)(n) == 0) { \

*cp++ = 0; \

cp[1] = (n); \

cp[0] = (n) >> 8; \

cp += 2; \

} else { \

*cp++ = (n); \

} \

}

/*

* DECODEL takes the (compressed) change at byte cp and adds it to the

Jacobson                                                       [Page 29]

RFC 1144               Compressing TCP/IP Headers          February 1990

* current value of packet field 'f' (which must be a 4-byte (long) integer

* in network byte order).  DECODES does the same for a 2-byte (short) field.

* DECODEU takes the change at cp and stuffs it into the (short) field f.

* 'cp' is updated to point to the next field in the compressed header.

*/

#define DECODEL(f) { \

if (*cp == 0) {\

(f) = htonl(ntohl(f) + ((cp[1] << 8) | cp[2])); \

cp += 3; \

} else { \

(f) = htonl(ntohl(f) + (u_long)*cp++); \

} \

}

#define DECODES(f) { \

if (*cp == 0) {\

(f) = htons(ntohs(f) + ((cp[1] << 8) | cp[2])); \

cp += 3; \

} else { \

(f) = htons(ntohs(f) + (u_long)*cp++); \

} \

}

#define DECODEU(f) { \

if (*cp == 0) {\

(f) = htons((cp[1] << 8) | cp[2]); \

cp += 3; \

} else { \

(f) = htons((u_long)*cp++); \

} \

}

Jacobson                                                       [Page 30]

RFC 1144               Compressing TCP/IP Headers          February 1990

A.2  Compression

This routine looks daunting but isn't really.  The code splits into four

approximately equal sized sections:  The first quarter manages a

circularly linked, least-recently-used list of `active' TCP

connections./47/  The second figures out the sequence/ack/window/urg

changes and builds the bulk of the compressed packet.  The third handles

the special-case encodings.  The last quarter does packet ID and

connection ID encoding and replaces the original packet header with the

compressed header.

The arguments to this routine are a pointer to a packet to be

compressed, a pointer to the compression state data for the serial line,

and a flag which enables or disables connection id (C bit) compression.

Compression is done `in-place' so, if a compressed packet is created,

both the start address and length of the incoming packet (the off and

len fields of m) will be updated to reflect the removal of the original

header and its replacement by the compressed header.  If either a

compressed or uncompressed packet is created, the compression state is

updated.  This routines returns the packet type for the transmit framer

(TYPE_IP, TYPE_UNCOMPRESSED_TCP or TYPE_COMPRESSED_TCP).

Because 16 and 32 bit arithmetic is done on various header fields, the

incoming IP packet must be aligned appropriately (e.g., on a SPARC, the

IP header is aligned on a 32-bit boundary).  Substantial changes would

have to be made to the code below if this were not true (and it would

probably be cheaper to byte copy the incoming header to somewhere

correctly aligned than to make those changes).

Note that the outgoing packet will be aligned arbitrarily (e.g., it

could easily start on an odd-byte boundary).

u_char

sl_compress_tcp(m, comp, compress_cid)

struct mbuf *m;

struct slcompress *comp;

int compress_cid;

{

register struct cstate *cs = comp->last_cs->cs_next;

register struct ip *ip = mtod(m, struct ip *);

register u_int hlen = ip->ip_hl;

register struct tcphdr *oth;       /* last TCP header */

register struct tcphdr *th;        /* current TCP header */

----------------------------

47. The two most common operations on the connection list are a `find'

that terminates at the first entry (a new packet for the most recently

used connection) and moving the last entry on the list to the head of

the list (the first packet from a new connection).  A circular list

efficiently handles these two operations.

Jacobson                                                       [Page 31]

RFC 1144               Compressing TCP/IP Headers          February 1990

register u_int deltaS, deltaA;     /* general purpose temporaries */

register u_int changes = 0;        /* change mask */

u_char new_seq[16];                /* changes from last to current */

register u_char *cp = new_seq;

/*

* Bail if this is an IP fragment or if the TCP packet isn't

* `compressible' (i.e., ACK isn't set or some other control bit is

* set).  (We assume that the caller has already made sure the packet

* is IP proto TCP).

*/

if ((ip->ip_off & htons(0x3fff)) || m->m_len < 40)

return (TYPE_IP);

th = (struct tcphdr *) & ((int *) ip)[hlen];

if ((th->th_flags & (TH_SYN | TH_FIN | TH_RST | TH_ACK)) != TH_ACK)

return (TYPE_IP);

/*

* Packet is compressible -- we're going to send either a

* COMPRESSED_TCP or UNCOMPRESSED_TCP packet.  Either way we need to

* locate (or create) the connection state.  Special case the most

* recently used connection since it's most likely to be used again &

* we don't have to do any reordering if it's used.

*/

if (ip->ip_src.s_addr != cs->cs_ip.ip_src.s_addr ||

ip->ip_dst.s_addr != cs->cs_ip.ip_dst.s_addr ||

*(int *) th != ((int *) &cs->cs_ip)[cs->cs_ip.ip_hl]) {

/*

* Wasn't the first -- search for it.

*

* States are kept in a circularly linked list with last_cs

* pointing to the end of the list.  The list is kept in lru

* order by moving a state to the head of the list whenever

* it is referenced.  Since the list is short and,

* empirically, the connection we want is almost always near

* the front, we locate states via linear search.  If we

* don't find a state for the datagram, the oldest state is

* (re-)used.

*/

register struct cstate *lcs;

register struct cstate *lastcs = comp->last_cs;

do {

lcs = cs;

cs = cs->cs_next;

if (ip->ip_src.s_addr == cs->cs_ip.ip_src.s_addr

&& ip->ip_dst.s_addr == cs->cs_ip.ip_dst.s_addr

&& *(int *) th == ((int *) &cs->cs_ip)[cs->cs_ip.ip_hl])

goto found;

Jacobson                                                       [Page 32]

RFC 1144               Compressing TCP/IP Headers          February 1990

} while (cs != lastcs);

/*

* Didn't find it -- re-use oldest cstate.  Send an

* uncompressed packet that tells the other side what

* connection number we're using for this conversation. Note

* that since the state list is circular, the oldest state

* points to the newest and we only need to set last_cs to

* update the lru linkage.

*/

comp->last_cs = lcs;

hlen += th->th_off;

hlen <<= 2;

goto uncompressed;

found:

/* Found it -- move to the front on the connection list. */

if (lastcs == cs)

comp->last_cs = lcs;

else {

lcs->cs_next = cs->cs_next;

cs->cs_next = lastcs->cs_next;

lastcs->cs_next = cs;

}

}

/*

* Make sure that only what we expect to change changed. The first

* line of the `if' checks the IP protocol version, header length &

* type of service.  The 2nd line checks the "Don't fragment" bit.

* The 3rd line checks the time-to-live and protocol (the protocol

* check is unnecessary but costless).  The 4th line checks the TCP

* header length.  The 5th line checks IP options, if any.  The 6th

* line checks TCP options, if any.  If any of these things are

* different between the previous & current datagram, we send the

* current datagram `uncompressed'.

*/

oth = (struct tcphdr *) & ((int *) &cs->cs_ip)[hlen];

deltaS = hlen;

hlen += th->th_off;

hlen <<= 2;

if (((u_short *) ip)[0] != ((u_short *) &cs->cs_ip)[0] ||

((u_short *) ip)[3] != ((u_short *) &cs->cs_ip)[3] ||

((u_short *) ip)[4] != ((u_short *) &cs->cs_ip)[4] ||

th->th_off != oth->th_off ||

(deltaS > 5 && BCMP(ip + 1, &cs->cs_ip + 1, (deltaS - 5) << 2)) ||

(th->th_off > 5 && BCMP(th + 1, oth + 1, (th->th_off - 5) << 2)))

goto uncompressed;

/*

* Figure out which of the changing fields changed.  The receiver

Jacobson                                                       [Page 33]

RFC 1144               Compressing TCP/IP Headers          February 1990

* expects changes in the order: urgent, window, ack, seq.

*/

if (th->th_flags & TH_URG) {

deltaS = ntohs(th->th_urp);

ENCODEZ(deltaS);

changes |= NEW_U;

} else if (th->th_urp != oth->th_urp)

/*

* argh! URG not set but urp changed -- a sensible

* implementation should never do this but RFC793 doesn't

* prohibit the change so we have to deal with it.

*/

goto uncompressed;

if (deltaS = (u_short) (ntohs(th->th_win) - ntohs(oth->th_win))) {

ENCODE(deltaS);

changes |= NEW_W;

}

if (deltaA = ntohl(th->th_ack) - ntohl(oth->th_ack)) {

if (deltaA > 0xffff)

goto uncompressed;

ENCODE(deltaA);

changes |= NEW_A;

}

if (deltaS = ntohl(th->th_seq) - ntohl(oth->th_seq)) {

if (deltaS > 0xffff)

goto uncompressed;

ENCODE(deltaS);

changes |= NEW_S;

}

/*

* Look for the special-case encodings.

*/

switch (changes) {

case 0:

/*

* Nothing changed. If this packet contains data and the last

* one didn't, this is probably a data packet following an

* ack (normal on an interactive connection) and we send it

* compressed.  Otherwise it's probably a retransmit,

* retransmitted ack or window probe.  Send it uncompressed

* in case the other side missed the compressed version.

*/

if (ip->ip_len != cs->cs_ip.ip_len &&

ntohs(cs->cs_ip.ip_len) == hlen)

break;

/* (fall through) */

case SPECIAL_I:

Jacobson                                                       [Page 34]

RFC 1144               Compressing TCP/IP Headers          February 1990

case SPECIAL_D:

/*

* Actual changes match one of our special case encodings --

* send packet uncompressed.

*/

goto uncompressed;

case NEW_S | NEW_A:

if (deltaS == deltaA &&

deltaS == ntohs(cs->cs_ip.ip_len) - hlen) {

/* special case for echoed terminal traffic */

changes = SPECIAL_I;

cp = new_seq;

}

break;

case NEW_S:

if (deltaS == ntohs(cs->cs_ip.ip_len) - hlen) {

/* special case for data xfer */

changes = SPECIAL_D;

cp = new_seq;

}

break;

}

deltaS = ntohs(ip->ip_id) - ntohs(cs->cs_ip.ip_id);

if (deltaS != 1) {

ENCODEZ(deltaS);

changes |= NEW_I;

}

if (th->th_flags & TH_PUSH)

changes |= TCP_PUSH_BIT;

/*

* Grab the cksum before we overwrite it below.  Then update our

* state with this packet's header.

*/

deltaA = ntohs(th->th_sum);

BCOPY(ip, &cs->cs_ip, hlen);

/*

* We want to use the original packet as our compressed packet. (cp -

* new_seq) is the number of bytes we need for compressed sequence

* numbers.  In addition we need one byte for the change mask, one

* for the connection id and two for the tcp checksum. So, (cp -

* new_seq) + 4 bytes of header are needed.  hlen is how many bytes

* of the original packet to toss so subtract the two to get the new

* packet size.

*/

deltaS = cp - new_seq;

cp = (u_char *) ip;

if (compress_cid == 0 || comp->last_xmit != cs->cs_id) {

comp->last_xmit = cs->cs_id;

Jacobson                                                       [Page 35]

RFC 1144               Compressing TCP/IP Headers          February 1990

hlen -= deltaS + 4;

cp += hlen;

*cp++ = changes | NEW_C;

*cp++ = cs->cs_id;

} else {

hlen -= deltaS + 3;

cp += hlen;

*cp++ = changes;

}

m->m_len -= hlen;

m->m_off += hlen;

*cp++ = deltaA >> 8;

*cp++ = deltaA;

BCOPY(new_seq, cp, deltaS);

return (TYPE_COMPRESSED_TCP);

uncompressed:

/*

* Update connection state cs & send uncompressed packet

* ('uncompressed' means a regular ip/tcp packet but with the

* 'conversation id' we hope to use on future compressed packets in

* the protocol field).

*/

BCOPY(ip, &cs->cs_ip, hlen);

ip->ip_p = cs->cs_id;

comp->last_xmit = cs->cs_id;

return (TYPE_UNCOMPRESSED_TCP);

}

Jacobson                                                       [Page 36]

RFC 1144               Compressing TCP/IP Headers          February 1990

A.3  Decompression

This routine decompresses a received packet.  It is called with a

pointer to the packet, the packet length and type, and a pointer to the

compression state structure for the incoming serial line.  It returns a

pointer to the resulting packet or zero if there were errors in the

incoming packet.  If the packet is COMPRESSED_TCP or UNCOMPRESSED_TCP,

the compression state will be updated.

The new packet will be constructed in-place.  That means that there must

be 128 bytes of free space in front of bufp to allow room for the

reconstructed IP and TCP headers.  The reconstructed packet will be

aligned on a 32-bit boundary.

u_char *

sl_uncompress_tcp(bufp, len, type, comp)

u_char *bufp;

int len;

u_int type;

struct slcompress *comp;

{

register u_char *cp;

register u_int hlen, changes;

register struct tcphdr *th;

register struct cstate *cs;

register struct ip *ip;

switch (type) {

case TYPE_ERROR:

default:

goto bad;

case TYPE_IP:

return (bufp);

case TYPE_UNCOMPRESSED_TCP:

/*

* Locate the saved state for this connection.  If the state

* index is legal, clear the 'discard' flag.

*/

ip = (struct ip *) bufp;

if (ip->ip_p >= MAX_STATES)

goto bad;

cs = &comp->rstate[comp->last_recv = ip->ip_p];

comp->flags &= ~SLF_TOSS;

/*

* Restore the IP protocol field then save a copy of this

* packet header.  (The checksum is zeroed in the copy so we

* don't have to zero it each time we process a compressed

Jacobson                                                       [Page 37]

RFC 1144               Compressing TCP/IP Headers          February 1990

* packet.

*/

ip->ip_p = IPPROTO_TCP;

hlen = ip->ip_hl;

hlen += ((struct tcphdr *) & ((int *) ip)[hlen])->th_off;

hlen <<= 2;

BCOPY(ip, &cs->cs_ip, hlen);

cs->cs_ip.ip_sum = 0;

cs->cs_hlen = hlen;

return (bufp);

case TYPE_COMPRESSED_TCP:

break;

}

/* We've got a compressed packet. */

cp = bufp;

changes = *cp++;

if (changes & NEW_C) {

/*

* Make sure the state index is in range, then grab the

* state. If we have a good state index, clear the 'discard'

* flag.

*/

if (*cp >= MAX_STATES)

goto bad;

comp->flags &= ~SLF_TOSS;

comp->last_recv = *cp++;

} else {

/*

* This packet has an implicit state index.  If we've had a

* line error since the last time we got an explicit state

* index, we have to toss the packet.

*/

if (comp->flags & SLF_TOSS)

return ((u_char *) 0);

}

/*

* Find the state then fill in the TCP checksum and PUSH bit.

*/

cs = &comp->rstate[comp->last_recv];

hlen = cs->cs_ip.ip_hl << 2;

th = (struct tcphdr *) & ((u_char *) &cs->cs_ip)[hlen];

th->th_sum = htons((*cp << 8) | cp[1]);

cp += 2;

if (changes & TCP_PUSH_BIT)

th->th_flags |= TH_PUSH;

else

th->th_flags &= ~TH_PUSH;

/*

Jacobson                                                       [Page 38]

RFC 1144               Compressing TCP/IP Headers          February 1990

* Fix up the state's ack, seq, urg and win fields based on the

* changemask.

*/

switch (changes & SPECIALS_MASK) {

case SPECIAL_I:

{

register u_int i = ntohs(cs->cs_ip.ip_len) - cs->cs_hlen;

th->th_ack = htonl(ntohl(th->th_ack) + i);

th->th_seq = htonl(ntohl(th->th_seq) + i);

}

break;

case SPECIAL_D:

th->th_seq = htonl(ntohl(th->th_seq) + ntohs(cs->cs_ip.ip_len)

- cs->cs_hlen);

break;

default:

if (changes & NEW_U) {

th->th_flags |= TH_URG;

DECODEU(th->th_urp)

} else

th->th_flags &= ~TH_URG;

if (changes & NEW_W)

DECODES(th->th_win)

if (changes & NEW_A)

DECODEL(th->th_ack)

if (changes & NEW_S)

DECODEL(th->th_seq)

break;

}

/* Update the IP ID */

if (changes & NEW_I)

DECODES(cs->cs_ip.ip_id)

else

cs->cs_ip.ip_id = htons(ntohs(cs->cs_ip.ip_id) + 1);

/*

* At this point, cp points to the first byte of data in the packet.

* If we're not aligned on a 4-byte boundary, copy the data down so

* the IP & TCP headers will be aligned.  Then back up cp by the

* TCP/IP header length to make room for the reconstructed header (we

* assume the packet we were handed has enough space to prepend 128

* bytes of header).  Adjust the lenth to account for the new header

* & fill in the IP total length.

*/

len -= (cp - bufp);

if (len < 0)

/*

* we must have dropped some characters (crc should detect

* this but the old slip framing won't)

Jacobson                                                       [Page 39]

RFC 1144               Compressing TCP/IP Headers          February 1990

*/

goto bad;

if ((int) cp & 3) {

if (len > 0)

OVBCOPY(cp, (int) cp & ~3, len);

cp = (u_char *) ((int) cp & ~3);

}

cp -= cs->cs_hlen;

len += cs->cs_hlen;

cs->cs_ip.ip_len = htons(len);

BCOPY(&cs->cs_ip, cp, cs->cs_hlen);

/* recompute the ip header checksum */

{

register u_short *bp = (u_short *) cp;

for (changes = 0; hlen > 0; hlen -= 2)

changes += *bp++;

changes = (changes & 0xffff) + (changes >> 16);

changes = (changes & 0xffff) + (changes >> 16);

((struct ip *) cp)->ip_sum = ~changes;

}

return (cp);

bad:

comp->flags |= SLF_TOSS;

return ((u_char *) 0);

}

Jacobson                                                       [Page 40]

RFC 1144               Compressing TCP/IP Headers          February 1990

A.4  Initialization

This routine initializes the state structure for both the transmit and

receive halves of some serial line.  It must be called each time the

line is brought up.

void

sl_compress_init(comp)

struct slcompress *comp;

{

register u_int i;

register struct cstate *tstate = comp->tstate;

/*

* Clean out any junk left from the last time line was used.

*/

bzero((char *) comp, sizeof(*comp));

/*

* Link the transmit states into a circular list.

*/

for (i = MAX_STATES - 1; i > 0; --i) {

tstate[i].cs_id = i;

tstate[i].cs_next = &tstate[i - 1];

}

tstate[0].cs_next = &tstate[MAX_STATES - 1];

tstate[0].cs_id = 0;

comp->last_cs = &tstate[0];

/*

* Make sure we don't accidentally do CID compression

* (assumes MAX_STATES < 255).

*/

comp->last_recv = 255;

comp->last_xmit = 255;

}

A.5  Berkeley Unix dependencies

Note:  The following is of interest only if you are trying to bring the

sample code up on a system that is not derived from 4BSD (Berkeley

Unix).

The code uses the normal Berkeley Unix header files (from

/usr/include/netinet) for definitions of the structure of IP and TCP

headers.  The structure tags tend to follow the protocol RFCs closely

and should be obvious even if you do not have access to a 4BSD

system./48/

----------------------------

48. In the event they are not obvious, the header files (and all the

Berkeley networking code) can be anonymous ftp'd from host

Jacobson                                                       [Page 41]

RFC 1144               Compressing TCP/IP Headers          February 1990

The macro BCOPY(src, dst, amt) is invoked to copy amt bytes from src to

dst.  In BSD, it translates into a call to bcopy.  If you have the

misfortune to be running System-V Unix, it can be translated into a call

to memcpy.  The macro OVBCOPY(src, dst, amt) is used to copy when src

and dst overlap (i.e., when doing the 4-byte alignment copy).  In the

BSD kernel, it translates into a call to ovbcopy.  Since AT&T botched

the definition of memcpy, this should probably translate into a copy

loop under System-V.

The macro BCMP(src, dst, amt) is invoked to compare amt bytes of src and

dst for equality.  In BSD, it translates into a call to bcmp.  In

System-V, it can be translated into a call to memcmp or you can write a

routine to do the compare.  The routine should return zero if all bytes

of src and dst are equal and non-zero otherwise.

The routine ntohl(dat) converts (4 byte) long dat from network byte

order to host byte order.  On a reasonable cpu this can be the no-op

macro:

#define ntohl(dat) (dat)

On a Vax or IBM PC (or anything with Intel byte order), you will have to

define a macro or routine to rearrange bytes.

The routine ntohs(dat) is like ntohl but converts (2 byte) shorts

instead of longs.  The routines htonl(dat) and htons(dat) do the inverse

transform (host to network byte order) for longs and shorts.

A struct mbuf is used in the call to sl_compress_tcp because that

routine needs to modify both the start address and length if the

incoming packet is compressed.  In BSD, an mbuf is the kernel's buffer

management structure.  If other systems, the following definition should

be sufficient:

struct mbuf {

u_char  *m_off; /* pointer to start of data */

int     m_len;  /* length of data */

};

#define mtod(m, t) ((t)(m->m_off))

----------------------------

ucbarpa.berkeley.edu, files pub/4.3/tcp.tar and pub/4.3/inet.tar.

Jacobson                                                       [Page 42]

RFC 1144               Compressing TCP/IP Headers          February 1990

B  Compatibility with past mistakes

When combined with the modern PPP serial line protocol[9], the use of

header compression is automatic and invisible to the user.

Unfortunately, many sites have existing users of the SLIP described in

[12] which doesn't allow for different protocol types to distinguish

header compressed packets from IP packets or for version numbers or an

option exchange that could be used to automatically negotiate header

compression.

The author has used the following tricks to allow header compressed SLIP

to interoperate with the existing servers and clients.  Note that these

are hacks for compatibility with past mistakes and should be offensive

to any right thinking person.  They are offered solely to ease the pain

of running SLIP while users wait patiently for vendors to release PPP.

B.1  Living without a framing `type' byte

The bizarre packet type numbers in sec. A.1 were chosen to allow a

`packet type' to be sent on lines where it is undesirable or impossible

to add an explicit type byte.  Note that the first byte of an IP packet

always contains `4' (the IP protocol version) in the top four bits.  And

that the most significant bit of the first byte of the compressed header

is ignored.  Using the packet types in sec. A.1, the type can be encoded

in the most significant bits of the outgoing packet using the code

p->dat[0] |= sl_compress_tcp(p, comp);

and decoded on the receive side by

if (p->dat[0] & 0x80)

type = TYPE_COMPRESSED_TCP;

else if (p->dat[0] >= 0x70) {

type = TYPE_UNCOMPRESSED_TCP;

p->dat[0] &=~ 0x30;

} else

type = TYPE_IP;

status = sl_uncompress_tcp(p, type, comp);

B.2  Backwards compatible SLIP servers

The SLIP described in [12] doesn't include any mechanism that could be

used to automatically negotiate header compression.  It would be nice to

Jacobson                                                       [Page 43]

RFC 1144               Compressing TCP/IP Headers          February 1990

allow users of this SLIP to use header compression but, when users of

the two SLIP varients share a common server, it would be annoying and

difficult to manually configure both ends of each connection to enable

compression.  The following procedure can be used to avoid manual

configuration.

Since there are two types of dial-in clients (those that implement

compression and those that don't) but one server for both types, it's

clear that the server will be reconfiguring for each new client session

but clients change configuration seldom if ever.  If manual

configuration has to be done, it should be done on the side that changes

infrequently --- the client.  This suggests that the server should

somehow learn from the client whether to use header compression.

Assuming symmetry (i.e., if compression is used at all it should be used

both directions) the server can use the receipt of a compressed packet

from some client to indicate that it can send compressed packets to that

client.  This leads to the following algorithm:

There are two bits per line to control header compression:  allowed and

on.  If on is set, compressed packets are sent, otherwise not.  If

allowed is set, compressed packets can be received and, if an

UNCOMPRESSED_TCP packet arrives when on is clear, on will be set./49/

If a compressed packet arrives when allowed is clear, it will be

ignored.

Clients are configured with both bits set (allowed is always set if on

is set) and the server starts each session with allowed set and on

clear.  The first compressed packet from the client (which must be a

UNCOMPRESSED_TCP packet) turns on compression for the server.

----------------------------

49. Since [12] framing doesn't include error detection, one should be

careful not to `false trigger' compression on the server.  The

UNCOMPRESSED_TCP packet should checked for consistency (e.g., IP

checksum correctness) before compression is enabled.  Arrival of

COMPRESSED_TCP packets should not be used to enable compression.

Jacobson                                                       [Page 44]

RFC 1144               Compressing TCP/IP Headers          February 1990

C  More aggressive compression

As noted in sec. 3.2.2, easily detected patterns exist in the stream of

compressed headers, indicating that more compression could be done.

Would this be worthwhile?

The average compressed datagram has only seven bits of header./50/  The

framing must be at least one bit (to encode the `type') and will

probably be more like two to three bytes.  In most interesting cases

there will be at least one byte of data.  Finally, the end-to-end

check---the TCP checksum---must be passed through unmodified./51/

The framing, data and checksum will remain even if the header is

completely compressed out so the change in average packet size is, at

best, four bytes down to three bytes and one bit --- roughly a 25%

improvement in delay./52/  While this may seem significant, on a 2400

bps line it means that typing echo response takes 25 rather than 29 ms.

At the present stage of human evolution, this difference is not

detectable.

However, the author sheepishly admits to perverting this compression

scheme for a very special case data-acquisition problem:  We had an

instrument and control package floating at 200KV, communicating with

ground level via a telemetry system.  For many reasons (multiplexed

communication, pipelining, error recovery, availability of well tested

implementations, etc.), it was convenient to talk to the package using

TCP/IP. However, since the primary use of the telemetry link was data

acquisition, it was designed with an uplink channel capacity <0.5% the

downlink's.  To meet application delay budgets, data packets were 100

bytes and, since TCP acks every other packet, the relative uplink

bandwidth for acks is a/200 where `a' is the total size of ack packets.

Using the scheme in this paper, the smallest ack is four bytes which

would imply an uplink bandwidth 2% of the downlink.  This wasn't

----------------------------

50. Tests run with several million packets from a mixed traffic load

(i.e., statistics kept on a year's traffic from my home to work) show

that 80% of packets use one of the two special encodings and, thus, the

only header is the change mask.

51. If someone tries to sell you a scheme that compresses the TCP

checksum `Just say no'.  Some poor fool has yet to have the sad

experience that reveals the end-to-end argument is gospel truth.  Worse,

since the fool is subverting your end-to-end error check, you may pay

the price for this education and they will be none the wiser.  What does

it profit a man to gain two byte times of delay and lose peace of mind?

52. Note again that we must be concerned about interactive delay to be

making this argument:  Bulk data transfer performance will be dominated

by the time to send the data and the difference between three and four

byte headers on a datagram containing tens or hundreds of data bytes is,

practically, no difference.

Jacobson                                                       [Page 45]

RFC 1144               Compressing TCP/IP Headers          February 1990

possible so we used the scheme described in footnote 15:  If the first

bit of the frame was one, it meant `same compressed header as last

time'.  Otherwise the next two bits gave one of the types described in

sec. 3.2.  Since the link had excellent forward error correction and

traffic made only a single hop, the TCP checksum was compressed out

(blush!) of the `same header' packet types/53/ so the total header size

for these packets was one bit.  Over several months of operation, more

than 99% of the 40 byte TCP/IP headers were compressed down to one

bit./54/

D  Security Considerations

Security considerations are not addressed in this memo.

E  Author's address

Address:  Van Jacobson

Real Time Systems Group

Mail Stop 46A

Lawrence Berkeley Laboratory

Berkeley, CA 94720

Phone:    Use email (author ignores his phone)

EMail:    van@helios.ee.lbl.gov

----------------------------

53. The checksum was re-generated in the decompressor and, of course,

the `toss' logic was made considerably more aggressive to prevent error

propagation.

54. We have heard the suggestion that `real-time' needs require

abandoning TCP/IP in favor of a `light-weight' protocol with smaller

headers.  It is difficult to envision a protocol that averages less than

one header bit per packet.

Jacobson                                                       [Page 46]