PAGE i

RIG-EQF Version 1.03

Control Program for Kenwood Radios.

Copyright (C) 1991

Thomas J. Dandrea N3EQF

------------

USER'S GUIDE

------------

RIG-EQF is copyrighted by the author, and has been released

for public distribution as "Shareware" or "User Supported"

software, for use by radio amateurs and SWL's.  The author

reserves all rights to this software under copyright laws.

Registered users of RIG-EQF are provided notification of

program revisions and technical support if required. The

registration fee for RIG-EQF is only $10.00.  If you enjoy

using RIG-EQF, please encourage further program development

by notifying the author at the address below.

-----------------------

Tom Dandrea  N3EQF

396 Sautter Drive

Coraopolis, PA 15108

USA

-----------------------

Improvements and revisions to RIG-EQF will be made in direct

proportion to the number of requests for them.  Your comments

about the program and suggestions for improvement are welcome.

PAGE ii

------------------------- CONTENTS -----------------------------

PAGE NUMBER

-----------

1.   RIG-EQF Overview............................1.0

2.   Installation................................2.0

3.   Operation...................................3.0

4.   PROGRAM SCAN CONTROL........................4.0

5.   LOCAL MEMORY CONTROL........................5.0

PAGE 1.0

1.  RIG-EQF OVERVIEW

--------------------

RIG-EQF provides computer control over Kenwood radios that utilize

the IF-232C interface.  Various radio parameters can be controlled

and monitored via the computer, increasing the power and flexibility

of your station.

RIG-EQF can even be used to control two radios, with fast easy

switching between the two communications ports.

Below are a few of the radio functions supported by RIG-EQF:

- Large digital display of current VFO frequency.

- Control over VFO frequency with direct entry.

- Shift current frequency up or down in 1MHz increments.

- Tune current frequency in small variable-rate increments.

- Switch between VFO's, select SPLIT, or set VFO's equal.

- Lock current frequency.

- RIT and XIT control for radios with these features.

- Sophisticated programmable scanning, including lower and upper

frequency limits, frequency increment, and scanning rate.

- LOCAL MEMORY control; an advanced memory system providing

storage of radio parameters on the computer's disk.

- Transmit/Receive control for transceivers.

PAGE 2.0

RIG-EQF INSTALLATION

-------------------

RIG-EQF may be installed on any IBM-compatible DOS computer, as long

as the DOS is Version 2.1 or higher.  The only other requirement is

that at least one communications (serial) port is installed on the

computer, either as COM1 or COM2.

To install RIG-EQF, just copy all files from the distribution disk

to a directory on your hard drive, or onto another floppy if your

computer does not have a hard drive.  There is only one file required

to operate RIG-EQF.  It is named RIG-EQF.EXE.

Once RIG-EQF runs, a file called RIG-EQF.CNF will be created.  This

file stores parameters like the serial port number and scanning

limits, for reuse each time the program runs.  Once you use the

LOCAL MEMORY feature in RIG-EQF, other files will be created to

store memory information.  Memory files always end in ".MEM".

Connect your Kenwood radio to either the COM1 or COM2 serial port of

your computer, following the recommendations in your Kenwood manual.

You should use a good quality serial cable, preferably sheided, and

one that uses (at least) 5 wires.  These 5 wires are standard for

RS-232C devices (like your Kenwood radio), but occasionally 3-wire

cables are sold.  A 3-wire "RS-232C" cable will not work correctly

with this program!  The names of the 5 signals (thus 5 wires) required

for proper operation are: Ground, Receive Data, Transmit Data, Clear

To Send, and Request to Send.

With your radio connected to one of the computer's serial ports, turn

on the radio and the IF-232C level translator (the IF-232C is not

required for some Kenwood models), and then start RIG-EQF.  RIG-EQF

will attempt to communicate with the COM1 port.  If the program does

not receive information from COM1, it will try the second port, COM2.

If the communication link is established successfully, the Main Screen

of RIG-EQF will appear.

If you have a radio connected to COM2, and some other device on COM1,

it is recommended that you turn off the device on COM1 until the

program starts.  Some devices will "hog" the channel, causing RIG-EQF

to become locked on COM1.  Another way to avoid this problem is to

type "RIG-EQF 2", to force the program to check the COM2 port first.

After the first time you run the program, RIG-EQF will check the

"known good" port first when you start it running again.

PAGE 3.0

3. RIG-EQF OPERATION

--------------------

Operation of RIG-EQF is straight-forward; there are no hidden

commands. All the available commands are displayed on the program

screens. The commands themselves consist of a single letter or

function key, and each command is shown on the screen next to its

corresponding function.

Note that some commands are not available with certain Kenwood

models.  In most cases, if the command is not available for the

radio currently in use, they will not appear on the screen.  For

example, the transmit/receive, Split, and XIT/RIT controls will

have no affect with the R5000 receiver.

The operation of each command is described below:

<A> Pressing the letter "A" allows you to enter a new frequency

in VFO A.  A cursor will appear at the upper right of the screen,

at the position where the existing frequency for VFO A is

displayed.  You may then enter the desired frequency.  There are

two ways to enter frequency; either using decimal points in the

proper places, or by stringing digits together in sequence like

you would at the radio's front panel.  When you hit <Enter> or

<Return>, the new frequency will be entered.

<B> The "B" key activates VFO B frequency entry.  This works exactly

the same way as for VFO A, above.

<=> Hitting the "=" key (equals sign) stores the currently displayed

(tuned) frequency into both VFO's.

<V> The "V" key causes a switch from the current VFO to the opposite

VFO. If VFO A is currently selected, VFO B will become the active

VFO.  The mode settings active when the opposite VFO were in use

will be recalled with that VFO, the same as if VFO's were switched

from the radio panel.

<S> Pressing the "S" key will turn the SPLIT function on if it was

off, or turn in off it was already on.  The word "SPLIT" will

appear at the upper left of the display, next to the large

frequency digits, when it is selected.

<D> The "D" key moves the present frequency DOWN 1 MHz.

<U> The "U" key moves the present frequency UP 1 MHz.

PAGE 3.1

<Page Down> key moves the frequency DOWN in small increments.  The

smallest increment, available when the key is first depreseed, is

10 Hz.  If you hold the key down for a few seconds, the rate will

increase to 100 Hz, and then finally to 1 Khz.  If you release

the key for a second or two, the rate will return again to the

slow 10 Hz. rate.  The displayed frequency will continuously be

updated as you tune.

<Page Up> key moves the frequency UP in the same manner as above.

<Space Bar> is used as the TRANSMIT/RECEIVE switch for transceivers.

When the radio is transmitting, a flashing TRANSMIT indicator

will appear at the top of the screen.

<M> The "M" key is used to change the radio MODE.  When you hit the

key, a box will appear, presenting the available modes to pick

from.  Use the <Up> and <Down> arrow keys to highlight the proper

mode, and then hit <Enter>.

<R> Hit the "R" key to turn the RIT function on or off. If the RIT

is on, the offset will be displayed at the upper right of the

display.  If the RIT is off, the word "off" will appear in its

place.  To adjust the RIT offset, use the <Right> and <Left>

arrow keys to tune up or down, respectively.

<X> The "X" key turns the XIT control on or off. Operation or the XIT

is the same as for RIT.

<Z> Pressing the letter "Z" will ZERO the XIT and RIT offsets. This

key zeros the offset reqardless of whether the RIT or XIT is on

or off.

<P> The letter "P" is used to select the PROGRAM SCAN CONTROL. A new

menu will be presented showing SCAN options.  See Section 4 of

this manual for descriptions of the SCAN controls.

<O> Hitting "O" (not zero, but "O") enables the LOCAL MEMORY CONTROL.

See Section 5 for a description of this system.

Page 4.0

PROGRAM SCAN CONTROL

--------------------

The PROGRAM SCAN CONTROL screen is used to control the parameters

related to the scanning function, and to start and stop the scanning

process.  The keys and their operation are described below:

<L> The "L" key is used to enter the LOWER, or starting frequency

for the scan.  The frequency is entered in Mhz.  The frequency

you enter here is where the scan process will start.

<U> Use the "U" key to enter the UPPER, or stopping frequency for

the scan.  This is the frequency limit (in Mhz) which causes the

scan to begin again at the lower frequency.

<I> The "I" key is used to enter the INCREMENT frequency.  This is

the size of the "step" between each frequency change during the

scan.  The increment frequency is entered in Khz, and the

smallest increment available is 10 Hz, or 0.01 Khz.

<P> The letter "P" sets the PAUSE time, in seconds, between scan

increments.  The smallest pause time possible is 0.25 seconds.

(NOTE: With some computer and radio combinations, scanning at

a very fast rate can result in intermitent errors.  If you

experience this problem, just make the pause time slightly

longer).

<Space Bar> is used to START and STOP the scan.  If the current VFO

frequency is between the LOWER and UPPER scan limits, the scan

will begin at the current frequency.  If the current VFO frequency

is outside the limits, the scan will begin at the lower limit.

You can use the <Space Bar> to momentarily pause during the scan,

and then start it again where it left off.

<Esc> key is used to leave the scan screen, and return to the Main

Screen functions.

PAGE 5.0

LOCAL MEMORY CONTROL

--------------------

The LOCAL MEMORY system allows you to store and recall radio

information, along with a brief description of each memory entry.

The following keys are available from this screen:

<Up> and <Down> Arrow Keys are used to select the memory line to be

saved or recalled.  Each "Page" of memories contains 22 lines, and

each line is one memory.

<F1> This function key is used to store the present radio information

(Frequency, VFO, Mode, Split), on the currently-highlighted line.

When you first hit this key, the current status of the radio (as

indicated by the window at the lower right corner of the screen)

will be captured and saved.  A cursor will appear in the

"COMMENT" column of the highlighted line.  You may then enter a

brief description of this memory for identification.

<F2> Hitting the <F2> function key will recall the memory shown by

the currently-highlighted line.  This means your radio will

change to the frequency and mode (as well as split, if that's

what the condition was when the memory was saved) to reflect

that memory line.  Hitting the <Return> or <Enter> key will do

the same thing as this function key.

<F3> This key will display a cursor at the "FREQUENCY" column of the

currently-highlighted memory line.  You may type in a frequency

different than the one displayed, or you can hit <Enter> to keep

the original frequency.  Once you hit <Enter>, the cursor will

move to the "MODE" column, the next time you hit <Enter> the

cursor will move to the "VFO" column, and finally to the

"COMMENT" column.  At each location, you may either type in new

information or just hit <Enter> to keep the old information.

<F4> The <F4> function key displays the existing memory files for the

current disk directory.  You can select an existing memory  by

positioning the highlight bar to the file name, and hitting

the <Enter> key.  The memories contained in that file will then

be displayed.  You can create new pages by hitting any other key

besides the arrow keys.  This will cause a prompt to appear at

the  bottom of the screen, asking you to enter a memory file

name.  A new file will be created, using the first 8 characters

you supply at the prompt.  The last four characters of the file

are always ".MEM", and the RIG-EQF program will automatically add

these characters.  Memory files are always located in the current

disk directory, as indicated at the lower left corner of the

memory page screen.

<Esc> Hitting the <Esc> key returns you to the Main Screen.

<Page Up> and <Page Down> will tune the radio frequency of the

current VFO up or down, the same as on the Main Screen.

------ Copyright (C) 1991 -- Thomas J. Dandrea -- N3EQF ----------------------