SATAIM  - -  Earth Station AZ/EL Chart Printer,  Version 2.3

_______________________________________________________________________________

What does it do?    SATAIM provides a printed chart which lists AZ/EL aiming

coordinates for geosynchronous satellites of interest to

North America.

___________________________________________________________

What you'll need    An IBM compatible AND a printer.  No screen display of

coordinates is provided...yet. The program's two files -

SATAIM.EXE and SAT.FIL - must be in the same directory.

___________________________________________________________

New to v2.3         The following enhancements have been made in version 2.3:

- Satellite information has been updated to include the

Galaxy 1 move, as well as several other "event" through

mid-July '91.

- "Frequency" column on the printout has been expanded to

accommodate user comments (examples such as "Inclined

Orbit" have been included).

___________________________________________________________

Using SATAIM        To print a satellite coordinate chart, follow these steps:

System Prompt...         Enter...

TERMINAL LOCATION?       Earth Station name (whatever name

you may use to refer to the dish

location).

LATITUDE?                Numbers in this format: xx,xx,xx

LONGITUDE?               (degrees, minutes, seconds).

NOTE:  Leading zeroes are not

needed.

Hit any key to Print,    Any key to print, "Q" to quit.

"Q" to quit...

Need extra copies?       Y or N.  If NO, program ends.  If

YES, program asks...

How many (1 to 5)?       1 to 5...SATAIM will print the

requested number of copies.

___________________________________________________________

SAT.FIL             SAT.FIL contains all of SATAIM's satellite information and

can be easily changed if you wish to add or delete

satellites.  SAT.FIL currently has a separate line for each

of 38 satellites, but may contain up to 40 lines.  If

SAT.FIL is edited to more than 40 lines, the program will

not continue.

SATAIM Documentation (continued)

_______________________________________________________________________________

Text Editing        Any ASCII text editor may be used, as long as it does not

embed any word processing codes.

___________________________________________________________

Deleting a          When deleting a satellite, be sure to delete the entire

Satellite          line (including carriage return/line feed).

___________________________________________________________

Adding a            To add a satellite, insert a new line containing the

Satellite          following information:

- the satellite's longitudinal position

- the name of the satellite (13 characters max.)

- a comment line (25 characters max.)

EXAMPLE:

999.9,"BOGUSTAR 2","Ku/X - Experimental craft"

___________________________________________________________

Helpful Hints       Keep in mind the following when editing satellite

information:

- use commas and quotes where indicated.

- make a separate line for each satellite's information.

- insert new lines in proper longitudinal order (program

does not sort list).

- be careful when editing, since no error-checking is

done on the information in SAT.FIL (the program only

checks for the file's presence and whether it has

40 lines or fewer).

___________________________________________________________

Input Problem?      Although SATAIM has been successfully tested on several

different machines and BIOSes, at least two users have

reported a strange incompatibility.  With their particular

systems, the program refuses to accept valid Latitude or

Longitude input.

It has been determined that if the input error-checking

code is disabled, the program works fine on their machines.

If you experience this problem, I'll be happy to upload a

similarly modified program to your mailbox.

___________________________________________________________

AUTHOR              Jim Perry

Chief Engineer, WTIC-TV, Hartford, CT 06103

CompuServe 74255,1607