SPEC525.DOC                Wednesday, August 8, 1990                 Page 1

+---------------------+

|                     |

|    S P E C 5 2 5    |

|   (SPECTRUM  525)   |

|                     |

+---------------------+

Version 3.1

A Mouse Control Program

for the

JRC Model NRD-525 General Coverage Receiver

by

James P. Hawkins

August 8, 1990

(First Release) April 24, 1990

Copyright (C) 1989-90, James P. Hawkins

All Rights Reserved

James P. Hawkins

P.O. Box 9146

Trenton, New Jersey 08650

SPEC525.DOC                Wednesday, August 8, 1990                 Page 2

* * * S P E C 5 2 5 * * *

(SPECTRUM 525)

INTRODUCTION

This program was written by James  P.  Hawkins  WA2WHV  as  a

visual  means  of  controlling  the JRC NRD-525.  Most of the

receiver controls  are  available  on  the  PC  screen.   The

salient   feature  of  this  program  is  the  display  of  a

multicolored frequency  spectrum  chart,  divided  into  band

allocations.  Each allocation can be clicked on as if it were

a menu selection which causes a frequency calibrated expanded

dial.   In  addition  to  the  frequency  marks, the dial can

contain station names  or  any  markings  the  user  desires.

These  markings  are stored in data files.  It requires a VGA

graphics as the resolution must be high enough to resolve the

frequency  dial.   The program is actually an ACTIVE spectrum

chart, broken up into 6 bands.  At this  time,  the  data  is

based  files  which  represent  the  hour  of the day and one

common file which contains markings which will appear for any

time of day.

INSTALLATION

Before proceeding, make a backup copy of  the  diskette.   To

install spec525, insert the diskette and type a:install.  The

installation will leave you in the C:  The program  is  setup

to  operate on port COM1 at 1200 baud.  If you wish to change

the  port  to  COM2,  edit  the  SPEC525.INI  file  so   that

port=COM2.  The same applies for ports COM3 and COM4.  To set

the baud rate to 1200  on  your  receiver,  hold  the  "memo"

button  down  and  press  the '8' button repeatedly until the

number 1200 is observed on the frequency  dial.   You  should

now be able to start the program by typing spec525.

If you want the program to be universally executable add  the

following line to the end of your autoexec.bat file.

path=%PATH%;c:\spec525

The program needs an SPEC525.INI  file  present  in  whatever

current directory you are in when you invoke it.  If there is

none present, it will attempt to create one.  The data  files

used  by  spec525 should reside in whatever path is specified

by the "path=" line in SPEC525.INI.  Initially it is  set  to

SPEC525.DOC                Wednesday, August 8, 1990                 Page 3

C:   If you have the demo, type DEMO525 to start the program.

If it's the registered version type SPEC525.

CONTROLS

This section describes the graphics control panel.

Dial Pointer

On the  top  of  the  screen  is  a  dial  pointer  which

indicates  the current selected position on the frequency

spectrum or dial.  It starts up at zero.

Spectrum/Magnified Bandspread

The area  below  the  dial  displays  a  portion  of  the

frequency  spectrum  between 100KHz and 34 MHz, depending

on what the  band  switch  is  set  to  (read  on).   The

spectrum  display  consists  of  color  coded areas, each

representing a specific allocation of the band.  The left

mouse  button  may  be  clicked  on or dragged across the

spectrum to change the  frequency  in  10KHz  increments.

The  dial pointer will follow and remain where it is left

when the mouse button is released.  If  the  right  mouse

button  is  clicked  on  one  of the allocations, it will

expand or magnify into a frequency calibrated dial  which

is  controllable  in  1KHz increments with the left mouse

button.  On the bandspread dial, is a  horizontal  center

line,  above  which  are  frequency markings.  Below this

line are labeled markings which may be added  or  changed

by the user.

Fine Tune

The fine tune dial, which is below and to the  center  of

the  spectrum  dial,  allows  the  user  to tune in 10 Hz

increments by clicking or dragging in the lower  half  of

the  box with the left mouse button.  The dial pointer in

the upper half will follow and  remain  where  the  mouse

button is released.

Fine Tune Offset Indicator

This indicator is to the right of the fine tune  bar  and

indicates  the plus or minus offset which is added to the

main dial when the fine tuning is changed.

Memory Channel Map - MCM

MCM is  a  visual  representation  of  the  200  receiver

channels  numbered every fifth square from 0 to 195.  The

squares seem small  at  first,  but  are  quite  easy  to

select.   The  channel  map is in halfway down the screen

and contains  two  rows  of  squares.   The  top  row  is

numbered  to 95 and the bottom row is numbered to 195.  A

channel is addressed by simply moving the cursor  onto  a

square and clicking the left mouse button.  There are two

selections which determine whether clicking a channel map

square  will  store  the  current  receiver  settings  or

retrieve  those  already  contained  in    the   receiver

channel.   They  are  labeled RECALL and STORE with "MCM"

SPEC525.DOC                Wednesday, August 8, 1990                 Page 4

appearing below them.  The highlighted option is the  one

in  effect  and  it will remain in effect until the other

option is chosen by clicking the  mouse  on  the  option.

Initially  the program starts in a RECALL mode to prevent

inadvertant destruction of  a  channel.   The  user  must

deliberately chose STORE.

With this method, the user may easily store large  groups

of  frequencies  very  quickly by alternately clicking to

the frequency, then clicking the memory channel while  in

STORE  mode.   The  user may also type in frequencies and

channels as will be shown.

Slew control

This is below and to the left of the Spectrum display and

consists of a circle and two arrows on either side.  This

control is only effective when the display  is  magnified

to  a  bandspread.   Clicking on the right or left arrows

steps the expanded display to the next allocation without

having  to  go  back  to  the  spectrum.  Clicking on the

circle,  simply  redisplays  the  current  band  display.

Slewing  is  limited  to  either  end of the current band

selection.

RECEIVER CONTROLS

In the lower half  of  the  screen  are  other  controls  and

indicators.  A selection is made by "clicking" the left mouse

button on your selection.  When a selection is made the  menu

item  displays  black  lettering  on  white  background  in a

reverse video fashion.  The NON-MODE  menu  (SET  FREQ,  DIAL

MARK, SAVE DB, CHAN STO. CHAN REC.)  does not highlight as it

does not put the receiver into any mode.

Digital Frequency and Time Indicator

Indicates the time and current frequency setting  of  the

receiver  PLUS  the value of timeshift in the SPEC525.INI

file.  The user  may  edit  the  timeshift  parameter  to

display  any  timezone shifted from his/her PC time.  The

best choice is probably Universal Time.

Mode Switch

Allows selection of RTTY, CW, USB, LSB, AM, FM, FAX using

the  left  mouse  button.   The  black  area  at  the top

indicates the current setting.

Bandwidth Switch

Allows selection of receiver  bandwith  to  WIDE,  INTER,

NARROW,  AUX.   Note  that  the NARROW selection will not

work without the optional CFL-232 filter mounted.

CHAN STO. and CHAN REC.

Allows setting of receiver channels.  Clicking on CHANnel

STOre  will  prompt  the user for a channel number.  Upon

entering the channel number, the receiver channel will be

set to the current program parameters.

SPEC525.DOC                Wednesday, August 8, 1990                 Page 5

Clicking the CHANnel RECall will prompt the  user  for  a

channel  number.   Upon entering the number, the receiver

will be set to the parameters contained in  that  channel

number.

I may remove these menu options to make room for  others,

as the previously explained channel map method of storing

and recalling receiver settings  is  a  visual  and  more

convenient way of setting channels.

DIAL MARK and SAVE DB

Allows setting of memory and dial labeling.  Clicking  on

the  DIAL  MARK  causes the program to ask for a typed in

input at the bottom of the screen.  When the  user  types

in a label, followed by a carriage return, the bandspread

dial will display a labeled tick mark at  the  point  the

the  frequency  was  set when the label was typed in.  If

the program  is  on  an  expanded  dial,  the  dial  will

redisplay to show the new mark, however, if the frequency

is not in range of the current expanded  dial,  the  mark

will  not show up unless the user goes to the appropriate

expanded dial.  This might happen in the case of a  typed

in  frequency.  REMOVAL of a dial mark is accomplished by

setting  the  dial  to  the  mark  to  be  removed,  then

following  the  above  procedure,  but  entering  only an

<ENTER>.   The  SAVE  DB  option  stores  the  dial  mark

information  in  the  data  base for the particular hour.

The labels are temporary unless the user clicks the  SAVE

DB option.

SET FREQ Option

When this option is "clicked", the prompt will ask for an

input frequency to be typed in KHz.  This allows the user

to type in a specific frequency manually.   This  feature

in conjunction with the DIAL MARK function can be used to

enter a series of frequencies  and  labels  from  a  list

without having to view them or move the frequency dial or

change the bandswitch.  Also by using the  DBASE  option,

the  entry  can  be  made for the appropriate hour of the

day.  It is important to remember, however that  SAVE  DB

must be clicked to store them away.

SEARCH and SRCH NEXT

This  selection  allows  the  program  to  search  for  a

specific  station  name  entry when typed in by the user.

First occurrences of a label will be found  by  selecting

SEARCH.  Subsequent occurrences of the same label will be

found by using SRCH NEXT.  If you click the SEARCH option

it  prompts  you  for a label. So, if you type in wwv, it

will  look  for  the  first  occurrence  or  lowest   wwv

frequency.  If you then click SRCH NEXT, it will look for

the next occurrence of wwv.  Each time  you  select  SRCH

NEXT it will look for the next occurrence.  So if you had

entered all the wwv frequencies, it will select  2.5  MHZ

SPEC525.DOC                Wednesday, August 8, 1990                 Page 6

first  then as you keep selecting SRCH NEXT it will cycle

through 5.0, 10.0, 15.0, 20.0 and 25.0 MHZ and then cycle

back  to  the  first.   So  if  I  want  to find the best

reception of WWV, I just  type  it  in  once,  then  keep

searching  for the best.  Input is not sensitive to upper

or  lower  case  so  it  does  not  matter  whether   the

characters are upper or lower case.

AGC

AGC FAST/SLOW/OFF

ATTEN

Attenuator ON/OFF.

REM

Remote control ON/OFF.  This frees up the receiver to  be

controlled  manually,  by  disabling program control.  If

receiver is turned on after program is started,  you  can

synchronize  the  receiver  with  the program by clicking

remote off, then remote on.  Turning on  remote,  outputs

ALL   of   the  saved  receiver  status.   This  is  also

convenient  if  you  want  to  turn   the   remote   off,

temporarily change the actual receiver controls, then put

it back to the state it was in by turning the remote  on.

The  idea  being that the receiver should be in the state

the program is in when the receiver is listening.

DBASE

Database file 0-23.  Selects which file to use  for  dial

markings.   The  numbers  represent  the hour of the day.

The program automatically  selects  the  data  file  that

represents  the current hour of the day as it is known by

the PC.  When the hour changes, the  data  file  changes,

thus changing the dial markings.  The datafiles are named

dbandN.525 where  N  represents  the  hour  of  the  day.

Another  file is cmnband.525.  This file contains station

names and frequencies that are to be displayed regardless

of  hour such as Citizens band channel numbers, WWV, etc.

Dbase files are found in the directory path specified  by

the  path setting in the SPEC525.INI file which exists in

whatever current directory the user is in at the time  of

program execution.

ADB

Automatic database, disables automatic time switching  of

data file.

Bandswitch

Selects the frequency range of the spectrum display.

RECEIVER STATUS SAVE

The uses editable data contained in the SPEC525.INI  file

to  initialize.   As  previously  stated,  SPEC525.INI is

located in whatever directory the user is in at the  time

of  program  invocation.   When  a user exits the program

SPEC525.DOC                Wednesday, August 8, 1990                 Page 7

normally by clicking on the exit menu selection,  control

settings  and frequency settings are saved SPEC525.INI in

the form of NRD-525 command strings.  The port name, baud

rate  and  datapath  and  timeshift are also specified in

this file.  When the program is started, it looks for the

values in this file to reset controls.  If a value is not

present, a default setting, hardcoded in the  program  is

used.

The  "timeshift="  entry  in  SPEC525.INI   allows   time

adjustment for time conversion to any time zone including

Universal time.  For example if Universal time is desired

and  your  PC is set to EDT, then set timeshift=4 for a 4

hour time difference.   The  program  will  then  display

Universal  time.  Set timeshift=0 if you want the program

to display the time set in your PC.

The user may edit these values manually and the order  is

unimportant.   When  the  program is terminated normally,

the file will  be  rewritten.   Missing  values  will  be

entered and modified values will be changed.

If the file does not exist or is empty, the program  will

create one with the default values.

This editable form makes it  possible  for  the  user  to

write   another   program  which  can  set  the  receiver

parameters as they are all documented  with  the  NRD-525

interface  card.   An example of the contents of the file

is as follows:

port=COM1

baud=1200

datapath=C:\USR\JIM\

timeshift=4

A0

B3

C001

D0

E1

F00113000

G0

I0

The default settings are as follows:

port=COM1

baud=1200

datapath=c:\spec525\

timeshift=0

A0

B0

C001

D4

E1

F00113000

SPEC525.DOC                Wednesday, August 8, 1990                 Page 8

G0

I0

FUTURE PLANS

When time allows, I am planning to add an additional menu

to  the  program  which  will  allow  the user to LOAD or

COMBINE station data from other  named  data  files  then

save  the  file  under  any desired name.  For example, a

single  file  may  contain  a  set  of  frequencies   for

HURRICANE  tracking; another might contain stations which

contain Spanish Speaking Stations.  These can  be  loaded

separately   or   combined.   Another  could  be  a  file

containing dipole antenna lengths so  that  when  loaded,

the  dial  will  be  a rule to calculate antenna lengths.

Any  ideas  on  ways  to  implement  these  features  are

welcome.   Send  me  postal  mail  or  contact me through

CompuServe ID 71760,3362.

SPEC525.DOC                Wednesday, August 8, 1990                 Page 9

SHAREWARE and COPYRIGHT NOTICE

Welcome to user supported software (commonly referred  to  as

"SHAREWARE").   User  supported  software  allows  you to try

software before you buy, and when you do  buy,  to  pay  much

less  than for commercially developed software.  This is true

because there are no costs  for  advertising,  sales  staffs,

fancy  packaging  and  distribution  costs  bundled  into the

software. Just good software, not glossy  packaging,  and  no

hassles.  Pay  only  if  you  like  it.  Your registration of

SPEC525 will encourage more development of this type software

and  ultimately  provide  better  software at lower prices to

everyone.  SPEC525 3.1 and  its  follow  on  products  depend

entirely upon your support for their existence.  This program

is a demob, but the only disabled feature is the  ability  to

save  new  label  entries (SAVE DB) and the number of station

labels is limited to 200.  If you register  this  program,  I

will mail you a fully working copy of the program.

SPEC525 V3.1 is protected by U.S. and international copyright

laws.   SPEC525  was  written  by  me,  James  P.  Hawkins in

Microsoft C and is Copyright (c) 1988-90 by James P. Hawkins.

All rights are reserved.

_________________________________________________________________

You  are specifically granted the right to use SPEC525 3.1 on

a 30 day trial basis.  If you like it and you  want  a  fully

functional  copy,  you  must  register  your  copy by sending

$36.50 to

James P. Hawkins

SPEC525.DOC                Wednesday, August 8, 1990                Page 10

P.O. Box 9146

Trenton, NJ 08650-1146

Residents of NJ must include sales tax.

(Currently 7%, so please send $39.05).

_________________________________________________________________

You are also encouraged to share SPEC525 V3.1 with others and

to distribute copies on computerized bulletin board  systems,

however you may not:

1)  Charge  for,  or  otherwise  sell,  lease  or  rent  this

program.  This  also  prohibits  bundling of this program

with some other product or service.

2) Modify the program or its documentation in any way.

3) Distribute the program without this .DOC file.

4) Distribute this program as part of  a  promotion  for  any

commercial  venture  or as an inducement to purchase some

product or service.

You MAY, however, charge a fee for  the  actual  distribution  of

this  program.  In this case, it must be made clear to the person

receiving SPEC525 that a registration fee is still  required,  if

the user intends to keep SPEC525.

Warrantee: This program is being provided "AS IS". It  is  up  to

the  user  to determine the suitability of this program for their

purposes. The user takes all responsibility  for  any  direct  or

consequential  damages.   It is written for VGA graphics only.  A

monochrome version may be made available in the future when I get

the chance.

Support: Please DO report all bugs, difficulties or  improvements

desired.   Registered  users will be notified of the availability

of the updates.

I have not yet determined the price of updates.  It  will  depend

on  the change.  If an update is only serious bug fixes, the cost

will be very minimal.   If  major  enhancements  are  added,  the

update will be more.

SPEC525.DOC                Wednesday, August 8, 1990                Page 11