THS - TNC Hostmode Server

=========================

HB9CVV

August 1989

Version 2.50

THS is a Packet Radio terminal  program for the IBM-PC. THS reqires  one

of the following devices:

- a TNC-1 with the WA8DED V1.3 Software.

- a TNC-2 with the WA8DED V2.3 software.

- a DRSI PC*Packet Adapter and the DRSI supplied TNC driver

software.

Major features of THS are:

- user configurable comm-port, colors and texts

- split screen operation

- command and parameter entry in windows

- extended help functions

- review of received text

- printing and snapshooting the review-buffer

- logging (capturing) to file

- online printer support

- send text from file

- binary file transfer using YAPP protocol

- multi-channel operation

- heard list showing the path

- Net/Rom frames are decoded

- wordwrapping is available

- temporary exit to DOS

- built-in message editor

- support for screens up to 80*60

- 16550A Chip support with FIFO

The list of current  revisions of THS version  2 is found at  the end of

this document.

THS has been developped  on a 386-AT with  VGA under IBM DOS  3.3. Tests

have been run on PC/AT's and  PC/XT's with EGA, CGA and Hercules  cards,

and under different versions of IBM DOS 3.x.  THS may run or may not run

on other  hardware, and  on earlier  or other  versions of the operating

system.

Tests show that on ordinary PCs running at 4.77 Mhz the program overhead

is too high  to serve a  9600 baud line,  if you experience  RS-232 port

overrun errors, you must reduce the baud rate to 4800 baud, or even 2400

baud.   However 8  Mhz or  10 Mhz  PCs run  well with  9600 baud  lines.

PC-ATs have no  problems. THS will  not run under  the pseudo multi-task

shells like Desqview or Windows.

The memory requirements of  THS are about 360  kB. In these days,  where

640kB memory is standard, this should  not be much of a problem.  If you

can start THS, but get once  the message "TERMINAL ERROR: cannot open  a

window" you  do not  have enough  memory to  run THS.  It is possible to

reduce the memory requirements of THS by reducing the window sizes.  The

minimum requirements will be about 250kB.

THS covers my needs for packet radio so I implemented the features which

I wished to have, and did not bother to fit all tastes and to match  all

possibilities.   THS may  well be  not adequate  for your  requirements,

please remember however that you are  not forced to use it, nor  did you

have to pay for it.

THS is not copyrighted, I release it to the Public Domain. Any  positive

feedback is  welcome, note  however that  I feel  no obligations  of any

kind.

1. Principles of Operation.

---------------------------

THS runs  the TNC  software exclusively  in HOST  mode, which  permits a

maxiumum of control and cooperation. THS is written in TURBO-C and  uses

a realtime kernel called CTASK. CTASK is public domain software  written

by Thomas  Wagner, Berlin,  West-Germany and  is found  on BIX  (a truly

remarkable piece of software !).

The internal  implementation of  THS in  form of  five concurrent tasks.

This permits  full parallel  service of  communications control, screen,

and keyboard - you  will never notice delays  when you type (opposed  to

other similar programs).

The windowing system  is implemented using  the VITAMIN C  library (with

some modifications), a commercially available software library.  Vitamin

C permits writing to non-active  windows, so scrolling in an  underlying

window is possible.

This feature, together with the real-time operation, permits to continue

the  receiving  and  displaying  of  received  packets even when support

windows (e.g. help windows or parameter dialog windows) are opened.

2. Installation.

----------------

It  is  expected  that  you  are  familiar  with  the operation of a TNC

containing the WA8DED control software and with the documents describing

that software.

THS consists of the following files:

THS.EXE         TNC Hostmode Server.

THS.CFG         Configuration file.

THSHELP.MSG     Help file.

THSHELP.IDX     Index file.

THS.DOC         Document file

The first four files must reside  in the same directory. Before you  can

run THS, you  must edit the  configuration file THS.CFG  to reflect your

environment, and possibly you might have to configure your TNC also.

The syntax to call THS is:  "THS [filename]" where filename is the  name

of  a  configuration  file.  If  filename  is  omitted, THS will use the

default, THS.CFG.

1.1 The configuration file THS.CFG.

-----------------------------------

THS  reads  the   configuration  file  whenever   you  start  THS.   The

configuration  file  contains  information  about  hardware and software

parameters. If this information is  not accurate, THS will be  unable to

proceed.

NOTE:  THS  does  not  do  any  consistency  checks or range checks when

reading from the  CFG-file. You must  not omit lines,  or use parameters

with values other than specified in this chapter.

Here is an example of a configuration file:

---------------------<beginning of file>-----------------------------

HB9CVV-02

2     Device is TNC-2 with 2.3

0x2F8 Address of TNC COMPORT

3     Interrupt of TNC COMPORT

9600  Baudrate

1     Printer number (LPT1)

1     Use direct video I/O

Operator ist OFFLINE, your message will be recorded - vy 73 de Peter.

*** Peter - Qth: Port/Biel - JN37OC ***

07,00,07,00,00,02,01,15,07,00  Receive (upper, permanent)

07,00,07,00,00,02,01,15,07,00  Transmit (lower, permanent)

04,07,07,04,03,00,07,00,04,15  Statusline (middle, permanent)

01,11,01,11,01,11,01,02,07,04  Immediate command entry

01,15,01,15,07,00,00,14,01,14  Opening, THS-Help, Review

01,11,01,11,03,00,03,01,07,00  Parameter Dialogs

01,11,01,11,01,15,01,11,07,00  Activity during YAPP xfer

00,02,00,02,01,14,00,00,07,00  Parameter Dialog Help Line

01,11,01,11,01,15,01,11,07,00  Parameter Dialog Extended Help (File)

01,11,01,11,01,15,01,11,07,00  Other

00,11,03,11                    Channel indicator attributes & echo text

#Name & Qth

My name is Peter, the qth is Port near Biel/Bienne

#The Rig

The rig here is:

Program   : THS (TNC Hostmode Server)

Computers : PC/AT, 3MB RAM, 80 MB harddisk.

386AT, 4MB RAM, 150 MB harddisk.

TRX       : Ten-Tec Paragon, 100W

IC-271H, 100W

TW-4100E 45W

Antenna   : Remotely tunable GP for HF

2*11 element Yagi (2m)

19 element Yagi (70cm)

TNC       : TNC-1 with WA8DED V1.3

TNC-2 with WA8DED V2.3

DRSI PC*Packet Adapter

%T 40

$C DK1SL VIA HB9F HB9W

$C HB9SDD

$C HB9PQX VIA HB9CTU

}0,200,200,200,100,100

------------------------<end of file>--------------------------------

The first line

contains your call. THS sends  an 'I'-command followed by this  callsign

string to the  TNC on channel  0 during the  initialization. No comments

are possible on this line.

The second line

defines the device type: '1' for  TNC-1 with WA8DED V1.3, '2' for  TNC-2

with WA8DED V2.3, and '8' for the PC*Packet Adapter. The numbers are not

validated  by  THS,  specifiying  another  number  will leave the system

inoperable.

The third line

defines the address of your communications port. You must specify it  in

this hexadecimal presentation (0xhhh).   THS will detect and report  the

access of a non-existent commport.  (Standard values are 0x3F8 for  COM1

and 0x2F8 for COM2).  This line is meaningless  if the device is  type 8

(PC*Packet Adapter), it must exist however.

The forth line

defines  the  interrupt  number  of  your communications port. (Standard

values are 4 for COM1 and 3  for COM2). This line is meaningless if  the

device is type 8 (PC*Packet Adapter), it must exist however.

The fifth line

defines the  baud-rate for  the communications  port. It  must of course

match the baudrate you selected for  the TNC.  This line is  meaningless

if the device is type 8 (PC*Packet Adapter), it must exist however.

*** NOTE: If THS is unable to contact the TNC, a wrong commport  address

and/or interrupt, or a  wrong baudrate could be  the cause. The same  is

true if you get permanent RS-232 error reports from THS after startup.

The sixth line

defines the printer number (printer port). Legal values are 1 to 3,  for

LPT1, LPT2  and LPT3.  Printer output  to printers  connected to  serial

lines is not possible.

The seventh line

defines the video access. A '1' is used for direct video i/o, and should

be used for all video cards other than CGA.  If a value of 1 causes snow

or other visual effects, switch to a value of 2. This changes THS to use

retrace synchronisation - which is requred for older CGA cards.

The eighth line

contains your connect message. The line may not exceed 72 characters, if

it does then  the line is  truncated. This line  is loaded into  the TNC

whenever THS is started. You can however alter your connect message  any

time with the ALT/U (Unattended Mode) command.

The ninth line

contains your beacon text. The line may not excced 72 characters, if  it

does, then the line is truncated. You can alter the beacon text with the

ALT/B command.

The next ten lines  specify the colour attributes  of all windows.   The

scheme is:

+------------------------------------ Border, background

|  +--------------------------------- Border, foreground

|  |

|  |  +------------------------------ Title, background

|  |  |  +--------------------------- Title, foreground

|  |  |  |

|  |  |  |  +------------------------ Text, background

|  |  |  |  |  +--------------------- Text, foreground

|  |  |  |  |  |

|  |  |  |  |  |  +------------------ Inactive Input field, background

|  |  |  |  |  |  |  +--------------- Inactive Input field, foreground

|  |  |  |  |  |  |  |

|  |  |  |  |  |  |  |  +------------ Active Input field, background

|  |  |  |  |  |  |  |  |  +--------- Active Input field, foreground

|  |  |  |  |  |  |  |  |  |

07,00,07,00,00,02,01,15,07,00  Receive (upper, permanent)

07,00,07,00,00,02,01,15,07,00  Transmit (lower, permanent)

04,07,07,04,03,00,07,00,04,15  Statusline (middle, permanent)

01,11,01,11,01,11,01,02,07,04  Immediate command entry

01,15,01,15,07,00,00,14,01,14  Opening, THS-Help, Review, Editor (Note 1.)

01,11,01,11,03,00,03,01,07,00  Parameter Dialogs

01,11,01,11,01,15,01,11,07,00  Activity during YAPP xfer

00,02,00,02,01,14,00,00,07,00  Parameter Dialog Help Line

01,11,01,11,01,15,01,11,07,00  Parameter Dialog Extended Help (File)

01,11,01,11,01,15,01,11,07,00  Other

The numbers are decimal. THS does not validate the range.

1.) An error in this version limits the foreground color for the editor

to black...

Number  Foreground      Backgrround

00      BLACK           BLACK

01      BLUE            BLUE

02      GREEN           GREEN

03      CYAN            CYAN

04      RED             RED

05      MAGENTA         MAGENTA

06      BROWN           BROWN

07      LIGHTGRAY       LIGHGRAY

08      DARKGRAY        |

09      LIGHTBLUE       |

10      LIGHTGREEN      |

11      LIGHTCYAN       |- as above, blinking

12      LIGHTRED        |

13      LIGHTMAGENTA    |

14      YELLOW          |

15      WHITE           |

Line twenty (this line is new in version 2)

must contain  four or  six numbers.  The first  four numbers defines the

attributes for the channel indicator. Please refer to the discussion  of

channels below.  The first  two numbers  in this  line define  the color

attributes of  the on-screen  channel, the  next two  numbers define the

attributes for an off-screen channel which has received data. Of the two

number pairs, the first is  the background attribute, the second  is the

foreground attribute. In the example, the statusline text attributes are

"03,00" which means 'black on cyan'. The example line 20 is:

00,11,03,11,02,00               Channel indicator attributes & echo text

which reads: the number of the on-screen channel is 'lightcyan on black'

(00,11) - and the number of a off-screen window which has received  data

is 'lightcyan on cyan' (03,11).

The third number pair defines the attributes for the transmitted text in

the receive window, and may be  omitted. The text you type is  echoed in

the lower (transmit) window. When you send the text by typing a carriage

return,  the  text  is  moved  to  the  upper  (receive) window, and the

transmit window  is cleared.  THS uses  different attributes  so you can

easily distinguish the  text which you  sent from the  text you receive.

By default, the text sent has the same text and background color as  the

received text, but is  intensified. This is what  you get if you  do not

specify the third number pair. If you do, THS will use these attributes.

Example:  if  you  have  defined  'green  on  black'  for  the text- and

background-color of the receive window,  which is the number pair  00,02

and you  want your  text reversed,  not intensified,  then define  02,00

here.

The next lines

are optional and are composed of four different types of strings:

- (#) text strings and titles for text strings.

- (%) initialisation command strings.

- ($) online command strings.

- (}) THS parameters

The text strings  are assigned to  the function keys  F01 to F10  in the

sequence they appear  in the configuration  file, when you  press such a

function  key,  THS  sends  the  associated  text  string to the TNC. By

pressing ALT/F THS shows the titles of the text strings in a window box.

The line with a hash-sign (#) in column one defines the title which  can

contain up to  30 charcters. Everything  exceeding 30 characters  is cut

off.  The lines following the  title line until the next title  line, or

the end of the file form the text string. The length of all text strings

together  is  limited  only  by  the  available  memory space.  The text

strings  are   intended  for   reasonable  sized   texts  like   station

descriptions etc.. Up to ten  #-lines are possible. If you  specify more

that ten #-lines, they are ignored.

You can also specify up to 10 lines with percent (%) sign in column one.

The text following the percent sign must not exceed 40 characters and is

sent as  a command  to channel  zero when  THS loads  the TNC parameters

during startup.  This is  done after  THS has  sent the  default set  of

parameter  commands  and  so  allows  the  user  to override the default

parameter set of  THS. This set  is described below.  You should however

take some care - the contents of the %-lines are sent unconditionally to

the TNC and  must be a  valid command to  be effective. A  %-line should

contain also only meaningfull commands  which do not interfere with  THS

operation. It makes no sense to have a line like:

%MN

to  disable  monitoring,  because  this  obviously  would  disable  THS'

heard-list feature.  It could make sense however to increase the TXDELAY

if your rig requires it.

The $-lines must  also contain valid  TNC commands. Up  to 20 lines  not

exceeding  72  characters  are  possible.  They  are  stored  by THS and

presented  together  with  a  selection  character when you press ALT/W.

When you press the  selection character or move  the bar to the  desired

entry, then the associated string is sent as a command to the  on-screen

channel.  The  $-lines are intended  to build a  call-sign directory for

connects, you may  however use other  commands there as  well, so it  is

called "Command String Directory".

The }-line  allows you  to modify  the THS  configuration. This  line is

optional. The format is:

}a0,b0,b1,b2,b3,b4,c0,d0,d1,e0

RS-232 control line usage: a0

If a0  is zero,  then THS  ignores the  RS-232 control-lines  (DCD, RTS,

CTS).  This allows just a 3-wire (TXD, RXD & GND) connection to the TNC.

Note however that in  this case THS cannot  detect if a TNC  is offline,

and will always  enter the TNC-initialisation  routine.  The  default is

non-zero.

Column size of the virtual receive windows: b0 to b4

b0 to b4 define the number  of lines in the virtual receive  windows for

channels zero  to four.  This defines  how many  lines are available for

reviewing.  The  minimum  value  is  40,  the  maximul value is 200. The

default is the maximum for all windows.

Software delay: c0

c0 is  a time  value in  microseconds for  a software  delay loop and is

effective only if the device is  a PC*PA. THS will wait c0  microseconds

before  every  access  to  the   TNCTSR.  This  may be required on  very

fast machines (e.g. 386 machines with cache).  The range of  c0 is 0  to

5000, the default is zero.  If you ever  experience a THS  error  report

(a  window appearing in the top half of the screen) with the PC*PA, then

you should experiment  with  this  value.  Start  with  c0 = 1000.  Note

that higher values may degrade the performance of THS however.

Editor window size: d0, d1

d0 and d1 define the editor window size, columns and rows. The range for

d0 (column)  is 64  to 79,  and includes  the window  border. If a given

value  exceeds  one  of  the  limits,  then the according limit value is

taken. If you omit this parameter,  or specify a zero, then the  default

value of 76 is taken. The range for d1 (row) depends on the size of  the

(upper) receive window, which itself  depends on the number of  lines of

your current video mode. The range if from half that window to full size

minus one. For the  standard 25 line mode,  the range is 10  to 20, also

including the border.  The default here  is the lowest  value, i.e. half

the size of the receive window.

Filter received characters: e0

e0 determines the initial value of the FILTER variable, see the

description of the ALT/T command. A zero disables the filtering, a one

enables it. The default (e.g. if omitted) is zero.

2. Starting THS.

----------------

When started, THS saves the current screen and presents three basic  and

one opening window. The basic windows are the receive window, the status

line (window)  and the  transmit window.  The basic  windwos are  always

present. THS has three major modes: dialog mode, review mode and  binary

file transfer (YAPP-)mode. The initial  mode is dialog mode.   Note that

THS will determine the current number  of rows used for the screen,  and

will adjust its windowing system accordingly.

To prevent overrun errors on the RS-232 port, THS disables the functions

of  the  CAPS-lock,  NUM-lock  and  SCROLL-lock  keys  when it used with

external TNCs connected to COM-ports with standard serial communications

controller chips. Such  controller chips are  the 8250, 16450  and 16550

chips. The  16550A chip  has a  built in  FIFO (a  16 byte hardware data

stack) and interrupt  servicing of this  chip is far  less critical. The

Dual Async Adapter/A from IBM in PS/2 Microchannel models is supposed to

use 16550A chips. For Industrial Standard Bus machines it is possible to

simply replace the 8250/16450/16550 chips on the serial adapters with  a

16550A chip to  benefit. THS will  detect a 16550A  chip and enable  the

FIFO (If the PC*PA is  used, no interrupt timing problems  exist because

the PC*PA's driver is polled).

The opening window  presents information about  the (hardware) state  of

the communications  port, and  informs you  about the  efforts of THS to

contact  the  TNC.  For  external  TNC's,  THS follows the guidelines of

KB5MU's "WA8DED  Host Mode  Guide" for  the initial  contact to the TNC.

For the PC*Packet Adapter, THS expects to find the TNC in terminal-mode,

not in host-mode.   Please note that when  THS exits, the TNC  is always

switched to terminal-mode.

If THS fails to contact the TNC properly, check the hardware  parameters

(comm-port address, irq, baudrate on  the THS side, and baudrate  on the

TNC side). After  a correction, a  new try should  be made only  after a

hardware reset on BOTH sides.

Note that  the standard  connection to  an external  TNC is  by a RS-232

cable which has the lines 1 to 8 and line 20 connected. This permits THS

to check the control signals to  determine if the TNC is switched  on or

not. If you disabled the use  of the control lines in the  configuration

file, THS is unable to detect an offline TNC immediately and will  start

the host resync pass (which then will fail).

THS  also  sets  the  parameters  of  the  TNC (indicated by the message

"loading TNC parameters". The callsign from line one of the CFG file  is

always set first,  followed by the  connect message and  the beacon text

line  (from  lines  8  and  9  of  the CFG file). Next, THS enables full

monitoring but excludes monitoring of  the own frames, i.e. frames  from

and to this callsign.

Finally, the following commands are sent if the device is an external

TNC:

"Y 4",        /* Enable all four channels */

"F 4",        /* Frame acknowledge (seconds) */

"N 10",       /* Number of tries (0=forever) */

"O 4",        /* Number of outstanding I frames */

"P 64",       /* P-persistence value */

"R 1",        /* Repeater enabled */

"T 40",       /* Transmitter delay 400mS */

"U 0",        /* Unattended mode disabled */

"V 2",        /* Version 2 protocol initiated */

"W 10",       /* Slot time interval (100ms) */

"X 1",        /* Transmitter PTT disabled/enabled */

"@D 0",       /* Full duplex disabled/enabled */

"@T2 100",    /* Timer T2 interval (1000ms) */

"@T3 18000",  /* Timer T3 interval (6 min) */

"@V 0"        /* Callsign validation disabled/enabled */

"P 64  and "W  16" are  sent only  to TNC-2's,  for a  TNC-1 with WA8DED

software version 1.3, a "W 16" is sent instead.

The following commands are sent to a PC*Packet Adapter:

"Y 4",        /* Enable all four channels */

"U 0",        /* Unattended mode disabled */

"V 2",        /* Version 2 protocol initiated */

"X 1",        /* Transmitter PTT disabled/enabled */

"@V 0",       /* Callsign validation disabled/enabled */

"P 0 1 64 10 4 4 10 100 18000 40",

"P 1 1 64 10 4 1 10 100 18000 30"

Next, for all TNCs, the %-lines read from the CFG file are sent to allow

an  overwrite  of  the  THS  defauts  (see  also  the description of the

configuration file above).

If you want to  accept less than four  connect-requests to you, you  can

alter this either online with  the "Yn" TNC-command in a  command window

(see the description of  the immediate-command-entry below), or  use the

Y-command in a %-line in the configuration file. E.g. %Y2 to accept only

two connect requests (from different stations).

THS finally comes up with channel ONE on-screen.

3. Operating THS.

-----------------

After a succesful  contact with the  TNC, you will  see the three  basic

windows:  the  receive  window,  the  statusline window and the transmit

window.

The presentation of the channels.

---------------------------------

The TNC software permits up  to four different and independent  channels

for simultaneous connections to different connectees. These channels are

channels one  to four.   Channel zero  is the  unconnected channel which

also receives all monitored frames. There is one receive window for each

channel.

On screen, there is  only space for one  receive window at a  time. This

receive window is called the 'on-screen window' and the channel which it

presents is called the 'on-screen channel'.

To switch between the channels,  you use the left-arrow and  right-arrow

keys. On the left side of  the statusline is the channel indicator,  the

string "01234", representing  channel 0 to  4. The on-screen  channel is

indicated  in  INVERSE  video.    The  number  of  a  off-screen   (i.e.

invisible) channel  which receives  data from  the TNC  changes to BOLD.

This is the  indication that there  is unseen activity  on that channel.

You can look  at that window  simply by making  it the on-screen  window

with the left/right arrow keys.

Note that these attributes (INVERS and BOLD) can be

changed for your taste in the configuration file.

The on-screen receive window.

-----------------------------

In  the  on-screen  receive  window  you  will  see  everything which is

received from the TNC on the associated channel - with the exception  of

packets during a binary file transfer (these go, of course, to a file).

If you  want to  review text  which has  scrolled off  the screen, press

ALT/R  or  Up-arrow.   During  review,  the  TNC  input to the on-screen

channel (only) is halted.  You should not leave THS in review mode for a

long time if  there is packet  activity on the  reviewed channel because

the TNCs buffer may overflow and cause loss of data.

The status line.

----------------

The status line  presents actual information  about the link,  the date,

and the time.  There are two  flavours of the  status line which  can be

toggled by the <INS>-key. The default is a line like:

01234 | NS:00 UA:01 TRY:01 | INFORMATION TRANSFER | PL | 05-AUG 23:46:45

|       |     |     |        |                      ||   |

|       |     |     |        current link stare     ||   date & time

|       |     |     current try number              |logging is ON

|       |     current number of unacked frames      printing is ON

|       current number of transmitted frames not sent.

channel indicator

NOTE: UnAacked frames have been  sent (once). NSent presents the  number

of  present  but  unprocessed  frames  in  the  TNC. Please refer to the

'L'-command of TNC's document.

The second flavour is a line like:

01234 | DK1SL VIA HB9VC HB9PUO                     | PL | 05-AUG 23:46:51

|

current connect state.

If the connected-state string length exceeds the available space, it  is

truncated.

The status line is always updated realtime (unless the receive window is

being reviewed). You can observe the quality of the link by watching the

time and number of tries used to acknowledge data frames.

The transmit window.

--------------------

The lowest three lines are the transmit status window, and all what  you

type for  your qso  is echoed  here. You  send the  entered text  to the

on-screen channel  as usual  by pressing  the ENTER-key  - the  transmit

window is cleared  and your text  is moved to  the receive window  using

bold characters  to easily  distinguish between  received text  and text

sent.

Note that these attributes (default: BOLD) can be

changed for your taste in the configuration file.

If your on-screen channel is 1,2,3 or 4, and you are not connected, then

the TNC simply  discards your input.  If you did  select channel 0,  the

text is send out UNPROTO (i.e. in an UI-frame) via the UNPROTO path.

The Commands

------------

We distinguish between  'normal' keys and  'special' keys. Special  keys

are  all  keys  not  found  on  the  main key field of the keyboard (i.e

function keys, keypad keys) and all ALT-key combinations of normal keys.

You use the normal  keys for data entry,  all special keys cause  THS to

perform some special function.

The following  special keys  are used  to control  THS (and  the TNC) in

dialog mode:

ALT/P   Device Parameter Setup Menu

ALT/T   THS Parameter Setup Menu

ALT/C   Connect Menu

ALT/D   Disconnect

ALT/U   Unattended Parameter Menu

ALT/G   Show Unformatted Heardlist

ALT/H   Show Formatted Heardlist

ALT/V   View DOS Directory

ALT/L   Logging (capture) ON/OFF

ALT/J   Printer ON/OFF

ALT/S   Send an ASCII file

ALT/R   Review Received Text

ALT/F   Review Function Key Setup

ALT/B   Beacon Text

ALT/W   Command string directory

ALT/K   Call Editor

PgUp    Binary File Upload (YAPP)

PgDn    Binary File Download (YAPP)

<INS>   Toggle Status Line Display

CTRL/ESC  Direct Command Entry

ALT/E   Temporary exit to DOS

ALT/X   Exit THS

With the  exception of  the function  keys proper,  pressing an 'unused'

special  key  causes  the  pop-up  of  a window which presents the above

information, this is to inform you about the 'used' special keys.

NOTE: All pop-up windows will disappear after some seconds. You can also

close the window by any keystoke.

ALT/P   TNC Parameter Setup Menu

--------------------------------

The Device Parameter  Window is shown,  and THS first  reads the current

value of all the parameters. You  then may alter one or more  parameters

(use the up- and down-arrow keys to navigate, and press F10 when you are

ready to apply the changes.  THS sets all parameters and  re-reads them.

If you specified a parameter to a value outside the permitted range, you

will see that  your change has  no effect. You  leave the TNC  Parameter

Setup  by  pressing  ESC.   Reading  and  writing the parameters is done

concurrently with  other activities  of course.  For your  assistance, a

one-line explanatory text is presented  for each parameter. If you  need

further help, you may press F1 to get a more detailed help information.

Most parameters are common to all channels. For all parameters which are

unique  for  each  channel,  only  the  on-screen  channels parameter is

influenced.

NOTE: If you change parameters and do not press F10 (the 'apply changes'

key) before you press the ESC key, then nothing is applied.

NOTE: If you change the HDLC  baudrate for a PC*Packet Adapter, the  you

MUST execute a TNC reset (via the ALT/T menu) next to make it effective.

ALT/T   THS Parameter Setup Menu

--------------------------------

The THS Parameter Setup Menu operates like the TNC Parameter Setup menu.

This  menu  is  used  to  modify  software parameters of THS rather than

software parameters of the TNC. You can:

-  enable  or  disable  timestamps  on  monitored  frames  (link status

messages are always stamped).

- enable  or disable  if you  want to  hear 'bells'  sent to you from a

connected station (you will never hear bells from monitored stations)

(BELL).

- enable or disable if you want  to hear a bell when you get  a connect

(CBELL).

- set the packet frame length for binary transfers (PKLEN).

-  set  the  maximum  number  of  pending  frames  for  transfer bursts

(MAXPFRAM).

- reset  the PC*Packet  Adapter. (Not  available for  external TNCs. To

reset such a TNC, exit THS, power off/on the TNC and call THS again).

- enable or disable word wrapping (WORDWRAP).

- define  the  window  which  gets  the monitored frames (MWINDOW).  By

default, this  is window  zero (because  channel zero  is the monitor

channel.  You  can  however  route  all  monitored  frames to another

channel window of your own choice.

- filter out non-printable charcters (FILTER). If this is enabled, only

BELL, HT, LF, VT,  FF, CR and the  characters with ASCII values  from

0x20 to 0x7E are displayed as is, the filtered charcter are shown  as

a small solid rectangle. If you use a national character set, MFILTER

should be disabled.

PKLEN needs an explanation. Obviously,  whenever you send a file  (ASCII

or binary), THS could send that file faster than the TNC is able to send

packets,  so  we  must  slow  down  somewhere  or the TNC's buffer would

overflow. Of the link state parameters which are readable from the  TNC,

there are two values which determine the number of packets which the TNC

has  already  got  for  transmission:  the  number  of  transmitted, but

unacknowledged frames, and the  number of sent (sent-to-the-TNC)  frames

not yet transmitted. The  sum of these to  numbers is MAXPFRAM. You  may

use this parameter together with PKLEN to tune the filetransmisson speed

to the quality of the packet link.

The maximum  numbers permit  the highest  possible speed  but requires a

high quality link.  This also implies  that the radio  channel is almost

exclusively  used  by  the  file-transfer,  leaving  no  room  for other

stations.  ! Don't do it on 'the qso channel' !

If you cannot withstand to do  a filetransfer when you have a  miserable

link, or if you insist to do a filetranfer on a channel with  activities

of other stations, then  use the minimum values.  In this case you  will

only have one short  packet outstanding and your  TNC will wait for  the

acknowledgement before sending out the next.

ALT/C   Connect Menu

--------------------

The Connect Menu lets you enter the destination callsign and ssid, up to

eight digipeaters, and the PC*Packet Adapter's port number. Again, press

F10 to execute  the command.   Pressing ESC exits  the connect menu  and

removes the window only.

On channel zero, this sets the UNPROTO path.

ALT/D   Disconnect

------------------

This causes a disconnect command for the TNC immediately. If you are not

connected, you  will get  a small  TNC status  window for  a few seconds

which tells you the TNC's return: CHANNEL NOT CONNECTED.

ALT/U   Unattended Parameter Menu

---------------------------------

Much  like  the  TNC  Parameter  Setup  menu, you can enable and disable

unattended operation here, and specify the connect-message.

ALT/H   Formatted Heardlist

ALT/G   Unformatted Heardlist

-----------------------------

A window is  presented with the  Heardlist. This is  a list of  the most

recent stations  heard, sorted  by descending  time (i.e.  last heard on

top).  The list can contain up to 30 different call signs, the  vertical

window size adujsts to the number of call signs but only up to 12 calls.

If more than  12 calls are  available, you can  scroll with up  and down

with the arrow keys. You also can scroll in the vertical direction if an

entry is larger than the vertical  window size. The window will stay  on

the screen 60 seconds if no action is done.

If you  used ALT/H  you get  exactly what  you would  have to  use for a

connection to such heard call.

If  you  pressed  ALT/G  instead  of  ALT/H  you  get a heard-list which

contains  the  uninterpreted  (unformatted)  lines,  i.e.  exactly   the

monitored headers as received by the TNC.

You will  see a  bar on  the first  entry which  moves when  you use the

up-arrow or down-arrow  key. When you  position the bar  on a line,  and

press F10 or <Enter> then a  connect command is send to the  source call

sign in the line, using the indicated path. Any key other than the arrow

keys, F10 or Enter removes the Heard-list window.

Please note the following:

The connect feature works also when you use the 'unformatted heard list'

window. If a source  call sign is heard  via a NetRom node,  it makes no

sense to connect it  this way.  Also  note that the heard  list gets not

updated when you switch monitoring off by a TNC command. The heard  list

processing is done by THS, *NOT* by the TNC - consequently it will  work

only  (a)  when  THS  is  active  and  (b)  when  THS  gets the required

information from the TNC.

ALT/V   View DOS Directory

--------------------------

You  are  asked  for  a  file  specification, and will get the directory

information.

ALT/L   Logging (capture) ON/OFF

--------------------------------

Logging means  writing out  to a  file everything  which appears  on the

screen for the logged channel. Logging is done on an individual  channel

basis,  i.e.  you  must  enable  and  disable  logging  for each channel

independently.

When you  press ALT/L  and logging  is currently  off for  the on-screen

channel, THS asks you for a filename. THS gives you a filename which  is

made up of the current date and time. If you want to accept this, simply

press ENTER, otherwise specify your  own filename. The extension of  the

filenmane is always "CAx" where x  is the channel number. The file  will

be created in the current directory,  on the current drive. If the  file

already exists, then THS-writes are appended to the existing file.

If logging is  already on, you  will be asked  if the logfile  should be

closed.

The letter "L"  appears on the  right hand side  of the statusline  when

logging is enabled. Logging is a channel attribute, so logging continues

for a channel as long as logging is enabled for the channel,  regardless

if the channel is on-screen or off-screen - until you disable it.

ALT/J   Printer ON/OFF

----------------------

Toggle the printer output on or off. Because you have multiple  channels

but only one printer, you can always print only one channel.

When you  press ALT/J  and printing  is currently  off for the on-screen

channel, then from that moment this channel (and only this channel) will

be printed. Printing continues when this channel goes off-screen.

The letter "P"  appears on the  right hand side  of the statusline  when

printing is switched on, and only one channel will show the "P".

Note that when you  are printing, and your  printer goes offline by  any

reason, THS will detect this and switch printer output OFF but will  not

hang.  Please  also  note  that  THS  has  an internal 1024 byte printer

buffer, so  with slow  printers, the  printer may  continue to print out

buffered characters for a while after you disabled printing.

Also, when you print a busy  channel with an almost maximum dataflow  of

almost 1200 bps on the HF side, your printer should be able to print  at

least 100 characters per second (!) continuously.

ALT/S   Send an ASCII file

--------------------------

You can send a  file containing text only  when you are connected.  When

you press ALT/S you are asked  for the filename, and if the  file exists

THS sends it out. You will see  the lines transferred to the TNC in  the

receive window  using bold  characters, just  like you  had typed in the

contents of the  file. You are  also informed about  how many lines  the

file contains, and how many  lines have been transferred.   Please refer

to the description of ALT/T for parmeters influencing file transfers.

Note: during ASCII file sends you cannot switch the on-screen channel.

ALT/R or Up-Arrow       Review Received Text

--------------------------------------------

ALT/R  lets  you  review  the  received  text  which has srolled off the

screen.  When reviewed, the  on-screen channel is polled, but  all other

channels are. THS' review buffer is 200 lines for each channel.

You can print the entire review buffer (that is, the used part if it  is

not filled up) by pressing ALT/J.  You can also write the review  buffer

into a file. The file is written in the current directory on the current

drive; the filename is generated by the current date and time, with  the

extension "SNP".

ALT/F   Review Function Key Setup

---------------------------------

ALT/F shows the titles of the text strings which can be called  pressing

the  function  keys.  Please  refer  to  the  description  of  the   THS

configuration file. If you have not used all function keys, pressing  an

unused function key will also show this window.

ALT/B   Beacon Text

-------------------

ALT/B shows the beacon text and lets you modify it.  Whenever you  press

Enter or F10, the beacon text  is sent (once) via the UNPROTO  path (see

ALT/0).

ALT/W   Command String Directory

--------------------------------

Please see the description of the $-lines in the configuration file.

ALT/K   Call Editor

-------------------

ALT/K calls the message editor. First you are asked for a filename, then

the  editor  loads  the  file  into  the  editor window. If no such file

exists, you are asked if the  file should be created. For a  description

of the editors  keys, press the  F1 help-key please.  When you exit  the

editor with the ESC-key, you are  asked if the file should be  saved. If

you agree, you can for example next send the file with ALT/S. If you  do

not want to  save to a  file, the editor  window is closed  and the next

time you  recall the  editor, you  are immediately  back where you left.

Note however that for to send the file, it must first be saved.

PgUp    Binary File Upload (YAPP)

PgDn    Binary File Download (YAPP)

-----------------------------------

Pressing PgUp or PgDn will start  a binary file transfer using the  YAPP

protocoll as  created and  specified by  Jeff Jacobson,  WA7MBL. THS  is

compatible   with   all   programs   which   have   'YAPP'  implemented.

Furthermore, the hostmode of the WA8DED software permits better  control

in the event link difficulties occur. For example, a disconnect during a

file  transfer  is  immediately  recognized  and the transfer is stopped

without sending  the remainder  of the  file out  via the  UNPROTO path.

Also, you can abort (cancel)  transfers by ALT/A. Note however  that the

cancellation will take  some time, because  the TNC must  first send out

all pending  frames until  the cancel  request frame  can be  sent, or a

incoming cancel request can be acknowledged.

This can be a severe problem when you have a bad link, but in this  case

you anyhow should better not  use automated transfers. Or have  at least

set the THS parameters accordingly -  see the description of ALT/T.   In

case of a bad link, you should also use AX25 level 1, not level 2.

At any time you can disconnect by ALT/D - this breaks the link but  also

flushes the TNC's buffers immediately.  Aborting a transfer by ALT/D  is

preferable to ALT/A. You  can also kill a  transfer with ALT/K. This  is

disconnects  the  link  and  reconnects  after  THS has switched back to

dialog mode.

THS uses the following timeout parameters:

(1) The sender will try to trigger the receiver (Send-Init) six times in

intervals of 20 seconds before timing out.

(2) The receive time-out (waiting for a YAPP packet) is 120 seconds.

You are able  to modify the  TNC or THS  parameters during ongoing  YAPP

transfers - the THS help window (which is called with an arrow-key,  for

example) will show you which keys have effect during YAPP transfers:

ALT/P   TNC Parameter Setup Menu

ALT/T   THS Parameter Setup Menu

ALT/D   Disconnect

ALT/A   Cancel Transfer

<INS>   Toggle Status Line Display

All other  keys are  ignored during  a YAPP  binary transfer.  It is not

possible to switch the channels during the transfer.

Before entering YAPP transfer mode, THS modifies the MAXFRAME  parameter

of the TNC to the maximum value of 7, sets RESPTIME to zero and disables

monitoring. The previous values are restored when YAPP transfer mode  is

left. This gives a maximum thoughput (and requires a high quality link).

You should realize that  if you have a  quality link on a  busy channel,

the other hams will surely not enjoy your transfers.

A  certain  amount  of  througput  control  is  done  with  the MAXPFRAM

parameter, see ALT/T.

<INS>   Toggle Status Line Display

----------------------------------

See the description of the status line above.

CTRL/ESC   Direct Command Entry

-------------------------------

When pressing CTRL/ESC,  you get a  command window and  a "CMD>" prompt.

Everything you enter is  sent to the TNC  as a command on  the on-screen

channel.  The  syntax  required  is  the  command  syntax  of the WA8DED

software.  If the  TNC returns a message  as an answer to  your command,

this message is  displayed. You terminate  the command entry  by another

ESC or by ENTER.

The direct or immediate command entry window call has been changed  from

ESC to  CTRL/ESC because  ESC is  commonly used  by THS  for escaping  a

window.

ALT/E   Temporary exit to DOS

-----------------------------

When you press ALT/E then THS suspends itself and calls a second command

processor (i.e. COMMAND.COM). THS is completely inactive until you  type

EXIT to the DOS prompt. This also means that the TNC is NOT polled,  and

all incoming frames remain buffered  in the TNC. You should  return back

to THS as soon as possible because the TNC's buffer are limited and  you

will loose information if the TNCs buffer overflow.

All  files  which  have  been  opened  by  THS  remain  open when THS is

suspended, you must NEVER change disks if your logfiles are on removable

disks !

NOTE

----

Exit to DOS requires DOS 3.20 or higher.

ALT/X   Exit THS

----------------

The ultimate key.  You will be  asked for a  confirmation. Anything else

but a 'Y'  or 'y' will  leave you in  THS. If THS  exits, it clears  the

screen and returns with your familiar DOS prompt.

However, THS  may also  report a  number of  device i/o  errors when  it

exits. A device i/o error results when one of several consistency checks

on  the  message  blocks  received  from  the  device fails. When such a

failure occurs, THS flushes the current message and continues. You shoud

normally not get any report at all. Note that errors resulting from  the

ACE chip ("RS-232 errors) are always reported (see below).

Finally, the secret ALT-key:

Whenever you give ALT/A, any incoming message to the currently  selected

window is accompanied by a deep summing tone. Another ALT/A disables the

sound. There is no visible indication of ALT/A, because obviously  there

is  an  audible  one.  I  often  use  ALT/A when my partner says "wait a

minute", because I have no klicking relais in my TRX which would attract

my attention after that minute.

4. Error Reports

----------------

Whenwever THS detects  an error, it  will open a  window and display  an

error message and  optionally parameters. Note  that it is  normally not

possible to recover from these errors. The message types are

"RS-232 Read error. Code received: 0xnnnn"  - where:

0xFFFF      timeout

0x02nn      overrun error

0x04nn      parity error

0x08nn      framing error

0x10nn      break interrupt.

These  errors  (besides   timeout)  are  signalled   by  the  ACE   chip

(8250/16450), i.e. hardware error reports or faults.

Parity and framing errors, or  break interrupt indicate a chip  problem,

this could  however be  on either  end of  the serial  line. Try  to use

another port on the PC, and/or reduce the transmission baud rate.

4.1. RS-232 Overrun Errors.

---------------------------

Whenever you  get an   overrun error  this indicates   that the   System

is not fast  enough to    process the  incoming   characters. This   can

be solved   by   either   reducing   the   frequency    in   which   the

incoming  characters  appear,   i.e.  by   reducing  the    transmission

baudrate,   or  by  using  a  more  sophisticated  serial communications

controller chip, or  by increasing the CPU speed.

Unfortunatly if the  System has not  enough time to  process an incoming

character  timely  this  may  be  caused  by  other  code which executes

concurrently and which disables the interrupt recognition for a too long

time period. Some sad examples:

Pressing the  CAPS-, NUM-  and SCROLL-lock  key of  an enhanced keyboard

cause the BIOS to toggle the  LED status indicators, which is done  with

interrupts  disabled  (but  takes  far  more  time  than other character

processing) and immediately causes an RS-232 data overrun error, even on

10Mhz AT's with 4800 Baud to the TNC.

THS disables the  functions of the  CAPS-lock, NUM-lock and  SCROLL-lock

keys when used with external TNCs (i.e. not when a PC*PA is used).

The next  problem of  interrups-off-too-long is  introduced by  national

keyboard drivers. ALL  keyboard drivers from  a DOS version  higher than

3.20 exhibit this problem. You could disable this driver by  CTRL/ALT/F1

which gives you the  US keyboard layout, or  try drivers from older  DOS

versions.  Sorry again, but DOS is truly NOT designed for concurrency.

Obviously, with the PC*PA you will not have any of these problems.

Another solution is to replace the standard serial controller chips by a

16550A chip which has a built-in  FIFO (a 16 byte hardware data  stack).

THS supports this chip,  and the use of  this chip generally solves  all

overrun problems.  If THS  detects such  a chip,  then you  will see the

string "FIFO:yes" in  the opening window,  and THS will  not lockout the

CAPS-, NUM- and SCROLL-lock keys. Otherwise, a "FIFO:no" is printed  and

the usage of these keys will be locked out.

4.2. Other ERRORS.

------------------

You  will  get  other  error  reports  following an RS-232 error report,

because THS gets  out of sync  with the TNC.   This is a  dead end,  you

should exit THS (if you can do so still) or reboot the PC.

5. Decoding of Net/Rom frames.

------------------------------

THS decodes Net/Rom routing broadcast messages and the inter-node frames

into a plain English text  format. For an interpretation of  the various

variables shown,  please refer  to "Net/Rom  Version 1.3  Documentation"

from Software 2000 Inc.

6. Final Remarks.

-----------------

I  would  be  interested  in  your  proposals  and comments. Please note

however that I will feel no  obligation of any kind. I am  a professinal

software designer, but THIS is a hobby project.

Finally, my address is:

Peter H. Heinrich

HB9CVV

Allemendstr. 25

CH-2562 Port

Switzerland.

You can contact me on CompuServe [71470,32] and on BIX (phh) also.

7. Revision list.

-----------------

(2.00)  01. Aug. 1988   Test release

* First multi connect version

* Restructuring of function tasks, using resources for

synchronisation

2.10   20. Aug. 1988   Initial release

2.20   18. Oct. 1988 released

* Heardlist (ALT/H, ALT/G) with connect to heard station

* NetRom frame decoding

2.30   24. Dec. 1988 released

* Exit to DOS (ALT/E) implemented

2.40   05. Feb. 1989 released

* MWINDOW introduced: channel 0 input to selectable window.

* Kill Yapp transfer (ALT/K) by disconnect plus reconnect

* Yapp transfers optimized, enhanced window.

2.50   16. Aug. 1989 released

* Message editor

* screen support for up to 60 lines.

* Support for 16550A serial communications controller

* FILTER variable in ALT/T

"*" denotes major addition and reason of version number change

--------------------------- end of file -------------------------------------

so far.