December 06, 1990

During the past year some great CD-ROM programs have been released for

schools.  Two of the best are World Book Information Finder(The World Book

Encyclopedia) and Compton's MultiMedia Encyclopedia.  These two programs

require 640K of RAM memory in order to operate.  In most instances, this

would mean they could not be run on a network without extra memory

upgrades and special memory management software for each workstation; the

network software just takes up too much memory.

There is one network that will allow your to network CD-ROMSs (even

more than one) and this is "Lantastic" from Artisoft.  Its network software

uses only 10K of memory for each workstation.  In our school we have a total

of nineteen computers, three in each classroom.  The server and CD-ROM are

located in the library.  We use the "Lantastic" network and "Comlink"

educational network software (Atari and Tandy use the same network software).

There are two portables on our site, but the students can still access the

encyclopedia in the library. I the case of the World Book program, it is

exactly the dame text as found in the World Book Encyclopedia on the shelf of

most libraries in schools.  In our school, students who use the CD-ROM for

research have some distinct advantages over those who still use the "book off

the shelf" method.  The students using the CD-ROM can access the encyclopedia

at the same time, no waiting for a particular volume to be returned.  A student

can search material far faster and more efficiently.  When students find the

articles they need, they can either save the selected text to disk to be

loaded into a word processor later, or print it out immediately from the

program.  Now they have an exact copy of what they wanted from the

encyclopedia.  They can them make rough notes right on the printout.  An

entire class of thirty students could perform their basic research in a couple

of days without going to the library.  This does not mean that students do not

use other sources of research;  however, it does make getting  started far

more efficient.  Students seem more motivated to do research when using the CD-

ROM.  I have seen students stay late after classes just to use the CD-ROM.

The basic things needed to network the CD-ROM are MS-DOS 3.3 (4.01 is

too large and will not work); MS-DOS Extensions 2.0 or later; CDLANFIX.sys

program from shareware (this one is free); Mkimage program that comes with

Comlink system which is used to make the boot image program that is used to

load DOS into the workstations; a text editor such a Qedit or an ordinary

wordprocessor and a shareware menu program such as Automaxx or Automenu.  The

menuing system that comes with Comlink is memory resident and takes up about

40K all the time.  However, when the workstations begin to "boot" up using the

setup listed below, they first load the non memory resident menu programs.  At

this point menu selections have been set up allowing the user to continue

loading the standard menu interface or to make an alternate CD-ROM selection.

When you are finished accessing the CD-ROM you are taken back to the original

menu and given the opportunity to load the Comlink menu.

Before you begin, only those people who understand DOS and their

network should do this.  A wrong move could shut down your entire network.

First, log on to your network as the administrator at the server where

your CD-ROM drive is connected.  Change over to drive A and  carefully follow

the installation instructions for installing the Micosoft Extensions software.

Be sure to install the proper driver for yours drive (eg. Hitachi, Nec, ect.).

When you install these extensions, they should be located in the root

directory.  Next copy into the root directory the CD Lanfix.sys program.  It

is necessary to install this in order to use the Microsoft extensions software

on more than one computer at a time.  You are  now ready to install the

applications software that came with your CD-ROM.  The example I will use here

is the one used for "World Book Information Finder."  Install the applications

software that came with the programs according to instructions.  It will make

a separate directory and install the program automatically.  Your autoexec.bat

and your  config.sys will be adjusted as well, however be sure that they are

the same as the ones listed below.  Also, remember that the MS extensions must

be loaded after the redir.exe command and before the server.exe command, as

shown in the example below.

SERVER AUTOEXEC.BAT

(partial listing)

redir server buffers = 8 size =  1024

cd\bin

mscdex.exe/d:cdrom 1 /m:10

cd\lantasti

server

SERVER CONFIG.SYS

DEVICE = C:\DEV\HITACHI.SYS/D:CDROM1/L:E/M:10/N:1/I:5/P:300

DEVICE = C:\CDLANFIX.SYS E:

FILES = 60

FCBS = 60,60

LASTDRIVE = Z

SHELL = C:\COMMAND.COM /E:1024 /P

Because the applications software writes a temporary files while you

are using the CD-ROm, you must copy the applications software into separate

directories equal to the number of workstations which will access CD-ROM

simultaneously.  Using the MD DOS command, we make directories labelled

World1, World2,ect.  The same is true with the Compton's Encycolpedia.  A

simple batch file is then written with the proper conditions defined so that

IF World1 is being used it will move ton and check for the availability of

World2 and so on.When the maximum users is reached, a message implanted in the

Batch file returns a "Sorry the maximum number of users has been reached, try

again later" message. The autoexec.bat (partial listing) file for each

workstation is as follows:

sti

redir 000000CBM_0#logins=10

net login\\server machines machines

net usen: \\server\c

net use1pt] \\server\@printer

net clock\\server

N:

cd\lanware

set comspec=command.com

mouse

net use e:\\server\e

cd\

mscdex/d:cdrom1/m:10

assign c=e or c=B

menu

It is also important that the CONFIG.SYS for each station be set as follows:

device=\system\hitachi.sys /D:CDROM1/L:F/M:10/N:1/I:5/P:300

device=\system\cdlanfix.sys C:

DRIVPARM=/D:00

files=15

buffers=2

lastdrive=z

These settings are for the "World Book Encyclopedia," In order for

this to work with Compton`s MultiMedia Encyclopedia you would have to change

the M:10 setting to M:4 in the autoexec.bat and config.sys files.

The cost of a network license for the "Wold Book"is very reasonable at

$995 per site.  The cost of networking the Compton`s Encyclopedia has not yet

been set but should be competitive with World Book. Many thanks to George Shaw

at Cobourg West Collegiate for his assistance.

Gordon Ghent teaches at Grant Sine

Public School in Cobourg, Ont.

Don Turnbull is the president

of Northumber PC Users `Group.

If you would like further assistance,

contact Mr. Ghent at:the school:

RR#5,Cobourg,Ont. K9A-1J8 or  phone(416)372-8528.

Both gentlemen may be reached via <NANET> using

Canada Remote Systems R105J:NANET

Don runs a BBS at 416-372-8668 up to 38,400

or at 416-372-2809 up to 9600 both at HST system.

Steven Cavanaugh