This README file accompanies ISADISK.OBJ for NetWare v2.15 revision c and

NetWare v2.2.  The following files are included for this revision:

ISA215   DSK      1030 12-10-91  11:45a

ISA22    DSK      1027 12-10-91  11:47a

ISAREM   DOC      (This file)

ISADISK  OBJ      6369 12-09-91  11:40a

Included with this driver are two .DSK files which are included for

NetWare v2.15c and v2.2 and are named accordingly.  Instructions for

their use is included below.

Purpose/Problems Addressed

1. On many computers, if the user selects a user-definable drive

type during the initial setup of the machine, the machine BIOS

would place the ROM PARAMETER TABLE below address segment C800.

Previous revisions of the ISADISK driver would not recognize the

table at this location and ABEND with the message: IMPROPER ROM

PARAMETER TABLE FOR DISK CONTROLLER.  This revision no longer

checks to see if the table is below address C800.

2. This version of ISADISK also addresses the problem with DOS drive types

greater than 127.  Previously, this would set the removable media flag

in the NetWare v2.2 installation. Once the flag is set, customers are

not able to modify volume size or directory entry settings in the File

Server Definition Window.  This driver will now accept drive types

greater than 127 without assuming that they are a removable drive.

3. This driver also includes an earlier fix that corrects a significant

performance drop of the first drive when a second drive is added to a

WD1003 or WD1006 controller (in the drive D position).  This has been

known to occur with the ISADISK.OBJ driver v2.10 and below.

4. This patch corrects problems where excessive noise is emitted from

drive 00.  This noise may be a grinding noise or sound as though the drive

is continually seeking.

To install this version of ISAREM.ZIP:

For 286 v2.15c, Floppy Installation:

Rename, or archive and delete, the existing ISADISK.OBJ on the working copy

of your DSK_DRV_001 diskette.  Copy the new ISADISK.OBJ onto the working copy

of your DSK_DRV_001 diskette.  Rename, or archive and delete, the existing

ISADISK.DSK on the working copy of your AUXGEN diskette.  Copy the new

ISA215.DSK to the working copy of your AUXGEN diskette as ISADISK.DSK.

For 286 v2.2, Floppy Installation:

Rename, or archive and delete, the existing ISADISK.OBJ on the working copy

of your SYSTEM-2 diskette.  Copy the new ISADISK.OBJ onto the working copy of

your SYSTEM-2 diskette.  Rename, or archive and delete, the existing

ISADISK.DSK on the working copy of your SYSTEM-1 diskette.  Copy the new

ISA22.DSK to the working copy of your SYSTEM-1 diskette as ISADISK.DSK.

To implement this driver:

1)  For NetWare 2.15c use Netgen -c.  Relink and configure the NetWare

Operating System and File Server Utilities.  Use the Custom installation path

and choose Miscellaneous Maintenance.  Under Misc. Maint. flag Load Operating

System and continue installation.  Reboot the server.

For NetWare v2.2 execute Install.exe and answer "NO" to "Will this machine be

the server?" in the OS Generation window.  Use Install -f.  Press [F10] at

the File Server Definition window and remove all tasks except Load Operating

System in the Actions to be Performed window.  Reboot the server.

*****************************************************************

DISCLAIMER

Novell, Inc. makes no representations or warranties with respect to

any NetWare software, and specifically disclaims any express or

implied warranties of merchantability, title, or fitness for a

particular purpose.

Distribution of any NetWare software is forbidden without the

express written consent of Novell, Inc.  Further, Novell reserves

the right to discontinue distribution of any NetWare software.

Novell is not responsible for lost profits or revenue, loss of use

of the software, loss of data, costs of re-creating lost data, the

cost of any substitute equipment or program, or claims by any party

other than you.  Novell strongly recommends a backup be made before

any software is installed.   Technical support for this software

may be provided at the discretion of Novell.

*****************************************************************

*****************************************************************************

CAUTION

ALWAYS backup your system before implementing any program/utility revision

involving the low-level functions of NetWare including re-linking of

operating system .OBJ files, Bindery utilities, drive and volume operation,

etc.

*****************************************************************************