This application note describes the setup procedure for

running the Novell SFT Netware 286 ver 2.15 rev C disk duplexing

feature using two ULTRASTOR controllers (ULTRA 12F ESDI) with the

firmware rev 28001-007 and BIOS rev 38001-007 or higher.

Controller Jumper Settings

on the primary controller card, the following jumpers should

be set as follows :

JP10-               1-2  OUT        Bios add at C800:0

3-4  OUT

5-6  IN

JP12-                    OUT Hardrive Primary I/O

port add (1F0-1F7,3F6-3F7)

JP17-                    IN  Enables Floppy controller

OUT IF MotherBoard has Floppy

ctlr/another one present

JP20-               1-2  OUT

2-3  IN           IRQ14 selected

on the secondary controller card, the following jumpers should

be set as followed :

JP10-               1-2  OUT         Bios add at CC00:0

3-4   IN

5-6   OUT

JP12-                    IN            Hard disk secondary I/O

port add (170-177,37)

JP17-                    OUT          Disable The Floppy ctlr

JP20-               1-2  IN           IRQ 15 Selected

2-3  OUT

Note : The primary card bios address can be at D000:0 or D800:0.

The secondary address can be at D400:0 or DC00:0. Make sure that

the primary card BIOS address is lower than the secondary card BIOS

address.

Hard Disk Setup :

Only two Ultrastor controller can co-reside in the system to

support Netware 286 disk duplexing function. Each controller can

support up to two hard disk drives. Two controller will support up

to 4 drives. Select the drive with similar sizes to be used as a

duplexing pair.

Steps :

1. Set jumpers in the desired configurtion for all the ESDI

drives. The drive selection shouldbe set to drive select 2

for all the drives (for twisting cable).

2. Connect the drive power to the system power supply, make

sure that the sytem power supply can support the attached

ESDI drives. Otherwise, a seperate power supply is needed

3. Insert the primary controller card into a 16 bit AT slots.

4. Connect the 34 pin daisy chain flat cable to J5 connector

on the controller. The edge connector near the twisted end

of the cable is connected to the first drive. The other

edge connector should be connected to the second drive if

second drive is to be attached.

5. Connect the 20 pin flat ribbon cable from the first drive

to J4connector on the controller. Connect the other 20 pin

flat cable from the second drive to J3 connector if second

drive is to be attached.

6. If the controller is the primary controller and system

motherboard does not support floppy, connect the 34 pin

daisy chain floppy cable to J1 connector on the controller

7. Insert in the secondary controller card into a 16 bit

AT slots. repeat step 4-6 for the secondary controller.

Hard Disk Drive Media Preparation :

Boot up the system with a DOS diskette in drive A:, and then

type DEBUG, then at the "-" prompt type g=c800:5 (or g=d000:5, or

G=d800:5 according to the BIOS addres setting in the primary

controller) and press enter to go into the Ultrastor Utility Main

menu. Select the " Drive Parameter Setup " option . Then Select the

following mapping mode ("native Mode ") for all the drives

connected to the controller :

One Spare Sector/Track   NO

Sector Mapping                  NO

Track Mapping                     NO

* Select 63 sector mapping mode if the drive sector per track

number is more than 63, as with most 20MHZ ESDI drive does.

Press F10 to save and F5 toconfirm execution. The utility will

update the system with the correct hard disk drive's parameter and

reboot from DOS automatically. After reboots, reload DEBUG, same

step as before, but this time select " Drive Preparation " Menu. Go

to AUTOFORMAT filed, select BRIEF-N or EXTENDED-N and press "ENTER"

and F5 to confirm it. and Begin the formatting process of the

drive. The Options with " -N" will perform an extra scanning pass

on the disk; this builds the NOVELL compatible defect list. If user

intends to use NOVELL COMPSURF for comprehensive drive scanning,

the BRIEF-N option is recommended; Otherwise the EXTENDED-N option

is suggested.

BRIEF-N :

1. Retrieve the manufacturer's defect list from the drive.

2. Low Level Format the drive

3. Mark Defect sectors as "BAD SECTORS"

4. Verify Drive

5. Mark Additional defects as "BAD SECTORS"

6. Re-scan the whole disk, converting and storing defect

addresses into a fromat recognizable By NOVELL COMPSURF

EXTENDED-N :

Almost the same as BRIEF-N but, you selected from AUTOFORMAT

and Select The EXTENDED-N Menu.

If second drive exists, select drive 1 and repeat the steps.

Otherwise exit to DOS by Hiiting ESC key twice. Also after you do

low level formatting fot the first/primary drive,you should also do

the sam steps to the secondary drives or all the next drives that

attached to the controller. Now the system is ready for installing

SFT Netware 286 ver 2.15 rev C.

Novell SFT Netware 286 v2.15 rev C Installation :

1. Replace DOS Boot diskette with the NOVELL NETGEN

2. Run Novell Netgen Utility by typing at A> Netgen

3. Select Custom Configuration Under System COnfiguration Level

4. Execute the "Network Configuration" By

4.0 Select Network Configuration

Select the appropriate options for Operating Systems

Select Resource Set

Select LAN Drivers

4.1 While selecting disk drivers, select the ISA Disk ctlr

for both Channel 0 and channel 1.

Select load items :

Channel 0 Industry Standard (ISA or AT Comp) Disk Ctlr

Channel 1 Industry Standard (ISA or AT Comp) Disk Ctlr

4.2 In configuring Drivers?Resources for Disks, Select :

ISADISK (IRQ14) Primary Verify on(or OFF) I/O=1F0H for

disk channel 0

ISADISK (IRQ15) secondary Verify=On(or OFF) I/O=170H for

disk channel 1

4.3 Save Selections and continue.

4.4 Begin Netware configuration by

Link/Configure Netware Operating Systems

Configure File Server Utilities.

5. Before installing the network, a short COMPSURF is required for

every drive to put the appropriate signatures on track 0

Execute "Analyze Disk Surface"

Select the drive to be tested : Either

Chan 0, Ctrl 0 drive 0 IBM AT Hardrive Type 001

or

Chan 0, Ctlr 0 drive 1 IBM ATHardrive Type 002

or

Chan 1, Ctrl 0 drive 0 IBM AT Hardrive Type 003

or

Chan 1 Ctlr 1 Drive 0 IBM AT Hardrive Type 004

Reply all those with the following answer for the

prompt from Compsurf.

Format the Disk :   NO

Maintain the current Media defect List :     YES

Number of Sequential Passes :      1

Are the Parameters Correct :  YES

After Track 0 is initialized the user may press ALT F10 to

disk surface analyze at any time, then repeat the same

proceddure for the rest of the drives and the controllers.

6. Continue Netware installation by selecting Netware Instalation

The Following table should appear :

IBM ISA hard disk Chan 0 ctlr 0 drive 0 status

IBM ISA hard disk Chan 0 ctlr 0 drive 1 status

IBM ISA hard disk Chan 1 ctlr 0 drive 0 status

IBM ISA hard disk Chan 1 ctlr 0 drive 1 status

Answer the question : Is the Drive List Correct ?  YES

7. Now you may proceed the rest like usual instalation. Please

Note however, Mirroring can not be accopolished if drives sizes

are equal.

Well, I guess i should wish you lots of luck, but remeber it's

not thatbad, infact it's fun!!. Happy Networking.......

Tris Kosasih

Multi Micro Inc

582 Folsom St

S.F - CA 94105