WESTERN DIGITAL

LOCAL AREA NETWORK ADAPTER SETUP PROGRAM

RELEASE INFORMATION

Version 1.04                            January 8, 1990

This directory contains version 1.04 of SETUP, a configuration

setup program for use with the new generation of Western Digital

LAN Adapters that are software configurable. In this release, the

SETUP program operates with:

- WD8003EB EtherCard PLUS adapter

- WD8003W  EtherCard PLUS10T adapter

MODIFICATION HISTORY

-- Version 1.04 (Current Version, January 8, 1990) --

This version of Setup program has the following changes.

1. Supports EtherCard PLUS10T (WD8003W) adapter.

2. Fixed problem in identifying adapter as intelligent

Ethernet adapter.

-- Version 1.03 (September 22, 1989) --

This version of Setup program has the following modifications.

1. Modified to support new boards that use interrupt IRQ5 as

well as earlier boards that use IRQ4.

2. Fixed these bugs found in the previous version:

- when checking for interrupt conflict.

- unable to change RAM size to 32K on the board with 32K RAM.

-- Version 1.02 (Initial Release, June 1989) --

Through normal use the following error messages may occur.

1. "SYS$ERR.DAT could not be found" or

"SYS$HLP.DAT could not be found"

If either of these error messages are displayed, the value of the

FILES parameter in the CONFIG.SYS file may be too small.  Try

enlarging this value, re-booting, and re-running.  Make sure you

do not have any network software drivers, such as those for

ViaNet or NetWare, loaded or active when you are using SETUP.

2. "Cannot find IBM$RUN.OVL"

If you try to run SETUP on a NetWare workstation, you will see

this error message.  You should not have the NetWare driver

loaded when running the SETUP program.

3.  You may encounter problems when chosing the Automatic

Configuration if you have installed any network software prior to

running SETUP.  To avoid problems of this nature, remove all

network drivers and other network software from the system, then

re-boot and run SETUP.

Refer to the User Installation Guide for additional information.

EtherCard PLUS is a registered trademark of Western Digital

Corporation.