A/V TechTalk

of

TEN FORWARD

Electronic Bulletin Board Systems

Running PCBoard v14.5

Pomona, New York

Data (914) 354-5463   12/2400 baud  24 hrs

presents

C O M P A C T   D I S C   A U T O S O U N D

Adding Compact Disc to Your Car

Including a special discussion concerning compact disc SKIPPING

Release 1.1  90May15

Written and compiled

by

Michael I. Saunders

Conference System Operator

Edited

by

Glenn Fischer

Executive System Operator

Look for other text information files in a variety

of formats from A/V TechTalk

ABOUT A/V TECHTALK

A/V TechTalk was opened as a dedicated conference on TEN FORWARD on May 14,

1990.  The conference is supervised by senior system operator Glenn Fischer

and conference system operator Michael Saunders.  General and specific

questions are welcomed concerning the topics of home and automobile audio and

video systems, more commonly referred to "home theater" and "autosound."

A/V TechTalk has access to the facilities of a small county-based consulting

firm, including a feature/specification database.  We do not solicit nor give

false information; at the same time, however, we cannot assume responsibility

for typographical errors and unintended mistakes in providing specs or

features.  Come join our conference; I'm sure you'll learn a lot about the

world of home and auto A/V, and you'll find a warm and friendly atmosphere.

And be sure to check us out before you go out and confront those dreaded

salesmen who won't always tell you the facts!

Also, be sure to check out TEN FORWARD in general.  With conferences from

Astronomy to Star Trek and over 30 ANSI-compatible doors, you're in for a lot

of fun!

DONATIONS

All consulting services provided by the SysOps are of course free, but any

donations would be greatly appreciated.  We work hard, especially when

compiling these information texts, and maybe you could reward the conference

with a little dough here and there!  Any donations should presented in check

form, and please make the check payable to "Greg I. Abel."  He is the

treasurer of my consulting firm and handles payments to the board's

conference.  Please send the check to:

Mike Saunders and Greg Abel c/o Ten Forward

Suite 331

49-3 Spring Valley Marketplace

Spring Valley, NY  10977

Also, if you have any additional info or questions to send us about your car

stereo, or you find any errors in this text, please send them to the address

listed above or tell us on Ten Forward...

C O M P A C T   D I S C   A U T O S O U N D

Adding Compact Disc to Your Car

JUMP ON THE BANDWAGON

The compact disc craze has hit your automobile and now is the time to get

with it.  Stop pressing rewind and fast forward and let's skip a few tracks

at a time!  Compact disc offers incredible sound, more choice of music at the

touch of a button, and faster access to your favorite music for far less

moola then you might think.  And you may have more ways to add compact disc

to your system than you may think....

THE CD PLAYER ISN'T THE WHOLE THING

Let's not get all excited, here.  Just because you add compact disc to your

system, it doesn't mean you will hear sonic nirvana.  You need a good main

head unit ("car stereo", "AM/FM stereo cassette," etc.) and decent speakers

as well.  The excellent frequency bandwidth a compact disc provides may

exceed your head unit and speaker's performance, in which case you aren't

getting ALL of the good sound compact disc has to offer; just part of it.

However, you are guaranteed to hear the difference between the sound of

compact disc and your tape or radio with the system you have now.  A compact

disc player is a worthwhile investment.

CD CHANGERS

Adding a CD changer to your car is like living in Arizona and getting air

conditioning in your apartment for the FIRST TIME.  You can't let it go.

Imagine access to 6 to 12 of your favorite discs at the touch of the remote

control.  Where do you put all of these discs?  Usually the unit that holds

and plays the discs is mounted in the trunk.  "Mounted" is the key word,

here, because the unit is semi-permanently BOLTED to the trunk floor.  A

remote control (USUALLY wired) lets you control the changer.  Sometimes the

manufacturer of the changer makes a head unit that is compatible with the

changer, so you can control the changer using the controls on the head unit.

Some changers let you program 50-plus tracks on any of the discs.  Some

changers (especially 6 and 10-disc) use identical "magazines" as many home

CD changers; you can take the magazine from your house and bring it to your

car, and vice versa!  Loading a magazine into the changer is similar to

loading a tape into a VCR; just open the trunk, insert the magazine and the

changer handles the rest.  Random play, repeat and digital oversampling round

out the features.  Changers range from $399 [Low] for a 6-disc changer to

$799 [Low] for a 12-disc.  Changers are usually best installed by

professionals, since there is a lot of wiring to mess around with, and lots

of room for error.

CD HEAD UNITS

If you had planned on upgrading your head unit, say for more power and better

tape performance, maybe a high-powered CD/Tuner head unit is a better choice.

They offer the same performance as changers, but only accept one disc at a

time.

There are two types of loading systems on CD head units.  The first is the

"standard" type, where you just take a disc out of its case and insert it

into a slit in the head unit.  The second is the "CD Cartridge" type, where

you must first insert the disc into a thin plastic cartridge and then insert

it into the head unit.  The advantage of the standard type is that you don't

have to keep taking a disc out of the cartridge, putting another disc into

the cartridge, and then inserting it into the head unit; you just pop the

disk in.  However, the cartridge type provides protection for the disc

outside as well as inside the head unit, where it slightly reduces the

occurrence of skipping.  Also, you can buy extra cartridges for around $5 a

piece and keep each disc in its own cartridge without having to put it back

in its case each time.

Compact disc head units usually provide tuners with identical performance of

standard AM/FM stereo cassette head units, some are removable, and some allow

you to control a compact disc changer as well as having its own disc.  Be

careful of "pre-amp" units: these do not have an amplifier inside the head

unit; you cannot use them without purchasing a separate amplifier which can

run you another $200.  Prices range from $349 to $599 [Low].

PORTABLE COMPACT DISC PLAYERS

The quickest and most affordable way to add compact disc to your existing

system (i.e., not having to get rid of your current head unit) is to go with

a portable CD player.  The typical player will include an LCD information

display (displaying track number, time, etc.), repeat play, and music search,

but sometimes lacks track programmability, which often is not desired in a

car, anyway.

You will probably need a special mounting kit to install a regular player

(meaning a player not specifically designed for auto use), so the player

doesn't skip at every bump (please see SKIPPING: MISCONCEPTIONS for more

info).  This kit consists of a "shock absorber" to cushion the player during

bumpy ride.  In addition to the kit, you need a "12 volt DC" power cord to

provide power to the player; you don't want to run the player off normal

alkaline battery power since it is a waste of money and convenience.

You also need a way to carry audio output from the player to the head unit.

There are three ways to do this.  The first is the simplest.  If you have a

head unit with a CD input jack on the FRONT of the unit, you can connect this

jack with the "line out" or "phones" jack on the player with a thin cable.

Sometimes it may be necessary to select the CD input on the head unit by

pressing a "CD" key.  The second is to "play it" through the cassette tape,

if your head unit has a tape player.  A "fake" cassette tape with a thin wire

attached to it is inserted into the tape deck.  The other end of the wire is

then connected to the "line out" jack on the player as mentioned above.  When

the "tape is playing," and the player is as well, the sound you hear from the

tape deck is actually coming from the player.  When you want to use your tape

again, simply eject the "CD tape," and insert your own tape.  The final

method is only necessary when you have but an AM/FM radio.  A special adapter

from Sony allows you to send audio from the player to the radio antenna.  By

tuning in to a specific frequency on your radio (usually 88.5 FM), you can

"pick up" sound from the CD.  This method is somewhat expensive; the adapter

sometimes sells for over $50.  You may be better off buying a better head

unit with a cassette player, and even consider getting a CD/Tuner head unit

if you want the sound of compact disc.

Portables aren't the most convenient option available.  Forget about changing

a disc while you are driving.  Changing a disc is hard enough with two hands;

you must open the player (the top swings up like a jewelry box), hold the

player with one hand (if it isn't mounted on top of your dash or armrest,

etc.) and take the disc out, change the disc, and close the player with the

other.  Some players are too large to be mounted elegantly (or you might not

have space to mount), so you may need to find a pocket somewhere in your

dash, maybe by removing the ashtray, or using a storage pocket under the head

unit.  If you do this, keep in mind that if you want to change a disc, you

must be able to slide the player in and out freely.

Your best bet when it comes to buying a portable CD player is the Sony model

D180K, $299 [List] $189 [Low].  It comes with a power cord, CD-to-cassette

adapter, and doesn't require a shock absorbing mount; it has one inside the

unit.  Just find a pocket in your dash, slide it in (and make sure you have

a somewhat tight fit, enough to keep the player from sliding around while

driving), and make the simple connections.  One nice feature of the D180K is

the amber illuminated buttons and LCD display, which are easily readable at

night.  As far as we know, this is the only player in the market with this

feature.

Other players require a mount kit.  Sony sells a kit for use with some of

their players that includes a shock absorbing mount, a CD-to-cassette

adapter, and a power cord for around $30.  Some other non-Sony players may

be compatible with the kit (as far as the fitting of the mount), so try

before you buy.

SKIPPING: MISCONCEPTIONS

A popular misconception concerning "skipping" or "mistracking" during the

playing of a compact disc is that the reason why it skips is because the

player has been jolted or shaken up and down.  Although this is true, a more

common condition exists.  More than three out of four times (75%), a player

skips because the disc being played has been "rotated" on its own, coming

loose from the stepping motor inside the player that rotates the disc.  This

phenomena occurs mostly in portables, and in home components, changers and

CD head units rarely.

A quick experiment illustrates this problem.  If you have a portable CD

player (it is not recommended to perform this with a home component), hold

the player in one hand (and make sure you have your headphones on!) and shake

it vertically.  It skips a bit if you shake it violently, but doesn't usually

skip if you move it up and down smoothly.  Now shake the player from side to

side, NOT rotating it, but just moving it parallel to the floor.  A little

skip here and there if you shake it enough.  Now ROTATE it, as if you were

screwing and unscrewing a light bulb, alternating left and right.  Wow, this

thing skips like the little girl down the block!

In the car, this "rotational" skipping problem usually only happens to

portables that aren't mounted with a kit, but are placed in a pocket.

To avoid this horrendous skipping problem, make sure there is little room on

both sides of the player and it may be wise to find a way to require you to

"stuff" the player into the pocket, forcing the pocket to be slightly "bent

out of shape."  This will prevent the player from sliding around, but will

still allow you to pull it out with minimal effort.

ASK A/V TECHTALK

A/V TechTalk at TEN FORWARD will be glad to answer any questions you have

about adding compact disc to your car stereo system, as well as questions

about other audio/video topics or equipment.  Call TEN FORWARD today, at

(914) 354-5463, 12/2400 baud.