+---------------------------------+

|                                 |

|   Loudspeaker Modeling Program  |

|                                 |

|             (LMP)               |

|                                 |

+---------------------------------+

A Speaker System Modeling Program

&

Passive Network Design Utility

For IBM and compatible computers

DOS v3.xx suggested

LMP was co-authored by Ralph Gonzalez and Bill Fitzpatrick.

Based on an original work by Ralph Gonzalez.

----------------------------------------------------------------------------

(C)1990 Sitting Duck Software   POBox 130   Veneta, OR 97487   (503)935-3982

All rights reserved

THIS MANUAL

This manual covers the use of LMP & P-FILTER, provides several example

designs and explains each input and menu choice in detail.  The complete

theory of loudspeakers, crossover networks, enclosures, "time alignment",

and the 1001 other details relevant to the construction of a quality loud-

speaker system from an infinite assortment of parts are far beyond the scope

of this manual.  The hobbyiest who wishes to explore the finer points of

system design is referred to the following:

-     Speaker Builder;  issues 1, 2 and 3          1987

-     Speaker Builder;  issues 1, 2, 3, 4 and 5    1988

-     Speaker Builder;  issue 2, 4 and 6           1989

-     Loudspeaker Design Cookbook by Vance Dickason

-     High Performance Loudspeaker Enclosures

-     Bullock On Boxes

-     Articles from the Journal of the Audio Engineering Society

All but the last item are available from Old Colony Sound Labs,

PO Box 243, Peterborough, NH 03458, (603) 924-6371

----- GETTING STARTED WITH LMP --------------------------------------------

Make a working copy of your distribution diskette and put the original in a

safe place.

+++++ THE FILES ON YOUR DISKETTE relating to LMP and P-FILTER

SD-LMP    .EXE        The Loudspeaker Modeling Progam run file.

SD-LMP    .DOC        This documentation.

XOVER <DIR>        The sub-directory containing the models.

X-OVER    .BAT        A batch file to run a filter program.

P-FILTER  .EXE        The Passive Filter design program run file.

FSGP      .DTA        A data file for P-Filter.

TSGPP     .DTA        A data file for P-Filter.

TSGPN     .DTA        A data file for P-Filter.

+++++ MONITOR SUPPORT

---------------------------------------------------------------------------

LMP supports Hercules, CGA, EGA color and VGA color monitors.  If you have

a Hercules monitor you must run your graphics support program before

running LMP.

+++++ DESCRIPTION OF PROGRAM

---------------------------------------------------------------------------

LMP is a computer program designed to model 2-way loudspeaker systems.

The resulting frequency and phase response curves predict the on-axis sound

pressure level produced by the interaction of your choice of crossover,

drivers and enclosure design.

LMP assumes that the impedance curve of the drivers in the modeled system

have been made reasonably flat by way of Zobel and/or impedance correcting

networks.  LMP also assumes that the drivers modeled have a flat frequency

response from the low frequency roll-off point to the high frequency roll-

off point.  Quality drivers on the market today meet this criteria.

LMP assumes that the user has had some experience with loudspeaker systems

and crossover networks and is familiar with the terminology commonly used

to describe the various parameters involved.

----- AN LMP WALK-THROUGH -------------------------------------------------

We are going to design a 2 way loudspeaker system with an 8" woofer and a

1" dome tweeter crossing over at 1500Hz.  Drivers that are available

commercially have specifications similar to the inputs we will provide to

LMP.  The crossover design for this example will be done with the P-Filter

program supplied.

Type  SD-LMP  to start the program.

+++++ THE MAIN MENU

Starting LMP will present you with a menu screen hereafter described as

the "main menu".

Select the menu choice "P" to make a file which describes your printer.

We suggest 150dpi, portrait mode for Laser Jet users.

Select the menu choice "C" to run the batch file which runs the P-Filter

program.  Exit and run P-FILTER separately if you don't have DOS 3.xx.

From the filter menu select choice "C" to create a new crossover

model.

Enter  2  for the number of drivers.

Type  "8 inch woofer"  for the 1st driver description.

Type  N  for NO polarity reversal.

Type  "1 inch tweeter" for the 2nd driver description.

Type  N  for NO polarity reversal.

Type  3  for crossover order.

Type  1500  for crossover frequency.

Type  8  for each of the impedance values.

YOU ARE NOW BACK AT THE MAIN MENU OF P-FILTER.

Select menu choice "S" to save this crossover design.  The design is

saved in the directory XFILES under the name PASSIVE.TMP

Select menu choice "V" to see the hook-up diagram and parts selection

for the network you have designed.  Print this diagaram if you wish

and return to the menu.

Select menu choice "R" to view the curve.  Press <ESC>.

Select menu choice "E" to exit the crossover design progam.

YOU ARE NOW BACK AT THE MAIN MENU OF LMP.

Select menu choice "L" to load the network you have designed.  From

the provided menu, highlight PASSIVE.TMP and press enter.

You are again at the main menu and your crossover design has been loaded.

+-------------------------------------------------------------------------+

| Although your "model" now contains information about the crossover net- |

| work, it does not yet have any information about the drivers you are    |

| going to be using.  You will provide that information now.              |

+-------------------------------------------------------------------------+

Select menu choice "M" to modify the model.

Note that information from the crossover design is already shown on this

screen.

Press <ENTER> to accept the 2 driver configuration.

YOU WILL NOW ENTER INFORMATION FOR THE WOOFER WHICH IS DRIVER #1.

Assume that the woofer is in a sealed box, the resulting Low

frequency corner is 40Hz and the Low frequency damping ratio is .7.

Input that information.

Assume that the woofer high frequency roll-off causes it to be

3db down at 3000Hz, the damping ratio is .7 and the roll-off is

order 4 (24db/o). Input that information.

Press <ENTER> to accept the Polarity Inversion default of "N".

According to the manufacturers data sheet, the sensitivity of the

woofer is 90db and it was measured on a EIC standard baffle (53" x

65") at 1 meter with 2.82 volts input.

Our enclosure is going to have an average baffle size (Height x

Width/2) of 16".  The on axis output will begin to DECREASE as the

wavelength of the sound gets longer and begins to equal the average

baffle size.  This frequency is provided by the formula:

F = 13500/BaffleSizeInInches.

and is called the Step Frequency.

For our example enclosure, the Step Frequency will be 843Hz.  Since

our small cabinet will be mounted on stands and placed well into the

room, the room boundaries provide no extension to the baffle.  In this

case the step will cause a loss of 6db.  This loss will just begin to

show up a 843Hz.  The shape of the curve which describes this

"diffraction loss", shown in Figure 1, is for 843Hz with a 6db step.

sorry, curve not available in text file

We need to allow for this loss within the program and will do so

by REDUCING the inputted value for Sensitivity by 6db.  So, rather

than accepting the 90db default sensitivity value provided by the

network design procedure, reduce the value to 84db.  Accept the 0

default for Depth Displacement, input 843 for the Step Frequency,

6 for the Step Height and accept the crossover default choice of 8.

+-------------------------------------------------------------------------+

|  The screen you now see is for entering the crossover component values. |

+-------------------------------------------------------------------------+

Press <ENTER> four times to accept the default values of your

crossover design.

YOU WILL NOW ENTER INFORMATION FOR THE TWEETER WHICH IS DRIVER #2.

Assume that the tweeter has a Low frequency corner of 600Hz, a Low

frequency damping ratio of .7, a High frequency corner of 20000Hz,

a High frequency damping ratio of .7, and a High frequency roll-off

order of 4 and no polarity inversion.

Because the tweeter is mounted on the same baffle as the woofer,

we need to duplicate the Step information and adjust the sensitivity

value as we did with the woofer.  Actually, we don't need to do this

with a 3rd order network since 843Hz is far enough from 3000Hz that

the step will make little difference.  But, it won't hurt.

Accept the displacement of 0, input 84 for the sensitivity, 843 for

the step frequency, 6 for the step height and accept the default

crossover choice.

Accept the defaults for the crossover component values.

YOU ARE NOW BACK AT THE MAIN MENU.

Select "R" to view the response and phase curves.

Make a print-out of this curve if desired.

Figure 2

sorry, curve not available in text file        Response of model

before modification.

We are assuming that Zobels or impedance correcting circuits are going

to be used where needed and that the tweeter will be set back on a

stepped baffle board so there is no offset.  This, then, is a fairly

accurate model.

The resulting design is not particularly good for 1990 standards though

it is similar to some designs of the 70's which were known as "West Coast"

speakers and had a forward quality.

Let's see if we can apply a couple of quick fixes.  Since reversing the

phase of one of the drivers does not help this example (try it and return

the phase to normal) we'll have to try something else.  One possible

approach is to reduce the amount of mid-range output and forget about the

90db effeciency that we were expecting.

Let's try increasing the first inductor (k3) in the Low Pass network to

2.2mh to restrict the amount of mid-band energy getting to the woofer.

Let's also reduce the first capacitor (k3) in the High Pass network to

7.0mfd to restrict the amount of mid-band energy getting to the tweeter.

Now, run the curve again.

Figure 3

sorry, curve not available in text file        Response of model

after modification.

Not too bad.  The room boundary effects are going to help lift the low

end so this might be a listenable speaker after all.

Note that the fix applied to our example does not represent the only fix

possible nor does it necessarily represent the best fix.

RETURN TO THE MAIN MENU.

+++++ SAVING YOUR DESIGN

Select "S" from the menu.

Assign a DOS legal name and short description of the model.  This

model is now saved and can be loaded in again during a subsequent

session with LMP.

YOU ARE NOW BACK AT THE MAIN MENU AND HAVE COMPLETED THE WALK THROUGH.

----- MAIN MENU CHOICES ---------------------------------------------------

The following are brief descriptions of the various choices available from

the main menu.

<L>oad model         Loads a model file from the sub-directory XFILES.

Loading a model overwrites the model currently

in memory.  You are presented with a loading

menu containing a list of models available.

<S>ave model         Saves the model currently in memory.  If it is

a new model you are given the opportunity to

assign a file name and brief description.  The

file name must be DOS legal; 8 legal characters,

a period and a 3 character extension.  The model

is saved in the subdirectory XFILES.

<M>odify model       If you have no model loaded, pressing "M" will

begin a new model.  If a model is currently in

memory, pressing "M" allows you to modify it.

<Q>uit               Exits to DOS, giving you the opportunity to save

the model currently in memory if it has not been

previously saved.

<P>rinter setup      Lets the program know about your printer.  This

information is written to a file and loaded the

next time you run the progam.

<R>esponse           Displays the frequency and phase response curves

of the model currently in memory.

<D>elete file        Deletes the disc file corresponding to the model

currently in memory and then removes the model

from memory.

<C>rossover          Temporarily suspends execution of LMP, exits to

DOS and runs the batch file named XOVER.BAT

which contains instructions to run the crossover

program and then return to LMP.

<B>ox                Temporarily suspend execution of LMP, exits to

DOS and runs the batch file named BOX.BAT

which contains instructions to run the box

program, QBOX, and then return to LMP.

<F>ree               Temporarily suspend execution of LMP, exits to

DOS and runs the batch file named FREE.BAT.

FREE.BAT may contain the name of any program you

might wish to run from within LMP.

+---------------------------------------------------------------------+

| In order to exit LMP and run the above batch files, DOS must be     |

| available.  Because leaving LMP loads an extra copy of COMMAND.COM  |

| and retains LMP in memory, the ability to run other programs from   |

| LMP will be dependent upon the available memory.  You should have   |

| no trouble running most of the box and crossover programs which are |

| available.  Note that the procedure for leaving and returning to a  |

| program with the original program data intact is called "SHELLing". |

| Shelling requires DOS 3.xx and if you have an earlier earlier       |

| version of DOS DO NOT use the Crossover, Box and Free menu choices. |       |

+---------------------------------------------------------------------+

<O>ptions          <T>  Allows you to toggle between Q and damping

ratio.  Use the form you are comfortable

with.

<R>  Sets the resolution for the response display.

<F>  Sets the frequency range the curves will

cover.

<!> Clear            Clears the model currently in memory so you can

begin anew with a clean slate.

----- ENTERING INFORMATION FOR THE MODEL ----------------------------------

+++++ THE DRIVER DATA FIELD

The following provides details for each item that is entered in the driver

model field.

+++++ LOW FREQUENCY CORNER

The resonant frequency of the driver.  Use an approximate value for

a woofer if the value is unknown or the system is other than an

acoustic suspension.  Mid-range drivers are almost always sealed

systems and tweeter are always sealed so use the manufacturers

resonant frequency specification.

+++++ LOW FREQUENCY DR/Q

Some prefer to work with Damping Ratio, others prefer to work with

Q.  The Options selection from the main menu allows you to choose.

Speaker manufacturers always specify Q so it may be more convenient

to follow suit.  You can convert with the formula:

DampingRatio = 1/2Q.

If the Q specification is not given but a response curve is provided,

compare the response curve to the figure below and pick the shape

with the closest match.  Some conversions are provided for your

convenience.

sorry, curve not available in text file

Figure 4         Low Frequency Damping, Second Order, 1KHz

+++++ HIGH FREQUENCY CORNER, DAMPING RATIO and ROLL-OFF ORDER

Refer to FIGURES 5 and 6.  The two sets of curves represent various

Damping Ratios for a 1KHz High frequency corner.  Compare the curves

to the high frequency roll-off of your driver as shown on the manu-

facturers data sheet.  Select a curve that most closely approximates

the high frequency roll-off of your driver.  You can estimate for

curves that fall in between those shown.

Example:  A manufacturers curve most closely approximates the curve

corresponding to DR=1.0/Q=.5 on the Second Order graph.

The curve is 6db down at 1KHz.  On the manufacturers curve,

the 6db down frequency is 2300Hz.

Then:  High Frequency corner = 2300Hz

sorry, curve not available in text file

Figure 5         High Frequency Damping, Second Order, 1KHz

sorry, curve not available in text file

Figure 6         High Frequency Damping, Fourth Order, 1KHz

+++++ SENSITIVITY

The sensitivity of the driver.  For a system, the sensitivity of each

driver should be determined by the same method.  The standard is 2.82

volts input and measured at 1 meter; this is equivalent to 2 volts in

and measured at .5 meter.

+++++ POLARITY

Input "N" if the - lead from the amplifier is connected to the - lead

of the driver.  Input "Y" if the - lead from the amplifier is

connected to the + lead of the driver.  This is based on the

assumption that a positive signal on the + lead causes the cone to

move foreward, which is almost always the case.

+++++ DEPTH DISPLACEMENT

Each driver will have a different "distance to the listener" when

mounted on a flat baffle.  The output of drivers further from the

listener will be delayed in time and thereby phase shifted.  The

acoustical center of each of the drivers in a system would be the

ideal value to input here.  This information will not generally

be available so we must do with an approximate physical measure-

ment.

Measure the height of the cone or dome of each driver and use one

half of that value.  Enter 0 for the driver having the forewardmost

cone.  Enter negative values in inches for the remainder of the

drivers based on how far behind 0 they will be when mounted on the

baffle.

For example:

We have a dome tweeter whose AVERAGE dome height will be .25"

forward of the flat baffle on which it is mounted.

We have a woofer whose AVERAGE cone depth will be  1" rearward of

the flat baffle on which it is mounted.

Effectively the woofer will be 1.25" behind the tweeter when

mounted on the baffle.

The proper inputs for this example would then be:

0"    depth displacement for the tweeter.

-1.25" depth displacement for the woofer.

If these drivers were mounted on a stepped baffle, where the woofers

mounting surface was 1.25" forward of that for the tweeters, you

would then input 0 for each of the depth displacement values.

Specifying the proper depth displacement does not "time align" the

drivers.

+++++ RESPONSE STEP FREQUENCY and HEIGHT

In the 2 way example model for the LMP walk-through, we used an

average value of the height and width for the baffle size to

determine the response step frequency.  In reality, the baffle

does not have a single size, but many.  For a driver which is

mounted asymmetrically on a baffle; measuring from one baffle

edge, across the center of the driver, to the other baffle edge

every 22.5 degrees on a circle will yield 4 DIFFERENT distances

and 4 different step frequencies, each contributing 1.5db to the

step.

Fortunately, the step curve produced by the averaging method will

be within 1 or 2db of a multi-step curve if the height to width

ratio of the baffle is not unusually large.

If sound absorbing foam is applied to the surface of the baffle,

the step frequency will increase to a degree related to the

absorbtion characteristics of the foam.  This should be considered

as somewhat unpredictable.

Speakers designed to be wall mounted will have NO step frequency

or step height as the wall acts as an extension of the baffle.

+++++ CROSSOVER CHOICES

Select the number corresponding to the crossover choice that you

want to use for the driver # that you are inputting information

for.  For a woofer this will usually be a low pass; the mid-range,

a band pass; the tweeter, a high pass.  You may often opt for no

network, allowing only the drivers natural roll-off to provide

filtering.

ASYMMETRICAL BAND PASS NETWORK SECTIONS MAY BE MODELED.  If you want

a 3rd order high pass cascaded with a 4th order low pass, select the

4th order band pass and "open" the second parallel inductor by

inputting a value of 1000 mh.

+++++ ENTERING INFORMATION IN THE CROSSOVER FIELD

Enter values for each element in the crossover when prompted.  All

inputs are in standard units; microfarads, millihenries and ohms.

To accept the displayed default value, just press <ENTER>.

Parallel  INDUCTORS  may be   OPENED   by inputting    1000      mh

Series    INDUCTORS  may be  SHORTED   by inputting        .001  mh

Parallel  CAPACITORS may be   OPENED   by inputting        .001 mfd

Series    CAPACITORS may be  SHORTED   by inputting    1000     mfd

Note that "asymmetricalizing" bandpass filter designs imported from

P-FILTER will cause the bandpass gain to be non-constant across the

bandwidth of the filter.

----- BI/TRI AMPLIFICATION  -----------------------------------------------

Some believe that bi or tri-amplification solves all the problems that are

inherent in systems with passive crossovers.  This is not true.  While

multi-amplification does eliminate the need to consider the drivers impe-

dance curve and apply Zobels or other correction networks, the systems

drivers still have their own roll-off characteristics, depth displacement

and step frequency considerations.

Multi-amplifier system designs can be modeled with LMP as easily as passive

network systems.  Each network choice in the P-Filter program and any

asymmetrical design you wish to create can be duplicated with active

circuits.

There are a number of active filter design programs available for the PC

from Old Colony Sound Lab, even one from Sitting Duck called A-Filter.