Documentation for QBOX.EXE a loudspeaker enclosure design program.

Quick Box allows you to design Vented and Sealed enclosures for woofers.

Although Quick Box is applicable to cone type mid-range drivers, this

documentation will confine itself to woofer enclosures.

If you are already familiar with the need for enclosures, driver parameters

and the terminology used for enclosure design, you can skip the explanatory

part of this document.  If you are not familiar with loudspeakers but wish to

learn more, it is suggested that you buy a copy of The Loudspeaker Design

Cookbook by Vance Dickason.

THIS DOCUMENTATION IS NOT A TUTORIAL ON VENTED OR SEALED ENCLOSURE DESIGN.

Sufficient information is presented only to enable an understanding of how the

various inputs interact and become a solution to the box design problem.

WHY WOOFERS MUST HAVE ENCLOSURES (simplistically stated)

While the woofer cone is moving FORWARD in response to a positive going

amplifier signal, it is creating, in FRONT of the cone, a high pressure area

by compressing the nearby air molecules.  This high pressure area must be

allowed to propagate into the room if it is to reach your ears.  At the same

time, a low pressure area is being created at the REAR of the cone.  If there

is no enclosure, the high pressure (seeking the path of least resistance)

simply propagates around to the rear of the woofer and fills the low pressure

area; almost everything cancels out and little remains to reach your ears.

Anything that prevents this cancellation will theoretically suffice and is

called a BAFFLE.

BAFFLES

Early on in the hi-fi game, baffles were VERY large boxes (hence the name

INFINITE BAFFLE) and woofers were designed to operate satisfactorily in these

large boxes.  In large boxes, the COMPRESSIBILITY of the volume of air inside

the box was of little significance.

In the early 1950s, it was decided that a woofer could be redesigned to work

properly in SMALLER boxes by adjusting some of mechanical and electrical

characteristics of the woofer itself.  Taking advantage of the fact that the

small volume of air trapped in a small box is difficult to compress, woofers

were designed so that the characteristics of the trapped air became part of the

final loudspeaker SYSTEM design.  These designs became known as AIR SUSPENSION

or ACOUSTIC SUSPENSION systems and because of the small box size needed to

confine the air, became very popular.

ACOUSTIC SUSPENSION or CLOSED BOX SYSTEMS

Since the compressibility of the air inside the box is part of the system

equation and since the compressibility is a function of the VOLUME of air

inside the box, the size of the box is an important variable.

The volume of the enclosure is one of the variables which the speaker system

designer (that's you) has control over.  For any given woofer, the Volume of

the Box, or Vb for short, can be varied over a fairly wide range and still

provide acceptable system performance.  However, if the volume is too small,

the air is overly difficult to compress; if the volume is too large, the air is

too easily compressed.

Qtc

Qtc is a variable and complex term which describes one aspect of an acoustic

suspension loudspeaker system.  Within the mathematical equation that describes

Qtc is Vb (the box volume).  So, by changing Vb you alter Qtc.  Unfortunately,

explaining the concept of the Q factor is beyond this disc document.  It is

enough for you to know, to use this program, that final values of Qtc which

fall into the range of .5 to about 1.5 are useful.  A Qtc of .7 provides flat

frequency response down to the cutoff of the system and is the target Qtc used

by many system designers.

As you use the Closed Box portion of the program, you will be able to see the

effects of Vb on Qtc.  Anybody can design a closed box woofer system with

Quick Box!

WOOFER PARAMETERS

Woofer manufacturers design their products for use within a specified range

of box types and sizes to achieve certain performance criteria such a Qtc,

power handling, maximum output, sensitivity and low frequency cutoff.  To

help the system designer, the manufacturer will publish a data sheet for the

woofer.  Data sheets contain the information required for YOU to describe a

woofer to Quick Box in order that the program can make the box calculations.

These pieces of information are called PARAMETERS.

When you first start the program, highlight "Driver" and press <ENTER>.  Begin

entering the parameters of the woofer you wish to use.  If you are a novice,

you will not have any parameters on hand, so for this design example we will

use the parameters of a popular 10", 8 ohm woofer from Audio Concepts, the

AC10-8.

The important parameters for the AC10-8 are:

Re = 6.1 ohms         Re is the DC resistance of the voice coil.

Fs = 23  Hz           Fs is the free air (unenclosed) resonant frequency

of the woofer.  Hz is the abbreviation for

Hertz and means "cycles per second".

ZMax = 56             The maximum free-air impedance of the woofer.

Qms = 3.6             Qms is the mechanical Q factor of the woofer.

Qes = .5              Qes is the electrical Q factor of the woofer.

Vas = 138             Vas is the enclosed volume of air which has the

same acoustical compliance as the woofer

suspension.

Le = 1.22             Le is the inductance of the voice coil.

Diam = 8.2 inches     Diam is the effective diameter of the woofer.

With this information the program can make the necessary calculations to

arrive at box volumes for various values of Qtc.  You need not input anything

for XMax or ZMax.  Notice as you input the data for Qms and Qes that the

program calculates Qts, which is the unenclosed (free-air) Q of the woofer.

When you have input the required information, pressing <ENTER> after each item,

press <ESC>, highlight "Closed" and press <ENTER>.

CLOSED BOXES

Notice that the program has assumed that your target Qtc is .7 and has already

calculated the proper box size.  BELIEVE IT OR NOT, YOU ARE ALMOST FINISHED!

For this woofer and for a Qtc of .7, the amount of compressible air volume

inside the box should be 88 liters or 3.1 cubic feet.  You must add to this the

volume taken away by the volume occupied by the woofer itself and, anything

else you may put inside the box.

Press <F2> to see a frequency response curve of the system.

Note that the cutoff frequency (F3) of the system is 37 Hz, that the output

rolls off (decreases) below 37Hz and that the response is flat, except for the

rolloff.  As evidenced by the plot, a Qtc of .7 does indeed provide a flat

response.

Press <ESC> to return to the Closed Box design section.

BOXES TOO SMALL

One of the mistakes many novices make is to take a woofer and put it into

"any-old-size-box".  Let's see what happens when this is done by changing Vb.

Press <F6> and input 14 liters which is .5 cu. ft.; a size that might be

selected for this woofer when space is at a premium such as in an automotive

installation.

Notice that the Qtc has risen to almost 1.5.  Run another plot and look at the

response.  Note the peak of 3 decibles at 90 Hz and the new value for F3.

This system will sound almost twice as loud at 90Hz than at any other

frequency.  Making a box that is too SMALL causes a bump in the response and

sacrifices some output at the lower frequencies.  This box will sound a little

"boomy" and be weak in the lower bass.  In addition, the high Qtc will cause

the amplifier to lose some control over the woofer; the cone will not stop and

start as rapidly as the input signal dictates.  Thus, the "TRANSIENT" response

is degraded.  Some loudspeaker manufacturers intentionally aim for Qtc values

of 1.2 to 1.5 in order that the system give the listener an "impression" of

better bass response.

BOXES TOO LARGE

At the other extreme, is a box which is much larger that it should be.  This

situation could be arrived at by mounting the woofer in a closet door, a common

practice in the "old days".

Change Vb to 2500 liters and run another curve.  Note that F3 has risen only

slightly and that Qtc has dropped to almost the Qts value of the woofer in

free air (Qtc can never be less than Qts).  Notice that the output of the

Qtc=.45 system in the 40 to 80 Hz range is not much different but that the

output BELOW cutoff is greater!  In addition, the transient response is better

because the transient response of a Qtc=.7 systems is known to be less than

perfect, even though considered acceptable.

While a Qtc of .7 has typically been a target value, some "high end" speaker

system manufacturers have been gravitating toward lower Qtc values; .5 or .6.

OTHER EFFECTS ON Qtc

Return to the Closed box design screen and reset Vb to 88 liters, the correct

value for a Qtc=.7 system.

Rg is the designator for "added resistance".  Added resistance is the DC

resistance of the wire between your amplifier and speaker PLUS the resistance

of any crossover coils which may appear in series with the woofer.

Change Rg to 1 ohm, not an unusual value for a reasonably priced crossover

coil plus speaker wire.  Note that Qtc has risen from .7 to .82 in response

to this added resistance.  Although the change with a 1 ohm Rg is not that

significant, systems with other woofers may exhibit more dramatic changes.

Rg is a important value which should be determined BEFORE you build your box.

-------

There are many other factors to take into account when designing a closed box

woofer system but they cannot all be covered here.  Some of these factors

include:

The woofers efficiency factor in relationship to the other drivers

in the system.

How much you may reduce the volume of a closed box by the use of

"stuffing" material.

Enclosure resonances, bracing and construction materials.

Crossover design and the integration of the mid-range and tweeter.

VENTED BOXES

Vented boxes are characterized by a port in the enclosure wall.  The port may

or may not have a tube behind it.  The port tunes the box to a particular

frequency which is a function of the box volume and the length and diameter of

the port.  Near Fb (the box frequency) both the cone and the vent are making

contributions to the total acoustic output of the system.  It should be noted

that at Fb the output of the woofer decreases by the same amount that the

output of the vent increases; there is no net GAIN in output.

Vented boxes have advantages and disadvantages; fans and enemies.  Many vented

box devotees have a love/hate relationship going with their bass systems.

The main ADVANTAGES of vented boxes are:

A.  A lower cutoff frequency using the same woofer (compared with a

sealed box).  With 10" and 12" woofers, large vented boxes can

easily have cut-off frequencies in the range of 15-20Hz.

B.  Less cone excursion near the box resonance frequency.

C.  For the same cut-off frequency, vented boxes can be smaller than

closed boxes.

Less cone excursion means that vented boxes can usually be played louder before

the excursion limits of the woofer are reached.  This is one of the reasons

that non-horn loaded speakers designed for sound reinforcement/rock music are

of the vented variety.

The main DISADVANTAGES of vented boxes are:

A.  The cut-off slope (the rate at which the output declines below

cut-off) is steeper than a sealed box and has poorer transient

response.

B.  At frequencies below Fb, the woofer cone loses its air load (the

"springiness" of the air volume in the box) and becomes free to

excurse without restriction.  These low frequency signals are due

mostly to record warp and can eat up a lot of amplifier power.  This

problem does not exist with CDs and can be eliminated with records

by the use of low frequency filters.

C.  Increased sensitivity to construction inaccuracies; box leaks,

Vb and Fb variations, and manufacturing tolerances of the woofers

themselves.

D.  The potential of a too-small-diameter vent making noise.

You must consider the various trade-offs and determine which design is more

appropriate for your application.

ALIGNMENT

Vented boxes are usually built to conform to a set of "alignments".  Standard

flat alignments result when the woofer Qts < .4 or so, and non-flat alignments

result when the woofer Qts > .4 or so.

QBOX will design boxes which conform to the popular B4, BB4, QB3 and C4

alignments.  Again, you must consider the trade-offs and determine which

alignment is appropriate for your application.

With the same woofer parameters as for the closed box design example, highlight

"Vented" and press <ENTER>.  Reset the value for Rg to 0.

Select alignments 1, 2, 3 and 4 by pressing the appropriate number keys in

quick succession and observe how the values for Vb, Fb and the vent length

change.  Note that for this particular woofer, each alignment produces a

similar Fb but the Vb ranges from 4 to 6 cubic feet.  Select the alignments

one at a time and plot the response of each.  The similarity of curves produced

by the 4 alignments will not always be this close with other woofers.

Change Rg to 1 ohm.  This will allow for a crossover coil with 1 ohm of DC

resistance.  Note how Vb and Fb have changed for each of the alignments.  Now

plot each one.  Quite a difference, and indicative of how sensitive vented

enclosures are to any added resistance.  High quality crossover coils might

have a DC resistance of .4 ohms whereas a cheap one may have a DC resistance

of 2 ohms.  For this, and other equally valid reasons, there is no such thing

as a "universal" crossover network.  It should be obvious that Rg must be

determined prior to the construction of a box.

For some exercise, experiment with different values for Vb, Fb and Rg and run

a plot after each change.  See what you can get away with, keeping in mind

that the port should be no less than 2" in diameter and the length must allow

it to fit into the box.  Variations in response of +/- 1.5 db could be deemed

acceptable, depending on how critical you want to be.

Getting a system "dead on" is NOT possible unless you are willing to:

1.  MEASURE the parameters of the actual woofer to be used in the system.

This removes the effects of the manufacturers production tolerances.

2.  Enter the MEASURED parameters into the program so it can calculate

the approximate box size.

3.  Build the box about 20-25% oversize to account for leakage.

4.  Measure the system and apply tweeks where appropriate.

A description of the equipment required and the procedures used for parameter

measurement and tweeking are given in The Loudspeaker Design Cookbook.

SMALL AND LARGE SIGNAL ANALYSIS

Quick Box performs a small signal analysis only.  A large signal analysis would

plot, for example, how much power you can input vs frequency before the voice

coil leaves the magnetic circuit.  Although of interest to the speaker system

manufacturers, hobbiests do not usually find themselves in the position of

having to wring the last ounce of performance from a system for the sake of

"price points" and marketability.  If a 10" woofer won't play orchestral or

rock at volume levels that suit you, you have the luxury of doing something

about it for little additional cost.