Perfect Communications 1.0

Perfect Communications

Copyright June 30, 1990

Russell Hildreth

Features:

Function key compatible with Word Perfect.

Menus similar to Word Perfect.

Mouse support.

Easy setup.

HandsFree Zmodem Downloading.

Introduction:

NOTE: *** This is a VERY early preliminary

release of the program.  Many features that you

might expect in a communcations program are not

yet to be found here.  I decided that it would be

better to release the program to the public so that I

can start to get some feedback on it, rather than

keep on working on it in silence.

This program has two features that distinguish it

from other communications programs--the keyboard

layout, and the extensive support for external

protocols and DSZ.  I began writing this software to

suit myself and these were the two things that I

wanted most.  I use Word Perfect just about

constantly, so it was logical to lay the keyboard out

similarly.  I also do a lot of file transfers, so I added

as much support for that as possible.

Perfect Communications 1.0 is the first release of a

terminal or communications program that uses a

keyboard setup and menu layout similar to the Word

Perfect word processing package.  I spend a great

deal of my time using Word Perfect, and was often

frustrated when using a popular terminal program

that the keyboard layout was completely different

from what I was familiar with in Word Perfect.

Hence, Perfect Communications was born.

The main objective in writing Perfect

Communications was to provide a communications

program that users of Word Perfect would be

comfortable with.  Perfect Communications uses the

function keys like Word Perfect does, and many keys

perform the same functions in both programs (i.e.,

F7 for exit).  I feel that frequent Word Perfect users

will find the keyboard layout in Perfect

Communications much more intuitive than some

other communications packages.

Perfect Communications also uses a look and feel

that is similar to that of Word Perfect.  Users of

Perfect Communications will find that the menu

layouts are identical to those of Word Perfect.

Again this was designed to make Word Perfect users

feel comfortable in Perfect Communications.

There are some other important features of Perfect

Communications.  It is smaller than most other

communications packages.  It provides more

flexibility in the form of user hotkeys than any other

package that the author is aware of.  I have added

little things that I found lacking in other packages,

such as the ability to use VGA's 50 line screen

mode.  Perfect Communications supports a mouse if

you have one.

Finally, let me ask you to keep in mind that this is

the first public release of this software package.  It is

by no means the last release!  I have a lot of ideas

for enhancements to this package, but I wanted to

get a version out to the public, so that other people

could begin to take advantage of the benefits offered

by a program such as this.

Legalities:

I sincerely hope that the Word Perfect Corporation

will not be offended by the release of this software.

It is not meant to infringe on any of the Word

Perfect Corporation's trademarks or intellectual

property rights.  I have a great deal of respect for

the work of the Word Perfect Corp., and would

never intentionally do anything to violate that

company's software rights.

Word Perfect and Word Perfect 5.1 are trademarks

of the Word Perfect Corportation.

Zmodem, Zmodem90, DSZ, and Autodownload are

trademarks of Omen Technology.

Perfect Communications 1.0 is copyright by Russell

B. Hildreth, June 1990.

Registration:

Perfect Communications 1.0 has taken many hours

to create.  If you use the software, you have an

obligation to register it with the author, Russell B.

Hildreth.  Registration costs only $25 per copy, and

assists the author in obtaining a higher education.

Perfect Communications 1.0 is shareware.  This

means that you have the opportunity to try out the

software to see if you like it.  If you do like and

continue to use it, you must register your copy with

the author.  If you do not wish to register, please

delete your copy of the software, or pass it along to

a friend.

Continued use of the software without registration is

unethical and a crime.

Registration provides certain benefits:  You receive

an updated release of the software, along with a

printed copy of the documentation.

Please use the PERFCOMM.REG form for

registration.

Installation:

This part is simple.

Copy all of the files in the PERFCOMM.ZIP archive

into a directory called PERFCOMM (or whatever

you choose).  Execute Perfect Communications by

running the PERFCOMM file.

When you load PERFCOMM the first time, you

should immediately go to the Setup menu (Shift-F1)

to configure the program for your system.  See the

section on Setup for more information.

There are two requirements for operation that you

should pay attention to:

If you wish to communicate with bulletin boards

using ANSI terminal emulation, you should specify

the line DEVICE=ANSI.SYS in your CONFIG.SYS

file.  Most people have this setup already.  In order

to keep down program size, Perfect Communications

does not provide its own ANSI driver (why be

redundant?).  If you call Compuserve, you should

GO TERMINAL and specify that you are running in

ANSI mode.

If you wish to transfer files to or from a host bulletin

board you will need to provide your own external

protocol programs.  Most BBS callers are now using

one or more of these external protocols (i.e.,

Zmodem, Puma, Jmodem, etc.) anyway, and again I

figured I would reduce redundancy.  I suggest using

DSZ Zmodem from Omen Technology as a

startingpoint.  It supports the big three transfer

protocols--X, Y, and Zmodem.  The sample setup

file included with Perfect Communications (my

personal setup) shows how to set up Zmodem and

Ymodem downloads.  Uploads currently need to be

done manually from DOS, or with an external shell

such as XFER32 (available on Compuserve--a good

idea for people who plan to do a lot of Zmodem

uploads).  Specify the shell as a hotkey.

Note also that Perfect Communications supports a

DSZ HandsFree feature.  Perfect Communications

monitors the stream of incoming characters and if it

detects a DSZ download it will automatically shell to

DOS and run DSZ for you--provided that DSZ is in

the same directory as Perfect Communications and

you have enabled the HandsFree Zmodem feature

in the Setup menu (Shift-F1, S).

If you frequently download files from Compuserve, I

suggest getting OZBEXT.ARC (GO IBMCOM on

Compuserve--you'll find it in the Protocols library,

along with DSZ).  This shell provides the

Compuserve B family of protocols, and is also

demostrated in the sample setup file.  If there is

enough demand for them, I will add the traditional

transfer protocols in an upcoming version of the

software.  I'd like to avoid doing so, however, since

most advanced users shy away from X and Ymodem

like the plague, and I don't want to bloat up the

software with unused features.  I still cannot get

over one communications package I once used which

supported about 16 different protocols--none of

which I ever used.

Setup:

Access the Setup Menu by pressing Shift-F1.

The Setup Menu offer the following choices:

1.  Dialing Directory

2.  Defaults

3.  Hotkeys

Briefly, the Dialing Directory Menu is where you

define the telephone numbers and line setting

characteristics for each of the BBS systems you

intend to call.  The Defaults Menu allows you define

options such as your communications port and

whether you want to operate in 25 or 50 line screen

modes.  The HotKeys Menu allows you define up to

10 user hot keys for external protocols, shells,

editors, et cetera (I always felt that the two hotkeys

offered by Procomm Plus were far too few).

For more information, see the next three sections.

Setup: Dialing Directory Menu

From the Setup Dialing Directory Menu you can

add new telephone numbers and delete old ones.

The Modify function appears on the menu but is not

yet implemented as of version 1.0 of Perfect

Communications.  If you wish to modify a Dialing

Directory entry, simply delete it and then create a

new entry with the correct information.  Adding and

Deleting entries should be fairly straightforward.

Setup: Defaults Menu

From the Defaults Menu you can set your

communications port, default line settings, screen

mode, modem initialization string, et cetera.

The COM port MUST be 1 or 2.  Ports 3 and up

are not supported as of version 1.0 of Perfect

Communications.

Your modem initialization string should enable

extended result codes.  For most modems this means

including the X4 command in the AT command

string.  Perfect Communications uses these long form

result codes to detect a BUSY signal or a

CONNECT.

The default initialization string is:

AT X4 S0=0 S11=50

Some modems may not support the last command

(S11=50), which is the length of the dialtone in

milliseconds.  If your modem does not support this

command, or does not dial as fast as 50 milliseconds,

remove or change this section of the initialization

string.

The modem initialization string is sent to the modem

each time you exit the Setup menu.  Because of this,

I suggest that you do not enter the Setup menu

while you are online.  Wait until you are not

connected to any BBS service.

You can Enable or Disable the Zmodem HandsFree

feature.  The default for this option is Disabled.  If

you do not have a copy of DSZ Zmodem, you

should not enable this feature.  If and when you do

get a copy of Zmodem, turn this option on.

The Zmodem Download Path option is implemented

for REGISTERED users of DSZ.  If you do not

have a registered copy of DSZ this option will do

nothing for you.  If you are running a registered

copy of DSZ, specify a path for your downloads and

this path will be appended to the call to DSZ for

downloads with PgDn or the HandsFree feature.

Setup: Hotkeys Menu

Perfect Communications supports up to 10 user

defined hotkeys.  You can call just about any

program that will fit in your available memory.  I

would suggest setting up some external protocol

drivers, a text editor, and whatever else you need for

online use.

Zmodem Downloading:

Perfect Communications supports automatic

Zmodem downloads, if you have a copy of

DSZ.COM in the same directory as Perfect

Communications.  In order to take advantage of the

automatic downloading feature you must do two

things:

Copy DSZ.COM to the same directory as Perfect

Communications.

Enable HandsFree Zmodem Downloading via the

Setup menu (Shift-F1, S).

If you have a registered copy of DSZ and wish to

download to a specific directory (only available with

the registered version of DSZ) you may define a

download directory via the Setup menu, by selecting

the Zmodem Download Path option.  Your

downloads will be sent to this directory.  This option

only functions with REGISTERED copies of DSZ.

Another Zmodem Downloading option is to use the

XFER DSZ Shell by the same author as Perfect

Communications.  This shell can be setup as a

HotKey, and supports point and shoot selection of

files for uploading, batch uploads and downloads,

downloading to any directory you choose (without a

registered copy of DSZ), and it plays Bach music.

This program is available on Compuserve (GO

IBMCOM) or direct from the author of Perfect

Communications for $10.  If you plan to use DSZ

frequently for uploading, XFER is a must.

Macros:

The macro feature, on Control-F10, is very simple at

this time.  Basically you can specify an ALT-

LetterKey for your macro (filenames and other key

combinations are not currently available, but

probably will be in the future), and record

keystrokes.

You need to be very careful when using this feature,

as there is currently no facility to slow down

playback of macros, so it may not be possible to

record long macros that need to pause to wait for

something to happen or for a specified time period.

Those features will be added in the future.

For now, you can use the macro feature to plug in

passwords etc.  Some experimentation is probably

going to be necessary.  Also, take note, there is

currently no macro editor, though that, too, will be

available at some point.

Function Keys:

Here's a quick list of what is available to you at this

time:

F1             Cancel (Hangup in Terminal)

Shift-F1       Setup

Control-F1     Shell to DOS

F3             Help

Shift-F3       Switch 25/50 line mode

Control-F3     Clear Screen

F5             List Dialing Directory

F7             Exit

Shift-F7       Dump Buffer to Printer

F8             Format (line settings N81 or E71)

F10            Screen Save (screen grab).

Control-F10    Macro Record On/Off

Conclusion:

I invite you to keep your Word Perfect template

right where it is and refer to it for available

keystrokes.  It's what I used when programming this

software.

I also invite you to send me your comments on this

software--tell me what you like and dislike, and what

you would like to see added.

I can be reached by Compuserve Easyplex mail at

71041,2132.  You can send US mail to:

Russell Hildreth

P.O. Box 72431

Davis, CA  95617-2431

As I said in the introduction, this is the first release

of the software.  It is not perfect, YET.  With your

support it will soon be.