CALL TIME

Compiled Basic V - 3.60

Shareware by BRAND Consultants

and

CALL TIME Version 4.40

Quality Software by BRAND Consultants

P.O. Box 310

Brattleboro, NY 12061

(C)Copyright, 1987-1990 All Rights Reserved

BRAND Consultants

----------------- Forward ----------------

As many of you may know, the U.S. Naval Observatory in Washington D.C. is a

source of much information for Military, Commercial and Civilian use.  They

have developed a system for disseminating time information in a variety of

different ways, for use by the general public.

They would like to make their system more useful to the public, but not

at the expense of regular users who must have access to certain vital

informtion.  They are even willing to add phone lines to carry the

additional traffic, if the need is there.

These programs have been developed with the full knowledge and consent of

the officials at the U.S. Naval Observatory.  They have also received

copies of the programs, so they are aware of how they work.

We have come through several major revisions, and a bunch of minor

revisions to this, the latest of all, Version 4.40.  In this we have

listened to many users, who have been kind enough to write and express

their opinions and suggestions, and added features that many have asked be

added to the program.

Things like, autosequencing, re-dial, automatic capture, and dial string

extensions for use with credit cards, are what you will find in this

version.

We are working on additional things like, pre-programmed command strings,

and a system for use anywhere in the world, with a world time display.

----------------- PLEASE NOTE ! ! ------------------

CALTYM36.EXE (CALLTIME Version 3.60) is Shareware and may be distributed

under the 'normal' Shareware conditions.  BUT ONLY as long as NOTHING is

changed in any of the system programs or document files.  Program names,

Copyright Information and Archived files must remain as orginally uploaded

by BRAND Consultants, and BRAND Consultants name and address information

MUST be included.

CT4.EXE (CALLTIME Version 4.00 and later) is NOT, repeat, IS NOT Shareware,

and may not be distributed in any form or by any means without express

written approval of BRAND Consultants.

Description:

CALL TIME is a communications program that will automatically dial the U.S.

Naval Observatory Computer in Washington, DC, establish handshaking with it

and download the correct time and date into your system, either under your

control or "no-hands" automatically.  It is corrected for leap years, and

allows you to select your particular time zone, in Standard or Daylight

time, PLUS Universal Time (Greenwich Time).

If you have a real-time clock, with compatible software, this clock will be

reset by CALLTIME if you started your real-time clock program in the

resident time/date mode.  Check your clock manual on the correct method.

For AT owners with the MOS clock on the motherboard, a selection within

CALLTIME allows re-setting this clock.

On another Naval Observatory telephone line, a variety of information is

available as General Information.  CALLTIME will automatically log you into

that system, and then allow you to enter requests of your own.  Other

information available include Tides & Moon Phases, Sidereal Time, Sunrise

and Sunset for any point...etc.  Enter  @TCO  when in manual mode, and you

will be given a list of commands.  You are limited to 30 minutes or 15

commands, whichever comes first.  This version contains a 1000 line capture

buffer, which you can turn on or off while on-line.  When you exit the

program you can save the buffer to a disk file called 'USNOCAP1.TXT'

The USN Observatory computer operates only at 1200 baud, Even parity with 7

data bits and 1 stop bit.

Versions 3.60 and 4.40 are essentially identical, but with some significant

differences.  V 4.40 is written in assembly language, is shorter, much

faster, is more accurate and contains many more enhancements.  Other

differences will be outlined in italics in the instructions.

Requirements:

IBM-PC,XT,AT with at least 256k of RAM

DOS 2.x or later.

Hayes 1200, 1200B or 1200 baud modem with a command set that is nearly

identical to Hayes command set.

To set real-time clock:

Requires software supplied by the maker of your Real Time Clock, that has

an option called a Resident time/date processor.  Usually activated by a

'slash' option. (For AST clocks, adding a /r to the 'ASTCLOCK')

For AT machines, special coding is included in CALLTIME

Operation:

The disk contains four or five files...

CALTYM36.EXE - The main program (Ver. 3.60)

CT360.DAT   - Configuration file used by the program. This file must be on

the same diskette or in the same directory on a hard disk as the main

program.

PRINTDOC.EXE - Document printing program

ATCLK.COM - Sets Motherboard MOS Clocks from System time.

CALLTIME.DOC - This document file, in ascii.

README.DOC - May be included for late information.

Registered users will also have:

CT44.EXE - The assembly language version of CALL TIME.

CT440.DAT - Configuration file for Version 4.40

INSTALLATION

First:

Copy the main program and the data file to a working diskette, or onto your

hard disk in whatever directory you wish.

Second:

To start, boot your computer, then at the DOS prompt, type:

CALTYM36  then press Enter (Return)

For V 4.40 - type CT44 then press Enter (Return).

NOTE:

ESCAPE KEY will allow you to 'escape' from most situations if you decide to

cancel your call, or some other problem occurs which causes the system to

'hang'.

Third:

Before attempting to log-in to the USN Computer, you must establish an

Identification String that the USN computer will accept.  It is quite

particular about the format of this ID String.  There is no ID string in

the data file, but the USN computer will not accept this, so you must press

'R' to revise the data file, at the main menu and create your own ID

string.  This ID (in caps) consists of 'YOUR NAME' a slash 'NAME OR

INITIALS OF YOUR ORGANIZATION' a slash 'CITY' a space '2 LETTER STATE

CODE'.

An example would be:

J.J.SMITH/R.C.A./ALBANY NY

The main criteria is to keep it short !

After the ID string, the next item is the Modem Reset string.  For Hayes

and Hayes compatible modems this is 'ATZ'.  This is sent to the modem to

cancel any prior set-up commands in preparation for a new set-up.  The

default set included in the data file is 'ATZ'.

MODEM SETUP STRING

Next is the modem set-up string that configures the modem parameters for

this program.  You will need your modem manual to be able to change this.

These are command strings sent to the modem to configure it for use with

CALLTIME.  We have found that this works for most modems that are Hayes

Command Set compatible.  We suggest you not change this unless you are

thoroghly familiar with your modem and the Command Set usage.

The next item that you can and may want to change is the modem dial string.

The default is 'ATDT1202' which is the Hayes command to start tone dialing

with the number 1 and area 202.  If you need to add a long-distance access

code, you would add it after the 'ATDT' and before the area code.  The USN

Observatory phones included in the program data, have no area codes, so if

you live in the 202 area, you would only need 'ATDT' or 'ATDT1'

MODEM PORT NUMBER

This is the port number that assigned to your modem, or to the RS232

connection where your modem is fastened.  These are identified as COM1 or

COM2.  Simply press the number, 1 or 2 to assign the correct port number.

The default is 1, for COM1.

FOR VERSION 4.40 ONLY

The next item is a Dial String Extension.  On some telephone systems it is

necessary to add a charge number code or credit card code, after the

telephone number is dialed.  Since the main phone number is buried in the

program data, we have added this new feature to V 4.40.  The Extension can

be up to 40 characters long, BUT it must begin with the four letter dialing

command 'ATDT', just as if you were starting to dial a phone number.  Since

this is a command line sent to the modem, you may include any 'legal' modem

commands in it, such as commas for delays, etc.  This Dial String Extension

is only transmitted if you add an option to the DOS start up line.  See the

section on 'Options'.

Starting with Version 4.40, two new questions and one new selection, have

been added to the 'Script' file Set-Up, which we have also decided to re-

name to 'Configuration' file, since it's purpose is more of a Configuration

nature now, than when first released.  We have also made the Configuration

easier, since ALL items now show the current setting, and simply pressing

the [RETURN] will allow you to 'step' thru the whole file, without making

any changes.

COUNT DOWN DELAY

The first new Question asks if you want to Active the Count-Down Delay.

Simply Pressing either 'Y' for yes, or 'N' for no will change the Count-

Down Delay from 'Active' or 'ON' to 'Not Active', or 'OFF'.

The purpose of this is for some machines such as the Olivetti and AT&T

which always set the number of seconds to zero, regardless of what is sent

to the interrupt.  This means that the time can only be accurately set when

the actual seconds equal zero.

If you are not sure if your machine needs the Count Down Delay, there

are several ways to test the system to find out.

One would be to have CALLTIME set the time, without Count-Down, then

with the main menu in operation, call the National Bureau of Standards, in

Fort Collins, Colorado (303) 499-7111.  You will then hear the same time

signal that NBS broadcasts over their station WWV.  This is a tone, with

clock Ticks.  At precisely 45 seconds the tone will stop, but the ticks

will continue.  About 10 seconds (seconds=50) before the exact Minute,

there will be a voice announcement giving the time, as 'Coordinated

Universal Time', then the first tone that sounds after the voice

announcement will be exactly at Zero Seconds. (This is one of the ways we

check CALLTIME)

Another way, would be to call the USNO two times, with the Count-Down

NOT Active and see if the second set of times listed on the screen, just

after the Time/Date are set, vary by more than one second.

BY EITHER method, if the times vary by more than one second, you

probably need the Count-Down ACTIVE.

After some consideration and work with several different approaches to

the Count Down Dely, we have chosen what we feel is the best.  Regardless

of how inaccurate a single machine's timer may be, over a long time

interval, it will be quite accurate enough for our purposes, over a maximum

of 59 seconds. So we have chosen to allow CALLTIME to get the first set of

correct data from the USNO Clock, then disconnect and internally count-

down the correct number of seconds, before setting the System Time, and the

Clock Time.  It is possible that even when the data appears correct, the

numbers may not be valid for a given date & time.  Should this occur, the

System Time may be set, but the Real Time Clock Time and Date will not be

changed.

Besides keeping your long distance phone time to a minimum, there is

another positive benefit in this approach.  If the approach was to stay on

line with the USNO Clock, waiting for it to count down to zero seconds, it

is possible that the data sent at the exact moment could contain noise or

something else that would prevent setting the System clocks.  This would

mean an additional call or maybe more...., since the USNO system will not

allow any single caller to stay on-line more than 60 seconds.  Our tests

with the USNO phone lines have indicated that some of them are very noisy,

and sometimes require several seconds before an error free string of data

comes through.

SET AT CLOCKS

The second new question, asks if you want the AT Clock software Active or

Not Active.  In the newest version we have included our program 'ATCLK.COM'

within CALLTIME, therefore it is not necessary for AT owners to run the

additional program after CALLTIME, when this is 'Active'.  There will be no

adverse effects if this is run on a machine, not an AT.  Since the

Time/Date are entered in Binary Coded Decimal form, instead of Binary, the

BIOS interrupt of a non-AT machine simply ignores them as being wrong data.

ALL VERSIONS

The last item you need to change is the time-zone in which you live.

Simply press the number in front of the proper time-zone then press

[RETURN] and it will be stored in the data file.  If you don't wish to

change the indicated zone, simply press [Return].

In the TIME-ZONE selection area of the Configuration file, we have now

added the ability to set your configuration for "Universal Time", or the

time that is transmitted by the USNO clock.  This is the time at

Longitude=0, or, the time at Greenwich, England.  This time zone is

referred to as "Zulu" which is the phonetic word for "Z".  If you should

ever see time written as 1201 Z, this means it is 1 minute after noon, in

time zone "Z".  This is common military practice, but they more often use

the letter for the time zone, where they may be located.  Just for

information...Eastern Time Zone='R', Central Zone = 'S', Mountain Zone =

'T' and Pacific Zone = 'U'.

The Revise data file option allows you to revise 5 (or 6) items in the data

file directly.  In prior versions, we suggested changing the data file,

using a word-processor program.  Beginning with Version 3.10, we ask you

not attempt to do this.  Some data is now being written in binary values,

which will not be displayed and can cause problems with the program.

CALLTIME now uses modem control signals to determine line status, and does

not rely on numeric or verbose feedback from the modem.

CALLTIME is configured to operate in color on a color monitor (either CGA,

EGA, PGA or...) and, without changes, will display properly on most

monochrome monitors, including Hercules and Hercules compatibles.  However

some compatibles with monochrome monitors, do not support the IBM extended

color attributes (Leading Edge, Model D for example).  So CALL TIME now

reads the video mode and assigns color attributes according to the video

mode in effect at the time of start-up.

If the data file is missing or damaged, never fear !  CALLTIME will create

a new one with the default values stored in the main program.

After you have selected the time-zone number the new data will be written

to the data file and CALTYM32 will return to modem set-up, so that any

changes you made will be initiated immediately.

CT44.EXE writes the data file, then returns to the main menu.  Terminal

startup and Modem initialization occurs after an option has been chosen

from the main menu.

You will then be returned to the main menu.

SYSTEM SWITCHES

A new item, on the main menu, is a display of the 'System Switches' and

their status, as either 'ON' or 'OFF'.  Along with this, is the ability to

'Toggle' each switch ON or OFF by pressing the highlighted letter inside

the brackets ([ ]).  This is temporary, and allows you to try different

settings without having to re-start the program or change the Configuration

file.  A word of caution, if you change the Count Down Switch or the AT

Clock Switch here, then run the [R]evise Configuration File, these switches

will be set as you had them on the main menu, unless you change them back,

while revising the Configuration.

The AutoSequence switch cannot be changed here, as this must be set

using the /a option at the start of the program.

LAST USNO CALL

In the middle of the main menu, the Date and Time of the last successful

call to the USNO Clock are displayed.  This is recorded in the

Configuration file whenever you are successful in setting the Time and Date

from the USNO Clock.  It does not record unsuccessful calls, or calls made

to the General Information Service.  This may be of help to those who are

interested in tracking the changes of their Real Time Clocks.

TIME-LINE

On the new face of CALLTIME, you see a linear Time Line, that looks much

like the Speedometer on some cars.  After investigating several different

graphic methods of showing elapsed Seconds, this appears to work the best.

We have not seen what we consider to be an adequate 'clock face' display,

save one that is in Microsoft's Windows, but not everyone has windows, and

not everyone has an EGA or similar display.

TICS

The clock 'Tics' remain as they were in the older version.  Every second

sounds a tic, with a higher pitched tic occurring at the point of 00

milliseconds, which occurs about every 5 seconds.  If the Tics are

annoying, you can turn them OFF and ON by just pressing the 'T' key.

TIME DISPLAY

We now show time in both 24 Hour Format, and 12 Hour Format.  We also have

eliminated the 100ths column from the 24 Hour Format, as the numbers ran so

fast, that it was useless to try and see them.

Between the two time displays, are the Weekday and Month, Day & Year with

the current Time Zone.

DETERIORATION OF TIME

We have sent to our users a bulletin entitled "Warning, Software can be

Hazardous to your Time!".  In this, we point out that all software causes

your System Time to loose time as a normal course of events.  The PCjr uses

a slightly different method with it's timers than the other machines, but

we don't know if it's timekeeping is much better.  And, until the

architecture of all systems is changed, so that a totally independent

timekeeping system is installed, there is no way that this problem can be

overcome.  Machines that use the 80386 processor, do have the ability to

run two program segments (called Threads) simultaneously as long as neither

segment requires more than 16 bits of data or addressing.  This could

overcome some of the hardware interrupts that cause the problem of lost

System time.  At this time, we know of no language capable of producing

"Threads" other than Assembly Language.

If you start your machine in the morning, and then run various

programs on it during the day, by the end of the day, the System Time can

be quite inaccurate.  If you want an accurate time, relatively speaking,

you should re-boot the machine, so that time from the Real Time Clock is

reloaded, then your System time will be as accurate as your Real Time

Clock.  In some cases, it is possible to simply run the software for your

Real Time Clock, to transfer it's time to the System.

One major problem with attempting to include software in CALLTIME to

access Real Time Clocks is that manufacturer's continue to change the

hardware and software of their clocks, and usually change the address of

the all important clock data at the same time.

OPTIONS FOR START-UP (Preset System Switches)

To activate the options a series of 'slash' commands must be entered after

the program name, at the time the name is entered behind the DOS prompt.

The options may be in lower case or capitals, and may be used in any order

and in any combination.

These options are:

/a - to set autosequence.  Autosequence

allows 'hands-off' operation of calltime.  It will dial USNO, set time and

return to DOS without any further entry.  An Exitcode will be set on DOS

return that allows use in batch files to determine if time-set was

successful.

/r - to activate redialing.  If the timeset

is not successful or if the phone is busy, or some other problem occurs

during the on-line session, CALL TIME will re-dial the USNO until a

successful time-set is accomplished, up to a maximum of 10 times.

VERSION 4.40 ONLY

/c - to activate capture buffer for time set call.  When '/c' is used on

the startup

command line, the capture buffer records the time/date information exactly

as you see it on the screen.  If autosequence is ON, this data is

automatically saved to disk as 'USNOCAP1.TXT'.  If autosequence is off you

will be asked if you desire to save data, or not.

/x - to send dial string extension AFTER the phone number.  If you want to

send a special

charge number or credit card number, after the phone number is sent, this

option must be added to the end of the command line, and the number to be

sent, must be put into the Dial String Extension of the configuration file.

Be sure to leave a space between the program name and the command option.

Examples:

ct44 /x/c/a

ct44 /a/x

Now, to set your system time, just press A and hang on!

AUTOMATIC TIME SET (MAIN MENU 'A' COMMAND)

Everything is automatic and does not require any input from you, and you

will be able to see the results on the screen.  Immediately after the time

and date are set, the system disconnects.

NOTE:

Days and Dates displayed are referenced to the LOCAL time zone you have

chosen.  The Modified Julian Date and Day-of-the-year sent by the USNO

system are based on Universal time, which is the time at the Greenwich

meridian (0o).  Therefore, the date referenced to Universal time changes 5

hours before midnight Eastern Standard Time.  CALLTIME takes this

difference into account to accurately set your system clock.

Version 3.60 attempts to filter 'noise' characters from the received data,

but if after 10 seconds it is unable to get a satisfactory set of data from

the USNO computer, it aborts with an error message.

Version 4.40 makes 15 tries at one second intervals to accomplish the same

purpose.  It also will abort after these tries, if unable to set time or

date.

GENERAL INFORMATION FROM US NAVAL OBSERVATORY ('G' COMMAND)

After you press 'G' at the main menu, the program will dial the USNO

general information phone, and then switch to manual entry mode to allow

you to enter your desired commands.  The capture buffer will be 'ON' and if

you should reach it's 1000 line limit, it will be switched off

automatically, and a message telling you it's full will be printed on the

screen. To switch the capture buffer ON or OFF, press <Alt> and <C> keys

together. The USN Observatory will not allow you to waste any time, and

will disconnect if you make too make mistakes or if 20 seconds elapse

without an entry. (or carriage return).  If you want to capture this

process on your printer, press Ctrl and PrtSc and be sure your printer is

on, and on-line with lots of paper.  To stop the printer, press Ctrl PrtSc

a second time.  Please be aware, if your printer is slow, the resulting

time wasted by the printer will severly limit your time on the system.

MANUAL MODE  ('M' COMMAND)

If you press M at the main menu, you will be switched to a normal 'dumb'

terminal mode.  You can manually enter phone numbers to dial and operate

this like any dumb terminal, operating at 1200 baud, Even parity, 7 data

bits and 1 stop bit

Escape KEY  (Esc COMMAND)

Pressing the Escape key at the main menu will return you to DOS.  It also

can be used to get you out of most situations, and back to the main menu

CORRECTING TIME OF REAL-TIME CLOCKS

Correcting your real-time clock board can be automatic, if your clock

software has an option that allows setting the board from DOS, by entering

a slash option at the end of the startup command.  For AST boards and some

other compatibles, adding /r to the end of the 'astclock' command will load

the resident time & date processors.

You must do this BEFORE running CALLTIME.

AT OWNERS NOTE:  CT44.EXE has special coding to transfer time to your

motherboard MOS Clock.  This code is only run when you set the System

switch 'Set AT Clock' 'ACTIVE' or 'ON'.  This code is run in addition to

the regular time interrupt codes, and will not affect non AT machines

The program ATCLK.COM will do the same if run anytime after running

CALTYM36.EXE.

CAPTURE BUFFER-FILE

When there is data saved in the capture buffer, a message asking if you

wish to save the buffer to disk, will be displayed after you log off the

General Information system.  If you answer yes, the ASCII data will be

saved in a file called 'USNOCAP1.TXT'.  It there is another file of the

same name, it will be overwritten.  If you desire to save these files and

not have them overwritten, we suggest you change the name.

You can display these data files with a word processing program, the EDLIN

program in your DOS, or print them using the DOS 'PRINT' command.

ADJUSTING TIME WITHIN CALLTIME

Version 4.40 has a minor improvement that can help reduce the number of

calls you make to the USNO.

When you have the Main Menu Screen showing, tapping the plus [+] or the

minus [-] key will add or subtract one second to the system time.  And if

you have your clock's resident time/date processor loaded, or the SET AT

CLOCK switch 'ON', the MOS clock will be reset.

If you keep track of the time drift of your MOS clock, and find that it

gains say 8 seconds in one week (this is not unusual), every Monday when

you start your system, you can start CALLTIME, then simply tap the minus

key 8 times, and this will reset your clock back 8 seconds.

NOTE: ONLY ATTEMPT THIS IMMEDIATELY AFTER STARTUP OR LOADING THE MOS clock

time into system time.  Otherwise software will change your system time and

your MOS clock will be set to an invalid time.

PROGRAM NOTES on Version 3.6

Version 3.60 was mainly to change address for BRAND Consultants, but the

code was altered to make the modem set-up occur when transferring from the

main menu to one of the dial-up systems.  This makes it easier to change

the Configuration file.  Another new feature is the 'linear' clock display

on the main menu screen.  This will give you an easier method to set your

watches, clocks ...or what-have-you.  You will also note that there is a

'TIC' sound made each second, with a slightly higher pitched tic on the 15,

30 and 45 second intervals.  A long high pitch tic will signal the start or

zero seconds of a new minute.  Pressing the 'T' key will toggle the TICS on

and off, so you won't be driven up the wall.

NOTE: The Version number can be found on the Start-up screen, directly

under the large CALL TIME letters.

PROGRAM NOTES on Version 4.40

Version 4.40 source code is assembly language, and duplicates Version 3.6,

but has many enhancements of it's own.  The capture buffer is limited to

65535 characters.  This version also will re-dial the Automatic Time-Set

phone number up to 10 times, if the line is busy and no connection is made.

An unsuported feature is an attempt to measure the round-trip delay time.

By pressing <F1> when connected to the USNO General Information system,

CALLTIME sends a carriage return (ASCII #13) and measures the delay time in

milliseconds, and displays the result on your screen.  This will not work

on the Automatic Time-Set line, as it doesn't respond to the carriage

return.  The one way delay time will be half of the time displayed on the

screen. (this also includes computer processing time at the other end.)

Version 4.40 will set an Exit Code or 'ERRORLEVEL' for use with Batch

files.  If the time-set is successful, the Exit Code will be Zero (0) or it

will be One (1), if time-set failed, or was not tried.  Refer to your DOS

manual, Batch File section, on use of 'ERRORLEVEL' codes.

An example of this type of batch file would be:

echo off

astclock /r

ct44 /a/r

if errorlevel==1 goto exit

atclk

echo Time Set according to US Naval Observatory

goto exit1

:exit

echo Time Setting Was Unsuccessful

:exit1

TIME ACCURACY NOTES

The time signal as transmitted by the Naval Observatory is offset from

their master time clock by 1.7ms +/- .4ms.  This is very accurate for most

purposes, but there are other problems associated with the final setting of

your PC's system clock.  One of the major obstacles is the time delay

introduced by the telephone system and the time it takes signals to travel

over a given distance.  From Washington DC to East Greenbush, Vermont we

were able to ascertain this transit time in the order of 33 to 34

milliseconds.  This would be by landline connection.  If the telephone

system should switch your call to a satelite, a delay of 200 milliseconds

for one way transmission is possible.  The telephone system can also

transmit part of your call by landline and send the return via satelite,

just to further confuse the issue.

A Remote Digital Loop Back (RDLB) feature is available from the USNO to

determine your roundtrip delay time.  But since there are a wide variety of

modems in use by PC owners, there is no attempt to utilize this in this

program.  Contact us if you have specific needs for this feature.

Assuming that you could determine the transit time from Washington to your

location, the next problem involves the setting of the PC system clock.

The Intel 8253 timer is supplied with a 1.19318 Mhz signal (regardless of

your systems clock pulse or system speed).

The ROM BIOS sets up the three timer/counters with a divisor of 65536,

which results in 18.2 pulses per second or one every 54.94 milliseconds.

These are used to update the system clock, refresh memory, provide disk

timing and timing for sound signals.  With assembly language it is possible

to reset any one or all of these timer counters, but should only be

attempted by very experienced programmers.  ROM BIOS only allows setting of

the system clock to 0.01 seconds (10 milliseconds), however, if we get a

time signal that is on the precise integer of one second (1.000 seconds) it

would be safe to assume that the system clock is set within a few

milliseconds, or at least within 1/2 second (500 ms).  How long it stays

that accurate involves detailed discussion of oscillator crystal accuracy,

phase-lock loops and other technical details which we will omit here.

Also, here again with proper programming, it would be possible to create a

system clock to replace the ROM-BIOS/DOS routines that would provide a

clock with greater accuracy.

Using CALLTIME you can expect your computer time to be set well within plus

or minus 1 second of the Naval Observatory clock, which should be accurate

enough for most people.

MODEM SWITCHES

Modem switches probably have caused more problems in the communications

field than any other single item.  Fortunately late model modems have done

away with most of the hardware switches and rely on software and modem

setup commands to configure modems correctly.

In order to be compatible with the greatest variety of modems, CALLTIME

reads the modem status register rather than words or numbers returned by

the modem, but the modem status register is affected by the modem switches.

On older modems, there are two switches that can cause minor problems with

CALLTIME, as neither can be overridden with software commands.  Refering to

the Hayes 1200 modem, these are switches #1 and #6.  For CALLTIME to

operate properly both should be UP.  This is the 'normal' setting for most

communications done with Bulletin Boards etc.

If Switch #1 is down the TR light on the modem will come on as soon as the

modem is turned on and stay on indefinitely.  CALLTIME will be unable to

hang-up, and you will have to turn the modem off to make it release the

telephone line.

If Switch #6 is down, CALLTIME will think that there is a carrier present

as soon as it has completed dialing, and will attempt to start looking for

time signals before the USNO modem has answered the call.  You will see

this as the communications screen, with the words 'Connected to the U.S.

Naval Observatory' in the top of the screen, will appear immediately after

dialing is complete, even before the USNO phone rings.  Usually this

presents no problem and everything works OK, but if for some reason the

USNO modem 'hangs' and doesn't answer the call, CALLTIME will 'timeout'

after 60 seconds and hangup, indicating received data was bad.

REGISTRATION

To become a registered user of this program, and receive new updates and

any new information regarding it's use, send $25.00 to:

Call Time Registration

Brand Consultants

P.O. Box 310

East Greenbush, NY 12061

We are always interested in your feedback, and will make every effort to

make additions and revisions according to your suggestions.

NOTE: Price quoted above is in effect as of Jan 1, 1990. If this comes to

you more than 1 year after this date, check for possible price changes

before sending for your registration.

COPYRIGHT NOTICE

Copyright 1987 thru 1990 by Bruce D. Anderson.  All Rights Reserved. This

publication and enclosed software has been provided pursuant to a

restricted agreement concerning it's use.  No part of either publication

may be disclosed to third parties by any means, or copied by any means, or

transmitted by any means without the express written permission of BRAND

Consultants, P.O. Box 310,  East Greenbush, NY 12061.

Failure to include this notice, or alteration of this document or any files

associated with these programs will result in revocation of license, and

can mean prosecution under the Copyright laws of the United States.

DISCLAIMER

Bruce D. Anderson dba BRAND Consultants makes no representation or

warranties with respect to the contents hereof and specifically disclaims

any warranties, however made, implied or otherwise, as to suitability,

merchantability or fitness for any particular purpose.  BRAND Consultants

reserves the right to revise this publication and associated software, and

make changes to the content hereof without obligation to notify, upgrade or

replace either product for any and all licensees of these products.

CALLTIME ORDER/REGISTRATION FORM

---------------------------------

To: BRAND Consultants

P.O. Box 310

East Greenbush, NY 12061

Date:____________

YES !    I want to know what time it is !    Please send me:

_____ Copies of CALLTIME, Version 4.40, @ $25.00 each.

I use:   ___ 5-1/4" 360k disk      ___ 3-1/2" 720k disk

AND/OR

_____ Copies of CALLTIME Pro, @ $35.00 each.

I use:   ___ 5-1/4" 360k disk      ___ 3-1/2" 720k disk

Enclosed is my check/money order for:     $ ________

Please ship to:

____________________________________

____________________________________

____________________________________

____________________________________

(Please note: - prices above are in effect as of Jan 1, 1990)