MNP COMMANDS

The modem control commands can be divided in several categories:

I. STANDARD HAYES COMMANDS (See mor-doc file for more detail)

A/	Repeat last command

A	Answer

Bn	CCITT/Bell 1200 bps operation

&Cn	DCD control

D	Dial

DS=n	Dial stored number

&Dn	DTR control in asynchronous mode

En	Command mode echo

&Fn	Restore factory defaults

&Gn	Select guard tone

Hn	Hook control

In	Identification

&Jn	Select jack

&Ln	Dial-up/Leased line

Ln	Speaker volume control

Mn	Speaker control

&M	Async/sync mode selection

On	Go back to on line state

P	pulse dial

&Pn	Dial pulse ratio

Qn	Disable result codes

&Rn	RTS/CTS control

Sn	Select S register

=	Set S register

?	Display S register

&Sn	DSR options

T	Tone dial

&Tn	Tests

Vn	short/long results codes

&V	show configuration

&Wn	Store configuration in EERAM

&Xn	Synchronous clock source

Xn	Call progress mode and connect messages

Zn	Recall EERAM configurations

&Zn=	Save phone number in EERAM

II. MICROCOM ADDITIONAL COMMANDS - GENERAL PURPOSE

\Bn	Transmit break to remote system

D/n	Dial stored number. Similar to DS=n. Added for MICROCOM

DL	Dial last number

%En	enable/disable auto retrain

\F	read stored phone numbers (subset of &V)

\Kn	Set break control

\Pn	Store phone number

%R	Display S registers

\S	Read modem status

%V	Display firmware version number (same as I3)

III. MICROCOM COMMANDS - MNP COMMANDS

\An	Maximum MNP block size

%An	Set Auto-Reliable Fallback character

\Cn	Set auto-reliable Buffer

%Cn	Enable/disable data compression (class 5)

\En	Set Data Echo in normal mode

\Gn	Set modem port flow control during NORMAL connection

\Jn	DTE interface Speed Adjust

\Nn	Set operating mode

\O	Originate reliable link

\Qn	Set DTE port flow control

\Tn	Inactivity Timer

\U	Accept reliable link

\Vn	Enable MNP result form

\Xn	Set XON/XOFF pass through

\Y	Switch to reliable

\Z	Switch to normal mode

All the commands in groups II and III, and some commands in group

I,  are documented here. It would be also useful to refer to  the

Microcom   "AX/1200  and  AX/2400  Series  Owner's  Manual"   for

additional details.

The  first  part  of the  documentation  specifies  each  command

individually.The  second  part contains  some  explanatory  notes

about  the  modem.The MNP S registers are specified in  the  last

section of the documentation.

DESCRIPTION OF THE COMMANDS

I. MODIFIED HAYES COMMANDS

&Fn	Recall factory configuration

Description

&F0	Recall Hayes factory default

&F1	Recall Microcom factory default

&F2	Recall CompuCom factory default (auto-reliable)

Action

if n = 0, recall configuration 0

if n = 1, recall configuration 1

if n = 2, recall configuration 2

if n > 2, say ERROR

Comments

&F0 is recalled at power-up. &F1 and &F2 is recalled only if  &F1

or &F2 commands are issued.

&Tn	Test

Description

&T0	stop test

&T1	local analog loop back

&T3	local digital loop back

&T4	enable response to digital loopback

&T5	disable response to digital loopback

&T6	initiate a remote digital loopback

&T7	RDLB with self test

&T8	local analog loop back with self test

any other parameter will return "ERROR"

&T1 and &T8 say ERROR if:

- not &M0

- already in test

- \N2 or \N3

&T3, &T6 and &T7 say ERROR if:

- not connected (therefore &M0 must be active for &T3/6/7 to work).

- already in test

- the connection is reliable

&M1, &M2, &M3 say ERROR if any test running (&T1/8/6/7/3).

&Wn	Store Current Configuration

Description

if n>=1, n=1. Store current configuration in EERAM conf. n.

Store registers:

S0, S14, S18, S21, S22, S23, S25, S26, S27 (hayes registers)

S36, S37, S38, S39, S40, S41 (MNP registers)

Stored parameters:

\A %A B \C &C &D E \E %E \G &G \J \K &L M &M \N P &P Q \Q &R T \T

&T4 &T5 V \V X \X S0 S18 S25 S26

Action Comments

Note that if the DTE speed is 110 bps, a speed of 300 bos is saved.

Y	Long space disconnect

Description

This  command was kept for compatibility with Hayes.  It  returns

"OK"  in  any  case but the command has an  effect  only  if  the

connection is direct.

ATY0 : ignore received long space

ATY1  : send a break for 4 seconds and hang-up if 1.6s  of  space

detected.

II. GENERAL PURPOSE ADDITIONAL COMMANDS

\Bn	Transmit break to remote system

Description

Send  a  scrambled  space to the  remote  system,  under  certain

conditions.

Action

* If not connected, say "NO CARRIER", do not send break.

* Else ...

D/n	Dial stored number.

Description

Dial  stored phone number specified by n. "n" must  be  comprised

between 0 and 3, otherwise "ERROR" is returned.

n  must  be  the last character in the  command  line,  otherwise

"ERROR" is returned.if n is omitted, n=0 is assumed

Action

(CMDBUF_PTR points to the character following "/")

If (CMDBUF_PTR = #BOTBUF+1) N=0;	/* end of buffer */

AL = [CMDBUF_PTR]

CMDBUF_PTR++;

If (AL = 0) N=0;			/* end of line */

Else If (AL < '0' or AL > '3') goto ERROR;	/* invalid */

Else If (CMDBUF_PTR!=#BOTBUF+1 AND [CMDBUF_PTR]!=0) goto ERROR;

Else N=n

Display NV_STORED[n];

Dial NV_STORED[n];

Comments

This  command is not a standard Hayes command. It was  added  for

compatibility  with  MICROCOM.  It is similar to  DS=n,  but  not

identical:  in  particular  DS=n will  ignore  the  command  line

following  the  n, D/n will return "ERROR" if n is not  the  last

character. The ATDS= command is still valid.

Differences with MICROCOM implementation:

MICROCOM modems allow 10 phone numbers, numbered from 1 to 10.

We only allow 4 phone numbers, numbered from 0 to 3.

DL	Dial last number

Description

Dial  last number (displayed on 2nd line of AT\S command).  Since

the  last  dialed number is only saved in RAM, it  is  not  saved

between  power-up. This command was added for compatibility  with

Microcom.

The  "L" must be the 1st character after "D", and any  characters

following the L are ignored.

%En	enable/disable auto retrain

Description

%E0	disable auto-retrain

%E1	 enable auto-retrain at 2400 bps when the signal  quality

is poor.

Action

If n>=1, n=1

S16.7 = n

Comments

S16.7 is checked in file DSP.ASM.

\F	read stored phone numbers

Description

If no ERAM present, send "NO NV MEM"

Else display the 4 phone numbers stored in ERAM.

No  parameter is needed. So AT\F0 will execute AT\F, then  return

ERROR because 0 is not a valid command

Comments

The code used is a subset of AT&V code. It was added for MICROCOM

compatibility

\Kn	Set Break Control

Description

Determines the type of action taken by the modem when a break  is

received from the DTE port or the modem port.

Action

If n>5, n=5

S40.7 6 5 = n

Comments

When breaks are received from the DTE or the modem line, or  when

a  "send  break"  command is issued, the  program  checks  the  S

register to take the appropriate action.

\Pn	Store phone number

Description

syntax:

AT..\P <CR>	equivalent to AT..\P0<CR>

AT..\Pn {optional string} <CR>

n  must be 0,1,2 or 3. Any other value following \P  will  return

"ERROR" (except for the AT..\P<CR> case, which assumes n=0).

The  valid  dial characters in the string are  stored  in  EERAM,

until the end of the command line, or 30 characters are stored.

Action

(CMDBUF_PTR points to the character following "\P")

If (CMDBUF_PTR=#BOTBUF+1) N=0	/* end of buffer */

AL = [CMDBUF_PTR]	/* read next char */

CMDBUF_PTR++		/* point to next */

if (AL=#0) N=0	/* end of line */

Else if (AL<'0' or AL>'3') ERROR	/* invalid parameter */

Else {	/* valid parameter */

N = AL - '0'

Store in NV_STORED[N]

}

Comments

This  command  is  Similar  to  Hayes &Z=  .  It  was  added  for

compatibility with Microcom, and the Hayes command AT&Z= is still

valid.

Unlike Microcom, only the valid dial characters in the string are

stored,  until 30 characters are stored. If the string  has  more

than 30 characters, the rest of the string is ignored.

Microcom allows phone numbers between 1 and 9, we allow between 0

and 3.

%R	Display Hayes S registers

Description

This  command  is provided for compatibility  with  Microcom.  It

displays registers S0 to S41 in hex and decimal format. The value

displayed  for  non existing registers (for example  S13)  is  0.

Example:

REG  DEC HEX    REG  DEC HEX    REG  DEC HEX    REG  DEC HEX

S00  000 00H    S01  000 00H    S02  043 2BH    S03  013 0DH

S04  010 0AH    S05  008 08H    S06  002 02H    S07  030 1EH

S08  002 02H    S09  006 06H    S10  014 0EH    S11  075 4BH

S12  050 32H    S13  000 00H    S14  170 AAH    S15  000 00H

S16  128 80H    S17  000 00H    S18  000 00H    S19  000 00H

S20  000 00H    S21  000 00H    S22  118 76H    S23  059 3BH

S24  000 00H    S25  005 05H    S26  001 01H    S27  064 40H

S28  000 00H    S29  000 00H    S30  003 03H    S31  054 36H

S32  020 14H    S33  157 9DH    S34  000 00H    S35  000 00H

S36  001 01H    S37  000 00H    S38  000 00H    S39  000 00H

S40  000 00H    S41  000 00H

Comment

No  parameter  is  read,  so AT%R0  will  read  %R,  display  the

registers, then read 0 and send an "ERROR" message.

\S	Read modem status

Description

Display current setting of the modem.

Action

We  don't  get  rid of eventual parameter  (so  \S0  will  return

"ERROR").  The  display  is identical to  the  Microcom  display,

except  for the options not implemented yet. The current  display

is as follows:

DIRECT	000:05:12

LAST DIAL	(408)2639300

MODEM BPS	2400	AT

MODEM MODE	DIR	AT\N1

MODEM FLOW	NONE	AT\G0

AUTO ANS.	OFF	ATS0=000

DTE BPS	2400	AT

BPS ADJUST	ON	AT\J1

DTE FLOW	OFF	AT\Q0

PASS XON/XOFF	OFF	AT\X0

PARITY	8N	AT

BREAK	5	AT\K5

EXIT CHAR	043	ATS2=043

COMMAND ECHO	ON	ATE1

RESULTS	ON	ATQ0

RESULT TYPE	LONG	ATV1\V0

DATA ECHO	OFF	AT\E0

INACT TIMER	000	AT\T000

AUTO RETRAIN	ON	AT%E1

COMPRESSION	OFF	AT%C0

MAX BLK SIZE	64	AT\A0

AUTO BUFF	0	AT\C0

AUTO CHAR	000	AT%A000

PAUSE TIME	002	ATS8=002

DTR	1	AT&D1

CARR DET	1	AT&C1

SPEAKER CTRL	1	ATM1

LEASE LINE	OFF	AT&L0

ASYNC/SYNC	0	AT&M0

CTS/RTS	ON	AT&R0

RDLB ENABLE	ON	AT&T4

DIAL MODE	3	ATX3

PULSE DIAL	US	AT&P0

GUARD TONE	0	AT&G0

BELL	OFF	ATB0

Comments

The  LAST  DIAL number is saved in RAM (not in EERAM), so  it  is

cleared when the power is turned off.

The  first  line  displays  the type  of  connection:  IDLE  (not

connection),  DIRECT,  NORMAL, RELIABLE  MNP4  (data  compression

disabled), RELIABLE MNP5 (data compression enabled).

The  time  displayed on the first line is  the  time  (HHH:MM:SS)

elapsed  since  the beginning of the current connection  (if  the

modem is on line), or the duration of the last connection (if the

modem is off line).

Note  that  if the S registers are set manually to  some  invalid

value  (example, S23.7,6=11), the \S display will show the  error

(i.e. GUARD TONE   3   AT&G3).

%V	Display firmware version number

Description

This  command  was  added for  compatibility  with  Microcom.  It

displays  the  same  message  as  I3  command.  No  parameter  is

expected, AT%V0 will display the identification message, then say

"ERROR".

III. MNP COMMANDS

\An	Maximum MNP block size

Description

\A0	block size max =  64 characters

\A1	block size max = 128 characters

\A2	block size max = 192 characters

\A3	block size max = 256 characters

\An, n>3 have the same effect as \A3.

Action

If n >= 3, assume n=3

S40.1,0 = n

%An	Set Auto-Reliable Fallback character

Description

Set  the  character  recognized as the  auto-reliable  fall  back

character  when  in answer mode and when option \C2  is  selected

(see \C command).

If n>=128, then 0 is assumed.

Action

If n>=128, n=0

S37 = n

\Cn	Set auto-reliable Buffer

Description

Determines  how  the  modem handles the data  received  from  the

remote modem, during the time interval in which it is  attempting

to  establish a reliable connection and only if it is  in  ANSWER

mode.  This option only takes effect at the end of  the  training

period  when  &M0  (async  mode)  and  \N3  (auto-reliable)   are

selected.

\C0   data  is  not buffered. if the remote modem is  a  non  MNP

modem,  any data sent by it during the 4 second wait period  will

be lost.

\C1  data buffered. If 200 characters buffered, switch to  normal

mode  and  pass  data  to the DTE. If  SYN  detected,  switch  to

reliable mode.

\C2   data  not buffered. If character defined  by  %A  received,

switch to normal mode, pass the character to the DTE

Action

n>2 => n=2

S38.1,0 = n

Comments

This  is  valid only if the modem is in  ANSWER  mode.  Originate

modems  ignore the \Cn setting and data is never buffered  during

the MNP handshake period.

%Cn	Enable/Disable Class 5

Description

%C0	Disable MNP5

%C1	Enable MNP5

Action

If n>1, n=1

S41.0 = n

Comments

If  MNP5  EQU  0 (in DEFINE.ASM), the code  is  assembled  for  a

SC11013  chip and MNP5 is not allowed. In that case, %C1  command

will return "ERROR" and data compression will always be disabled,

even if S41.0 is set "manually".

\En	Set Data Echo in normal mode

Description

\E0	Don't echo data sent by local DTE in normal data mode

\E1	Echo data sent by local DTE in normal data mode

Only valid if data mode is normal. Other data modes disable data

echo.

Action

n>2 => n=1

S38.3 = n

Comments

valid only in NORMAL mode

\Gn	Set modem port flow control during NORMAL connection

Description

Sets the type of flow control used to pace the data received from

the  remote modem (only in NORMAL mode. In direct mode,  no  flow

control  is  used.  In reliable mode, a special  scheme  of  flow

control is used and \Gn setting is ignored).

\G0	no flow control

\G1	XON/XOFF flow control

Action

if n>1 => n=1

Set S36.2 = n

\Jn	DTE interface Speed Adjust

Description

\J0	Don't change the speed of the serial port.

\J1	Change the speed of the serial port to match the bps rate

of the connection.

\Jn, n>1 behaves as \J1

Action

if n>1, set n=1

S38.2 = n

Comment

This  option is examined after the end of the MNP handshake:  the

"connect ..." message is always sent at the previous speed.  Then

if the speed of the connection is different from the speed of the

serial  port  and if \J1 is selected, the speed of  the  UART  is

changed.

In direct mode (\N1 selected), this option is ignored, it  always

behaves as \J1.

\Nn	Set operating mode

Description

Control  the type of connection when asynchronous mode  (&M0)  is

selected.

\N0	normal mode (non MNP, buffered)

\N1	do not use on SPEEDMODEM

\N2	reliable mode (MNP)

\N3	auto-reliable mode (MNP if possible, otherwise, normal)

Action

If n >= 3, assume n=3

S36.1,0 = n

Comments

Do  not  use during an active connection.  This  command  doesn't

affect the setting of &M command (S27.1,0):

After dialing, if &M0 is selected (S27.10=00), then the \N option

will  take effect, and the modem will eventually attempt  an  MNP

connection.  If  an asynchronous mode is selected, \N  option  is

ignored, and a synchronous connection (non MNP) is made.

Also, the MNP light is only turned on when a reliable  connection

is  established.  It  is not turned on as the  result  of  a  \Nn

command.

\O	Originate Reliable Link

Description

Initiate a reliable link, when the modem is in a direct or normal

connection.  If  the remote modem doesn't respond, go  to  normal

mode  (the remote modem must issue a AT\U command to establish  a

reliable connection).

*  If  the command was issued when not connected, NO  CARRIER  is

returned.

* If the modem was already in reliable mode, when AT\O is issued,

it goes back on line, immediately (without waiting for a response

from the remote modem).

* The rest of the command line (following \O) is ignored.

\Qn	Set DTE port flow control

Description

\Q0	disable DTE flow control

\Q1	XON/XOFF bidirectional flow control

\Q2	CTS unidirectional flow control

\Q3	CTS/RTS bidirectional flow control

When \Q1 is selected, the modem can tell the DTE to stop  sending

data by sending an XOFF character to the DTE. The modem will send

an XON character to tell the DTE to resume transmission. The  DTE

will tell the modem to stop sending data to the DTE by sending  a

XOFF  character,  and  to resume sending data by  sending  a  XON

character.

When \Q2 is selected, the modem uses CTS line to control the data

coming  from the DTE. The DTE has no way to control the  flow  of

data coming from the modem.

When \Q3 is selected, the modem uses CTS line to control the data

coming from the DTE, The DTE uses RTS line to control the flow of

data coming from the modem has no way to control the flow of data

coming from the modem.

(See flow control documentation)

Action

If n>=3, n=3

S36.7,6 = n

Comments

Simple  command.  This  command can be sent  when  the  modem  is

online,  by  returning  to  the  command  mode  using  an  escape

sequence. The new option will take effect as soon as returning to

data mode (using ATO for example).

\Tn	Inactivity Timer

Description

Defines  the number of minutes the modem waits before  hanging-up

when  no  data  is  sent  or  received  in  normal  or   reliable

connections.

\T0	disable the inactivity timer

\Tn	n = number of minutes. 0<n<=90

Action

If n>=90, n=90 (limit n to 90)

S39 = n

Comments

The  MNP  program is in charge of the counter and of  taking  the

appropriate action.

\U	Accept Reliable Link

Description

Accept  a reliable link, when the modem is in a direct or  normal

connection.  If  the remote modem doesn't respond, go  to  normal

mode  (the remote modem must issue a AT\O command to establish  a

reliable connection).

*  If  the command was issued when not connected, NO  CARRIER  is

returned.

* If the modem was already in reliable mode, when AT\U is issued,

it goes back on line immediately (without waiting for a  response

from the remote modem).

* The rest of the command line (following \U) is ignored.

\Vn	Enable MNP result form

Description

\V0	use standard Hayes result codes

\V1	use Microcom connect messages, in normal or MNP connect.

MNP connection

DTE speed	Long form		Short form

300		CONNECT 0300/REL	20

1200		CONNECT 1200/REL	22

2400		CONNECT 2400/REL	23

4800		CONNECT 4800/REL	24

9600		CONNECT 9600/REL	25

normal connection

DTE speed	Long form		Short form

300		CONNECT			1

1200		CONNECT 1200		5

2400		CONNECT 2400		10

4800		CONNECT 4800		11

9600		CONNECT 9600		12

(The  DTE speed here, is after eventual BPS adjust, depending  on

\Jn  option.  See  additional  documentation  below  about  speed

issues.)

Action

if n>=1, n=1

S38.4 = n

Comments

This option is checked by CON_M_MRQ routine in MNPIOCS.ASM.

\Xn	Set XON/XOFF pass through

Description

This command takes effect in normal and reliable connections.

\X0	 If  XON/XOFF  flow control  is  disabled,  the  XON/XOFF

characters  are  always passed through as regular  data.  If  \Q1

(XON/XOFF  DTE  flow  control) is  enabled,  XON/XOFF  characters

received  from the DTE are not sent to the remote system. If  \G1

(modem  flow  control) is selected, XON/XOFF  received  from  the

remote system is not sent to the local DTE.

\X1	 XON/XOFF  characters are always passed through  in  both

directions  (from  local  DTE to remote system  and  from  remote

system to local DTE). Action is taken if flow control is  enabled

(\Q  for DTE flow control or \G for line flow control  in  normal

mode).

Any parameter bigger than 1 produces the same effect as \X1.

(See flow control documentation for more details)

Action

if n>=1, n=1

S36.3 = n

Comments

This  command can be sent when the modem is online, by  returning

to the command mode using an escape sequence. The new option will

take effect when going back to data mode (using ATO for  example,

or at next connection).

\Y	Switch to reliable mode

Description

Initiate (if the modem dialed) or accept (if the modem  answered)

a  reliable  link,  when  the modem is  in  a  direct  or  normal

connection.  If  the remote modem doesn't respond, go  to  normal

mode  (the remote modem must issue a AT\Y command to establish  a

reliable connection).

*  If  the command was issued when not connected, NO  CARRIER  is

returned.

* If the modem was already in reliable mode, when AT\Y is issued,

it goes back on line immediately (without waiting for a  response

from the remote modem).

* The rest of the command line (following \Y) is ignored.

\Z	Go from reliable to normal mode

Description

*  If  the command was issued when not connected, NO  CARRIER  is

returned.

* If the modem was in direct mode, it stays in direct mode.

* If the modem was in normal mode, it stays in normal mode.

* The rest of the command line (following \Z) is ignored.

NOTES

1. FLOW CONTROL

DO NOT USE XON/XOFF FLOW CONTROL WITH THE SPEEDMODEM USE ONLY

HARDWARE FLOW CONTROL,  RTS/CTS.

The  flow control settings (\Gn, \Xn, \Qn) can be changed  during

an active connection by escaping to command mode (using +++), and

issuing  the desired command. The new flow control settings  will

take place after returning in data mode (using ATO).

\Xn COMMAND

The  \Xn  option  is used to decide what happens  with  the  flow

control characters when flow control is enabled.

When \X0 is selected:

* DTE flow control: if DTE flow control is not selected, XON/XOFF

received  from  the  local DTE are treated as  regular  data  and

therefore  sent  to  the remote modem. If  DTE  flow  control  is

selected  (  &M0  async mode + Normal  (\N0)  or  reliable  (\N2)

connection  + \Q1 option ) when receiving XON/XOFF  flow  control

characters  from the local DTE, the modem takes  the  appropriate

action (stop or resume sending data to the DTE), but the XON/XOFF

characters are not sent to the remote modem.

* Modem port flow control: if modem flow control is not selected,

XON/XOFF  received from the remote system are treated as  regular

data  and therefore sent to the local DTE. If modem flow  control

is  selected  ( &M0 async mode + Normal (\N0)  connection  +  \G1

option ) when receiving XON/XOFF flow control characters from the

remote  system, the modem takes the appropriate action  (stop  or

resume  sending  data  to the remote system),  but  the  XON/XOFF

characters are not sent to the local modem.

When \X1 is selected:

* The XON/XOFF characters received from the local DTE are sent to

the remote system.

Then  if XON/XOFF DTE flow control is enabled ( &M0 async mode  +

Normal  (\N0)  or reliable (\N2) connection + \Q1  option  )  the

modem  stops sending characters to the DTE after  receiving  XOFF

from  DTE,  and  resumes  sending characters  to  the  DTE  after

receiving XON from the DTE.

*  The  XON/XOFF characters received from the remote  system  are

sent to the local DTE as regular data.

Then, if modem XON/XOFF flow control is enabled ( &M0 async  mode

+  Normal \N0 connection + \G1), the modem stops sending data  to

the remote system when XOFF is received over the phone line,  and

resumes sending data when XON is received over the phone line.

In conclusion, the way \Xn works is shown in the table below  (we

suppose a normal or reliable connection ).

XON/XOFF received from local DTE

\Q0		: pass to remote system through phone line

\Q1	\X0	: do not pass to remote system

\X1	: pass

XON/XOFF received from the remote system over the phone line

\G0		: pass to local DTE

\G1	\X0	: do not pass

\X1	: pass to local DTE

When \X1 is selected, XON/XOFF characters are always passed through.

When \X0 is selected, XON/XOFF characters are not passed  through

if  the  corresponding flow control is enabled; they  are  passed

through  if  the  corresponding flow control  is  disabled.  (see

figure).  Note that \G option has no effect on characters  coming

from local DTE; \Q option has no effect on characters coming from

the remote system.

NOTE:  RIGHT  NOW \G1\X0 WILL PASS THE CHARACTER.  THIS  WILL  BE

CHANGED LATER ON.

2. AUTO-RELIABLE BUFFER: \Cn AND \Jn COMMAND

In  auto-reliable mode (\N3), after the end of the  DSP  training

sequence  (CCITT  V.22  or  V.22  bis),  the  modem  attempts  to

establish an MNP "reliable" connection with the remote modem.  If

a  reliable connection is not possible (because the remote  modem

doesn't support it, or because the line is extremely noisy),  the

modem  falls back to a normal or a direct connection  (see  table

below). This process can take a few seconds.

The remote modem might start sending data before the local  modem

is  ready (for example if the remote modem doesn't support  MNP),

that  is, before the local modem has finished the  MNP  handshake

and decided to fall back to normal mode. Option \Cn decides  what

is going to happen to these characters received before the end of

the MNP handshake: (see flow charts)

If  \C0  is  selected,  the  characters  are  not  buffered.  Any

characters  sent by the remote modem before the local  modem  has

gone into data mode are lost. They are not sent to the local DTE.

If \C1 is selected, the data is buffered in the ANSWERING  modem.

If  the  buffer  fills  up  before  the  reliable  connection  is

established, the local modem goes into normal mode and passes the

contents  of  the buffer to the local DTE. The local  modem  also

goes  into  normal  mode if 4 seconds  elapse  without  making  a

successful MNP reliable connection.

If \C2 is selected, the data is not buffered. The modem  attempts

to  establish  a reliable connection for 4 seconds,  but  if  the

character defined by %A command (S37) is received from the remote

modem,  it switches immediately to normal mode. %A  character  is

recognized only by ANSWERING modems.

Note: the description below relates to the data received from the

remote modem over the phone line. The data from the local DTE  is

never  buffered  (  if a key is hit on the  local  terminal,  the

handshake is aborted.). %Cn setting is only valid in ANSWER mode.

In  ORIGINATE  mode,  the received data  is  never  buffered,  %A

characters  are  not recognized, and \Cn setting is  ignored:  it

makes sense because it is assumed that the answer modem won't try

to type before the originate modem.

When \N3 is selected, the final type of connection depends on the

remote modem and on options \C and \J. This is summarized in  the

following  table:  (the table describes the state  of  the  local

modem). The table is valid for ORIGINATE and ANSWER mode modems.

\N0, \N1	\N2, \N3   (remote modem)

-----------------------------------

|     normal       |     normal     |

\N0 |    ignore \C     |   ignore \C    |

|                  |                |

-----------------------------------

| direct           | direct         |

\N1 | ignore \C \J     | ignore \C \J   |

|                  |                |

-----------------------------------

|                  |                |

\N2 |  no connection   |      MNP       |

|                  |                |

-----------------------------------

|\C1,\C2: norm     |                |

\N3 |\C0: \J0 dir      |      MNP       |

|     \J1 norm     |                |

-----------------------------------

(local modem)

3. SPEED ISSUES: DTE SPEED ADJUST, CONNECT MESSAGE, HANG-UP

DTE speed adjust

SPEEDMODEM NOTE...DTE SPEED SHOULD BE FIXED AT 19,200 BPS, NO AUTOBAUDING

When  establishing  a connection over the modem port,  the  final

type  of connection established depends on various factors,  such

as  the  type  of modem involved, the speed of each  of  the  DTE

connected to each modem, the condition of the phone line,  etc...

The final speed of the modem connection might not match the speed

of the DTE:

For example, suppose the local DTE is 2400 bps and the remote DTE

is 1200 bps. After establishing a normal or reliable  connection,

the  final speed of the connection will be 1200 bps (figures  [1]

and [2] ).

If  option \J0 is selected on the local modem (figure  [3]):  the

DTE port speed stays at 2400 bps. Flow control between the  modem

and the DTE will most likely be needed since the DTE will be able

to send data to the modem much faster than the modem will be able

to send them to the remote system.

If \J1 is selected (figure [4]), after the end of the  handshake,

the  modem will change the speed of the DTE port to 1200 bps.  If

the  DTE speed is not changed also to 1200 bps, the DTE  will  be

unable  to  communicate with the modem. The DTE will have  to  be

changed  to  1200. This will be done manually most of  the  time,

unless you are using a software that recognizes speed changes and

adjusts automatically.

Note

\Jn option is only valid for normal or reliable connection. In  a

direct  connection,  the  speed  of the  serial  port  is  always

adjusted,  regardless of the \Jn setting. It makes sense: if  not

buffering  and  flow control is possible, the speed  of  all  the

elements of the connection must be the same.

At hang-up, the old speed is never restored, except if \N1\J0  is

selected.

Using parallel modems:

If  \J1 is selected, the modem writes directly to the  UART,  and

there  is  no  need to give the "change  speed"  command  to  the

communication  software.  Note  that  in  that  case,  the  speed

displayed  by the communication software might not correspond  to

the actual speed of the DTE port.

Connect message

The message sent after a connection was established depends on  a

lot of factors as shown on the flow chart:

1)  If the connection is in direct mode, the messages  depend  on

the modem speed, which is also the DTE adjusted speed:

DTE adjusted BPS	direct connection

= modem speed

300		CONNECT

1200		CONNECT 1200

2400		CONNECT 2400

2) if X0 is selected, the message is always "CONNECT", no  matter

what.

3)  If  \V0  is selected, we use  the  standard  Hayes  messages,

depending on the modem speed:

modem speed	message

300		CONNECT

1200		CONNECT 1200

2400		CONNECT 2400

4) If \V1 is selected the message will reflect the DTE speed:

- If \J1 is active, the DTE speed is adjusted to match the  modem

speed (300, 1200 or 2400):

DTE adjusted bps	normal conn.	MNP connection

= modem speed

300		CONNECT	CONNECT 0300/REL

1200		CONNECT 1200	CONNECT 1200/REL

2400		CONNECT 2400	CONNECT 2400/REL

-  If  \J0 is active, the connect message will  reflect  the  DTE

speed (the modem speed can be different)

DTE speed	normal	MNP

300		CONNECT	CONNECT 0300/REL

1200		CONNECT 1200	CONNECT 1200/REL

2400		CONNECT 2400	CONNECT 2400/REL

4800		CONNECT 4800	CONNECT 4800/REL

9600		CONNECT 9600	CONNECT 9600/REL

Restoring UART speed at hang-up

For  normal and MNP connection, the previous UART speed is  never

restored:  if \J1 is selected and the UART speed was adjusted  at

the beginning of the connection, it stays the same after hang-up.

If \J0 is selected, the DTE speed is not adjusted, and  therefore

it never needs to be restored.

For direct connections: the DTE port speed is always adjusted, as

said  above.  But:  if  \J1 is active, the  DTE  speed  is  never

restored. If \J0 is active: the speed is restored if the  hang-up

occurs  when  not in command mode (you don't want to  change  the

port speed all of the sudden, if the user is in command mode).

For  synchronous  connections (&M1, &M2, &M3): the DTE  speed  ia

always restored.

4. BREAK COMMANDS

3 things to distinguish between:

1. Break received from local DTE

2. Break received from remote modem

3. send break command \B

1.a Break received from local DTE, during a direct connection

\K0,\K2,\K4	go into command mode

\K1,\K3,\K5	no action

In  a direct connection, break (scrambled space) will be  treated

as normal data, and therefore will be sent to the remote modem.

This is not implemented right now.

1.b Break received from local DTE, during a normal or reliable con.

\K0,2,4	Go to command mode (don't send break to remote)

\K1		Empty buffers and send break

\K3		Send break immediately

\K5		Send break in sequence with TX data

2.a Break received from remote modem during a normal connection

\K0,\K1	Empty buffers, send break to DTE

\K2,\K3	Send break immediately to DTE

\K4,\K5	Send break in sequence with RX data

2.b Break received from remote modem during direct  connections:

the action depends on Yn option (long space disconnect

3.a  Send  break  (AT\B) command, during  a  reliable  or  normal

connection

\K0,\K1	Empty buffers, send break to remote system

\K2,\K3	Send break immediately to remote system

\K4,\K5	Send break in sequence with TX buffered data.

3.a Send break (AT\B) command, during a direct connection

just send a break (there is no buffering problem here).

Comments

How do all the modes work ? In particular, what's the  difference

in 2.a between \K0,\K1 and \K4,\K5 ?

How do the other modes work ?

Need to add and document 2 functions for detecting break from DTE

and break from modem port5.

FACTORY SETTINGS AND EERAM

Three  factory  configurations are now  available:  AT&F  command

accepts  a  parameter: n=0, 1, 2. If n>2, ERROR is  sent.  If  no

parameter  is given, n=0 is assumed, this  ensures  compatibility

with the previous version. Factory configuration 0 is recalled at

power-up  if  no  EERAM  is  present,  or  using  command  AT&F0.

Configuration  1  and  2 are recalled by  using  commands  AT&F1,

AT&F2.

At  the first power-up (new EERAM), the EERAM configuration 0  is

loaded with factory configuration 0, and EERAM configuration 1 is

loaded with factory configuration 1.

&F0 - Hayes default. The modem behaves like a regular Hayes modem.

&F1  -  Microcom default. This is the default  configuration,  as

defined by Microcom.

&F2  - Auto-reliable configuration. This is a configuration  that

should suit most uses of MNP4/MNP5.

AT&F0 : "HAYES DEFAULT"

This  is the factory default as defined by Hayes: it is  recalled

at power-up, if no EERAM is present. S0 to S27 are as defined  by

hayes. The MNP registers are set to disable MNP operation.

Hayes registers

S0	0	disable auto-answer

S1	0	Ring count

S2	43	Escape code character '+'

S3	13	Carriage return

S4	10	Line feed

S5	8	Backspace

S6	2	Wait for dial tone

S7	30	Wait for carrier

S8	2	Pause for ',' dial

S9	6	Carrier detect response time

S10	14	Lost carrier to hang-up delay

S11	75	DTMF time (not Hayes)

S12	50	Escape code guard time

S14	0AAH	E1, Q0, V1, no dumb, Pulse dial, Originate

S16	80H	Enable auto-retrain

S18	0	Test timer

S21	20H	&J0, &R0, &D0, &C1, &S0, Y0

S22	76H	L2, M1, X4, &P0

S23	7	2400 bps, Even, &G0

S25	5	Delay to DTR

S26	1	RTS/CTS Delay

S27	40H	&M0, &L0, &X0, B1

MNP registers

S36	01H	\N1 (direct mode connection)

S37	00H	N/A

S38	00H	\J0 (restore DTE speed), \V0 (hayes results)

S39	00H	N/A

S40	00H	N/A

S41	00H	N/A

AT&F1 : "MICROCOM DEFAULT"

In  this  mode, the Hayes registers S0 to S27 are as  defined  in

&F0. The MNP registers are different:

MNP registers

S36	01H	\N1, \G0, \X0, \Q0

S37	00H	%A0

S38	24H	\C0, \J1, \E0, \V0

S39	00H	\T0

S40	A3H	\A3, \K5

S41	01H	%C1, &C0		if MNP5 EQU 1

00H	%C0, &C0		if MNP5 EQU 0

AT&F2 : MNP, AUTO-RELIABLE CONFIGURATION

The Hayes registers S0 to S27 are the same. The MNP registers are

as  follows:  auto  reliable mode is selected,  flow  control  is

enabled. The fall back character is set to a Carriage Return.

S36	47H	\N3, \G1, \X0, \Q1

S37	0DH	%A13

S38	32H	\C2, \J0, \E0, \V1

S39	00H	\T0

S40	A3H	\A3, \K5

S41	01H	%C1, &C0		if MNP5 EQU 1

00H	%C0, &C0		if MNP5 EQU 0

\C1	auto-reliable buffer

\G1	enable modem flow control

\J0	disable bps adjust

\N3	Auto-reliable mode

\Q1	XON/XOFF DTE flow control

\V1	enable MNP result codes

OPERATIONAL MODES - SUMMARY

(These  modes apply only if asynchronous mode is selected,  using

AT&M0.  If  &M1, &M2 or &M3 are selected, the modem  will  behave

like a regular Hayes synchronous modem)

\N0 - NORMAL MODE

If  \N0 is selected, the modem will only make normal  connections

(as opposed to \N3 setting, which can result in direct, normal or

reliable connections, depending on various factors).

For  normal  connections, flow control and  buffering  are  used.

There  is no MNP error correction. The relevant settings are  the

following (in order of importance):

\Gn  -  Modem port flow control : this setting is valid  only  in

normal mode.

\Qn - DTE port flow control

\Xn - XON/XOFF pass through

\Jn  - BPS rate adjust : \J0, the DTE port speed is  not  changed

when the connection is established, flow control might be needed.

\J1, the DTE port speed is adjusted to match the modem bps.

\En  - Data echo : this setting is only valid in normal mode.  E0

disables  the echo-ing of the data sent by the local DTE in  data

mode, E1 enables the echo.

\Kn - Break control

\Vn  - Connect message : if \V0 is used, the messages  are  Hayes

compatible  and reflect the modem speed (CONNECT,  CONNECT  1200,

CONNECT  2400).  If \V1 is used, the messages  will  reflect  the

(eventually  adjusted)  DTE speed: CONNECT  0300,  CONNECT  1200,

CONNECT 2400, CONNECT 4800, CONNECT 9600.

\Tn - Inactivity timer : (for now, only works in reliable modes)

%Dn - Disconnect buffer delay : (not implemented yet).

\N1 - DO NOT USE WITH SPEEDMODEM

COMMUNICATION SOFTWARE

(ABOUT FOREIGN LANGUAGES...)

Suppose the communication package you use is set to 8 data  bits,

no parity: if the remote DTE uses parity, the parity bit will  be

considered as the 8th data bit by your communication package, and

the  8  bit character sent to the terminal will appear  as  Greek

letters and other strange characters.

For ASCII only connections and file transfers:

If  the  local communication package is set to 8 data  bits,  the

remote communication package should also be set to 8 data bits (7

bits + space parity will also work). If the setting of the remote

modem  is unknown, or if it cannot be changed, you must set  your

local comm. package to 7 data bits.

General file transfers:

If  you want to be able to transfer binary files, both local  and

remote communication packages should be set to 8 data bits.

HOW TO TRANSFER FILES

USE OF COMMUNICATION PACKAGES:

NEW S REGISTERS

S36 REGISTER - FLOW CONTROL AND TYPE OF CONNECTION

bit 1,0  \Nn command

0 0	\N0	normal mode

0 1	\N1	direct (default)

1 0	\N2	reliable

1 1	\N3	auto-reliable

Bit 2	\G command

0	 \G0	 no modem flow control  for  normal  connections

(default)

1	\G1	use XON/XOFF flow control during normal connection

bit 3	\X command

0	\X0	don't pass through XON/XOFF (default)

1	\X1	pass through XON/XOFF

bit 5,4	Status of the connection

0 0	Normal	(no MNP buffered

0 1	Direct	(no MNP, non buffered)

1 0	MNP4 Reliable

1 1	MNP5 data compression

Note that these bits are meaningful only in asynchronous mode. If

a  synchronous connection is established, or if no connection  is

established, they must be ignored.

bit 7,6	DTE flow control

0 0	\Q0	no flow control (default)

0 1	\Q1	XON/XOFF bidirectional flow control

1 0	\Q2	CTS unidirectional flow control

1 1	\Q3	RTS/CTS bidirectional flow control

S37 REGISTER - AUTO-RELIABLE FALL BACK CHARACTER

bits 7 to 0

n	%An  n (between 0 and 127) will be recognized as the fall

back character in auto-reliable (\N3) mode. (default = 0). Bit  7

is always 0.

S38 REGISTER - MISCELLANEOUS OPTIONS

bits 1,0 \Cn command - Set Auto-reliable buffer

0 0	\C0 Do not buffer data (default)

0 1	\C1 Buffer data for 4 seconds

1  0	 \C2  do not buffer data. Switch  to  normal  mode  when

character define by %An option is received.

1 1	not used

bit 2	\Jn - BPS adjust

0	\J0	disable bps adjust

1	\J1	Enable bps adjust (default)

bit 3	\En - Set data mode echo

0	\E0	don't echo in data mode (default)

1	\E1	echo in data mode, if non reliable

bit 4	\Vn - set MNP result codes

0	\V0	disable MNP result codes (default)

1	\V1	Enable MNP results

bit 5,6	free

bit 7

(reserved. Break request)

S39 REGISTER - INACTIVITY TIMER

S40 REGISTER - MISCELLANEOUS OPTIONS

bits 1,0 - \A command : maximum MNP block size for stream link con.

0 0	\A0	max block size = 64  characters  (default)

0 1	\A1	max block size = 128 characters

1 0	\A2	max block size = 192 characters

1 1	\A3	max block size = 256 characters

bits 2,4,3 free

bits 7,6,5 \K command - break control

0 0 0	\K0

0 0 1	\K1

0 1 0	\K2

0 1 1	\K3

1 0 0	\K4

1 0 1	\K5	(default)

S41 REGISTER - MISCELLANEOUS OPTIONS

bit 0	%Cn command

0	%C0	disable MNP5

1	%C1	enable MNP5 (default)

#################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################################