Enclosed you will find a number of different text files. You may print these

files if you wish, though I believe you will find the most important commands

and information within the printed manual that comes with your CompuCom modem.

Please note: Roms after 4/1/91 no longer accept or use the init +F for fax

send disable. If you have this in your modem init string, and your rom or

modem is 2.61 or installed after 4/1/91, please remove this init.

Modem.doc

Modem.doc includes more detailed information about the modem settings,

including what the default settings are, and what these settings do. If for

example you wished to have more information about a setting such as how

the modem reacts to DTR (Data Terminal Ready), you could consult this

document for more information.

Result_C.ode

This file includes the numeric result codes that the modem returns IF

you have V0 in your modem initialization string. Certain Bulletin

boards require numeric result codes (Such as Wildcat!) in their setup.

This document is useful for this purpose, but is generally not needed

for setting up most com programs or bbs's. The default setting for

result codes is verbal, which is fine for most com programs or bbs's.

Speedbbs.txt

This file includes a listing of Speemodem equipped bbs's. This file

is constantly being updated, and the most recent version is always

available on the CompuCom Support bbs at 408-738-4990, or the So.

California System (The General_CompuCom) at 619-281-2622. If you

wish to add or change a listing, please leave a comment to sysop

on the CompuCom Support bbs. Please encourage your sysop

(or yourself, if you are a sysop), to obtain the various echo's

or conferences relating to the CompuCom modems. These include Fido

Net (SPEEDMODEM), RelayNet (RIME) (SPEEMODEM #107), and Network

(COMPUCOM_SPEEDMODEM). These message conferences are frequented

by many CompuCom Sysops and users, and are very helpful for obtaining

news and other information about your CompuCom modem.

Mnp.doc

Mnp, or Microcom Networking Protocol, is a feature included in your Compucom

modem. Mnp2 through Mnp4 are error correcting protocols built into the modem,

which enable you to connect with other Mnp equipped modems with hardware

error correction. In doing this, you can use file transfer protocols such

as Ymodem -G (batch), 1K-Xmodem -G, Zmodem, and various other protocols

to obtain faster transfers. When you have an Mnp connection, your modem will

do the error correction itself, thereby allowing you to use these fast

protocols to their best efficiency. In order to take full advantage of Mnp

error correction, you need to use the recommended settings in the printed

manual (IE: AT&F2*H2&D2 etc etc etc.), Lock (Set) your com program or

bulletin board com port speed at 19,200 baud, Lock (Set) your dialing

directory entries to 19,200 baud, and turn on Hardware flow control (IE:

CTS/RTS Enabled, Xon-Xoff Disabled). This will allow your modem to communicate

faster with the other Mnp modem then a regular 2400 baud modem! Mnp.doc

includes descriptions of the Mnp commands in detail, though the recommended

settings in the printed manual have proven to be very effective in most any

situation. Mnp5 is a Data Compression protocol, which is very useful in

transfering uncompressed files between Mnp5 equipped modems. If you are

transfering compressed (.ZIP, .ARC, .LZH, .ARJ, .ZOO, Etc) files, you will

probably wish to disable Mnp5 with the %C0 command in your setup string.

Mnp5 does not help, and actually can slow down the transfer of compressed

files.

Hayescmd.doc

This file includes a brief description of the Standard Hayes Command sets,

including what the "X" registers, "S" registers, and other commands actually

are for. The printed manual includes recommended settings, though your com

program or bbs may require specialized commands. This document would be useful

in the event you need to change any of the default settings. The Hayes command

set is consistant between most modems today, though many many modems use

different default settings. These settings are set as soon as you turn on

your computer, and may require modification to suit your needs. When you

turn on your CompuCom modem, it is setup in Non-Mnp, Standard 2400 mode.

You will probably want to consult the printed manual to change the settings

to enable Mnp and High Speed Modes.

Speed_up.asc

This is the product brochure file. You may wish to post this file

on yours or other bbs's. It includes certain technical information

about the modem, including specifications.

*.TXT, CompuCom.mdm, Qmodem.mdf, etc..

Included are some sample setups for various com and bbs programs. Also

included are two modem configuration files. Qmodem.mdf is for Qmodem

Quick config, version 42+, CompuCom.mdm is for Mustang Software's Wildcat!

2.xx. The sample setup files end in .TXT, and more are being added all

the time. Please call The CompuCom Support Bbs at 408-738-4990, or

The General CompuCom BBS at 619-281-2622 for Updated Files. You may download

from either bbs on your first call.

Sysopfrm.txt

This is the Sysop Application form. If you are a sysop, and wish to request

the Sysop discount special for the Speedmodem Combo, Please fill out and

return this file to CompuCom.

Other Information..

Please note the following helpful tips and information...

If you have other serial I/O devices in your computer (such as other

modems, serial port cards, scanner interfaces, VGA cards that use IRQ's,

Network cards, etc), You will have to make sure that these devices do

not "Conflict" with the Com Port you wish to use for your Speedmodem.

As it comes out of the box, Dos recognizes 4 com ports (1-4). You will

need to check your other devices to see which Com Port addresses or IRQ's

they use, to make sure they are different. This is true with most any

High Speed Modem, and can cause many problems at any speed!

When calling another CompuCom Speedmodem or Champ Modem, you may use

AT*H1 in your setup or dial string to increase throughput. This will

increase your throughput up to 10% when using Zmodem, but is *Not*

recommended when receiving calls (BBS's), or when you have noisy lines.

Some BBS's Require Fossil Drivers. The Speedmodem and Champ have been

tested with BNU, X00, FDCD, OpusCom And Others. Please make sure you

lock the port at 19,200, and use the proper IRQ and Address (If Req'd).

If you are having problems with your Speedmodem or Champ, Please type

ATI3 from any Com Program and write down your Version Number. When

you call Customer Support, you should have this version number, the

approximate date you bought your modem, The type of bbs or com program

that you are having problems with, along with the setups you are using.

This will speed up the process of helping you a lot! Most of the

difficulties experienced involve Setup Strings and/or Com Port Conflicts,

and are remedied over the telephone quickly! Most of the problems seem

to be involving the proper setups for Hardware Flow control, and

Initialization strings.

Rev. 041491

CompuCom Corp. Customer Service Line 408-733-3290, Fax 408-732-4570.

All rights Reserved, 1990, CompuCom Corp. 1180-J Miraloma Way, Sunnyvale,CA.

Wildcat! is a Copyright of Mustang Software, Zmodem is Omen Technologies,

Qmodem is John Friel, MNP is Microcom Networking Protocol.