============= 2400B HAYES MODEM COMMANDS LIST =============

Here is a list of the S-registers.

ALL S REGISTER COMMANDS ARE ENTERED IN THE AT COMMAND LINE.

EXAMPLE: AT S8=2 S2=43 S7=30

TO READ THE VALUE IN ANY REGISTER USE

AT SX? <CR>

THE MODEM SHOULD ECHO THE VALUE FOUND IN THE REGISTER X

REG  Values      Register function

S0   0           AUTO ANSWER OFF

1-255       AUTO ANSWER ON, ANSWER ON INDICATED # OF RINGS

S1 **0-255       RING COUNT (THIS IS A READ-ONLY REGISTER)

S2   0-127       ESCAPE CHARACTER  NORMALLY SET TO 43 OR "+"

127-255     ESCAPE FUNCTION DISABLED

S3  0-127        END OF LINE CHARACTER  NORMALLY SET TO 13 (CR)

S4  0-127        LINE FEED CHARACTER  NORMALLY SET TO 10 (LF)

S5  0-127        BACKSPACE CHARACTER  NORMALLY SET TO 8 (BS)

S6  2-255  SEC   PAUSE BEFORE DIALING (NORMALLY 2 SEC)

S7  1-255  SEC   WAIT FOR CARRIER (NORMALLY SET TO 30 SEC)

S8  0-255  SEC   PAUSE FOR COMMA IN DIAL STRING (NORMALLY SET TO 2 SEC)

S9  1-255  .1SEC CARRIER VALIDATION TIME NORMALLY 6 (.6 SEC)

S10 1-255  .1SEC LOSS OF CARRIER DISCONNECT DELAY NORMALLY 14 (1.4 SEC)

S11 50-127 .1SEC TOUCH TONE DURATION. THE DEFAULT IS 90.

S12 20-255 .02SE ESCAPE SEQENCE PAUSE  NORMALLY 50 (1.0 SEC)

S13 *            POWER UP ASYNC DATA FORMAT (USE CHART BELOW)

S14 *            OPTION STATUS DEFULT AA (HEX CODE) (USE CHART BELOW)

S16 *            DEFULT 0. TEST STATUS (USE CHART BELOW)

S18 0-255 SEC    DEFULT 0. TEST TIMER (REMOTE TEST)

S21 *            DEFULT 0.OPTION STATUS  (USE CHART BELOW)

S22 *            OPTION STATUS, DEFULT 76. (USE CHART BELOW)

S23 *            OPTION STATUS  NORMALLY 07 (USE CHART BELOW)

S25 0-255 SEC    DTR DETECT DELAY NORMALLY 5 SEC

ASYNC MODE, SYIC AUTO-DIAL AND SYIC MANUAL DIAL MODE .05 SEC

S26 0-255 MS     RTS TO CTS DELAY (1 MS)

S27 *            OPTION STATUS, DEFULT 40 (HEX CODE)  (USE CHART BELOW)

* REGISTERS USED BY MODEM PROCESSOR. DO NOT ALTER THESE

** READ ONLY REGISTERS

S13 DATA FORMAT:

POSITION  VALUE  FUNCTION

BIT 0      -    NOT USED

BIT 1      -    NOT USED

BIT 2      0    PARITY OFF

1    PARITY ON

BIT 3      0    ODD PARITY

1    EVEN PARITY

BIT 4      0    7 DATA BITS

1    8 DATA BITS

BIT 5      0    BIT 8=0

1    BIT 8=1

BIT 6      -    NOT USED

BIT 7      -    NOT USED

S14 DATA FORMAT:

BIT 0      -    NOT USED

BIT 1      0    LOCAL CHARACTER ECHO OFF

1    LOCAL CHARACTER ECHO ON

BIT 2      0    STATUS DISPLAYS ON

1    STATUS DISPLAYS OFF

BIT 3      0    STATUS CODES SENT

1    STATUS MESSAGES SENT

BIT 4      0    MODEM RESPONDS TO COMMANDS

1    MODEM IGNORES COMMANDS

BIT 5      0    TONE-DIALING

1    PULSE-DIALING

BIT 6      -    NOT USED

BIT 7      0    ANSWER MODE

1    ORIGINATE MODE

S16 DATA FORMAT:

BIT 0      0    ANALOG LOOP OFF

1    ANALOG LOOP ACTIVE

BIT 1      -    NOT USED

BIT 2      0    DIGITAL LOOP OFF

1    DIGITAL LOOP ACTIVE

BIT 3      0    REMOTE DIGITAL LOOP REQUESTED BY OTHER MODEM IS OFF

1    REMOTE DIGITAL LOOP REQUESTED BY OTHER MODEM IS ACTIVE

BIT 4      0    REMOTE DIGITAL LOOP OFF

1    REMOTE DIGITAL LOOP ACTIVE

BIT 5      0    SELF TEST RDL OFF

1    SELF TEST RDL ACTIVE

BIT 6      0    SELF TEST ANALOG LOOP OFF

1    SELF TEST ANALOG LOOP ACTIVE

BIT 7      -    NOT USED

S21 DATA FORMAT:

BIT 0      0    RJ11/RJ41/45 JACK TYPE

1    RJ12/FJ13 JACK TYPE

BIT 1      -    NOT USED

BIT 2      0    CTS FOLLOWS RTS

1    CTS FORCED ON

BIT 3&4    0    MODEM IGNORES DTR

1    MODEM ASSUMES COMMAND STATE WHEN DTR TURNS OFF

2    MODEM ASSUMES COMMAND STATE AND DISABLES AUTO-ANSWER

WHEN DTR TURNS OFF

3    MODEM RESETS WHEN DTR TURNS OFF

BIT 5      0    DCD FORCED ON

1    DCD FOLLOWS CARRIER

BIT 6      0    DSR FORCED ON

1    DSR BEHAVES NORMALLY

BIT 7      0    LONG SPACE DISCONNECT DISABLED

1    LONG SPACE DISCONNECT ENABLED

S22 DATA FORMAT:

BIT 0&1    1    LOW SPEAKER VOLUME

2    MEDIUM SPEAKER VOLUME

3    HIGH SPEAKER VOLUME

BIT 2&3    0    SPEAKER ALWAYS OFF

1    SPEAKER ON UNTIL CARRIER DETECTED

2    SPEAKER ALWAYS ON

3    SPEAKER OFF DURING DIALING THEN ON UNTIL CARRIER DETECTED

BIT 4&5&6  0    X COMMAND IN EFFECT

4    X 1 COMMAND IN EFFECT

5    X 2 COMMAND IN EFFECT

6    X 3 COMMAND IN EFFECT

7    X 4 COMMAND IN EFFECT

BIT 7      0    39/61 MAKE/BREAK RATIO (PULSE DIALING)

1    33/67 MAKE/BREAK RATIO

S23 DATA FORMAT:

BIT 0      0    REMOTE DIGITAL LOOP REQUEST DENIED

1    REMOTE DIGITAL LOOP REQUEST GRANTED

BIT 1&2    0    0-300 BPS

1    600 BPS

2    1200 BPS

3    2400 BPS

BIT 3      -    NOT USED

BIT 4&5    0    EVEN PARITY

1    PARITY BIT ALWAYS 0

2    ODD PARITY

3    PARITY BIT ALWAYS 1

BIT 6&7    0    NO GUARD TONES

1    550 HZ GUARD TONE

2    1800 HZ GUARD TONE

S27 DATA FORMAT:

BIT 0&1    0    ASYNCRONOUS MODE

1    ASYNC/SYNC MODE

2    SYNC AUTO-DIAL

3    SYNC MANUAL DIAL

BIT 2      0    PUBLIC PHONE LINE

1    LEASED LINE

BIT 3      -    NOT USED

BIT 4&5    0    INTERNAL CLOCK USED

1    EXTERNAL CLOCK USED

2    SLAVED CLOCK USED

BIT 6      0    CCITT V.22 PROTOCOL (1200 BPS)

1    BELL 212A (1200 BPS)

BIT 7      -    NOT USED

OTHER MODEM COMMANDS:

AT       STANDARD LEAD IN SEQUENCE (ATTENTION)

ATA      FORCE ANSWER

ATD      DIAL (ATD ###-#####)

P     PULSE (ATDP ###-####)

T     TONE  (ATDT ###-####)

ATE1     ENABLE LOCAL CHARACTER ECHO

ATE0     DISABLE LOCAL CHARACTER ECHO

ATH1     FORCE LINE CONNECT

ATH0     FORCE LINE DISCONNECT

ATI      ECHO VERSION TYPE

ATL1     SPEAKER VOLUME LOW

ATL2     SPEAKER VOLUME MEDIUM

ATL3     SPEAKER VOLUME HIGH

ATM0     SPEAKER ALWAYS OFF

ATM1     SPEAKER ON EXCEPT WHILE CARRIER PRESENT

ATM2     SPEAKER ALWAYS ON

ATM3     SPEAKER ON EXCEPT WHILE DIALING AND CARRIER PRESENT

ATO      GO ONLINE AFTER ESCAPE OR WHILE IN COMMAND MODE

ATQ0     STATUS DISPLAY ON

ATQ1     STATUS DISPLAY OFF

ATV0     STATUS MESSAGES SENT

ATV1     STATUS CODES SENT

ATX0     "CONNECT" (CODE 1)  ALL SPEEDS NO DIAL TONE DETECTION

ATX1     "CONNECT" (CODE 1)  0-300 BPS OR "CONNECT 1200" FOR 1200 OR

"CONNECT 2400" FOR 2400 BPS NO DIAL TONE DETECTION

ATX2     WAIT FOR DIAL TONE (CONNECT SPEEDS SHOWN AS FOR X1)

ATX3     DETECT BUSY SIGNAL (CONNECT SPEEDS SHOWS AS FOR X1)

ATX4     WAIT FOR DIAL TONE, DETECT BUSY SIGNAL (CONNECT SPEEDS SHOWS AS FOR X1)

ATZ      RESET TO POWER UP CONDITION (CLEARS ALL REGISTERS)

2400 MODEM SPECIAL COMMANDS:    (* = FACTORY SETTING STANDARD)

ATB0     CCITT V.22 AT 1200 BPS

ATB1     BELL 212A AT 1200 BPS *

AT &F    RESTORE TO FACTORY C0NFIGURATION

AT &G    NO GUARD TONE *

AT &G1   550 HZ GUARD TONE

AT &G2   1800 HZ GUARD TONE

AT &J    RJ11/RJ41S/RJ45 JACK *

AT &J2   RJ12/RJ13 JACK

AT &L    REGULAR PHONE LINE *

AT &L1   LEASED LINE

AT &M    ASYNC OPERATION *

AT &M1   ASYNC/SYNC OPERATION

AT &M2   SYNC AUTO DIAL

AT &M3   SYNC MANUAL DIAL

AT &P    39/61 PULSE MAKE/BREAK RATIO *

AT &P1   33/67 PULSE MAKE/BREAK RATIO

AT &T4   GRANT RDL TEST REQUEST *

AT &T5   DENY RDL TEST REQUEST

AT &V    VIEW STATUS OF ALL MODEM SETTINGS. SEE ALL S REGISTER SETTINGS AND

INTERNAL MODEM COMMAND SETTINGS ON ONE SCREEN!

AT &X    SYNC CLOCK INTERNAL *

AT &X1   SYNC CLOCK EXTERNAL

AT &X2   SYNC CLOCK SLAVED

AT &Z    STORE DIAL COMMAND LINE

DATA INTERFACE (2400)

AT &C    DCD ALWAYS ON

AT &C1   DCD ON WHILE CARRIER PRESENT *

AT &D    DTR IGNORED *

AT &D1   DTR FAIL DISCONNECT ENABLED (MODEM RETURNS TO COMMAND MODE)

AT &D2   DTR FAIL DISCONNECT ENABLED (MODEM RETURNS TO COMMAND MODE, TURN

AUTO ANSWER OFF)

AT &D3   DTR FAIL DISCONNECT ENABLED (RESET MODEM)