This is general information which applies to ALL CompuCom modems.

Other files may provide additional detail on specific modem models.

COMMAND SUMMARY

AT        ATTENTION COMMAND

A/        REPEAT COMMAND

A         ANSWER MODE COMMAND

B/B0      SELECT CCITT V.22 IN 1200 BPS

B1        SELECT BELL 212A

Dn        DIAL COMMAND (n = DIAL STRING, 0-9, *, #, !, W, R, AND,

E         DISABLE ECHO

E1        ENABLE ECHO

H         ON HOOK (HANG UP)

H1        OFF HOOK

I/I0      IDENTIFY

I1        DISPLAY 000

I2        DISPLAY OK

I3        DISPLAY MESSAGE PROGRAM ROM

L/L0      LOW VOLUME

L1        LOW VOLUME

L2        MEDIUM VOLUME

L3        MAXIMUM SPEAKER VOLUME

M/M0      DISABLE SPEAKER

M1        SPEAKER ON UNTIL CARRIER

M2        SPEAKER ALWAYS ON

M3        DISABLE SPEAKER WHEN DIALING OR CARRIER

O         GO ON LINE

O1        RETRAIN (2400 BPS MODE ONLY)

Q/Q0      ENABLE RESULT CODES

Q1        DISABLE RESULT CODES

Sn=X      S REGISTER WRITE (n = VALID S REG, X = ALLOWED VALUE SEE

&F COMMAND)

Sn?       READ SPECIFIED S REGISTER (n = ANY VALID S REGISTER)

V0        SELECT DIGIT RESULT CODE

V1        SELECT WORD RESULT CODE

X0        BLIND DIAL, DISPLAY "CONNECT"

X1        BLIND DIAL, DISPLAY "CONNECT 1200/2400"

X2        DETECT DIAL TONE, DISPLAY "CONNECT 1200/2400"

X3        DETECT BUSY TONE, DISPLAY "CONNECT 1200/2400"

X4        DIAL AND BUSY TONE DETECT, DISPLAY "CONNECT 1200/2400"

Y0        DISABLE LONG SPACE DISCONNECT

Y1        ENABLE LONG SPACE DISCONNECT

Z         SOFTWARE RESET

+++       ESCAPE CODE (GO TO COMMAND MODE IF ON LINE)

&C0       DCD ALWAYS ON

&C1       DCD FOLLOWS REMOTE CARRIER STATUS

&D0       IGNORE DTR

&D1       GO TO COMMAND WHEN DTR ON TO OFF

&D2       HANGUP AND GO TO COMMAND WHEN DTR ON TO OFF

&D3       POWER UP RESET WHEN DTR ON TO OFF

&F        FETCH FACTORY CONFIGURATION for S REGESTERS

&G0       NO GUARD TONE

&G1       550 HZ GUARD TONE

&G2       1800 HZ GUARD TONE

&J0       SELECT RJ-11/RJ-41S/RJ-45S

&J1       SELECT RJ-12/RJ-13

&M0       SELECT ASYNCHRONOUS MODE

&P0       MAKE BREAK RATIO FOR US (40/60)

&P1       MAKE BREAK RATIO FOR FOREIGN STANDARD (33/66)

&R0       CTS TRACKS RTS

&R1       CTS ON WHEN MODEM READY IN DATA MODE

&S0       DSR ALWAYS ON

&S1       DSR FOLLOWS RE232 SPECIFICATION

&T0       STOP TEST

&T1       LOCAL ALB TEST

&T3       LOCAL DLB

&T        ENABLE DLB REQUEST

&T5       DISABLE DLB REQUEST

&T6       REMOTE DLB

&T7       REMOTE DLB AND SELF TEST

&T8       LOCAL ALB AND SELF TEST

&V        VIEW PROFILE

S-REG   ADDRESS  DEFAULT  VALID RANGE   DESCRIPTION

S0      228      0        0-255         RING TO ANSWER ON

S1      229      0        0-255         RING COUNT

S2      230      43       0-127         ESCAPE CODE CHARACTER

S3      231      13       0-127         CR CHARACTER

S4      232      10       0-127         LF CHARACTER

S5      233      8        0-127         BS CHARACTER

S6      234      2        0-255         WAIT FOR DIAL TONE

S7      235      30       1-99          WAIT FOR DATA CARRIER

S8      236      2        0-255         PAUSE TIME FOR COMMA

S9      237      6        1-255(.1SEC)  CARRIER DET RESP. TIME

S10     238      14       1-255(.1SEC)  LOST CARRIER TO HANG UP

DELAY

S11     239                             TOUCH TONE DURATION

S12     240      50       0-255         ESCAPE CODE GUARD TIME

(1/50SEC)

S13     241                             NOT USED

S14     242      AAH                    BIT MAPPED OPTION REG

S15     243                             NOT USED

S16     244      0                      MODEM TEST OPTIONS

S17     245                             NOT USED

S18     246      0        0-255         TEST TIMER

S19     247                             NOT USED

S20     248                             NOT USED

S21     249      0                      BIT MAPPED OPTIONS REG

S22     250      76                     BIT MAPPED OPTIONS REG

S23     251      7                      BIT MAPPED OPTIONS REG

S24     252                             NOT USED

S25     253      5        0-255(.01SEC) DELAY TO DTR (SYNC ONLY)

S26     254      1        0-255(.01SEC) RTS TO CTS DELAY (HALF

DUP)

S27     255      40H                    BIT MAPPED OPTION REG

2

HOST INTERFACE

This section describes the way in which the host interfaces to the

modem.  The  modem communicates to the host in either the  command

mode  or the data mode. The Model 2400 controls the switching  be-

tween  the  two modes. The staus of the modem is communicated  via

result codes.

COMMUNICATION MODES

The  modem communicates to the host in one of two possible  modes:

the  command  mode or the data mode. Upon powerup or after  reset,

the  modem  performs a selftest. Immediately following this  test,

the  modem  enters the command mode and configures itself  in  the

default  configuration. The modem then enters the autobaud routine

and waits for the attention code 'AT' from the host.

4.1.1  Command Mode

In  the command mode, the user communicates with the modem via the

host  computer.  This  mode  is used for  modem  configuration  or

dialing.  The modem executes local commands from the host computer

and  returns  the  result codes. After the autodialing  and  modem

handshake sequences are completed, the modem enters the data mode,

unless otherwise specified by the dial subcommand.

4.1.2  Data Mode

In  the data mode, the modem receives characters from the host and

processes  the data before transmitting it to the telephone  line.

The  modem  also checks the data from the host for an escape  code

sequence. It does not execute any local commands in the data mode.

The modem returns to the command mode only when it receives an es-

cape code sequence from the host, or one of the data call termina-

tion procedures described in Section  6.5 occurs.

4.2  MODE CONTROL

Upon  powerup,  the Model 2400 modem is in the command  mode.  The

modem  can enter the data mode while connected to another modem or

when  using diagnostics. The escape code is used to return to  the

command  mode.  To return back to the data mode, the ATO  (online)

command is used.

The  escape  code forces the modem to return to the  command  mode

from  the data (online) mode. The escape code character and escape

code guard time are defined by the values set in register S12. The

escape  code  guard time is defined as the time delay between  the

last  character  of  escape  code transcitted and  the  next  data

character. The default guard time is defined as the time delay be-

tween  the last character of escape code transmitted and the  next

data  character.  The  default guard time is one  second  and  the

default  code character is "+". The escape code character must  be

entered  three  consecutive  times.  To execute  the  escape  code

(default values), proceed as followe:

3

1)  Type "+++"

2)  Wait one second before transmitting another command character.

The  modem  returns to the command mode and sends the result  code

"OK".  The  telephone  line connection is retained or  aborted  as

defined  by  the value in register S10. The connection is  aborted

automatically if the carrier signal from the remote modem is lost.

If  the  modem is forced into the command mode from the data  mode

using the escape sequence, the ATO (online) command can be used to

return  to  the  data mode. The modem can be reconfigured  to  the

default state by executing the software reset ATZ command.

4.3  RESULT CODES

Result  codes  are  the  responses  to  commands  that  have  been

executed. Result codes may be selected as English words or numeric

digits.   Word   result   codes  are  followed   by   a   carriage

return/linefeed  sequence; numeric result codes are followed by  a

carriage return only. Table 3-1 lists the result codes.

DIGIT     WORD            RESULT

CODE      CODE

0         OK              Command executed without error.

1         CONNECT         Carrier detected (300 bbs or 1200 bbs if

X0 is set.)

2         RING            Phone is ringing (will answer only if

auto answer is enabled.)

3         NO CARRIER      Carrier lost or never detected.

4         ERROR           Error in command line, line too long, or

invalid character format.

5         CONNECT 1200    Carrier detected at 1200 bbs if X1 set.

6         NO DIALTONE     Displayed if no dialtone is detected

within 5 seconds and X2 or X4 is set.

7         BUSY            Displayed when a busy signal is

detected and Z2 or X4 is set.

8         NO ANSWER       Displayed when "@" dial modifier is used

and phone continues to ring.

10        CONNECT 2400    Carrier detected at 2400 bbs. X1 or X2

must be set.

4.4  DATA TRANSMISSION RATE

The  Model  2400  will  transmit  and receive  data  at  the  rate

specified  by the table below. The transmission rate is determined

at the time of connection by three facrtors:  the data rate of the

data  terminal  connected to the Model 2400 modem; the  speed  and

standard of the remote modem; and whether or not the Model 2400 is

answering or originating the call.

4

ANSWERING MODEM

ORIGINATING

MODEL 2400     BELL 103    BELL 212    V.22        V.22 bis

300         1200        1200        2400

BELL 300       300         300         300**       300**

BELL 1200      1200*       1200        1200        1200

CCITT 1200     1200*       1200        1200        1200

CCITT 2400     2400*       1200        1200        2400

ORIGINATING MODEM

ANSWERING

MODEL 2400     BELL 103    BELL 212     V.22        V.22 bis

300         1200         1200        2400

BELL 300       300         1200         1200         2400

BELL 1200      300         1200         1200         2400

CCITT 1200     300*        1200         1200         2400

CCITT 2400     300*        1200         1200         2400

*  The connect rate shown results when a Model 2400 calls  another

Model  2400. The connect rate may differ when a Model 2400 is con-

necting with a different type of modem.

** Connection will be made if the answering modem is configured to

fallback  to  Bell 103. No connection will be made if fallback  is

set to V.21.

5.  MODEM COMMAND SET

See also HAYESCMD.DOC and MNP.DOC files.

Direct  communication  with the modem is performed using the  "AT"

command  set.  The modem accepts commands from the host when it is

in the command mode only. To issue a command while the modem is in

the data mode, it must be placed in the command mode by issuing an

escape code sequence.

MODEL 2400 "AT" COMMANDS

COMMAND             DESCRIPTION

AT                  COMMAND LINE PREFIX (ATTENTION CODE);

PRECEDES COMMAND LINES EXCEPT +++ (ESCAPE)

AND A/ (REPEAT) COMMANDS.

A/                 RE-EXECUTE LAST COMMAND LINE; (A/ IS NOT

FOLLOWED BY A CARRIAGE RETURN).

A                  OFF-HOOK IN ANSWER MODE.

5

B                  SELECTS CCITT V.22 OPERATION WHEN

COMMUNICATING AT 1200 BPS.

B1  (Default)      SELECTS BELL 212A OPERATION WHEN

COMMUNICATING AT 1200 BPS.

D                  DIAL NUMBER WHICH FOLLOWWD IN THE COMMAND

LINE.

E                  MODEM DOES NOT "ECHO" COMMANDS BACK TO

TERMINAL.

E1  (Default)      MODEM SECHOS COMMANDS BACK TO TERMINAL.

H  (Default)       ON-HOOK (HANG-UP) COMMAND.

H1                 GO OFF-HOOK.

I                  REQUEST PRODUCT IDENTIFICATION CODE.

I1                 PERFORMS CHECKSUM ON FIRMWARE ROM; RETURNS

CHECKSUM.

I2                 PERFORMS CHECKSUM ON FIRMWARE ROM; RETURNS

OK OR ERROR.

M                  SPEAKER OFF.

M1  (Default)      SPEAKER ON UNTIL CARRIER DETECTED.

M2                 SPEAKER ALWAYS ON.

M3                 SPEAKER ON UNTIL CARRIER DETECTED EXCEPT

DURING DIALING.

O                  RETURN TO ON-LINE STATE.

O1                 RETURN TO ON-LINE STATE AND INITIATE RETRAIN.

Q  (Default)       MODEM RETURNS RESULT CODES.

Q1                 MODEM DOES NOT RETURN RESULT CODES.

SRT=N              SET REGISTER R TO VALUE N.

SR?                REQUEST CONTENTS OF REGISTER R.

V                  SHORT FORM NUMERIC RESULT CODES.

V1  (Default)      FULL WORD RESULT CODES.

X                  300 BPS COMPATIBILITY MODE; CONNECT RESULT

6

CODE ENABLED.

X1                 MODEM BLIND DIALS; ALL CONNECT XXXX RESULT

CODES ENABLED; BUSY SIGNAL NOT DETECTED.

X2                 MODEM WAITS FOR DIAL TONE BEFORE DIALING;

ALL CONNECT XXXX RESULT CODES ENABLED; MODEM

SEND BUSY RESULT CODE IF BUSY DETECTED.

X3                 MODEM BLIND DIALS; ALL CONNECT XXXX RESULT

CODES ENABLED; MODEM SENDS BUSY CODE IF BUSY

DETECTED.

X4  (Default)      MODEM WAITS FOR DIAL TONE BEFORE DIALING;

ALL CONNECT XXXX RESULT CODES ENABLED; BUSY

SIGNAL DETECTED.

Z                  LOAD STORED CONFIGURATION PROFILE.

&C                 DCD ALWAYS ON.

&C1  (Default)     DCD ON INDICATES PRESENCE OF DATA CARRIER.

&D   (Default)     MODEM IGNORES DTR.

&D1                MODEM ASSUMES COMMAND STATE WHEN ON-TO-OFF

TRANSITION DETECTED ON DTR.

&D2                MODEM HANGS UP & ASSUMES COMMAND STATE.

AUTO ANSWER NOT DISABLED UNLESS EXTERNALLY

IMPLEMENTED BY (RI- # DTR-)= A6-.

&D3                MODEM ASSUMES INITIALIZATION STATE UPON

DETECTING AN ON-TO-OFF TRANSITION ON DTR.

&F                 LOAD FACTORY CONFIGURATION PROFILE.

&P  (Default)      PULSE DIAL MAKE/BREAK RATIO = 39/61 (UK/HK).

&P1                PULSE DIAL MAKE/BREAK RATIO = 33/67 (UK/HK).

&S                 DSR ALWAYS ON.

&S1                DSR OPERATES IN ACCORDANCE WITH EIA RS-232-C

SPEC.

&T                 TERMINATE TEST PROGRESS.

&T1                INITIATE LOCAL ANALOG LOOPBACK TEST.

&T8                INITIATE LOCAL ANALOG LOOPBACK WITH SELF TEST.

&Z                 STORE TELEPHONE NUMBER.

7

MODEL 2400 "AT" COMMAND DEFINITIONS

A     Go off hook and answer phone immediately. The Model 2400

immediately goes off hook, transmits the answer tone and

waits for a carrier from the remote mode.

Commands entered after the A command are ignored.

A/    Repeat previous command. The last command executed by the

Model 2400 will execute again. This command does not require

the "AT" prefix or <cr> termination.

Bn    Bell/CCITT mode select.

B0 - Selects CCITT V. 22 operation at 1200 bps.

B1 - Selects BELL212A operation at 1200 bps (Default).

Most 1200 bps modems in the United States operate using the

BELL212A specification. In order to communicate with a modem

conforming to the international CCITT V.22 standard, issue

the B0 command before dialing.

Dn    Dial the number immediately following this command where

n = digits 0 through 9 together with the dial modifiers

listed below.

The Model 2400 will dial the number specified in the manner

determined by the dial modifiers. The dial modifiers may

appear anywhere within the dial string with the exception

of the ; modifier which must appear at the end of the

string.

Dial Modifiers:  P  Pulse dial

T  Tone dial

R  Originate call in answer mode

W  Wait for dial tone before dialing

,  Pause before dialing next digit

;  Return to command mode after dialing

@  Wait for quiet answer before dialing

!  Flash-hook

S  Dial the number stores in RAM

EXAMPLE:  ATDP9WT713,SS;

In this example the Model 2400 pulse dials a 9 digit, waits

for a dial tone, them tome dials the digits 713. Next, the

modem pauses the length of time specified in register S8. It

them dials the number stored in the Model 2400 controller's

RAM which can be up to 32 digits in length. Finally, the

modem returns to the command mode after the final digit has

been dialed.

The P modifier causes the modem to pulse or rotary dial at a

fixed rate of 10 pulses per second. (The make/break ratio

for pulse dialing is determined by the &P command.)

8

DIAL MODIFIERS

P                   PULSE DIAL

T                   TONE DIAL

,                   PAUSE

!                   FLASH

@                   WAIT FOR SILENCE

W                   WAIT FOR SECOND DIAL TONE

;                   RETURN TO COMMAND STATE AFTER DIALING

R                   ORIGINATE CALL IN ANSWER MODE

D                   DIAL STORED NUMBER

6.  DIALING

The  Model 2400 originates a call by dialing a remote modem, using

either  pulse or DTMF (dual-tone multifrequency) dialing depending

on  the dial subcommand issued by the user. The tandem mode can be

used  in call progress dialing to switch between pulse and tone in

the middle of a dial sequence.

Dialing can be performed in one of two possible modes:

1)  Call  progress mode where the modem monitors the  progress  of

the call, or

2)  Blind mode where the modem does not monitor the call  progress

signal after dialing is completed.

When  a  dial  command  is entered, the  modem  places  itself  in

originate  mode  unless  reverse  mode dialing  is  specified.  If

reverse  mode dialing is specified, the modem reconfigures  itself

in the answer mode when dialing is completed.

If  call  progress  mode  is enabled, the modem  will  detect  the

presence  of  a  dial tone before proceeding with  the  dialing  o

numbers. When call progress mode is disabled, the modem uses blind

dialing  in  which the numbers are dialed without waiting for  the

presence  of a dial tone. The modem is also capable of  overriding

its  dialer,  thus  allowing  manual  dialing  using  an  external

telephone  handset.  After dialing, the modem waits for an  answer

tone from the remote modem.

6.1  CALL PROGRESS DIALING

Once  the dialing is entered, the modem places itself off hook and

looks  for the dial tone indicate signal. It assumes tone  dialing

(unless pulse dialing is requested) and dials the entered number.

9

If  a "W" is encountered while numbers are being dialed, the modem

searches  for another dial tone indicate signal before  proceeding

with  the remainder of the dialing sequence. When the dialing  se-

quence  is complete, the modem searches for a remote ring  signal.

If this signal is detected, it them searches for an answer carrier

tone. The modem waits for an originate carrier tone if the reverse

mode is selected (see "R" command).

When  the  carrier  tone is detected, the modem is placed  in  the

originate  mode  and completes the modem handshake  sequence.  The

modem is then automatically placed in the data mode.

6.2  BLIND DIALING

Blind  dialing is similar to call progress dialng except that  the

call  progress signals are disabled and ignored by the modem. Once

offhook,  the  modem  delays  the dialing  sequence  by  the  time

specified  in status register S5 (wait for dial tone). If a "+" is

encountered  in the dialing sequence, it is ignored. To wait for a

second dial tone (for PBX systems), a comma "," (pause subcommand)

must be inserted between the numbers to be dialed. After the dial-

ing  sequence is complete, the modem searches for an answer  tone.

The  remainder  of the sequence is identical to the call  progress

dialing modem handshake sequence, described in Section 6.1.

6.3  AUTO ANSWER MODE

When   the  auto-answer  configuration  is  selected,  the   modem

automatically  answers the phone after "n" rings. The parameter  n

is  set using the command "Sr=n". The value in status register  S0

controls  the  number of rings that must occur  before  answering.

After  the  telephone  is answered, the modem  waits  two  seconds

(billing  delay) during which the modem sends only an answer  car-

rier  tone  to the originating modem, and then proceeds  with  the

call  establishment  handshake  as  specified  by  the  respective

protocol.  After  the  handshake is completed, the modem  sends  a

result  code  to  the host (unless disabled) and enters  the  data

mode.

If the "A" command is used and status register S0 is set to 0, the

modem answers the phone without wating for a ring.

6.4  DATA CALL TERMINATION

Connection may be teminated by either the host or the remote modem

in the following ways:

Data  Terminal Ready.....When the Data Terminal Ready (DTR) signal

goes  low  for more than 50 ms, the modem begins a disconnect  se-

quence  (except for certain test modes). The modem then returns to

the command mode.

Carrier  Loss.....If the carrier signal is lost for more than  300

ms, the modem returns to the command mode. The telephone line con-

10

nection is automatically broken and the modem placed onhook if the

command mode is re-entered as a result of a carrier loss.

Abort.....If the carrier signal is not detected within a specified

time  interval  after  dialing,  the modem goes  onhook  and  then

returns to the command mode.

6.5  HANDSHAKE PROTOCOL

Modem handshake protocols for Bell 103J, Bell 212A, V.22, and V.22

bis   are   specified   by   the   respective   Bell   and   CCITT

recommendations.  After  the handshake sequence is  complete,  the

modem  automatically enters the data mode. An exception occurs  if

the modem has been given the command to return to the command mode

after dialing.

11