DropDTR.COM

Version 1.0

Memory-resident DOS utility for

quickly disconnecting a modem

AUTHOR:  Pete Petrakis, Everett, Washington (CIS 76555,1175)

DESCRIPTION:    DropDTR is  a  memory-resident program  that will

instantly hang  up a  properly configured  Hayes-compatible modem

when you press a certain key combination.   The serial port to be

controlled  (ports  1-4)  is  designated   when  the  program  is

installed.  DropDTR occupies about 5.8  kbytes of DOS memory when

installed and  is removed  from memory  by a  second run using  a

command line switch.   Its  usefulness to anybody  remains to  be

seen.

OPERATION

Configuring the Modem

DropDTR will not  hang up a  modem unless the modem  is

configured  to follow  the  data  terminal ready  (DTR)

signal.     This  adjustment  is  made  with  software-

configurable  modems  by entering  terminal  mode  in a

communications  program and  typing,  AT&D2&W.   Modems

that are  not  configurable through  software  must  be

configured  to follow  DTR  by  setting an  appropriate

physical switch (consult your modem manual).

Installing the Program

To install DropDTR type the following at the DOS prompt

and press return:

dropdtr n

where n is the  number of the serial port  to which the

modem is attached.   The program can work with  port 1,

2, 3, or  4, but once installed it will  work only with

the port you designated when you loaded it.

Hanging Up a Modem

To disconnect  the modem, hold  down the Ctrl  and left

Shift keys and press H (Ctrl-LShift-H).

Removing the Program from Memory

To remove DropDTR from memory type the following at the

DOS prompt:

dropdtr /u

As with  any memory-resident program, DropDTR cannot be

removed from  memory  if one  or  more  memory-resident

programs were loaded after  it; the subsequent programs

have to be removed first.

Known Incompatibility

I am aware of only one situation where DropDTR will not

work   properly.     If   DropDTR   is  invoked   in  a

communications  program   that  is  loaded   under  the

WordPerfect  Library, there  will  be  a system  crash.

This  has happened  to  me with  TAPCIS  and with  YAM.

Presumably  it  will   happen  with  any  communication

program run under  the Library.   However, there is  no

problem  at  all   if  DropDTR  is  invoked   in  those

communications  programs when  they have  been directly

loaded from DOS -- the loading method most people use.

DISTRIBUTION:   Though copyrighted and  not in the public domain,

DropDTR may  be freely distributed  provided no money  is charged

for it and it is not modified.

DISCLAIMER:  Pete Petrakis accepts no responsibility or liability

for any damage or  loss caused by using DropDTR.   You accept any

risks in using it.

Peter L. Petrakis

Everett, WA 98208