*************************

*	DCOM.DOC        *

*************************

FB.DRUILHE Copyright (C) 1989, 1990

INTRODUCTION:

This program is designed to test High Speed Multirate

Asynchronous Modem. It requires to have into an IBM-PC

or compatible TWO communication boards COM1 and COM2.

OVERVIEW:

This program have two different modes:

SCREEN MODE: interractive communication with 2 different Modem

allowing FULL DUPLEX exchanges of DATA.

TEST MODE: powerfull AUTOMATIC TESTING of connexion and Character

error Rate.

A Hierarchical MENU (accessed by F10) is used to set or Modify

the differents parameters of the program.

SCREEN MODE:

While into the SCREEN MODE the screen is divided into TWO

windows where the received characters of both Modem are

displayed at the same time.

COM1 is displayed into the WHITE lower window and COM2

into the BLACK upper window.

A small third SYSTEM window is composed by the last line

of the screen where current FORMAT and Status are displayed.

By using the Right or Left Arrow, the user can select one

COMMUNICATION PORT and Modify the associated parameters:

Alt_P:	Show Parameters Menu:

Alt_S:	Change the SOURCE of data Transmited

to the PORT:

Keyboard: any normal characters comming

from the keyboard.

QBF1: "QUICK BROWN FOX" pattern.

QBF2: alternate QBF pattern.

File: Containt of a file.

AAAA .. xxxx: Single character.

Alt_D:	Change DESTINATION of received characters:

Screen: display on WINDOW.

QBF1_Error_count: Count Errors on Received

characters, assuming we are receiving QBF1

pattern.

none: received characters are flushed and lost.

Alt_C:	Edit COMMAND FORMAT.

Alt_U:	Edit USER FORMAT.

By using the F1 key the user can change from COMMAND format to USER

format or reverse.

By using the F2 key the user can call an external program (called

INTERPRETOR) with a parameter "/x" where x is 1 or 2 depending

of the CURRENT COMMUNICATION PORT selected.

At any time if a full page (24 lines by 80 rows) is needed the user

can press the Alt_Z (ZOOM MODE) to display the CURRENT COMMUNICATION

PORT received characters.

The HELP DISPLAY can be accessed by the HOME Key.

EDIT:

With any QUESTION (as File name, Predefined Key, Test Strings ..)

the allowed key for editting are:

BS:  (Back Space) delete left character and move left.

INS: (Insert) toggle INSERT MODE (default is REPLACE).

Left-Arrow: move left.

Right-Arrow: move Right.

Home: move to the biginning of the string.

End: move to the end of the string.

CR: (Enter) Valid the String Modification and Return.

ESC: (Escape) Return without Modification.

FORMAT:

For both Communication boards (COM) the user can define

two formats of Transmition:

COMMAND FORMAT: used to send COMMANDS to the Modem.

USER FORMAT: used to exchange DATA.

Both of the 4 differents FORMATS include:

COMMUNICATION SPEED: from 75 to 19200 bps

PARITY: parity of characters (N, E, O, M, S).

LENGHT: lenght of characters (5, 6, 7, 8).

STOP: number of stop bits (1, 2).

note: the communication speed can be ANY number (not only

multiple of 600) so Overspeed or Underspeed of the Asynchronous

to Synchronous converter can be tested (main clock is 1.152 MHz).

TEST MODE:

The test Mode is an Automatic Mode where the program

execute repetitive connexions and errors count.

SINGLE TEST:

The SETUP AUTO TEST MENU allows to define:

Title: Facultative 75 Characters String.

Test Mode: Full Duplex, Simplex, Analog Loop Back.

Number of Tests: number of same tests executed.

Size of Test: number of Characters sended (full

duplex tests are HALF the duration of

a simplex one).

Stop on Error: Allows to Break the Loop (and remain

in the same Communication state) when, at the

end of a test some errors (Characters) occurs.

FORMATS: Formats (same as those of SCREEN MODE).

AT STRINGS: Strings sended to the COM(s) to setup

the Modem(s).

note:	use Alt_126 for a 1sec Delay.

| for a BREAK.

^ for Control Prefix (eg "^M" is CR).

The test is Executed as Follow:

1)  set COMs to COMMAND FORMAT.

2)  send INITIAL AT STRING to COM(s).

3)  send CONNEXION AT STRING to COM(s).

4)  Wait CD, CTS of COM(s). With a TIMEOUT of 30sec.

5)  set COMs to USER FORMAT.

6)  Send and Count QBF1 Errors on COM(s).

7)  Display Results.

8)  set COMs to COMMAND FORMAT.

9)  Stop if STOP ON ERROR and ERROR, or Last Test.

10) send END AT STRING to COM(s).

11) Go To 3.

CHAIN TEST:

When the SETUP AUTO TEST are saved, using the SAVE SETUP command,

they can AUTOMATICALY be CHAINED and the Results saved into a

File by using the CHAIN TESTS command.

The following Menu is Displayed (Example):

�� CHAIN TESTS ������������������������������������������������������Ŀ

�   ************************** RUN TESTS *************************    �

�   External Line Program   : SETPTT 2                                �

�   Generic Line File Name  : LINE*.PTT                               �

�   Generic Setup File Name : DA?.TST                                 �

�   Output Results File Name: CX_ST.RES                               �

�   Size of Test Scaling    : 1.000000                                �

�   Number of Tests Scaling : 1.000000                                �

�   Resume Results          : NO                                      �

�����������������������������������������������������������������������

Where:

EXTERNAL LINE PROGRAM is the name of a DOS valid program

to be Executed as ofen as GENERIC LINE FILE NAME is

found.

Within this Loop as ofen as GENERIC SETUP FILE NAME is found,

the according test is run.

All of the Tests Results are saved into the OUTPUT RESULTS

FILE NAME.

All the tests can be Scaled (number of Characters sended) using

the SIZE OF TEST SCALING.

All the tests number of trails can be Scaled using the NUMBER OF

TEST SCALING.

To start the complete Test select the "**** RUN TESTS ***" line.

Use DOS wihldcart characters "*" or "?" to define the sets of files

to be executed.

note: 1) there is no garanty of the order of execution.

2) External line program and Generic setup file name are

optional.