FILE:   HST9600.DOC  FORMATED FOR OUTPUT:

JUST ENTER    COPY HST.DOC PRN   AT 'DOS' LEVEL

TO PRINT IT

AUTHOR:  PAT KELLY

ADDRESS COMMENTS TO:   PAT KELLY ON  EXEC-PC BBS 1-414-789-4300 [9600 8 N 1]

or    RG MORROW ON  CRS     BBS 1-416-629-0168 [9600 8 N 1]

US ROBOTICS COURIER HST MODEM

(9600/2400/1200/300 BPS MODEM)

as shown on the cover of my manual

PRELIMINARY CONSIDERATIONS

==========================

I AM ASSUMING YOUR MODEM MANUAL IS SIMILAR TO THE ABOVE.

MY COMMENTS ETC. WERE TESTED USING 'TELIX V3.12' AND 'TELEMATE V2.10'

ON A SOUPED UP IBM XT WITH A NEC V20 CHIP, A 8MHZ SECONDARY CLOCK, AND

A MATH CO-PROCESSOR. THEY WERE ALSO TESTED ON AN ALR 386/33 MACHINE.

THE RESULTS SHOULD BE SIMILAR ON OTHER COMPUTERS AND WITH OTHER

COMMUNICATION PROGRAMS.

THE REASON FOR THIS DOCUMENT IS THAT I HAVE DOWN-LOADED VARIOUS

'HST' SET-UP FILES, ETC. FROM ASSORTED BBS'S AND HAVE FOUND THAT

WHILE THEY ALL SAY THEY ARE DOING THE SAME THING, THEY SURE GO ABOUT

IT IN A VARIETY OF WAYS. IN ALL THE DOWN-LOADS I HAVE FOUND MANY

ITEMS WHICH WERE UNCLEAR AND SEEMED TO BE CONTRADICTORY FROM ONE

EXPLANATION TO ANOTHER. THUS, I HAVE MADE MY OWN AND TRIED TO GIVE

REASONS, EXPLANATIONS, AND REFERENCES FOR EACH OF MY ACTIONS. IN

SO DOING I HAVE GONE THROUGH THE HST MANUAL AND VARIOUS PUBLICATIONS

IN ORDER TO UNDERSTAND (?) AND BE ABLE TO EXPLAIN WHAT I HAVE DONE.

ANOTHER REASON FOR DOING IT THIS WAY IS TO ENABLE ME TO EXPLAIN IT

TO MY COMPUTER STUDENTS ( HA - THERE WAS ANOTHER MOTIVE AFTER ALL ).

COMMENTS, CORRECTIONS, CONTRIBUTIONS (MONEY OR OTHER) MAY BE SENT

TO ME VIA THE ABOVE BBS'S.

HARDWARE

========

THE HST HAS 10 DIP SWITCHES:

1 DATA TERMINAL READY OVERRIDE    6 CARRIER DETECT OVERRIDE

2 VERBAL/NUMERIC RESPONSE CODES   7 SINGLE/MULTI PHONE INSTALLATION

3 RESULT CODE DISPLAY             8 'AT' COMMAND SET RECOGNITION

4 COMMAND MODE LOCAL ECHO         9 NORMAL MODE ESCAPE CODE

5 AUTO ANSWER                    10 POWER-ON SOFTWARE DEFAULTS

IN THE FOLLOWING: U=UP open or off       D=DOWN closed or on

1   2   3   4   5   6   7   8   9  10

FACTORY SETTINGS    U       U           U              U   rest are D

MY CHOICES          U   U       U       U   U       U  U   rest are D

PAGE 2

COMMENTS:

1 - U  DTR NORMAL: RS232C PIN 20 TO COMPUTER CONTROLS MODEM

2 - U  ENABLE VERBAL (WORD) MESSAGES FOR MODEM CODES

3 - D  DISPLAY RESULT CODES ( BUSY, RINGING, ETC.)

4 - U  ECHO OFF-LINE COMMANDS (SO YOU CAN SEE WHAT YOU TYPE)

5 - D  SUPPRESS AUTO ANSWER (MODEM WILL not ANSWER PHONE)

( SET AT S0=1 FOR host MODE {EXPLAINED BELOW} )

6 - U  NORMAL CARRIER DETECT: RS232C PIN 8 INDICATES MODEM IS ON-LINE

7 - U  SINGLE PHONE USAGE

8 - D  ENABLE 'AT' COMMAND SET (TO PROGRAM THE MODEM'S nram MEMORY)

9 - U  DISCONNECT WITH +++ (MY LONG DISTANCE BILL SUFFERED WHEN I

DID not DO THIS)

10 - U  USE NRAM DEFAULTS ( AS OPPOSED TO FACTORY DEFAULTS)

WHEN YOU NOW TURN ON YOUR MODEM THE    TR   MR   RS   CS

(Terminal Ready-Modem Ready-Request to Send-Clear to Send)

LIGHTS SHOULD BE ON. YOU MAY ONLY HAVE THE 'MR' AND 'CS' LIGHTS

ON UNTIL YOU RUN YOUR COMMUNICATION PROGRAM.

SOFTWARE

========

THE DISCUSSION AT HAND WILL BE IN TERMS OF TWO (2) COMMUNICATION

PACKAGES ( TELIX V3.12  and  TELEMATE V2.10). IN BOTH CASES I ASSUME

THERE IS no CONFIGURATION FILE PRESENT.

FOR TELIX:

QUESTION                        RESPONSE

--------                        --------

MONITOR TYPE AND CARD           I USED COLOR MONITOR AND COLOR CARD

MAXIMUM BAUD                    19200

COM PORT                        1   (YOURS MAY VARY)

STATUS LINE                     YES (I LIKE TO 'SEE' WHAT IS HAPPENING)

TELIX NOW RESPONDS: ( explanations of actual items will come later)

ATZ                             RESET MODEM TO NRAM DEFAULTS

OK                              MODEM SAYS     OK

AT S7=45 S0=0 V1 X1             SET VARIOUS MODEM DEFAULTS

OK                              MODEM SAYS     OK

FOR TELEMATE:

RUN THE PROGRAM 'TMINST.EXE' TO CREATE THE CONFIGURATION FILE 'TM.CNF'.

WHEN THE MENU 'POPS UP' EXIT AND SAVE THE SET-UP (WE WILL ALTER IT

LATER). NOW RUN THE PROGRAM 'TM.EXE'.

TELEMATE NOW RESPONDS:

AT S0=0 Q0 E1 X4                SET VARIOUS MODEM DEFAULTS

OK                              MODEM SAYS     OK

PROGRAM ENTER PHONE DIRECTORY   PRESS THE 'ESC'KEY TO GET TO COMMAND

MODE AND YOU WILL SEE THE 'AT ...' LINE

(shown at left side of page.)

PAGE 3

PROGRAM USAGE TO SET MODEM RAM MEMORY (NRAM)

============================================

IT IS NOW ASSUMED THAT YOU ARE IN YOUR COMMUNICATION PACKAGE AT THE

COMMAND (TERMINAL) LEVEL.

IE.: THE 'AT...' AND 'OK' LINES ARE SHOWING ON THE SCREEN.

DEFINITION: 'AT' IS THE PREFIX FOR GIVING COMMANDS TO YOUR HST MODEM.

----------

THE FOLLOWING ARE SOME COMMANDS YOU CAN ENTER TO SEE HOW YOUR MODEM

IS SET-UP.

ACTION                                COMMENT

======                                =======

AT   (YOU TYPE 'AT')                  SEE IF COMPUTER & MODEM ARE COMMUNICATING

OK                                    MODEM SAYS   OK

ATZ  (YOU TYPE 'ATZ')                 LOAD NRAM DEFAULTS (EXPLAINED BELOW)

OK                                    MODEM SAYS   OK

ATI0 (YOU TYPE 'ATI0')                ASK FOR MODEM'S PRODUCT CODE

964                                   MAY BE 961 TO 964 (964 IS THE NEWEST)

OK

ATI1 (YOU TYPE 'ATI1')                ASK FOR ROM CHECKSUM

038     56      9419                  MODEM DISPLAYS CHECKSUM

OK

ATI2 (YOU TYPE 'ATI2')                ASK MODEM TO CHECK IT'S 'RAM'

OK

ATI3 (YOU TYPE 'ATI3')                SHOW MODEM'S CALL DURATION TIME

00:00:00                              ZERO AS WE HAVEN'T GONE ON-LINE YET

OK

PAGE 4

ACTION                                  COMMENT

======                                  =======

ATI4 (YOU TYPE 'ATI4')                     SHOW factory SETTINGS WITH SLIGHT

USRobotics Courier 9600 HST Settings....   VARIANCES AS WE HAVE CONFIGURED OUR

TERMINAL PROGRAM WHICH WILL AFFECT

C=1  E=1  F=1  M=1  Q=0  V=1  X=6  B=1  SOME OF THESE VALUES.

BAUD=19200  PARITY=N  WORDLEN=8         THESE VALUES & THEIR MEANINGS ARE:

DIAL=PULSE  ON HOOK    TIMER            C=1 TRANSMITTER ENABLED

E=1 COMMAND MODE ECHO ON

&A1  &B1  &G0  &H3  &I0  &K0            F=1 ON-LINE ECHO OFF

&M4  &N0  &P0  &R2  &S1  &Y1            M=1 SPEAKER ON UNTIL CARRIER DETECT

Q=0 SHOW RESULT CODES-BUSY,RING, ETC

S00=000  S01=000  S02=043  S03=013      V=1 SHOW CODES AS MESSAGES

S04=010  S05=008  S06=002  S07=060      X=6 ENABLE ALL RESULT CODE OPTIONS

S08=002  S09=006  S10=007  S11=070      B=1 USE U.S.A. ANSWER SEQUENCE

S12=050  S13=000  S14=001  S15=000

S16=000  S17=000  S18=000  S19=000    NOTE: V=1 MATCHES DIP SWITCH 2 UP

S20=000  S21=010  S22=017  S23=019          Q=0 MATCHES DIP SWITCH 3 DOWN

OK                                             E=1 MATCHES DIP SWITCH 4 UP

MORE NOTES: THESE PERTAIN TO THE ABOVE '&' OPTIONS (FACTORY DEFAULTS).

WE WILL CHANGE SOME OF THESE AND PUT THEM IN 'NRAM' MEMORY.

(THIS IS WHY WE SET DIP SWITCH 10 UP.)

CODE  MEANING                                 COMMENT

====  =======                                 =======

&A1 - /ARQ CODES WILL BE DISPLAYED            WE WILL KEEP THIS

&B1 - LOCK FIXED BAUD RATE (LOCK HIGH SPEED)  WE WILL KEEP THIS

&G0 - NO GUARD TONE (U.S/CANADA DEFAULT)      WE WILL KEEP THIS

&H3 - HARDWARE/SOFTWARE FLOW CONTROL          WE WILL CHANGE THIS

(TO H1 HARDWARE CONTROL ONLY)

&I0 - FLOW CONTROL DISABLED (DEFAULT)         WE WILL KEEP THIS (NO XON/XOFF)

&K0 - DATA COMPRESSION DISABLED               WE WILL KEEP THIS (AS MOST

FILES ON BBS'S ARE 'ZIPPED')

&M4 - NORMAL/ARQ MODE                         WE WILL KEEP THIS (MODEM WILL

TRY HST ARQ MODE IF POSSIBLE)

&N0 - GET HIGHEST POSSIBLE BAUD WITH REMOTE   WE WILL KEEP THIS

&P0 - NORTH AMERICAN DIAL PULSE RATE          WE WILL KEEP THIS

&R2 - MODEM SENDS DATA TO TERMINAL ONLY       WE WILL KEEP THIS

WHEN 'RequestToSend' IS HIGH

&S1 - MODEM WILL CONTROL 'DataSetReady'       WE WILL KEEP THIS

&Y1 - DESTRUCTIVE BREAK (ABORT TRANSFER)      WE WILL KEEP THIS

THE Sxx ARE REGISTERS AS OUTLINED IN THE HST USER MANUAL. ONES WHICH MIGHT

CHANGE ARE: S07=DIAL TIME IN SECONDS, S15= TURN MNP LEVEL 4 (DATA FRAME

OPTIMIZATION) ON=0 OR OFF=1, S19=INACTIVITY TIMER, S22=XON CHAR (ASCII 17),

S23=XOFF CHAR (ASCII 19).

WE WILL ALTER SOME OF THE ABOVE SHORTLY.

PAGE 5

ACTION                                  COMMENT

======                                  =======

ATI5 (YOU TYPE 'ATI5')                           SEE THE NRAM (Nonvolatile

USRobotics Courier 9600 HST NRAM Settings....    Random Access Memory) SET-UP

DIAL=PULSE  M=1  X=6  F=1  B=1

BAUD=19200  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H3  &I0  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S02=043  S03=013  S04=010  S05=008

S06=002  S07=060  S08=002  S09=006

S10=007  S11=070  S12=050  S13=000

S15=000  S19=000  S21=010  S22=017

S23=019

STORED PHONE #0:                              NOTE: YOU CAN ALSO STORE UP

#1:                                    TO 4 PHONE NUMBERS

#2:

#3:

OK

ATI6 (YOU TYPE 'ATI6')                           SEE A DIAGNOSTIC SUMMARY OF

A MODEM SESSION

Chars sent                     0        Chars received                 0

Chars lost                     0

Octets sent                    0        Octets received                0

Blocks sent                    0        Blocks received                0

Blocks resent                  0

Retrains Requested             0        Retrains Granted               0

Line Reversals                 0        Blers                          0

Link Timeouts                  0        Link Naks                      0

Data Compression        Off

Equalization            Long

Fallback                Disabled

No Connection

OK

ATI7 (YOU TYPE 'ATI7')                           SEE YOUR MODEM'S CONFIGURATION

PROFILE

USRobotics Courier 9600 HST Configuration Profile:

Product type            External

Custom chip             U100

Clock freq              14.976

Options                 Standard

Supervisor date         11/25/87

Supervisor rev          4

Transmitter rev         2

Receiver rev            2

OK

PAGE 6

ACTION                                  COMMENT

======                                  =======

ATI8 (YOU TYPE 'ATI8')                ASK MODEM WITH AN INVALID COMMAND

ERROR                                 MODEM ANSWERS 'ERROR'

OTHER COMMANDS YOU CAN TRY ARE:

AT$     DISPLAY THE BASIC 'AT' COMMAND SET

AT&$    DISPLAY THE EXTENDED 'AT' COMMAND SET

ATS$    DISPLAY THE S-REGISTER FUNCTIONS

ATD$    DISPLAY THE MODEM'S DIAL COMMAND SUMMARY

THE ABOVE WILL PRODUCE:

AT$

HELP,  Command Quick Reference (CTRL-S to Stop, CTRL-C to Cancel)

&$    HELP, Extended Commands        In    n=5  NRAM Settings

A     Answer Call                          n=6  Link Diagnostics

A/    Repeat Last Command            Kn    n=0  Call Duration Mode

Bn    n=0  CCITT Answer Sequence           n=1  Real Time Clock Mode

n=1  Bell Answer Sequence      Mn    n=0  Speaker Off

Cn    n=0  Transmitter Off                 n=1  Speaker On Until CD

n=1  Transmitter On                  n=2  Speaker Always On

Dn    Dial a Telephone Number              n=3  Speaker Off During Dial

n=0..9#*TPR,;"W@!()-           O     Return On-Line

DSn   Dial Stored Phone Number       Qn    n=0  Result Codes Sent

D$    HELP, Dial Commands                  n=1  Quiet (No Result Codes)

En    n=0  Do not echo Chars         Sr=n  Sets Register "r" to "n"

n=1  echo Command Chars        Sr?   Query Register "r"

Fn    n=0  Half duplex               S$    HELP, S Registers

n=1  Full duplex               Vn    n=0  Numeric Responses

Hn    n=0  On Hook (Hang Up)               n=1  Verbal Responses

n=1  Off Hook                  Xn    n=0  Basic Result Codes

In    n=0  Product Code                    n=1  Extended Result Codes

n=1  Checksum                        n=2-7  Advanced Result Codes

n=2  RAM Test                  Z     Software Reset

n=3  Call Duration/Clock       >     Repeat Command String

n=4  Current Settings          $     HELP,  Command Summary

OK

PAGE 7

AT&$

HELP, Extended Commands (CTRL-S to Stop, CTRL-C to Cancel)

&An   n=0  Disable /ARQ  Result Codes     &Mn   n=0  Normal Mode

n=1  Allow /ARQ  Result Codes             n=4  ARQ/Normal Mode

&Bn   n=0  Variable DTE Speed                   n=5  ARQ Mode

n=1  Fixed DTE Speed                &Nn   n=0  Highest Link Speed

&F    Load Factory Configuration                n=1  300 bps      n=3  2400 bps

&Hn   n=0  Disable TX data flow control         n=2  1200 bps     n=6  9600 bps

n=1  CTS                            &Pn   n=0  USA Pulse Dial

n=2  Xon/Xoff                             n=1  UK Pulse Dial

n=3  CTS and Xon/Xoff               &Rn   n=1  Ignore RTS

&In   n=0  Disable RX data flow control         n=2  RX Data to DTE / RTS high

n=1  Xon/Xoff                       &Sn   n=0  DSR always On

n=2  Xon/Xoff chars filtered              n=1  Modem controls DSR

n=3  HP Enq/Ack Host Mode           &W    Store Current Configuration

n=4  HP Enq/Ack Terminal Mode       &Yn   Break options

&Kn   n=0  Data Compression Disabled      &Zn=s Store Phone Number

n=1  Auto Enable/Disable            &Zn?  Query Phone Number

n=2  Enabled

OK

ATS$

HELP, S Register Functions (CTRL-S to Stop, CTRL-C to Cancel)

S0  Ring to Answer On                S11 Touch Tone Spacing (msec)

S1  Counts # of Rings                S12 Escape Code Time (1/50sec)

S2  Escape Code Char                 S13 Bit Mapped

S3  Carriage Return Char             S16 Test Modes

S4  Line Feed Char                   S17 LD Reason

S5  Backspace Char                   S19 Inactivity Timeout (min)

S6  Wait Time/Dial Tone (sec)        S20 Disconnect Reason

S7  Wait Time/Carrier (sec)          S21 Break Length

S8  Comma and Repeat Pause (sec)     S22 Xon Char

S9  Carrier Detect Time (1/10sec)    S23 Xoff Char

S10 Carrier Loss Time (1/10sec)

OK

ATD$

HELP, Dial Commands (CTRL-S to Stop, CTRL-C to Cancel)

0-9 Digits to Dial

*#  Auxiliary Digits

T   Tone Dialing

P   Pulse Dialing

R   Call an Originate Only Modem

,   Pause

;   Remain in Command Mode

"   Used to Dial Alpha Phone #'s

W   Wait for 2nd Dial Tone (X3-X7)

@   Wait for an Answer (X3-X7)

!   Flash Switch Hook

OK

PAGE 8

PROGRAMMING THE MODEM'S NRAM

============================

NOW EXIT YOUR TERMINAL PROGRAM AND RE-ENTER IT (I USED TELIX). WE ARE

NOW GOING TO PROGRAM THE MODEM'S NRAM. MAKE SURE YOU ARE AT TERMINAL

MODE.

ACTION                                        COMMENTS

======                                        ========

ATZ                                           SET MODEM TO NRAM DEFAULTS

OK

AT B1 C1 E1 F1 M1 Q0 V1 X6                    AT - INVOKE COMMAND LEVEL

OK					      B1 - SET U.S. ANSWER SEQUENCE

C1 - ENABLE TRAMSMITTER

E1 - COMMAND MODE ECHO ON

F1 - ONLINE ECHO OFF

M1 - SPEAKER ON THROUGH CD(DIAL)

Q0 - SHOW RESULT CODES

V1 - VERBAL RESULT CODES

X6 - ALL ADVANCED RESULT CODES

AT &A1 &M4 &K0 &H1 &R2 &I0 &B1 &N		AT - INVOKE COMMAND LEVEL

OK						     THE & INDICATES EXTENDED

COMMAND LEVEL COMMANDS

&A1- SHOW /ARQ RESULT CODES

&M4- SET NORMAL/ARQ MODE

&K0- DISABLE DATA COMPRESSION

&H1- HARDWARE (CTS) FLOW CTRL

&R2- ENABLE HARDWARE RTS CTRL

&I0- DISABLE RECEIVE DATA CTRL

&B1- FIX DTE BAUD RATE

&N - SET HIGHEST BAUD POSSIBLE

AT &W						WRITE IT ALL TO RNAM

OK

ATI5						CHECK IT OUT!

USRobotics Courier 9600 HST NRAM Settings....

DIAL=PULSE  M=1  X=6  F=1  B=1

BAUD=19200  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I0  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S02=043  S03=013  S04=010  S05=008

S06=002  S07=060  S08=002  S09=006

S10=007  S11=070  S12=050  S13=000

S15=000  S19=000  S21=010  S22=017

S23=019

STORED PHONE #0:

#1:

#2:

#3:

OK

PAGE 9

NOW I EXITED THE COMMUNICATION PROGRAM AND RE-ENTERED IT TO SEE HOW

THINGS WORKED OUT!

ACTION                                  COMMENT

======                                  =======

ATI4                                            SEE HST SETTINGS

USRobotics Courier 9600 HST Settings....        (THEY ARE CHANGED)

C=1  E=1  F=1  M=1  Q=0  V=1  X=1  B=1

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=PULSE  ON HOOK    TIMER

&A1  &B1  &G0  &H1  &I0  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S00=000  S01=000  S02=043  S03=013

S04=010  S05=008  S06=002  S07=045

S08=002  S09=006  S10=007  S11=070

S12=050  S13=000  S14=001  S15=000

S16=000  S17=000  S18=000  S19=000

S20=000  S21=010  S22=017  S23=019

OK

ATI5                                            CHECK OUT NRAM SETTINGS

USRobotics Courier 9600 HST NRAM Settings....

DIAL=PULSE  M=1  X=6  F=1  B=1

BAUD=19200  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I0  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S02=043  S03=013  S04=010  S05=008

S06=002  S07=060  S08=002  S09=006

S10=007  S11=070  S12=050  S13=000

S15=000  S19=000  S21=010  S22=017

S23=019

STORED PHONE #0:

#1:

#2:

#3:

OK

AT&F                                            LOAD ROM FACTORY SETTINGS

OK

PAGE 10

ACTION                                  COMMENT

======                                  =======

ATI4                                            SEE, THEY ARE THERE!

USRobotics Courier 9600 HST Settings....

C=1  E=1  F=1  M=1  Q=0  V=1  X=1  B=1

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=PULSE  ON HOOK    TIMER

&A1  &B0  &G0  &H0  &I0  &K1

&M4  &N0  &P0  &R1  &S0  &Y1

S00=000  S01=000  S02=043  S03=013

S04=010  S05=008  S06=002  S07=030

S08=002  S09=006  S10=007  S11=070

S12=050  S13=000  S14=001  S15=000

S16=000  S17=000  S18=000  S19=000

S20=000  S21=010  S22=017  S23=019

OK

ATI5                                            CHECK OUT THE NRAM SETTINGS

USRobotics Courier 9600 HST NRAM Settings....

DIAL=PULSE  M=1  X=6  F=1  B=1

BAUD=19200  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I0  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S02=043  S03=013  S04=010  S05=008

S06=002  S07=060  S08=002  S09=006

S10=007  S11=070  S12=050  S13=000

S15=000  S19=000  S21=010  S22=017

S23=019

STORED PHONE #0:

#1:

#2:

#3:

OK

ATZ                                             LOAD NRAM SETTINGS AS DEFAULT

OK

PAGE 11

ACTION                                  COMMENT

======                                  =======

ATI4                                            AGAIN, SEE, THEY ARE THERE!

USRobotics Courier 9600 HST Settings....

C=1  E=1  F=1  M=1  Q=0  V=1  X=6  B=1

BAUD=19200  PARITY=N  WORDLEN=8

DIAL=PULSE  ON HOOK    TIMER

&A1  &B1  &G0  &H1  &I0  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S00=000  S01=000  S02=043  S03=013

S04=010  S05=008  S06=002  S07=060

S08=002  S09=006  S10=007  S11=070

S12=050  S13=000  S14=001  S15=000

S16=000  S17=000  S18=000  S19=000

S20=000  S21=010  S22=017  S23=019

OK

ATI5

USRobotics Courier 9600 HST NRAM Settings....   VIEW THE NRAM SETTINGS

DIAL=PULSE  M=1  X=6  F=1  B=1

BAUD=19200  PARITY=N  WORDLEN=8

&A1  &B1  &G0  &H1  &I0  &K0

&M4  &N0  &P0  &R2  &S1  &Y1

S02=043  S03=013  S04=010  S05=008

S06=002  S07=060  S08=002  S09=006

S10=007  S11=070  S12=050  S13=000

S15=000  S19=000  S21=010  S22=017

S23=019

STORED PHONE #0:

#1:

#2:

#3:

OK

GREAT, SO FAR EVERYTHING IS PROGRESSING NICELY.

SETTING COMMUNICATION PROGRAM PARAMETERS

========================================

TELIX

-----

FIRST WE WILL DO IT USING TELIX V3.12:

ENTER TELIX: (and set the options as shown below)

N O T E:                     ( *'ed lines show where changes are.)

========

PAGE 12

Telix Copyright (C) 1986-89 Exis Inc. and Colin Sampaleanu. All rights reserved.

Version 3.12, released 12-1-89.

Press ALT-Z for help on special keys.

ATZ

OK

AT S7=60 S0=0 V1 X6

OK

PRESS ALT O FOR MENU

�͵ Configure Telix ������������͸

�                                �

� Change which default settings? �

�                                �

�      Screen and color          �

--->   �      Terminal options          �

�      General options           �

�      Modem and dialing         �

�      Filenames and paths       �

�      Ascii transfers           �

�      Protocol options          �

�      Kermit transfers          �

�      Comm port setup           �

�      Write setup to disk       �

�      Exit                      �

�                                �

��������������������������������;

�͵ Terminal options ���������������������������������������������������͸

�                                                                        �

*  � A - Baud 19200   Parity None    Data length 8    Stop bits 1           �

� B - COM1                                                               �

�                                                                        �

� C - Default terminal type ........... ANSI-BBS                         �

� D - Status line ..................... Bottom                           �

� E - Local echo ...................... Off                              �

� F - Add Line Feeds after CRs ........ Off                              �

� G - Strip high bit (incoming data) .. Off                              �

� H - Received Backspace destructive .. On                               �

� I - Backspace key sends ............. BS                               �

*  � J - XON/XOFF software flow control .. Off                              �

*  � K - CTS/RTS hardware flow control ... On                               �

*  � L - DSR/DTR hardware flow control ... Off                              �

� M - Compuserve Quick B transfers .... On                               �

� N - Zmodem auto-downloads ........... On                               �

� O - Answerback string (ENQ) .........                                  �

�                                                                        �

�                                                                        �

� Change which setting?       (Return or Esc to exit)                    �

������������������������������������������������������������������������;

PAGE 13

�͵ General settings ���������������������������������������������������͸

�                                                                        �

� A - Sound ...................... On                                    �

� B - Alarm sound ................ On                                    �

� C - Keep aborted downloads ..... On                                    �

� D - Show free disk space ....... On                                    �

� E - Use of Quick Dialing bar ... On                                    �

� F - Confirm hang-up ............ Off                                   �

� G - Usage Log on by default .... Off                                   �

� H - Scroll Back buffer size .... 10  (KBytes)                          �

*  � I - Capture file buffer size ... 4   (KBytes)                          �

� J - Date format ................ MMDDYY                                �

� K - Date separator character ... -                                     �

*  � L - Time format ................ 24-Hour                               �

� M - Time separator character ... :                                     �

� N - Use enhanced keyboard ...... On                                    �

� O - Shell: 16550 buffer on ..... On                                    �

�                                                                        �

�                                                                        �

� Change which setting?       (Return or Esc to exit)                    �

������������������������������������������������������������������������;

�͵ Modem and dialing parameter setup ��������������������������������������͸

�                                                                            �

*� A - Init string ......... ATZ^M~~~AT S7=60 S0=0 V1 X6^M~                   �

*� B - Dialing prefix 1 .... ATDP     (use ATDT for touch tone LINE)          �

*� C - Dialing prefix 2 .... ATDP                                             �

*� D - Dialing prefix 3 .... ATDP                                             �

� E - Dialing suffix ...... ^M                                               �

� F - Connect string ...... CONNECT                                          �

� G - No connect strings .. NO CARRIER             BUSY                      �

�                           VOICE                  NO DIAL TONE              �

� H - Hang-up string ...... ~~~+++~~~ATH0^M                                  �

� I - Auto answer string .. ATS0=1^M                                         �

� J - Dial cancel string .. ^M                                               �

�                                                                            �

*� K - Dial time ........... 60                                               �

� L - Redial pause ........ 1                                                �

*� M - Auto baud detect .... Off                                              �

� N - Drop DTR to hangup .. On                                               �

�                                                                            �

�                                                                            �

� Change which setting?       (Return or Esc to exit)                        �

����������������������������������������������������������������������������;

PAGE 14

�͵ Filenames and paths ������������������������������������������������͸

�                                                                        �

� A - Download directory:                                                �

�                                                                        �

� B - Upload directory:                                                  �

�                                                                        �

� C - Script directory:                                                  �

�                                                                        �

� D - Default Capture file:                                              �

*  �      EXEC.DOC                                                          �

� E - Default Usage Log name:                                            �

�      TELIX.USE                                                         �

� F - Directory program:                                                 �

�                                                                        �

� G - Screen Image file:                                                 �

�      TELIX.IMG                                                         �

� H - Editor name:                                                       �

�                                                                        �

�                                                                        �

� Change which setting?       (Return or Esc to exit)                    �

������������������������������������������������������������������������;

�͵ ASCII file transfer settings ��������������������͸

�                                                     �

�  A - Strip high bit ...... Off                      �

�  B - Remote abort char ... 0    (ASCII)             �

�                                                     �

� Uploads                                             �

�  C - Local echo .......... Off                      �

�  D - Expand blank lines .. On                       �

�  E - Pace character ...... 0    (ASCII)             �

�  F - Line pacing ......... 10   (1/10 seconds)      �

�  G - Character pacing .... 0                        �

�  H - CR translation ...... None                     �

�  I - LF translation ...... None                     �

�                                                     �

� Downloads                                           �

�  J - CR translation ...... None                     �

�  K - LF translation ...... None                     �

�                                                     �

�                                                     �

� Change which setting?       (Return or Esc to exit) �

�����������������������������������������������������;

PAGE 15

�͵ File transfer protocols ������������������������������������������������͸

�                                                                            �

� External           Key   Protocol     Upload    Download   BAT or    DL    �

� Protocols                  Name      Filename   Filename   Script   Name   �

�                                                                            �

� A.................                                                         �

� B.................                                                         �

� C.................                                                         �

� D.................                                                         �

�                                                                            �

*� E: File xfer disk buf. size (Kb) ... 4                                     �

� F: Relaxed Xmodem timing ........... Off                                   �

� G: Filename guessing ............... Only if sure                          �

� H: Zmodem receive crash recovery ... Off                                   �

� I: Zmodem send crash recovery ...... Off                                   �

*� J: Allow Zmodem 32 bit CRCs ........ On                                    �

� K: Zmodem window size (Kb) ......... 0                                     �

� L: Zmodem file type (translation) .. Either                                �

�                                                                            �

�                                                                            �

� Change which setting?       (Return or Esc to exit)                        �

����������������������������������������������������������������������������;

�͵ Kermit Settings �����������������������������������������͸

�                                                             �

� A - Control quote character .. 35   (Ascii)                 �

� B - Padding character ........ 0    (Ascii)                 �

� C - Number of pad chars ...... 0                            �

� D - Maximum packet size ...... 94                           �

� E - Start of packet char ..... 1    (Ascii)                 �

� F - End-of-line-character .... 13   (Ascii)                 �

� G - File type ................ Binary                       �

�                                                             �

� Change which setting?       (Return or Esc to exit)         �

�������������������������������������������������������������;

�͵ Comm port setup ���������������������������������͸

�                                                     �

�              Start     IRQ                          �

�             Address    Line                         �

�                                                     �

� A: COM1 ..... 3f8       4                           �

�                                                     �

� B: COM2 ..... 2f8       3                           �

�                                                     �

� C: COM3 ..... 3e8       4                           �

�                                                     �

� D: COM4 ..... 2e8       3                           �

�                                                     �

� E: COM5 ..... 3f8       4                           �

�                                                     �

� F: COM6 ..... 3f8       4                           �

�                                                     �

� G: COM7 ..... 3f8       4                           �

�                                                     �

� H: COM8 ..... 3f8       4                           �

�                                                     �

� Change which setting?       (Return or Esc to exit) �

�����������������������������������������������������;

PAGE 16

NOW SAVE ALL CHANGES. EXIT TELIX AND RE-ENTER IT TO ACTIVATE THE

CHANGES. NOW GO TO DIALING DIRECTORY AND ENTER A LINE. FOR EXAMPLE,

FOR EXEC-PC  ENTER     1-414-789-4300     9600 8 N 1

FOR CRS      ENTER     1-416-629-0168     9600 8 N 1

NOTE THAT THE MODEM DEFAULT IS 19200, BUT WE USE 9600 ABOVE. I HAVE

FOUND THAT IF ONES USES 19200 ABOVE, CRC ERRORS CREEP IN AND THE

BAUD RATE DROPS TO A VERY LOW VALUE. USING 9600 ON THE OTHER HAND

WORKS VERY WELL AND BECAUSE 19200 IS A HARDWARE DEFAULT, THE ACTUAL

DATA TRANSFER OF A 'ZIP' FILE USUALLY EXCEEDS 9600 BAUD VERY NICELY.

TELEMATE

--------

NOW WE WILL DO IT USING TELEMATE V2.10: (TM /O)

ENTER TELEMATE: (and set the options as shown below)

( *'ed lines show where changes are.)

Telemate Version 2.10                  Registered to: [ REGISTERED ]

Copyright (c) 1988-1990 Tsung Hu.  All rights reserved.

Press F1 for help on function keys.

ATZ

OK

ENTER  ALT O FOR OPTIONS MENU

�����������������͸

�  General        �

�  Dial setup     �

�  Directory      �

�  Terminal       �

�  Communication  �

�  Protocol       �

� ��������������� �

�  Load options   �

�  Save windows   �

�  Save options   �

�����������������;

PAGE 17

General

���������������������������������������������������������Ŀ

�  �� DOS Shell ��������������������������������������Ŀ  �

�  �  [ ] Swap Telemate to XMS/EMS                     �  �

�  �  [X] Swap Telemate to disk                        �  �

�  �����������������������������������������������������  �

�  �� Mouse ������������������������������������������Ŀ  �

*          �  �  Type  ( ) None   (x) MicroSoft  ( ) MouseSystem  �  �

*          �  �  Port  (x) None   ( ) COM1       ( ) COM2         �  �

�  �  Speed ( ) Slow   ( ) Medium     (x) Fast         �  �

�  �����������������������������������������������������  �

�  �� Display �������������Ŀ   �� Log ���������������Ŀ  �

�  �  [X] Scroll Bars       �   �  [X] Log Filter      �  �

�  �  [X] Menu Bar          �   �  [X] Usage Log       �  �

*          �  �  [X] Status Line       �   ������������������������  �

*          �  �  [X] 24 hour format    �   �� Sound �������������Ŀ  �

�  �  Scroll Back      100  �   �  [X] Music           �  �

�  �  Editor Margin     70  �   �  [X] Bell            �  �

�  �  Message Pause      1  �   �  Alarm Sound      1  �  �

�  �  Error Pause        2  �   �  Alarm Time       3  �  �

�  ��������������������������   ������������������������  �

�              ����ͻ              ��������Ŀ             �

�              � OK �              � Cancel �             �

�              ����ͼ              ����������             �

�����������������������������������������������������������

Dial setup

���������������������������������������������������������������������Ŀ

�                                                                     �

*    �  Prefix   (x) 1 "ATDP "       (ATDP=Pulse  ATDT=Touch Phones)       �

�           ( ) 2 "ATDT "                                             �

�           ( ) 3 ATDT 001,                                           �

�           ( ) 4 ATDT 9,                                             �

�                                                                     �

�  Suffix   (x) 1 ^M                                                  �

�           ( ) 2 ^M                                                  �

�           ( ) 3 ^M                                                  �

�           ( ) 4 ^M                                                  �

�                                                                     �

�  Connect String     CONNECT                [ ] Auto Baud Detect     �

�                                                                     �

*    �                                            Dial Time           60   �

�                                            Redial Pause         2   �

�  Busy String        NO CARRIER             Redial Attempt       0   �

�                     BUSY                                            �

�                     NO CONNECT             ����ͻ    ��������Ŀ     �

�                     NO DIALTONE            � OK �    � Cancel �     �

�  Cancel String      ^M                     ����ͼ    ����������     �

�                                                                     �

�����������������������������������������������������������������������

PAGE 18

Directory

����������������������������������������������������������Ŀ

�                                                          �

�  Telemate    C:\COMMUN\TM\                               �

�                                                          �

�  Upload                                                  �

�                                                          �

�  Download                                                �

�                                                          �

�  Script      C:\COMMUN\TM\                               �

�                                                          �

�  Edit                                                    �

�                                                          �

�  Log         C:\COMMUN\TM\                               �

�                                                          �

�  V. Memory   H:   (Ram disk)                             �

�                                                          �

�             ����ͻ                  ��������Ŀ           �

�             � OK �                  � Cancel �           �

�             ����ͼ                  ����������           �

������������������������������������������������������������

Terminal

��������������������������������������������������������������������Ŀ

�                                                                    �

�  Terminal     ( ) TTY  (x) ANSI  ( ) VT52  ( ) VT102               �

�                                                                    �

�  Connection   (x) Modem          ( ) Computer                      �

�                                                                    �

�  �� Toggles �����������������Ŀ   �� Paste/ASCII Upload �������Ŀ  �

�  �  [ ] Add Line Feed         �   �  [X] Expand Blank Line      �  �

�  �  [X] Add Return            �   �  Line Pacing             0  �  �

�  �  [ ] Local Echo            �   �  Char Pacing             0  �  �

�  �  [X] Auto Wrap             �   �  Pace Character          0  �  �

�  �  [X] Destructive Backspace �   �������������������������������  �

�  �  [ ] Strip High Bit        �   �� Macro/Script �������������Ŀ  �

�  �  [ ] Confirm Hang Up       �   �  Extended Pacing         0  �  �

�  ������������������������������   �������������������������������  �

�  �� Control �����������������Ŀ   �� Quote ��������������������Ŀ  �

*    �  �  [ ] XON/XOFF Flow         �   �  [X] Initial(@) Guessing    �  �

*    �  �  [X] RTS/CTS Flow          �   �  Prefix         @>          �  �

�  ������������������������������   �������������������������������  �

�                                                                    �

�                  ����ͻ                ��������Ŀ                  �

�                  � OK �                � Cancel �                  �

�                  ����ͼ                ����������                  �

����������������������������������������������������������������������

PAGE 19

Communication

��������������������������������������������������������������Ŀ

�                                                              �

*       �  Modem Init String     ATZ~~~AT S0=0 E1 M1 &K0^M~            �

*       �  Modem HangUp String   ^#~~~+++~~~ATH0^M~                    �

*       �  Auto Answer String    ~~~+++~~~AT S0=1^M~                   �

�  Answer Back String                                          �

�                                                              �

�                                                              �

*       �  Port          (x) COM1  ( ) COM3  ( ) COM5  ( ) COM7        �

�                ( ) COM2  ( ) COM4  ( ) COM6  ( ) COM8        �

�                                                              �

�  Baud Rate     ( ) 300        ( ) 4800       ( ) 38400       �

*       �                ( ) 1200       ( ) 9600       ( ) 57600       �

�                ( ) 2400       (x) 19200      ( ) 115200      �

�                                                              �

�  Parity        (x) None       ( ) Even       ( ) Odd         �

�  Data Bits     ( ) 7          (x) 8                          �

�  Stop Bits     (x) 1          ( ) 2                          �

�                                                              �

�                 ����ͻ              ��������Ŀ               �

�                 � OK �              � Cancel �               �

�                 ����ͼ              ����������               �

����������������������������������������������������������������

Protocol

������������������������������������������������������������������������Ŀ

�                              �� ASCII �����������������������������Ŀ  �

*  �   [X] Filename Guessing      � Upload  Download                     �  �

*  �   [X] Check Disk Space       �   [ ]     [ ]    Strip High Bit      �  �

�                              �   [ ]     [ ]    Translation         �  �

�  �� Zmodem �������������Ŀ   �   [ ]     [ ]    Strip CR            �  �

�  �  [X] Recovery         �   �   [ ]     [ ]    Strip LF            �  �

*  �  �  [X] Auto-Download    �   �   [ ]     [ ]    Add CR before LF    �  �

�  �  [ ] ASCII-Upload     �   �   [ ]     [ ]    Add LF after  CR    �  �

�  �������������������������   ����������������������������������������  �

�                                                                        �

�  �� External Protocols ��������������������������������������������Ŀ  �

�  �  Protocol       Upload      Download   Prompt   Auto-Download    �  �

�  �  Name      Key  Batch       Batch      DL Name  Sequence         �  �

�  �  BiModem    I   BI-MODEM    BI-MODEM     [ ]                     �  �

�  �  Jmodem     J   JMODEM-U    JMODEM-D     [X]                     �  �

�  �  Kermit     K   KERMIT-U    KERMIT-D     [ ]                     �  �

�  �  Puma       P   PUMA-U      PUMA-D       [ ]    ^V^H^V^H^VPuma^  �  �

�  ��������������������������������������������������������������������  �

�                                                                        �

�                     ����ͻ                 ��������Ŀ                  �

�                     � OK �                 � Cancel �                  �

�                     ����ͼ                 ����������                  �

��������������������������������������������������������������������������

BE SURE TO SAVE THE NEW SET-UP.

PAGE 20

MODEM NRAM TELEPHONE NUMBERS

============================

LASTLY, AT THE TERMINAL LEVEL ONE CAN ENTER UP TO 4 PHONE NUMBERS.

FOR EXAMPLE:

AT&Z0=P1 414 789 4300  FOR EXEC-PC   and

AT&Z1=P1 416 629 0168  FOR CRS

NOW WHEN ONE USES THE MODEM ONE CAN DIAL USING THE NRAM OPTIONS

AS FOLLOWS:

ATDS1                                   (MAKE SURE 9600 BAUD IS USED)

Dialing...P14166290168

RINGING

CONNECT 9600/ARQ

CONNECT 19200 / 06-27-90 (20:31)

(Error Correcting Modem Detected)

CRS Premium Bulletin Board - Node 38 - USR HST 14400

PCBoard (R) v14.5/U (beta 06/26/90 13:44) - Node 38

NOW ENTER  CTRL H TO HANG UP AND WE GET:

NO CARRIER

+++ATH0

OK

ATDS0                                   (MAKE SURE 9600 BAUD IS USED)

Dialing...P14147894300

RINGING

CONNECT 9600/ARQ

(NODE# 206)

Exec-PC(tm) Network BBS v3.73  Copyright (c) 1980-1990 Exec-PC

EXEC-PC (R) Reg. U.S. Pat. and Tm. Off.

The Business Knowledge Exchange

WELL THERE IT IS!  MY MODEM NOW WORKS WITH NO PROBLEM ON ALL THE

BBS'S I HAVE TRIED.

FILES INCLUDED:

TELIXOLD.CNF    TELIX CONFIG FILE AT START

TMOLD.CFG       TELEMATE CONFIG FILE AT START

TM.CFG          TELEMATE CONFIG FILE AFTER FULL SET UP

TELIX.CNF       TELIX CONFIG FILE AFTER FULL SET UP

HST9600.DOC     THIS MAMMOTH TOME!

COMMENTS, CORRECTIONS, ETC. ARE WELCOME.

P.A. KELLY

30 SOUTHAMPTON DR. S.W.

CALGARY, ALBERTA, CANADA, T2W 0T5

- END OF FILE -