PEAK PERFORMANCE for USR HST Modems

Version 2.0

Copyright(c), 1991 Robert M. Walker

PEAKDS is a revision to PEAK14K, a stand-a-lone

executable for setting up US

Robotics Courier HST and Dual Standard modems.

PEAKDS is designed to set up the HST for calling BBS's.

This means PEAKDS does

not configure the modem to run PC Board.  I have chosen

what I have found to be

the optimal settings for fast, efficient downloads of

compressed files (.ZIP, .ARC) and

also mail packets.

The program will ask you if you have a 14.4K HST.  If you

answered yes, then your

modem will be set up to run as efficiently as possible for an

HST 14.4k.

If you answered no, then we know that it is a Dual Standard

you wish to set up.  You

are then asked if you want to run it in V.32bis mode.  This

will enable you to connect

with other V.32bis modems including HAYES and ATI at

9600 baud.  It will also let you

connect with an HST DS in V.32/V.42/HST mode.

If you answer no, then your modem will be set up in

HST/V.42 mode which is the

mode I suggest.  My experience has been that the HST in

V.32bis mode will usually

make a 9600 negotiation before it makes a 14,400 connect.

If you are calling Com-

puserve and the 9600 baud lines are installed, then you

want the V.32bis con-

figuration.

After PEAK has written the new settings to your modem,

you are asked if you want to

write the new setting to the NRAM of the modem.  If you

answer yes, these now

become the new defaults each time you turn the modem

on.  This is why the modem

init string has been changed to the reset command, ATZ.

If you want to make any adjustments to the modem set-

tings, you can do it through

the dialling string prefix.  In Telix for example, prefix 1 may

just be ATDT to do a tone

dial.  If you wanted to turn on data compression for calling

Compuserve, then you

would assign your dialling entry for Compuserve the prefix

2.  Prefix 2 could be

AT&K2DT to turn on compression.  PEAK turns the speaker

off until the number is

dialled.  If you do not like this setting, you could turn it on

in the dialling string by

entering ATM2DT.

If for any reason you do not like the settings, I have two

suggestions.  Before you run

the program, go into your communication program and

issue the command ATI5.

This will give you a display of the current settings of your

modem in the NRAM.  Do a

PrintScreen of this for your reference.  If you want to set

your modem back to the

factory defaults, issue the command AT&F.  If you want to

write the factory settings

back intro the NRAM, issue the command AT&W.

Due to the overwhelming response I received from

PEAK14K, I decided not to put my

phone number in this time for all the Texans and Califor-

nians who forgot about the

time zone changes in the middle of the night.  I have

however included my FAX

number if you have any comments.

If this program saves you a lot of headaches, please show

me your gratitude by

popping a $20 bill in the mail.  In return I'll mail you a disk

with the source code and

answer your inquiries promptly.  I can also be reached on

NANET, SMARTNET, and

INTERLINK - using hiding in the IBM/TECHNICAL conferen-

ces. If you are a Com-

puserve member, my address is 70143,2370.

Thanks and please enjoy!

Robert Walker

698 Lansdowne Avenue

Toronto, Ontario M6H 3Y8

FAX (416) 533-9074